3GPP TSG-RAN WG2 Meeting #127	R2-2406952

Maastricht, NL, Aug 19th – 23rd, 2024

Agenda Item:	8.8.2

Source: 	CSCN

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

RAN2 has discussed potential RAN2 impact on downlink coverage enhancements in NR NTN, the following agreements were achieved in the previous meetings [1][2]. In RAN2#126, RAN2 had identified some questions to RAN1 for aspects where the input is required and an LS [3] was sent.

RAN2#126 Agreements:

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input.

RAN2#125bis Agreements：

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input.

In this contribution, we discuss system-level downlink coverage enhancements in NR NTN.

Discussion

2.1 Analyses of RAN2 impact due to SSB periodicity extension

According to the latest WID [4], SSB periodicity extension can be discussed for downlink coverage enhancement, but the final decision is up to RAN1 discussion. Based on our analyses in [5], given a limited number of simultaneously active beams compared with the total number of beam footprints, SSB periodicity extension is necessary since SSB transmission may not be completed in every beam footprint with the maximum period (i.e., 160ms) if beam hopping mechanism is adopted. Furthermore, whether to add SSB pattern (e.g. SSB position in burst, SSB index number, etc.) is up to RAN1, and RAN2 needs to wait for reply from RAN1. In this contribution, we only discuss the impact of SSB periodicity extension on RAN2.

For UEs in RRC_IDLE or RRC_INACTIVED, the longer SSB periodicity implies the longer common signalling periodicity (e.g. SIB1, paging, MSG1, etc). Consequently, the latency of cell search/research processes, RA procedures and paging would increase. Therefore, it is necessary for RAN2 to check whether the relevant timer durations are sufficient. For UEs in RRC_CONNECTED, since the UE shall setup the SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (which is affected by extended SSB periodicity), measurement configurations may also be affected. The other impacts of SSB periodicity extension can be further discussed in RAN2.

Proposal 1: RAN2 should consider the impact on SSB periodicity extension, e.g. cell search/research, RA process, paging and measurement.

2.2 Whether cell/beam level DTX is considered

Considering that satellite service area may consist of thousands of satellite beams, only a portion of these beams can be activated at any given time due to limited power and the nominal EIRP density per satellite beam. Furthermore, RAN1 has discussed various scenarios for satellite parameters and agreed that the total number of simultaneously active beams is 10.02% for FR1 or 1.5% for FR2 or 1.5% for FR1 in different cases. In order to enhance downlink coverage, beam hopping is an efficient technology that allows dynamic and flexible power sharing between satellite beams. Therefore, it is possible that some satellite beams within a cell are active while others remain inactive at the same time based on UE traffic requirements and network implementation. If cell level DTX is adopted, all satellite beams within a cell would be activated or deactivated simultaneously. Therefore, beam level DTX is more suitable.

Proposal 2: RAN2 assumes beam level DTX is preferred for downlink coverage enhancements.

2.3 Whether cell/beam level DRX is considered

Considering that R18 NES contains DTX and DRX, the next topic for discussion should be the necessity of cell/beam level DRX. To initiate this discussion, it is important to determine whether UL beam hopping is required. RAN2 needs to wait for reply from RAN1.

Proposal 3: RAN2 needs to wait for reply from RAN1 to determine whether cell/beam level DRX is needed.

Conclusions

Based on above analysis, proposals are summarized as follow.

Proposal 1: RAN2 should consider the impact on SSB periodicity extension, e.g. cell search/research, RA process, paging and measurement.

Proposal 2: RAN2 assumes beam level DTX is preferred for downlink coverage enhancements.

Proposal 3: RAN2 needs to wait for reply from RAN1 to determine whether cell/beam level DRX is needed.

Reference

[1] R2-2403733, Report from Break - out session on NR - NTN and IoT - NTN

[2] R2-2405703, Report from Break - out session on NR - NTN and IoT - NTN

[3] R2-2405767, LS on DL coverage enhancements

[4] RP -241667, Revised WID : Non - Terrestrial Networks ( NTN ) for NR Phase 3

[5] R1-2405257, Discussion on downlink coverage enhancement for NR - NTN