3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500086

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.18

Source:	vivo

Title:	On LTE TN to NB-IoT NTN Mobility Handling

Document for:	Discussion and Decision

- Introduction

In the previous meeting, RAN2 discussed the LTE TN to NB-IoT NTN mobility. The discussion progress and the achieved agreements are summarized below [1],

R2-2410793 Inclusion of the NB-IoT satellite information in E-UTRAN Google, Samsung, MediaTek Inc. discussion Rel-19 - Revised in R2-2411085 R2-2411085 Inclusion of the NB-IoT satellite information in E-UTRAN Google, Samsung, MediaTek Inc., Inmarsat, Viasat, Thales, Nordic discussion Rel-19 Proposal 1 Enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27 (TP attached). - QC thinks there should be no RRM impacts. Google agrees - CMCC thinks we can also avoid impacts to 36.304, i.e. to cell reselection - We can work on the provision of the NB-IoT satellite information in E-UTRAN as part of TEI19 work. RAN2 assumes that this will not impact other WGs (e.g. no RRM impacts) and will only require changes to 36.331, 36.306 (to indicate this is an optional feature without capability) and Stage 2 - We enable the inclusion of the satellite assistance information for NB-IoT NTN in SIB33, and create the association between the satellite assistance information in SIB33 and the NB-IoT frequency information in SIB27 - TP for SIB27 in R2-2411085 is considered as a baseline - Continue the discussion in the next meeting, e.g. on the impacts to SIB33

In this contribution, we would like to provide our understanding of the LTE TN to NB-IoT NTN mobility handling.

- Discussion

The first thing that comes to us is how to provide the NB-IoT satellite assistance information in SIB33. Based on the achieved outcome of LTE TN to NR NTN mobility, it seems a spontaneous logic to define new IE for NB-IoT satellite assistance information and reuse the existing neighbour satellite information list to include detailed satellite assistance information in SIB33. This is because using a new IE is simple and clean, and also helps to include more satellite information, considering the maximum number of satellites within the one NeighSatelliteInfoList cannot exceed 4. The corresponding ASN.1 change for SIB33 is given below as an example.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

... ,

[[

neighSatelliteInfoListNB-r19	NeighSatelliteInfoListNB-r19			OPTIONAL	-- Need OR

]]

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfoListNB-r19 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

Furthermore, in Rel-19, both LTE TN to NB-IoT NTN mobility and LTE TN to NR NTN mobility are supported. We are wondering whether a given satellite may simultaneously support both NB-IoT NTN and NR NTN (If so,signalling optimization may be considered for ephemeris overhead reduction). Frankly, considering the significant difference between NB-IoT NTN and NR NTN in terms of coverage performance, we think a given satellite would not provide both NB-IoT NTN and NR NTN at the same time. The same logic might also be suitable for NB-IoT NTN and eMTC. Thus, we think ephemeris sharing between NB-IoT NTN and eMTC NTN/NR NTN is not needed. Besides, even introducing a new IE for NB-IoT satellite assistance information, the enhanced SIB33 is supposed not to exceed the SIB size. Last, we think the detailed NB-IoT satellite information can be provided via NeighSatelliteInfo-r18.

Proposal 1: Define a new IE for NB-IoT satellite assistance information and a new neighbour satellite information list to provide the NB-IoT satellite information in SIB33.

Proposal 2: maxSat-r17 (i.e. 4) is reused for the maximum number of NB-IoT satellites.

Proposal 3: NeighSatelliteInfo-r18 is reused for including NB-IoT satellite information.

3 Conclusion

In this contribution, we have provided our understanding of handling the LTE TN to NB-IoT NTN mobility. The following proposals are made,

Proposal 1: Define a new IE for NB-IoT satellite assistance information and a new neighbour satellite information list to provide the NB-IoT satellite information in SIB33.

Proposal 2: maxSat-r17 (i.e. 4) is reused for the maximum number of NB-IoT satellites.

Proposal 3: NeighSatelliteInfo-r18 is reused for including NB-IoT satellite information.

4 Reference

[1] R2-2500002, RAN2#128 Meeting Report.