3GPP TSG-RAN WG2 Meeting #129	R2-2500098

Athens, Greece, Feb. 17th - 21st, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#128 meeting, RAN2 achieved the following agreements about the impacts on random access operation by the evolution of duplex operation:

- The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). - FFS on the following options Option 1 Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. - Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

In this contribution, we will further analyse the RAN2 impacts to support random access in SBFD symbols.

# 2	Discussion

## 2.1 RACH events

In the RAN2#127 meeting, there was one working assumption on the appliable RACH events for SBFD resource. Considering that random access in SBFD symbols aims to reduce the random access latency, to reduce the PRACH collision probability and to improve the coverage of PRACH and Msg3 transmission, we do not see strong reason to drop any of the RACH events and all of them should benefit from the advantages of RACH operation in SBFD resource.

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

## 2.2 RACH Configuration

In the last RAN2 meeting, there was agreement about RACH configuration:

- Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Based on this agreement, it is understood that enabling both options at the same time from network side is not to be supported. Considering that, though UE is not required to support both options, it is possible that one UE could support both option 1 and option 2 based on the related UE capability.

In the RAN1#119 meeting, there was agreement about RACH configuration Option1:

Agreement Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

According to this, the network will explicitly indicate the Option 1 is enabled or not for SBFD RACH operation, with a new field e.g., expressed by "ENUMERATED {true} indicating Option 1 enabled" . The absence of this field indicates Option 1 is disabled. The UE behaviour in different scenarios can be summarized as following:

Table 1. UE behaviour in different scenarios regarding RACH resource configuration options

|  | gNB indicates Option 1 enabled | gNB doesn’t indicate Option 1 enabled and provides RACH configuration by Option 2 |
| --- | --- | --- |
| UE supports Option 1 only | Follow Option 1 config | No SBFD RACH config |
| UE supports Option 2 only | No SBFD RACH config | Follow Option 2 config |
| UE supports both Option1, 2 | Follow Option 1 config | Follow Option 2 config |

We understand for SBFD-aware UEs supporting Option 1 configuration only, even if the UE receives RACH configuration with Option 2, it will ignore the SBFD RACH resource configured by Option 2; For SBFD-aware UEs supporting Option 2 configuration only, if the UE receives RACH configuration with Option 1, it will ignore the SBFD RACH resource configured by Option 1 and consider no SBFD RACH resource is available. For the UE supporting both configurations, it will follow the SBFD RACH resource configuration used by gNB: if the network indicates Option 1 enabled, the UE supporting both Options will follow Option1 configuration to perform RACH procedure. Eventually if the network doesn’t indicate Option 1 is enabled while provides SBFD RACH configuration by Option 2, the UE supporting Option 2 or both Options will follow Option 2 configuration to perform RACH procedure.

Proposal 2: RAN2 confirms the understanding that Table 1 is the UE behaviour depending on UE capability regarding supporting Option 1 and Option 2 and the gNB indication of whether Option 1 is enabled or not.

## 2.3 RACH resource selection

In the last meeting, RAN2 discussed the RACH resource selection issue, and achieved the following agreements:

- FFS on the following options Option 1 Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

There are shortcomings for both options: if the selection of RO type is dependent on network indication with a cell-specific signalling used by all the UEs in the cell, it is apparently not suitable for all the UEs which can be in different locations within the cell. For example, using SBFD recourse can be beneficial for latency reduction for UEs at cell-center, it can cause higher CLI by cell-edge UEs using SBFD resource.

On the other hand, the concern on the UE specific, RSRP-like threshold based selection of RO type is that it is not possible for UE to benefit from latency reduction and coverage enhancement at the same time. If cell border UEs with lower RSRP measurement than a RSRP threshold, they are filtered to use conventional RO instead SBFD resource, hence cannot achieve coverage enhancement by using long PRACH sequences on the SBFD symbols.

As can be seen from above, decision between the cell-border UEs to use SBFD resource and not to use SBFD resource needs to be done and it can be done by the gNB, as the gNB would have better knowledge than the UEs about the related information such as cell size, interference between cells etc.

So instead of signalling the direct priority on using additional ROs or on using legacy ROs, the gNB can signal the priority on "latency reduction" or on "coverage enhancement". For cell border UEs, they can select legacy ROs based on the RSRP comparison, if the gNB signals priority of "latency reduction"; and select additional ROs based on the RSFP comparison, if the gNB signals priority of "coverage enhancement". For cell-centre UEs, as there is no issue on coverage enhancement, they can always select the additional ROs for their first RACH attempt.

Proposal 3: Support the network indicates for which purpose SBFD RACH resource is to be used, i.e. indicates "prioritizing latency reduction" or indicates "prioritizing coverage enhancement".

Proposal 4: A UE is considered as a cell-border UE if its RSRP measurement is lower than a configured RSRP threshold, and is a cell-centre UE if its RSRP measurement is higher than the configured RSRP threshold.

Proposal 5: The cell-border UE selects the additional ROs if the gNB signals "prioritizing latency reduction" and selects the legacy ROs if the gNB signals "prioritizing coverage enhancement". The cell-centre UE always selects the additional ROs for its first RACH transmission.

## 2.4 Resource fall-back behaviour

Another aspect to discuss on resource selection is the UE fall-back behaviour. For example, after multiple times of RACH failures with resource in SBFD symbols, UE shall be allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols at least. The fall-back from non-SBFD symbols to SBFD symbols can also be supported. This is based on the consideration that using addition ROs can achieve coverage enhancement for cell-border UEs.

Proposal 6: Support that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

## 2.5 Msg3 early indication

In the last RAN2 meeting, Msg1 resource-based early identification was agreed, whether Msg3 based EI is needed is to be further discussed. We think it is still useful for network to know the UE capability earlier, when Msg1 based EI is not used, so that network can schedule subsequent transmissions/receptions in SBFD symbols. For Msg3 early indication solution, the reserved LCID can be allocated to the SBFD aware UEs to indicate the supporting of SBFD, which will have no impact to legacy UE. As there are plenty of reserved LCID, the concern on the limited LCID shall not be critical.

Proposal 7: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

## 2.6 RACH-RNTI calculation

In the last RAN2 meeting, some contributions mentioned the RA-RNTI calculation issue caused by SBFD RACH resource configuration.

In the RAN1#116bis meeting, the configuration related to legacy-ROs and additional-ROs was discussed with the following agreements:

Agreement For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

In the RAN1#118 meeting, there was further agreement about the RACH configuration:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

In the RAN1#119 meeting, there was agreement about RO determination:

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

According to these agreements, we understand there shall be no need to enhance the calculation of RACH-RNTI. For RACH configuration Option 1, the legacy ROs and additional ROs share the same space of RO index, hence no RA-RNTI collision issue. For RACH configuration Option 2, if the additional ROs  overlapping with legacy-ROs are invalid for SBFD-aware UEs, there would be no RA-RNTI collision issue as well. Anyway, we can wait the conclusion of RAN1 regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

Proposal 8: RAN2 to wait for RAN1 conclusion regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

RACH events

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

RACH Configuration

Proposal 2: RAN2 confirms the understanding that Table 1 is the UE behaviour depending on UE capability regarding supporting Option 1 and Option 2 and indication of whether Option 1 is enabled or not.

RACH resource selection

Proposal 3: Support the network indicates for which purpose SBFD RACH resource is to be used, i.e. indicates "prioritizing latency reduction" or indicates "prioritizing coverage enhancement".

Proposal 4: A UE is considered as a cell-border UE if its RSRP measurement is lower than a configured RSRP threshold, and is a cell-centre UE if its RSRP measurement is higher than the configured RSRP threshold.

Proposal 5: The cell-border UE selects the additional ROs if the gNB signals "prioritizing latency reduction" and selects the legacy ROs if the gNB signals "prioritizing coverage enhancement". The cell-centre UE always selects the additional ROs for its first RACH transmission.

Resource fall-back behaviour

Proposal 6: Support that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

Msg3 early indication

Proposal 7: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

RACH-RNTI calculation

Proposal 8: RAN2 to wait for RAN1 conclusion regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)