3GPP TSG-RAN WG2 Meeting #129	                                                                              R2-2500103

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.12.3

Source:	Nokia

Title:	RAN2 Aspects of the NR MIMO

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding requirements that RAN2 need to discuss in relation to the WID, in particular about:

- Objective 1: UE-initiated beam management;

- Objective 3: Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions:

# 2	UE-initiated beam management

The latest agreements from RAN1 on UEIBM related to RAN2 discussion are:

Agreement RAN1#117 Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement(RAN1 #118bis) Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). Agreement RAN1#119 On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

Rel-15 to Rel-18 has specified periodic, semi-persistent and aperiodic CSI reporting as described in TS 38.214. In the scope of Rel-19, some use cases where UE could benefit from initiating the beam management/reporting are being identified. The UE-initiated beam management (UEIBM) feature refers to the case where the UE may be configured with at least one event/condition, and then the UE may start beam reporting if this at least one event/condition occurs or is satisfied. The improvements aimed by this feature are twofold:

- Reduce overhead, such that beam reports are sent by the UE only when needed, avoiding unnecessary beam reports that we may have in case of periodic reporting configured with small periodicities.

- Reduce latency, such that, as soon as certain events/conditions are met, a beam report can be sent, to avoid long delay that we may have in case of periodic reporting configured with large periodicities.

From the RAN1 agreements, so far Event-2 has been agreed as the main event that can trigger a UEIBM report. Event-2 is the event where the quality of at least one new beam becomes a “threshold value” better than the current beam. For example, the network may configure the UE via RRC with a certain threshold, e.g. 6 dB, and when the UE measures a new beam to have a L1-RSRP which is 6 dB better than the L1-RSRP of the current beam, then a UEIBM report is triggered.

Besides Event-2, also Event-1 and Event-7 have been recently agreed at RAN1#119, where:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

It has also been agreed that those additional events will reuse the same design of Event-2.

Observation 1: RAN1 has defined two additional events, Event-1 and Event-7, which reuse the design of Event-2.

At RAN1#117, RAN1 made the following agreement on Event-2 triggering determination:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Although the definition of Event-2 and the condition for triggering the beam report depend on the counter value, the complete procedure is still not clear for now and up to detailed discussion in RAN1. So, it is not necessary for RAN2 to start a parallel discussion. The same applies for Event-1 and Event-7 since they will reuse the design of Event-2.

Proposal 1: RAN2 waits for further progress from RAN1 regarding the event evaluation and triggering procedures.

RAN1 has been discussing about introducing a prohibit timer to enable first PUCCH re-transmissions. Few candidates are on the table:

- Option-1: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

- Option-2: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

Both options are relevant.

Option-2 is a prohibit timer after the first PUCCH, and it is for Mode A: in case the UE does not receive a DCI after the first PUCCH within a certain time, then it is allowed to re-transmit the first PUCCH.

Option-1 is a prohibit timer after the PUSCH, i.e., after the UE sends a UEIBM report. When the UE sends a UEIBM report to the gNB triggered by an event, e.g., Event-2, in case the gNB does not do any beam switching after the UEIBM report sent by the UE, it may happen that the conditions triggering the same UEIBM report may persist over a period of time (the same Event-2, i.e., for same new beam(s) and for same conditions, e.g., same RSRP value(s) of the new beam(s) and of the current beam, keep repeating over time). In the current framework (even considering the enhancement where multiple Event-2 instances within a time window need to take place to trigger a UEIBM report) this causes the transmission of multiple consecutive UEIBM reports with same content, i.e., same top-N beams with same RSRP values, which bring no additional information to the network and just cause additional overhead (see also Figure 1 below). Generally, we think in Option-1 it would be beneficial to allow the transmission of a first PUCCH within the timer in case for example different beams triggered the same event or another event has been triggered.

Figure 1. Multiple consecutive UEIBM reports with same contents transmitted by the UE under the current framework.

Although prohibit timers are handled in RAN2 spec, the exact procedures are still not clear for now and up to detailed discussion in RAN1 (with also the possibility that no prohibit timer will be introduced at all). Therefore, also on this topic, we suggest to wait for further RAN1 progress.

Observation 2: RAN1 has not concluded on the details of the prohibit timer, or even whether to introduce a timer.

Proposal 2: RAN2 waits for further progress from RAN1 regarding prohibit timer for first PUCCH re-transmissions.

# 3	3-Tx UL codebook-based transmissions

The 3-port SRS resource is agreed to reuse the legacy 4-port SRS resource, where the 4th SRS port is muted, based on the agreement in RAN1 #117:

Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

For a UE with 4-port capability, the UE can support up to max 4-port SRS transmission. Usually, this type of UE can be configured to support 3-port uplink transmission. Therefore, 3-port SRS resource can be configured. Based on the 3-port SRS resource design, the legacy 4-port SRS resource would be configured for the UE. When the legacy 4-port SRS resource is configured, the UE won’t be able to differentiate whether the 3-port or the 4-port SRS resource shall be used.  Therefore, one RRC parameter would be useful to differentiate the 3-port SRS resource configuration. RAN1 has made following agreement in RAN1#118:

Agreement For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.

The RRC configuration for 3 Tx could be placed in either SRS-Config or PUSCH-Config. While it may seem logical to associate the number of Tx chains with the number of SRS ports by adding "3 ports" in SRS-Config, this approach may not be suitable because we are not enhancing the SRS resource to support 3 ports in this work item, and dynamic switching of SRS resource size for codebook based operation isn’t supported for 3 Tx. PUSCH-Config, which has configurations that are closely related to SRS-Config, could be a better option. By considering 3 Tx as a variant of transmission using a 4-port SRS, configuring this in PUSCH-Config avoids the need to redefine the SRS. Alternatively, configuring it in SRS-Resource or SRS-ResourceSet could allow for future enhancements.

Proposal 3: RAN2 to  define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration. Discuss further if it is per SRS resource or SRS resource set vs. if it is per PUSCH.

# Conclusion

This document has made the following observations:

Observation 1: RAN1 has defined two additional events, Event-1 and Event-7, which reuse the design of Event-2.

Observation 2: RAN1 has not concluded on the details of the prohibit timer, or even whether to introduce a timer.

And proposed the following:

Proposal 1: RAN2 waits for further progress from RAN1 regarding the event evaluation and triggering procedures.

Proposal 2: RAN2 waits for further progress from RAN1 regarding prohibit timer for first PUCCH re-transmissions.

Proposal 3: RAN2 to  define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration. Discuss further if it is per SRS resource or SRS resource set vs. if it is per PUSCH.