3GPP TSG-RAN WG2 Meeting #129	R2-2500104

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding requirements that RAN2 need to discuss in relation to the WID, in particular about:

- Objective 5: Asymmetric DL sTRP/UL mTRP deployment scenarios.

# 1	Asymmetric DL sTRP/UL mTRP

Figure 1 shows examples of the asymmetric DL sTRP / UL mTRP scenario. In such scenario, the DL transmission is from only a single TRP, depicted in the figure as “DL/UL capable TRP”. When using this feature, one or more “UL-only TRPs” may be deployed, potentially close to the cell edge in order to improve coverage. A UE may be scheduled for UL transmissions toward more than one TRP. UL transmissions are scheduled by the DL/UL capable TRP, which is the only TRP sending data and control information to each UE. Moreover, there is no restriction for a certain UE on which TRPs the UE is scheduled for UL transmissions, as we may have with reference to the figure below:

- UE1/UE2 transmitting toward an UL-only TRP and the DL/UL capable TRP,

- UE3 transmitting only toward the DL/UL capable TRP (legacy UE),

- UE4 transmitting only to an UL-only TRP,

- UEs (not depicted in the figure) transmitting to even more than one UL-only TRP.

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

An UL-only TRP exclusively receives signals and does not transmit any DL signals. In the absence of DL pathloss reference signal (PL-RS) transmission from an UL-only TRP, estimating the pathloss to the UL-only TRP within the current NR framework is not feasible for determining the UL transmit power needed for transmission to the UL-only TRP. However, a UE can estimate the pathloss to the UL-only TRP by using the pathloss of the Anchor TRP along with a pathloss offset specified by the NW. The NW indicates an UL RSRP difference as a pathloss (PL) offset to assist the UE in estimating the pathloss between itself and the UL-only TRP. This specified PL offset is standard and typically applies to power control for all UL signals/channels (e.g., PUCCH, PUSCH, SRS, PRACH). During the RAN1#116 meeting, it was agreed that the initial PL offset would be conveyed to the UE through RRC signaling. Prior to establishing a connection with the UL-only TRP, the UL-only TRP may lack reliable uplink measurements, thus the NW can only provide the UE with an initial PL offset, ensuring a significant reduction in transmit power towards the UL-only TRP to mitigate potential interference. The UE utilizes this initial PL offset to adjust its initial transmission power to the UL-only TRP. After the first UL signal is received from the UE, the NW can precisely estimate the actual PL offset between the UL-only TRP and the Anchor TRP and update the PL offset through a MAC Control Element (CE) message. The UE should then revise the initially incorrect RRC-configured PL offset according to the updated, more accurate offset received from the NW measurement. In the RAN2#128 the following agreement is made on path-loss offset indication

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

RAN1 has agreed to apply the path loss (PL) offset to the Type 1 PHR calculation for actual/reference PUSCH. Further study is needed to determine if and how the PL offset should be applied to the Type 3 PHR calculation. From the RAN2 perspective, a point of discussion is whether the PHR trigger might be impacted. According to TS 38.321, one of the PHR trigger events is defined such that any change in path loss for a given PL reference signal (PL RS) is taken into account to determine the PHR trigger.

The path loss value for the PL reference signal (PL RS) is calculated within the PHY layer, remaining transparent to the MAC layer. We believe the PL offset should be applied directly during this calculation in the PHY layer. Thus, the path loss used to determine the PHR trigger would already account for the PL offset at that stage. Consequently, no modifications to the PHR trigger condition in the MAC layer should be necessary.

Proposal 1: RAN2 to confirm that PL offset has no impact on PHR triggering condition as the PL offset is applied in the  PHY during the calculation of PL value.

When a UE goes through a RACH process and receives the random-access response from the serving gNB it has to apply the power ramping procedure based on pathloss, RRC indicated powerRampingStep, and other parameters specified in TS 38.213 for the PUSCH and PUCCH power control. In the asymmetric DL sTRP and UL mTRP deployment with pathloss offsets, the impacts on the power control of the PUSCH and PUCCH transmissions after the successful transmission of the RACH preamble (message 3 or message A) needs to be studied.

Proposal 2: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

In the Random Access (RA) procedure, the UE measures  the RSRP of the downlink PL-RS and compares it with an RSRP threshold for RA resource selection. In an asymmetric downlink/uplink scenario, the UE may measure RSRP from a downlink TRP and transmit PRACH to an uplink-only TRP. Due to a PL offset, the measured RSRP from a downlink TRP might differ from the actual RSRP of the uplink-only TRP used in the RSRP comparison. This offset could influence the RSRP comparison involved in RA resource selection.

Proposal 3: RAN2 to examine whether the PL offset affects the RSRP comparison in RA resource selection.

# 5	Conclusion

Proposal 1: RAN2 to confirm that PL offset has no impact on PHR triggering condition  as the PL offset is applied in the  PHY during the calculation of PL value.

Proposal 2: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

Proposal 3: RAN2 to examine whether the PL offset affects the RSRP comparison in RA resource selection.