3GPP TSG RAN WG2 #129	R2-2500106

Athens, Attica, Greece, February 17th – 21st, 2025

Source:	Panasonic

Title:	Store & Forward enhancements

Agenda Item:	8.9.2  Support of Store & Forward

Document for:	Discussion, decision

# 1. Introduction

Store & Forward operation of satellites is seen as a key enhancement of IoT-NTN in order to facilitate cost-effective deployment of IoT-NTN to allow for delay-tolerant, non-real-time IoT-NTN services [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

The aim of this document is to provide RAN2 with proposals for enhancing S&F-related functionality with amendments to the already reached set of agreements – see the annex of this document.

# 2. Discussion on deficiencies of the Store & Forward mode

# 2.1 Store & Forward illustration

  - Figure 1 below shows an S&F situation with two visible satellites – alpha and beta – in a Low Earth Orbit (LEO). The entire constellation might consist of four satellites in the same orbit.

  - The period of time no satellite is visible for a particular UE shall be handled in a way that the mobile communication part of the UE powers down as much as possible for extending the battery life. Suitable modes for those “dark phases”, i.e. no satellite illumination present, are eDRX and/or PSM.

  - Observation 1:	In the absence of any satellite overhead – i.e. during the time period the location of the UE is not illuminated due to a sparsely populated orbit in an S&F environment – power saving might be mandatory for IoT devices.

  - Proposal 1:	The periods of time the UE is not illuminated by a satellite shall be handled by powering the mobile communication part of the UE down as much as possible. Suitable methods are eDRX and/or PSM.

  - Figure 1: Store & Forward satellite constellation

  - In the S&F example presented here the two satellites alpha and beta illuminate four neighbouring cells each. The two regions illuminated – A/B/C/D and E/F/G/H – are far apart from one another.

  - The only gateway/ground station is located near the zone the cells A/B/C/D currently span. At its current position in the orbit, satellite alpha is connected to the gateway, i.e. a feeder link exists. At the same time there might also be service links existing between satellite alpha and UEs on the ground. So satellite alpha is either in S&F mode or in real-time mode - depending on the existence of service links between the satellite and the UEs at its current position.

  - At its current position, satellite beta has no feeder link connection to the gateway, but service links to the UEs on the ground exist – which corresponds to an S&F scenario.

  - To illustrate a practical LEO set-up, here are some example attributes of the satellites and their orbit:

- Altitude:					550 km

- Speed above ground:				7589 m/s

- Satellite spot beam aperture:			40°

- Diameter of each cell on the ground:		461,5 km

- Duration of illumination along cell centre:	60.8 s

- One cycle in that orbit takes			95 minutes

  - It is obvious that the duration of the existence of the service link between one particular satellite and a UE is restricted – especially in case the UE is not located in the centre of the zone of illumination. Hence data might need to be segmented in order to transfer it piece-wise from the UE to the network. Also restrictions regarding the storage space aboard the satellite might lead to a segmentation of data sets available from the UEs.

  - The gaps detected in the current signalling for such an S&F scenario are these:

- Current signalling done with SIB31(-NB), SIB32(-NB) and SIB33(-NB) is designed for a single beam per satellite. Although it is not impossible to signal multiple beams (and cells), the related restrictions are significant.

- There is no indication of the delivery time, i.e. the time it takes from the upload of data from the UE to the eNB on board the satellite and the subsequent download from the eNB to the network.

- Indicating remaining storage space aboard the satellite is currently not enabled.

  - 2.2 Multi-cell scenario

Currently, three SIBs are covering the basic NTN-specific needs in conjunction with LTE-based IoT-NTN – as follows:

- SIB31(-NB): Information about the currently serving satellite

- SIB32(-NB): Information about satellites not covering neighbouring areas

- SIB33(-NB): Information about satellites covering neighbouring areas

The information provided is satellite-specific, but multiple spot beams per satellite can only be covered in a way that each spot beam is handled like a single beam satellite consuming precious signalling resources likely reaching the maximum of four satelliteIds.

## Observation 2:	Multi-beam scenarios can be described with the existing signalling, but that is connected to undue restrictions in terms of number of spot beams per satellite and/or number of satellites that can be described.

Introducing a spot beam level for all three SIBs (or six including their narrowband (NB) variants) is suggested to align to state-of-the-art satellite technology with up to 48 spot beams per satellite illuminating up to 48 areas on the ground (not necessarily all at the same time).

Proposal 2:	Introduce a spot beam signalling level underneath the satellite level in SIB31(-NB), SIB32(-NB) and SIB33(-NB) in order to reflect state-of-the-art satellite technology.

For the three/six SIBs in question that would affect the following parts of the signalling:

- SIB31(-NB):	ServingSatelliteInfo-v1820 consisting of reference location and distance threshold location-based measurement initiation

- SIB32(-NB):	footprintInfo-r17, which provides information of the RF footprints, i.e. the illuminated areas, of remote satellites

- SIB33(-NB):	A new footprint information is added to enable similar functionality as described above for SIB32(-NB) – here in conjunction with neighbouring cells

The amended coding of the three/six SIBs in question might look like this:

SIB31(-NB):

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING			OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

[[	servingSatelliteInfo-v1910	ServingSatelliteInfo-v1910		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=		SEQUENCE {

ephemerisInfo-r17		CHOICE {

stateVectors		EphemerisStateVectors-r17,

orbitalParameters		EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17		SEQUENCE {

nta-Common-r17		INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17	INTEGER (-261935..261935)		OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120,

s180, s240, s900},

epochTime-r17			SEQUENCE {

startSFN-r17		INTEGER (0..1023),

startSubFrame-r17		INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17			INTEGER (0..1023),

k-Mac-r17				INTEGER (1..512)			OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18			SatelliteId-r18			OPTIONAL,	-- Need OR

referenceLocation-r18		CHOICE {

fixedReferenceLocation-r18		ReferenceLocation-r18,

movingReferenceLocation-r18		ReferenceLocation-r18

}								OPTIONAL,	-- Need OR

distanceThresh-r18			INTEGER(0..65535)			OPTIONAL	-- Need OR

}

ServingSatelliteInfo-v1910 ::=	SEQUENCE {

satelliteId-r19			SatelliteId-r18			OPTIONAL,	-- Need OR

SpotBeamList-r19 ::=	SEQUENCE (SIZE   (1..maxSpotBeam-r19)) OF SpotBeamInfo-r19

SpotBeamInfo-r19	SEQUENCE {

spotBeamId	INTEGER (0..255),

referenceLocation-r18		CHOICE {

fixedReferenceLocation-r18		ReferenceLocation-r18,

movingReferenceLocation-r18	ReferenceLocation-r18

}								OPTIONAL,	-- Need OR

distanceThresh-r18			INTEGER(0..65535)			OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

spotBeamId Identifies one of n spot beams emitted by the satellite identified with satelliteId.

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.

Figure 2: Proposed extension of SIB31(-NB) towards multi-beam scenario

SIB32(-NB):

-- ASN1START

SystemInformationBlockType32-r17 ::= SEQUENCE {

satelliteInfoList-r17				SatelliteInfoList-r17		OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING		OPTIONAL,

...,

[[	satelliteInfoList-v1800		SatelliteInfoList-v1800	OPTIONAL	-- Need OR

]],

[[	satelliteInfoList-v1830		SatelliteInfoList-v1830	OPTIONAL	-- Need OR

]]

[[	satelliteInfoList-r19		SatelliteInfoList-r19	OPTIONAL	-- Need OR

]]

}

SatelliteInfoList-r17 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r17

SatelliteInfoList-v1800 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF CarrierFreqList-v1800

SatelliteInfoList-v1830 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF CarrierFreqList-v1830

SatelliteInfoList-r19 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r19

SatelliteInfo-r17 ::=		SEQUENCE {

satelliteId-r17	INTEGER (0..255),

serviceInfo-r17	SEQUENCE {

tle-EphemerisParameters-r17	TLE-EphemerisParameters-r17		OPTIONAL,	-- Need OR

t-ServiceStart-r17		TimeOffsetUTC-r17			OPTIONAL	-- Need OR

},

footprintInfo-r17			SEQUENCE {

referencePoint-r17		SEQUENCE {

longitude-r17	INTEGER (-131072..131071),

latitude-r17	INTEGER (-131072..131071)

}							 OPTIONAL,	-- Need OR

elevationAngles-r17		SEQUENCE {

elevationAngleRight-r17	INTEGER (-14..14),

elevationAngleLeft-r17	INTEGER (-14..14)		OPTIONAL	-- Need OP

} 							OPTIONAL,	-- Need OR

radius-r17			INTEGER (1..256)			OPTIONAL	-- Need OR

}

}

CarrierFreqList-v1800 ::=		SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA

CarrierFreqList-v1830 ::=		SEQUENCE {

carrierFreqList-r18		SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA-r9

SatelliteInfo-r19 ::=		SEQUENCE {

satelliteId-r19	INTEGER (0..255),

serviceInfo-r19	SEQUENCE {

tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17	OPTIONAL,	-- Need OR

t-ServiceStart-r17		TimeOffsetUTC-r17			OPTIONAL	-- Need OR

},

SpotBeamList-r19 ::=	SEQUENCE (SIZE   (1..maxSpotBeam-r19)) OF SpotBeamInfo-r19

SpotBeamInfo-r19	SEQUENCE {

spotBeamId	INTEGER (0..255),

footprintInfo-r19			SEQUENCE {

referencePoint-r17		SEQUENCE {

longitude-r17	INTEGER (-131072..131071),

latitude-r17	INTEGER (-131072..131071)

}					 OPTIONAL,	-- Need OR

elevationAngles-r17		SEQUENCE {

elevationAngleRight-r17	INTEGER (-14..14),

elevationAngleLeft-r17	INTEGER (-14..14)	OPTIONAL              -- Need OP

}					OPTIONAL,	-- Need OR

radius-r17			INTEGER (1..256)	OPTIONAL              -- Need OR

}

}

-- ASN1STOP

SystemInformationBlockType32 field descriptions

carrierFreqList Includes a list of E-UTRA frequencies, see TS 36.304 [4].

elevationAngleLeft, elevationAngleRight Leftmost and rightmost (with reference to the satellite direction) elevation angle. Unit in degree. Step of 5 degree. Actual value = field value * 5. If the field elevationAngleLeft is absent, the leftmost elevation angle is equal to the value of field elevationAngleRight.

footprintInfo-r17 Satellite footprint. E-UTRAN may configure elevationAngles and/or radius for earth moving cell. E-UTRAN may configure referencePoint and radius for quasi-earth fixed cell.

footprintInfo-r19 Spot beam footprint. E-UTRAN may configure elevationAngles and/or referencePoint and radius together for earth moving cell. E-UTRAN may configure referencePoint and radius for quasi-earth fixed cell.

Latitude Latitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

Longitude Longitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

Radius Distance between the reference point and the edge of the satellite or spot beam coverage. Unit in km. Step of 10 km. Actual value = field value * 10.

satelliteInfoList List of satellite information. If E-UTRAN includes satelliteInfoList-v1830, it includes the same number of entries, and listed in the same order, as in satelliteInfoList-r17. In this version of the specification, E-UTRAN does notneither include satelliteInfoList-r17 nor does it include satelliteInfoList-v1800.

serviceInfo Information on when the satellite will provide coverage. E-UTRAN always configures tle-EphemerisParameters for a satellite with earth moving cell(s) and always configures t-ServiceStart for a quasi-earth fixed cell.

spotBeamId Identifies one of n spot beams emitted by the satellite identified with satelliteId.

spotBeamInfoList List of spot beam information.

tle-EphemerisParameters Mean values of the satellite orbital parameters based on the TLE set format for estimating in-coverage and out-of-coverage periods for a satellite with earth moving cell(s), see TS 36.304 [4].

t-ServiceStart Time information on when the incoming satellite spot beam identified with spotBeamId is going to start serving the area for quasi-earth fixed cell.

Figure 3: Proposed extension of SIB32(-NB) towards multi-beam scenario

SIB33(-NB):

-- ASN1START

SystemInformationBlockType33-r18 ::= 	SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18	ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240,

s900}			OPTIONAL,	-- Need OP

lateNonCriticalExtension	OCTET STRING				OPTIONAL,

...

[[	neighsatelliteInfoList-r19	NeighSatelliteInfoList-r19		OPTIONAL	-- Need OR

]]

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfoList-r19 ::=	SEQUENCE (SIZE (1..maxSat-r19)) OF NeighSatelliteInfo-r19

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18	SatelliteId-r18,

ephemerisInfo-r18	CHOICE {

stateVectors-r18		EphemerisStateVectors-r17,

orbitalParameters-r18	EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18	SEQUENCE {

nta-Common-r18		INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18	INTEGER (-261935..261935)		OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18		SEQUENCE {

startSFN-r18		INTEGER (0..1023),

startSubFrame-r18		INTEGER (0..9)

}								OPTIONAL,	-- Need OP

k-Mac-r18			INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18	TimeOffsetUTC-r17				OPTIONAL	-- Need OR

NeighSatelliteInfo-r19 ::=	SEQUENCE {

satelliteId-r19	SatelliteId-r19,

ephemerisInfo-r18	CHOICE {

stateVectors-r18		EphemerisStateVectors-r17,

orbitalParameters-r18	EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18	SEQUENCE {

nta-Common-r18		INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18	INTEGER (-261935..261935)		OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)		OPTIONAL	-- Need OP

},

epochTime-r18		SEQUENCE {

startSFN-r18		INTEGER (0..1023),

startSubFrame-r18		INTEGER (0..9)

}								OPTIONAL,	-- Need OP

k-Mac-r18			INTEGER (1..512)			OPTIONAL,	-- Need OP

SpotBeamList-r19 ::=	SEQUENCE (SIZE   (1..maxSpotBeam-r19)) OF SpotBeamInfo-r19

SpotBeamInfo-r19	SEQUENCE {

spotBeamId	INTEGER (0..255),

footprintInfo-r17			SEQUENCE {

referencePoint-r17		SEQUENCE {

longitude-r17	INTEGER (-131072..131071),

latitude-r17	INTEGER (-131072..131071)

}					 OPTIONAL,	-- Need OR

elevationAngles-r17	SEQUENCE {

elevationAngleRight-r17	INTEGER (-14..14),

elevationAngleLeft-r17	INTEGER (-14..14)	OPTIONAL              -- Need OP

}					OPTIONAL,	-- Need OR

radius-r17			INTEGER (1..256)	OPTIONAL              -- Need OR

t-ServiceStartNeigh-r18	TimeOffsetUTC-r17		OPTIONAL              -- Need OR

}

}

}

-- ASN1STOP

SystemInformationBlockType33 field descriptions

epochTime Epoch time of the neighbour satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the neighbour satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the eNB when this field is provided in a TN cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If this field is absent in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. The startSFN indicates the SFN nearest to the frame where the message indicating the epochTime is received. If this field is absent in a TN cell, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

neighValidityDuration Validity duration of the neighbour satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. If this field is absent in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in a TN cell, how the UE sets validity duration is left to UE implementation.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) identified with spotBeamId served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

Figure 4: Proposed extension of SIB33(-NB) towards multi-beam scenario

Proposal 3:	Adopt the proposed amendments to SIB31(-NB), SIB32(-NB) and SIB33(-NB) as outlined with R2-2500106.

  - 2.3 Indication of delivery time

So far, the eNB doesn’t provide any information to the UE about the time it takes until the data set about to be uploaded reaches the network (or the application provider/server in the end). A related proposal is formulated here.

Observation 3:	Delivery time of data between upload and download points is not described yet. That is equivalent to a best effort approach under the current conditions.

Due to the intermittent nature of service links and feeder links in S&F mode – they never occur together – there is an inevitable delay from uploading data from the UE to the eNB aboard a satellite and the later downloading from that satellite to the core network, when a feeder link is established. See figure 1 in clause 2.1 above for a related illustration.

The mentioned delay mainly depends on the distance between the position the satellite occupies when the data is uploaded from the UE to the satellite and the position, at which the same satellite is able to establish a feeder link to the gateway on the ground. Still, other factors could come into play, e.g. the existence of Inter-Satellite Links shortening the delay or an extension of the mentioned delay based on a download to another than the first gateway that comes into view – for whatever reason.

The indication of the delivery time to the UE might be used by the latter to decide, if the upload makes sense at all (if the data is required at the application provider/server within e.g. 20 minutes, a delivery period of 30 minutes would render the upload obsolete). Also the selection between different providers – based on different delivery times (and related tariffs) might be supported by the indication of the delivery time. Even the same provider might offer different options with different delivery times (and tariffs) – e.g. conventional S&F, S&F combined with ISL and real-time mode. Furthermore, the UE could also be positioned at the crossroads of two or more orbits.

Proposal 4:	Introduce an indication of delivery time in an NTN-related SIB.

There is also a relationship between the delivery time and the available storage space aboard the satellite.

That delay between the upload from the UE to the eNB and the offload from the eNB to the gateway might be indicated with SIB31(-NB), because that Service Information Block Type is suited for the S&F situation, because it describes the serving satellite.

The entire delivery time from source to sink consists of four components:

- t1: Time it takes to transfer the data from UE to satellite

- t2: Time the satellite needs to travel from the aforementioned uplink position in its orbit to the position where it (re)establishes the feeder link to the gateway on the ground

- t3: Time it takes to transfer the data from the satellite to the gateway

- t4: Time it takes to transfer the data from the gateway to the application provider (via the Internet)

Since t4 is not within 3GPP’s scope, the total delivery time is TD = t1 + t2 + t3  (major portion represented by t2). For preventing frequent updates, delivery time is indicate as the absolute point of time the data reaches the gateway/network. In order to calculate the time period that will elapse until data reaches its destination, the time indication being part of SIB16 or GNSS can be used by the UE.

Proposal 5:	Indicate delivery time in SIB31(-NB) as outlined with R2-2500106.

The coding might be the following (also applicable to the NB version of SIB31, i.e. SIB31-NB):

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING			OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

[[	servingSatelliteInfo-v1910	ServingSatelliteInfo-v1910		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=		SEQUENCE {

ephemerisInfo-r17		CHOICE {

stateVectors		EphemerisStateVectors-r17,

orbitalParameters		EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17		SEQUENCE {

nta-Common-r17		INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17	INTEGER (-261935..261935)		OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120,

s180, s240, s900},

epochTime-r17			SEQUENCE {

startSFN-r17		INTEGER (0..1023),

startSubFrame-r17		INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17			INTEGER (0..1023),

k-Mac-r17				INTEGER (1..512)			OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18			SatelliteId-r18			OPTIONAL,	-- Need OR

referenceLocation-r18		CHOICE {

fixedReferenceLocation-r18		ReferenceLocation-r18,

movingReferenceLocation-r18		ReferenceLocation-r18

}								OPTIONAL,	-- Need OR

distanceThresh-r18			INTEGER(0..65535)			OPTIONAL	-- Need OR

}

servingSatelliteInfo-v1910 ::=	  SEQUENCE {

deliveryTime-r19			TimeOffsetUTC-r17			OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

deliveryTime Point of time, at which in Store & Forward mode (see SIB1 signalling) the data the UE uploads to the satellite (service link exists) will be downloaded from the satellite to the remote gateway (feeder link exists).

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.

Figure 5: Proposed extension of SIB31(-NB) for indication of delivery time

Observation 4: 	There is a close relationship between the delivery time indication and the indication of the remaining time until a satellite changes from S&F mode to real-time (RT) mode or vice-versa.

Proposal 6:	If the indication of delivery time becomes adopted, indicate the change between S&F and RT mode and delivery time in an aligned way.

Observation 5:	Since both indications – change between S&F and RT on the one hand side and delivery time on the other hand – are closely related to one another, those should be coded in a combined way in order to prevent redundancies.

Proposal 7:	Indicate point of time a satellite changes from S&F to RT mode (or vice-versa) as the related absolute time instance in SIB31(-NB).

  - 2.4 Indication of remaining storage space available aboard the satellite/eNB

A satellite that operates in S&F mode permanently or part-time needs to store the data uploaded from the UEs in the presence of the service link – until that data can be offloaded to the gateway on the ground, i.e. when a feeder link is established.

Since storage space is limited, it might not always be possible to accept all amounts of data that UEs want to upload to the eNB. In such a case, there might be two options for limiting the amount of data received from the UEs:

- eNB does not schedule UL transmission for a number of UEs when storage space becomes scarce

- Limit the amount of data UEs are permitted to upload (can be different for different UEs, e.g. depending on tariffs) and tell the UEs about that limit

Observation 6:	Limited storage space aboard an S&F satellite can lead to nearing or reaching storage overflow – in an uncontrolled environment.

Not scheduling UL transmission of MO data is a tool already available – also for the NTN case.

Observation 7:	Currently, a countermeasure against storage overflow is to not schedule uploads from UEs to the satellite anymore when nearing storage overflow.

But the provision of information about available memory space aboard the satellite is not defined yet. Should the memory in question become scarce and UEs should still be enabled to upload data when a service link has been established, the amount of data that each UE is permitted to upload could be restricted. On the basis of the related information, the UE can decide in favour of uploading only a subset of the data it has stored or in favour with refraining from uploading entirely during this service link period.

Proposal 8:	Introduce an indication of remaining available storage space per UE in an NTN-related SIB.

Here is a proposal for an indication of that space. Because the information won’t be available in advance of the related satellite’s appearance in the area of the UE, the signalling should be realized in conjunction with the serving satellite and hence with SIB31(-NB). In the first place – up to now SIB31(-NB) is only defined for broadcast distribution – the information provided would be the same for all UEs receiving SIB31(-NB) from the particular satellite. The coding might look like this (also applicable to the NB version of SIB31, i.e. SIB31-NB):

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING			OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

[[	servingSatelliteInfo-v1910	ServingSatelliteInfo-v1910		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=		SEQUENCE {

ephemerisInfo-r17		CHOICE {

stateVectors		EphemerisStateVectors-r17,

orbitalParameters		EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17		SEQUENCE {

nta-Common-r17		INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17	INTEGER (-261935..261935)		OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120,

s180, s240, s900},

epochTime-r17			SEQUENCE {

startSFN-r17		INTEGER (0..1023),

startSubFrame-r17		INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17			INTEGER (0..1023),

k-Mac-r17				INTEGER (1..512)			OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18			SatelliteId-r18			OPTIONAL,	-- Need OR

referenceLocation-r18		CHOICE {

fixedReferenceLocation-r18		ReferenceLocation-r18,

movingReferenceLocation-r18		ReferenceLocation-r18

}								OPTIONAL,	-- Need OR

distanceThresh-r18			INTEGER(0..65535)			OPTIONAL	-- Need OR

}

servingSatelliteInfo-v1910 ::=	  SEQUENCE {

storageSpace-r19			INTEGER(0..65535)			OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

storageSpace Remaining storage space per UE aboard the satellite. Indication in Megabytes (MB).

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.

Figure 4: Proposed extension of SIB31(-NB) for indication of remaining storage space per UE

Proposal 8:	Indicate remaining available storage space per UE in SIB31(-NB) as outlined with R2-2500106.

3. Conclusions

In this document, we discussed how to enhance the S&F mode of LTE-based IoT-NTN – based on what has been agreed already.

The following observations and proposals were made:

  - Observation 1:	In the absence of any satellite overhead – i.e. during the time period the location of the UE is not illuminated due to a sparsely populated orbit in an S&F environment – power saving might be mandatory for IoT devices.

## Observation 2:	Multi-beam scenarios can be described with the existing signalling, but that is connected to undue restrictions in terms of number of spot beams per satellite and/or number of satellites that can be described.

Observation 3:	Delivery time of data between upload and download points is not described yet. That is equivalent to a best effort approach under the current conditions.

Observation 4: 	There is a close relationship between the delivery time indication and the indication of the remaining time until a satellite changes from S&F mode to real-time (RT) mode or vice-versa.

Observation 5:	Since both indications – change between S&F and RT on the one hand side and delivery time on the other hand – are closely related to one another, those should be coded in a combined way in order to prevent redundancies.

Observation 6:	Limited storage space aboard an S&F satellite can lead to nearing or reaching storage overflow – in an uncontrolled environment.

Observation 7:	Currently, a countermeasure against storage overflow is to not schedule uploads from UEs to the satellite anymore when nearing storage overflow.

  - Proposal 1:	The periods of time the UE is not illuminated by a satellite shall be handled by powering the mobile communication part of the UE down as much as possible. Suitable methods are eDRX and/or PSM.

Proposal 2:	Introduce a spot beam signalling level underneath the satellite level in SIB31(-NB), SIB32(-NB) and SIB33(-NB) in order to reflect state-of-the-art satellite technology.

Proposal 3:	Adopt the amendments to SIB31(-NB), SIB32(-NB) and SIB33(-NB) as outlined with R2-2500106.

Proposal 4:	Introduce an indication of delivery time in an NTN-related SIB.

Proposal 5:	Indicate delivery time in SIB31(-NB) as outlined with R2-2500106.

Proposal 6:	If the indication of delivery time becomes adopted, indicate the change between S&F and RT mode and delivery time in an aligned way.

Proposal 7:	Indicate point of time a satellite changes from S&F to RT mode (or vice-versa) as the related absolute time instance in SIB31(-NB).

Proposal 8:	Introduce an indication of remaining available storage space per UE in an NTN-related SIB.

Proposal 9:	Indicate remaining available storage space per UE in SIB31(-NB) as outlined with R2-2500106.

- References

[1]	RP-240776 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	3GPP TS 36.331 V18.4.0 (2024-12)				3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 18)

ANNEX – RAN2’s S&F agreements so far

- Meeting R2-125-bis (Changsha):

  - S&F implies that at least the full eNB will be onboard

  - An IoT-NTN network shall be able to inform UE(s) whether S&F satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

  - The S&F satellite operation is common for NB-IoT and eMTC.

  - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture).

- Meeting R2-126 (Fukuoka):

  - For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

    - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

    - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

    - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

    - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again.

  - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed.

  - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN.

- Meeting R2-127 (Maastricht):

  - RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and split-MME payload options)

  - RAN2 will consider both single satellite pass and multiple satellite pass scenarios

  - RAN2 will consider both MO and MT data within scope

  - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

  - Come back to the topic “UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast” in the next meeting

  - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs is expected.

  - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow Rel.19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication).

- Meeting R2-127-bis (Hefei):

  - The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed).

  - RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by an NTN cell that do not support S&F capability and the case that the UE is served by an NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

  - An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed.

  - If the “S&F operation” indication is not broadcast, a Rel.19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure.

  - When present, the “S&F operation” indication has two possible settings:

    - ‘1’: the cell is operating in S&F mode for all UEs (Rel.19 UEs supporting S&F are allowed to access the cell).

    - ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel.19 UEs supporting S&F are barred (Rel.19 UEs not supporting S&F will follow legacy barring procedure).

- Meeting R2-128 (Orlando):

  - Time information can be broadcast to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a Rel.19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for Rel.19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start).

  - It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable.

  - “S&F operation” indication is provided in SIB1.

  - RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

  - RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation.

  - RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT-NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite.

  - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.