3GPP TSG-RAN2 Meeting#129	R2-2500112

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	8.0

Source:	OPPO

Title:	Discussion on RAT Restriction

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of RAT restriction, as indicated in C1 LS C1-247193 as follows.

The following aspect is not yet covered in any stage-3 specification and CT1 would like to involve RAN2:

If an access technology is restricted in a PLMN, the UE shall not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service.

- Discussion

Given there is no corresponding RAN WI for this CT1 WI, while there seems a need for R2 normative work for stage-3, the R2 workload has to be evaluated, i.e., whether to proceed the work without RAN WI and thus no explicit TU planning.

- R2 discuss whether / how to handle this cross-TSG issue without corresponding RAN WG WI, by evaluating the load of necessary normative work (e.g., regarding impact on 304 spec) by R2.

Intention wise, as stated in the LS, C1 would like to

CT1 is introducing an optional feature for the network to indicate RAT utilization restriction information to the UE with EMM and 5GMM procedures.

[…]

The following access technologies can be restricted: GERAN, UTRAN, E-UTRAN, satellite E-UTRAN, NG-RAN, satellite NG-RAN. E-UTRAN access technology encompasses all terrestrial E-UTRAN cells. NG-RAN access technology encompasses all terrestrial NG-RAN cells.

For RRC_CONNECTED, the related tools seem already in place. According to 23.501

Mobility Restrictions consists of RAT restriction, Forbidden Area, Service Area Restrictions, Core Network type restriction and Closed Access Group information as follows:

-	RAT restriction:

Defines the 3GPP and non-3GPP Radio Access Technology(ies), a UE is not allowed to access in a PLMN. In a restricted RAT a UE based on subscription is not permitted access to the network for this PLMN. For 3GPP access and CM-CONNECTED state, when radio access network determines target RAT and target PLMN during Handover or redirection procedure, it should take per PLMN RAT restriction into consideration. The RAT restriction is enforced in the network, and not provided to the UE.

And corresponding NGAP signalling is in place in 38.413

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary RAT Restriction | M |  | BIT STRING { e-UTRA (0), nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6), e-UTRA-LEO (7), e-UTRA-MEO (8), e-UTRA-GEO (9), e-UTRA-OTHERSAT (10)} (SIZE(8, …, 16)) | Each position in the bitmap represents a Primary RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 11-15 reserved for future use. The Primary RAT is the RAT used in the access cell, or target cell. |

The RAT restriction mechanism for RRC_CONNECTED mobility can be handled via existing signalling.

Then back to the RRC_IDLE / RRC_INACTIVE mobility, i.e., the cell (re)selection, as asked by C1 LS.

In the current 304 spec (both 36 and 38), so far NAS layer can impose control to cell selection via the ‘list of preferred PLMN/access technology combinations’, which has been captured in 304 as follows

If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in TS 23.122 [9].

[…]

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

On the other hand, since the associated RAT(s) are in our understanding the operator preferred list, and the user preferred list, which is a whitelist, where the entries only provides preference (not restriction), without mandating the UE to restrict to the associated RAT(s).

So, there is a need to enhance the cell (re)selection functions, if R2 would like to support what C1 asked for.

The RAT restriction mechanism for RRC_IDLE/RRC_INACTIVE mobility is now limited to cell selection, which is based on a white preferred list still allowing UE to make use of the RATs out of the list, and does not include TN/NTN differentiation.

For cell selection, the description currently in R2 is quite general already, as cited above. Considering that the detailed usage of the NAS-provided PLMN/RAT combination list is specified in C1 spec (23.122).

Considering C1 request differentiates between normal / limited-service state, it seems technically reasonable to revise the ‘suitable cell’ concept to align with the C1 intention, e.g.,

suitable cell:

For UE not operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

[…]

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" (TS 22.011 [18]), which belongs to a PLMN that fulfils the first bullet above.

-	The RAT of the cell is not restricted by NAS (as specified in TS 23.122 [9])

For cell reselection, currently it is a priority-based procedure

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If any fields with cellReselectionPriority or nsag-CellReselectionPriority are provided in dedicated signalling, the UE shall ignore any fields with cellReselectionPriority and nsag-CellReselectionPriority provided in system information.

[…]

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

I.e., theoretically it is possible to set UE-specific priority for the concerned RAT/frequency via RRCRelease message, based on the RAT-Restriction IE sent from CN to RAN. Yet considering

1) In order to avoid the searching of a specific RAT (e.g., NTN) for some UEs (not all), the concerned frequency has to be included in system information, yet without associated priority, and fully rely on RRCRelease to set the priority for the concerned frequency (if the access is allowed) or not (if the access is restricted).

2) Although so far, NTN is deployed at specific bands, which are different from bands defined for TN, there could be deployment in the future to have both TN and NTN co-exist in the same band(s), for which the frequency specific differentiation may not be sufficient.

Therefore, this solution, although has the advantage of minimized R2 spec impact, has some drawbacks.

The other solution is to mimic what we had for cell selection, i.e., to further restrict the definition of ‘suitable cell’

- If R2 decides to procced with normative work for this issue, R2 aims at minimum change to R2 spec, e.g., revising the suitable cell definition to reflect the RAT restriction asked by C1 (TP as above).

Another issue is whether we want to implement the CR for 2G and 3G in R2 specification (considering as shown in the following figure, C1 introduces RAT restriction since 2G and 3G)

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  |  |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT utilization control IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of RAT utilization control contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  |  | 0 spare | Type of RAT utilization control |  |  |  | octet 3* |
| 0 spare | 0 spare |  | Sat-NG-RAN |  | Sat-E-UTRAN |  | NG-RAN |  |  | E-UTRAN |  | UTRAN |  | GERAN |  | octet 4* |

Figure 9.9.3.3A.1: RAT utilization control information element (from TS 24.301)

Note that CT1 spec of TS 23.122 covers different RATs in general, but R2 has different spec for different RATs:

- 2G: 03.22 (closed at Rel-8), 43.022 (without no CRs since Rel-13)

- 3G: 25.304 (have no real CRs since Rel-12)

So for 2G and 3G, it seems too late to go for a CR, yet if we do not do it, and if we take 2G as an example (the same issue applies to 3G if we do not do it)

      - For cell selection, if NAS selects PLMN-1, which includes both 2G and 4G RATs, and let's say 2G is now restricted, without 2G CR at R2 side, whether the restriction on selection-to-2G would take effect during cell selection?

      - For cell reselection, in the example above, if the UE selects a 2G cell during cell selection, the follow-up cell reselection-to-2G cannot be prevented either. In another example, if a UE now camps on 2G (not restricted) for PLMN-2, but 4G of the same PLMN is restricted, without 2G CR at R2, whether the restriction on reselection-to-4G would take effect during cell reselection?

I.e., the restriction would fail to take effect as long as the UE initially camps on a 2G cell (the reselection-to-4G cannot be restricted either). And it seems can only be solved via either

- different MCC/MNC code for 2G/3G, i.e., NAS can from the very beginning prevent the UE to camp on 2G via PLMN selection step, or

- different TAI for 2G/3G using the same MCC/MNC, i.e., the legacy solution;

- If R2 decides to procced with normative work for this issue, R2 discuss whether to do the change for 2G and 3G specification. And if no, clarify the status/consequence to C1 in the reply LS.

- Conclusion

Proposal 1	R2 discuss whether / how to handle this cross-TSG issue without corresponding RAN WG WI, by evaluating the load of necessary normative work (e.g., regarding impact on 304 spec) by R2.

Proposal 2	If R2 decides to procced with normative work for this issue, R2 aims at minimum change to R2 spec, e.g., revising the suitable cell definition to reflect the RAT restriction asked by C1 (TP as above).

Proposal 3	If R2 decides to procced with normative work for this issue, R2 discuss whether to do the change for 2G and 3G specification. And if no, clarify the status/consequence to C1 in the reply LS.