3GPP TSG-RAN WG2 Meeting #129	R2-2500128

Athens, Greece, 17th – 21st Feb, 2025

Source:	vivo

Title:	Discussion on signaling feasibility of dataset and parameter sharing

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

During the RAN1#119 meeting, RAN1 discussed the options for resolving issues related to inter-vendor training collaboration of the CSI-compression use case. RAN1 concluded that standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity. In addition, to facilitate the model parameter and/or dataset sharing, RAN1 concluded the following two candidate standardized signaling approaches:

- over-the-air

- other approaches

After that, RAN1 sent the LS [1] to RAN2 on the feasibility of the standardized signaling for the down-selected options:

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

In this contribution, we will provide our analysis of the feasibility of the potential two approaches.

# Discussion

### 2.1 Analysis of over-the-air Approach

During Rel-18, to support AI/ML model transfer/delivery, the following solutions were studied and considered in [2]:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

However, the necessity of standardized solutions for model transfer was unclear in the SI phase. In addition, the typical model size was not provided by RAN1 at that time. As a consequence, only analysis of current status and gaps, and potential specification impact for potential solutions were captured in the TR.

In the LS from RAN1, both model parameters and dataset are required to be shared. However, the format of model parameter and dataset can be agnostic from RAN2 perspective. Therefore, RAN2 can further study and down-select the model transfer solutions in Rel-18 to facilitate dataset/parameter sharing based on RAN1 input about the dataset/parameter size.

Besides, both CP-based and UP-based solutions can be considered for further study and down-selection. Specifically:

- Solutions 3a and 3b can be excluded as only CSI compression use case is considered in the LS.

- Solution 1b can be de-prioritized as no UP tunnel is terminated at gNB.

- Solution 4a is out of scope as the model is delivered from OTT server, which is not the scenario in the LS.

- Based on RAN1 input about the dataset/parameter size, RAN2 to down-select the model transfer solutions in Rel-18 SI to enable dataset/parameter sharing.

- For the standardized signaling of dataset/parameter sharing, both CP-based and UP-based solutions can be considered for further study.

- Solutions 1a (gNB to UE via RRC) and 2a (CN to UE via NAS) are candidates as CP-based solutions;

- Solutions 2b (CN to UE) and 4b (OAM to UE) are candidates as UP-based solutions.

#### 2.1.2 CP-based solution

For CP-based solutions, one key issue is the dataset/parameter size. In the LS, the typical size of selected options is indicated as follows:

- For Option 4-1, sharing {target CSI, CSI feedback} dataset, the typical size of dataset is 225 MB.

- For Option 3a-1 without target CSI: sharing encoder parameter, the typical size of model parameter is 11.6 MB.

- For Option 3a-1 with target CSI: sharing encoder parameters along with {target CSI} dataset, assuming N2=N1, the typical size of dataset and model parameter is 161.6 MB.

From our understanding, two solutions can be considered to address the size issue for CP-based dataset/parameter sharing:

- Solution 1: The network splits the dataset/parameter into multiple subsets and shares them with different UEs, and the UEs deliver the subsets to the server for model training.

- Solution 2: The network performs dataset/parameter segmentation and transfers them to the UE, the UE or the server reassembles the segments.

Note that the above two solutions can work together, i.e., one split subset can be further segmented and transferred to one UE.

- RAN2 to study how to address the size issue for CP-based dataset/parameter sharing, the following two solutions can be considered:

- Solution 1: The network splits the dataset/parameter into multiple subsets and shares them with different UEs.

- Solution 2: The network performs dataset/parameter segmentation before transferring them to one UE.

Considering the dataset size is quite huge and it cannot always be assumed that the UE involved in the dataset/parameter sharing is in large number, Solution 2 should be considered as a starting point. And RAN2 should study suitable segmentation methods to support CP-based dataset/parameter sharing.

- RAN2 to study suitable segmentation methods to support CP-based dataset/parameter sharing.

The maximum size of a PDCP SDU is 9k bytes. Assuming the dataset/parameter is shared with one single UE, the segment number is illustrated in Table 2.1-1.

Table 2.1-1 Segment number for dataset/parameter sharing

| Option | Segment number |
| --- | --- |
| Option 4-1 (225 MB) | 25000 |
| Option 3a-1 without target CSI (11.6 MB) | 1289 |
| Option 3a-1 with target CSI (161.6 MB) | 17956 |

From Table 2.1-1, it can be seen that dataset/parameter sharing requires a large number of segments, especially dataset sharing, which needs to be divided into 25,000 segments.

For the existing downlink CP-based message, the maximum size of one message can be extended with segmentation:

- RRC segmentation is realized with a sequence number and end marker, allowing a maximum of 5 segments for downlink. Therefore, the largest downlink message that RRC can send cannot exceed 45k (9k*5).

- LPP uses a segmentation method with end marker only and has no limit on the number of segments. The LPP layer is responsible for reassembling segments.

Since the current supported segment number of RRC message is far less than the required segment number, it is not recommended to consider increasing the RRC segment number to support CP-based dataset/parameter sharing.

On the other hand, the LPP-like segmentation can be considered, i.e., based on the end marker.

- Do not consider increasing the RRC segment number to support CP-based dataset/parameter sharing.

- LPP-like segmentation can be considered, i.e., based on end marker, to support CP-based dataset/parameter sharing.

For dataset transfer, it is worth noting that in some scenarios there may be some security or user consent issues that may need to be considered. For example, in case the dataset for training a two-sided model is typically composed of data samples collected from different UEs/vendors, sharing datasets would mean sharing data samples from one UE/vendor to other UEs/vendors. Whether there are security issues and whether an additional mechanism of user consent is needed should be referred to SA3.

- LS to SA3 to study whether there is security issue of sharing data samples collected from one UE/vendor to other UEs/vendors and whether an additional user consent mechanism is needed.

#### 2.1.3 UP-based solution

At the RAN#106 meeting, it was agreed that the transfer of data over UP for Solution 2 should be studied by SA2 and the transfer of data over UP for Solution 3 can be studied by SA5 [3].

2. Actions: To TSG SA, WG SA2, WG SA5. ACTION: TSG RAN kindly asks TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2 and transfer of data over UP for solution 3 in WG SA5 and provide feedback to TSG RAN and WG RAN2.

From our understanding, the dataset/parameter transfer can be seen as a reverse direction of Option 2/3 of UE side data collection. Therefore, RAN2 should send LS to SA2 and SA5 to study the feasibility of UP-based dataset/parameter sharing via the air interface.

- LS to SA2 and SA5 to study the feasibility of UP-based dataset/parameter sharing via the air interface.

### 2.2 Analysis of Other Approaches

Data collection for UE-side model training, the following options were discussed in RAN2:

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

As to the other approach indicated by RAN1, NW-side training entity provides the dataset/parameter to the server for data collection for UE-side model training/OTT server directly. From our understanding, this approach is part of the data transfer path of Option 2/3 of UE side data collection, i.e., from CN/OAM to the server. Thus the conclusion of feasibility analysis for UE-side data collection solutions 2/3 can be reused.

- For the other approaches of dataset/parameter sharing, NW-side training entity provides the dataset/parameter to the server for data collection for UE-side model training/OTT server directly, which can reuse the conclusion of feasibility analysis for UE-side data collection solutions 2/3.

- Conclusion

This contribution concludes with the following:

Analysis of over-the-air Approach

- Based on RAN1 input about the dataset/parameter size, RAN2 to down-select the model transfer solutions in Rel-18 SI to enable dataset/parameter sharing.

- For the standardized signaling of dataset/parameter sharing, both CP-based and UP-based solutions can be considered for further study.

- Solutions 1a (gNB to UE via RRC) and 2a (CN to UE via NAS) are candidates as CP-based solutions;

- Solutions 2b (CN to UE) and 4b (OAM to UE) are candidates as UP-based solutions.

- RAN2 to study how to address the size issue for CP-based dataset/parameter sharing, the following two solutions can be considered:

- Solution 1: The network splits the dataset/parameter into multiple subsets and shares them with different UEs.

- Solution 2: The network performs dataset/parameter segmentation before transferring them to one UE.

- RAN2 to study suitable segmentation methods to support CP-based dataset/parameter sharing.

- Do not consider increasing the RRC segment number to support CP-based dataset/parameter sharing.

- LPP-like segmentation can be considered, i.e., based on end marker, to support CP-based dataset/parameter sharing.

- LS to SA3 to study whether there is security issue of sharing data samples collected from one UE/vendor to other UEs/vendors and whether an additional user consent mechanism is needed.

- LS to SA2 and SA5 to study the feasibility of UP-based dataset/parameter sharing via the air interface.

Analysis of Other Approaches

- For the other approaches of dataset/parameter sharing, NW-side training entity provides the dataset/parameter to the server for data collection for UE-side model training/OTT server directly, which can reuse the conclusion of feasibility analysis for UE-side data collection solutions 2/3.

- Reference

- R1-2410922	LS on signalling feasibility of dataset and parameter sharing

- TR 38.843		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-243316	LS on AI/ML UE sided data collection