3GPP TSG-RAN WG2 Meeting#129	R2-2500129

Athens, Greece, 17th – 21st, Feb. 2025

Source:	vivo

Title:	Discussion on A-IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

RAN#106 meeting has agreed on the new work item of solution for Ambient IoT in NR [1], with the following aspects within RAN2 scope:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, with the guidance of the highlighted part, we investigate several issues to realize the functionality of A-IoT paging in terms of the content, format of the paging message, and the device behaviour upon receiving the paging message.

- Discussion

## Content

### Paging Identifier

WID has given the instruction that, for one paging message, there can be one or no paging identifier included. We go to the SA2 design on device identifier [3].

The permanent Ambient IoT Device Identifier includes the following information: 1) Part1information: - The ID type, including - information indicating whether the network identifier is included or not. - information indicating whether information used to identify a 3rd party is included or not. - the Part2 type indicating EPC or the other format. - A network identifier (i.e. MCC+MNC and/or NID), when the ID includes the network identifier. - Information used to identify a 3rd party when the ID includes the information used to identify a 3rd party. 2) Part2 information: - The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information. Editor's note: Part2 information for the operator assigned and 3rd party assigned ID needs further study. NOTE 3: The coding for the above information is left to stage 3. Figure 8.2.1-1: The example of Operator allocated ID For operator allocated Part 1 information, the network identifier is mandatory and can be used to index the authentication server or not. If it is not used to index the authentication server, the ID may further include the information used to identify a 3rd party. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server. Figure 8.2.1-2: The example third party allocated ID For third party allocated Part 1 information, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server. NOTE 4: Within each type of ID, the length of Part 1 is fixed (i.e. the length of ID type, the Network Identifier (if present) and the Information used to identify a third party (if present) are fixed) and the length of Part2 is dynamic. The detail is left to stage 3. With the above information, the AIoT device ID is globally unique. Editor's note: Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

Based on the above information, the length of device identifier defined for NR support of A-IoT can be fixed or dynamical when allocated by the operator or the third party.

In turn, when it comes to the design of paging identifier from MAC layer perspective, we only notice that the paging identifier is a kind of MAC SDU, which may have variable length. And we do not care about the detailed content inside this MAC SDU, which is to be passed to A-IoT layer to handle.

- The paging identifier is transparent to the A-IoT MAC Layer, in a form of MAC SDU with a variable size.

### Service type

The issue of whether service type is needed in paging message was raised and upon pendency. We see little benefit from inclusion of paging cause to indicate the service type in the A-IoT paging message. In NR, paging cause is introduced to assist a device to determine whether to respond to the paging message immediately or to ignore the paging message. For example, for the MUSIM UE, if a USIM is paged with a low priority cause value while the other USIM has a high priority service ongoing, the UE may ignore the paging triggered by low priority service, assuming that dual-Rx/Tx is not supported. On the contrary, due to the low capability of A-IoT device, paging is what it must respond to once it is paged. Besides, since paging message is used to address all types of the A-IoT service, to finish any service, A-IoT device should respond to paging message with no differentiation in the service type. Thus, there is no need for a device to be informed about the cause why it is being paged for.

- NR paging cause helps UE to decide on service priority and whether to respond; while A-IoT device needs to respond to A-IoT paging message regardless that the paging message is related to any A-IoT service type.

- No need to include an explicit paging cause in A-IoT paging message to indicate A-IoT service type at A-IoT MAC layer, i.e., device identifies the service directly at A-IoT layer.

### ID to avoid duplicate responses

It is agreed and captured in TR [2] that a certain kind of ID information to avoid duplicate responses from the device to a reader can be included in the A-IoT paging message. Moreover, the duplication is on the level of one reader to one device.

The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information.

Other than that, RAN3 is obliged to support device proximity functions, and has come to the following conclusions in SI phase [2].

WID - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity TR description 6.9 Locating ambient IoT devices 6.9.1 General A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

- RAN3 supports device locating with the following objectives:

- CN may send a Command to multiple readers to improve A-IoT operation;

- The device locating reporting to CN is at the granularity of reader ID.

However, RAN2 made no consensus on the multi-reader scenarios. In our understanding, to support the locating purpose, the multi-readers scenario should also be confirmed for signaling design over the air.

- RAN2 to support the scenario that one A-IoT device can be required to respond respectively to A-IoT paging messages from different readers associated with the same service.

On the whole, when the issue is extended to multiple readers to one device, we consider that how to avoid duplicate responses from the A-IoT device perspective needs further discussion. We therefore analyze these issues together.

- For a single reader

The issue of duplicate responses comes up in the realm of one device interacting with one reader. Since the reader may send multiple paging messages without changing the paged device to accomplish one A-IoT service considering the contention resolution, from the device perspective, if it has successfully access to the reader, it normally may not need to reply to the paging message triggered by the same service.

As shown in Figure 2.1-1, it is easily to come out that, for paging messages corresponding to a same service, the ID information in these paging messages should be the same. Whereas, the A-IoT reader is able to know the service information from CN, thus it is viable that the A-IoT reader either directly reuses information to identify the service from CN, or autonomously determines the ID information to carry in the paging message.

Figure 2.1-1: single reader scenario to avoid duplicate responses for a same service

- For multiple readers

However, if Proposal 3 is endorsed and the multi-reader scenario is considered, it becomes a bit complex than a single reader scenario.

First of all, the duplicate responses to the same service from different readers should also be avoided in such scenario in the normal case. As shown in Figure 2.1-2, if the same inventory service is allocated to different readers towards the same device, for which the device is expected to only respond to one reader and to report its device ID, the repeated responses to the same service from different readers should be avoided. Under this situation, it is not necessary for readers themselves to generate local information to identify whether the paging is triggered by the same or different service from another reader. For example, Reader 1 may generate ID=1 for Service 1; while Reader 2 may generate ID=3 for Service 1. From the device point of view, it is hard for it to tell if the paging message with ID=1 and the one with ID=3 is for the same or different service. And the normal behavior is to see it as for different services and to respond. In this understanding, it is only viable for reader to directly reuse a global ID to manage service from CN; or to employ a unified function to obtain information based on this global ID as an input, in case of device’s repeated responses for a same service.

Figure 2.1-2: multi-reader scenario to avoid repeated responses for a same service

As for the proximity service reversely, the device is expected to respond to such service from different readers repeatedly, in order to let more readers aware of the device existence around and to improve the location accuracy. Analyzed from the above, it is not controllable to rely on the reader to locally assign the ID. And for the option that reader employs the ID from CN in the paging message, the proximity service can be satisfied as well by smart CN implementation depicted in Figure 2.1-3. CN, i.e., AIOTF can allocate different global IDs for this same service; in turn, the device and readers barely see these IDs and handles as two individual services. It totally depends on CN to attach these service responses together by implementation.

Figure 2.1-3: multi-reader scenario to allow repeated responses for a same service

From Table 2.1-1, it can be seen that the ID carried in paging message should be a global ID unique per AIOTF used across readers to associate a certain A-IoT service, which is able to cope with issue to avoid duplicate responses and to support multiple reader scenario with different service requirements.

Table 2.1-1: Feasibility analysis on ID carried in paging message from CN or reader

|  | To avoid duplicated responses from one reader | To avoid repeated responses from multiple readers for a same service | To allow repeated responses from multiple readers for a same service |
| --- | --- | --- | --- |
| Global ID from CN (per AIOTF) | √ | √ | √ |
| Local ID per reader | √ | × | × |

- A global ID carried in A-IoT paging message is able to not only avoid duplicate responses from a single reader, but also support multi-reader scenario.

To be carried over the air, the service ID in the paging message can be processed from the ID uniquely identified an A-IoT service within AIOTF. Since this service ID should be short and simple, and to consider the good extendibility, 3-bit length can be feasible as a starting point.

- The information to avoid duplicated responses in paging message for the same service request can be a global service ID, with a possible length of 3-bit.

## Format

In regard to the paging message format, incorporated with the above analyses on the content of the paging message and compatible with the overall design of A-IoT MAC PDU, we came out with two possible designs.

- Candidate 1

A-IoT paging message is A-IoT MAC PDU with one MAC subPDU, whose header indicates the message type and the succeeding field combinations.

- The Header of paging message consist of the following field:

- The format field indicates the type of this message (i.e., the format of the MAC PDU) is for A-IoT paging.

- The field indication field indicates whether Paging identifier, Q, RA resource allocation parameters or Service ID are existed or not. It can be in a form of 4-bit bitmap.

Moreover, these two fields can be joint-coded, that is, each possible combination of field in paging message can be of a dedicated header

- The length indication field indicates the length of the corresponding paging identifier.

- The Paging Identifier field is in essence a MAC SDU to be delivered to A-IoT layer.

- The Q field is to indicate the number of access slots in one paging round. Details are up to RAN1 and a typical number of devices taking part in A-IoT paging.

- The RA resource allocation field is the dedicated or common resources for MSG1 transmission. Details are up to RAN1. If RA resource allocation needs to be with flexible length, there will be an extra LI field in front of such field.

- The SI field is the service ID.

Figure 2.2-1: A-IoT MAC PDU with one MAC subPDU for A-IoT paging message

- Candidate 2

Any message is an A-IoT MAC PDU with flexible combinations of one or multiple MAC subPDU(s), each corresponding to one or more functional fields. Figure 2.2-2 gives one possible format of MAC PDU to realize paging.

Figure 2.2-2: A-IoT MAC PDU with one or multiple MAC subPDU(s) to realize paging

Candidate 2 reuses the NR MAC PDU format as much as possible, with each part paging content is designed into a A-IoT MAC subPDU.

With no message type indication and the design with only one PRDCH physical channel, the device has to descramble the A-IoT MAC PDU completely to know the function of such A-IoT PDU. Besides, with the fixed length of some fields, i.e., Q, RA resource allocation if confirmed by RAN1, little information can be designed in the subheader, but a fixed waste of size is taken.

  - The MAC subheader for MAC SDU or MAC CE consists of the following fields:

- L: the length field indicates the length of the corresponding MAC SDU or MAC CE (in case that RA resource allocation is of variable length).

- T: The type field indicates the type of the corresponding MAC SDU/MAC CE, e.g., whether it is paging identifier, Q, service ID or RA resource allocation.

We slightly prefer the first one. Owing to the simplicity of device, it is straightforward to fix the message format rather than to allow some flexibility of one message. This can further reduce the device complexity, processing delay and signaling overhead for transmission.

- The A-IoT MAC PDU of paging message comprises only one MAC subPDU, whose header indicates the message type and the succeeding field combinations.

## Device behavior upon receiving paging messages

In consideration to information to avoid duplicate response in a paging message, it is only agreed that device can determine whether to skip sending the response to the paging message. We would like to further talk about the detailed criterion on how the device makes the determination, which should be captured as device behavior.

First and foremost, the device should firstly ascertain that it is the one to be paged by the paging message. And this assertion is made by A-IoT layer, by comparing the paging identifier and the device identifier. On the other side, if no upper layer container (i.e., paging identifier), the MAC layer itself of device is aware that it is to be paged and execute the following checking procedure.

Then, the A-IoT layer should inform the A-IoT MAC layer that the device is to be paged. And the device’s MAC layer can evaluate whether to respond the current paging message. Based on the progress discussed in Random access, the re-access can be triggered for contention resolution failure as a baseline scheme, and an optional success/failure indication about MSG3/the following D2R data transmission. For CFRA case, the success/failure indication is also applied for a failed MSG2 transmission to let device aware of the re-access necessity. In this understanding, from the device perspective, the respond to the current paging is determined by 1) whether this is the first paging message; 2) if this is the subsequent paging message (i.e., the device has received another paging message before corresponding to the same service as the current one), whether the random access is successful for that received paging message. If one of the conditions are not satisfied, the device’s MAC layer should respond to the current paging message.

In sum, entering WI phase, the device behavior upon receiving the A-IoT paging message should be considered and specified.

- Upon receiving an A-IoT paging message, the MAC layer of the A-IoT device should:

- Forward the paging identifier to the A-IoT layer, if any;

- If no paging identifier is contained, confirmed itself to be paged;

- After confirmed by the A-IoT layer or the MAC layer itself that the device is paged:

  - Check if this is the first paging message of the service ID; if yes, respond to the current paging message;

  - If this is the subsequent paging message, check if the access procedure has been successfully completed for the former paging message of the same service ID:

    - If the access procedure is not successful for the former paging message of the same service ID, respond to the current paging message.

- Conclusion

On a basis of above analysis, we make the following proposals regarding A-IoT paging:

Content

Paging identifier

- The paging identifier is transparent to the A-IoT MAC Layer, in a form of MAC SDU with a variable size.

Paging Cause (service type)

- NR paging cause helps UE to decide on service priority and whether to respond; while A-IoT device needs to respond to A-IoT paging message regardless that the paging message is related to any A-IoT service type.

- No need to include an explicit paging cause in A-IoT paging message to indicate A-IoT service type at A-IoT MAC layer, i.e., device identifies the service directly at A-IoT layer.

ID to avoid duplicate responses and to support multi-reader scenario

- RAN3 supports device locating with the following objectives:

- CN may send a Command to multiple readers to improve A-IoT operation;

- The device locating reporting to CN is at the granularity of reader ID.

- RAN2 to support the scenario that one A-IoT device can be required to respond respectively to A-IoT paging messages from different readers associated with the same service.

- A global ID carried in A-IoT paging message is able to not only avoid duplicate responses from a single reader, but also support multi-reader scenario.

- The information to avoid duplicated responses in paging message for the same service request can be a global service ID, with a possible length of 3-bit.

Format

- The A-IoT MAC PDU of paging message comprises only one MAC subPDU, whose header indicates the message type and the succeeding field combinations.

Device behavior upon receiving paging message

- Upon receiving an A-IoT paging message, the MAC layer of the A-IoT device should:

- Forward the paging identifier to the A-IoT layer, if any;

- If no paging identifier is contained, confirmed itself to be paged;

- After confirmed by the A-IoT layer or the MAC layer itself that the device is paged:

- Check if this is the first paging message of the service ID; if yes, respond to the current paging message;

- If this is the subsequent paging message, check if the access procedure has been successfully completed for the former paging message of the same service ID:

    - If the access procedure is not successful for the former paging message of the same service ID, respond to the current paging message.

- Reference

- RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 38.769		Study on solutions for ambient IoT (Internet of Things)

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things