3GPP TSG-RAN WG2 Meeting #129      	       	R2-2500130

Athens, Greece, Feb. 17th – 21st, 2025

Source:	vivo

Title:	Random Access Procedure for A-IoT Device

Agenda Item:	8.2.3

Document for:	Discussion and Decision

# Introduction

In the SI stage, random access procedure of A-IoT device was discussed and some conclusions and recommendations were achieved as follows [1]:

# 8 Conclusions and recommendations From RAN2 perspective, RAN2 has studied Topology 1 and Topology 2, and has concluded they are feasible. In details: - General AS procedures, A-IoT paging, A-IoT random access, A-IoT data transmission and Topology 2 aspects have been studied: - ***************Omit other items******************************. - See sub-clause 6.3.4) for the A-IoT random access, including the contention-free access and contention-based random access procedure, the message contents, the re-access solutions, etc. - From RAN2 perspective, the descriptions under clause 6.3 and 6.10 are considered as the baseline conclusions. - Following is further concluded: - At least following features are recommended for the normative phase in the A-IoT MAC layer: - A-IoT paging - A-IoT random access: - For contention-based random access, further down-selection between Solution 1 (3step-only) and Solution 3 (unified solution) in sub-clause 6.3.4. - A-IoT data transmission - ***************Omit other content******************************

In this contribution, we continue to discuss A-IoT random access procedure, which includes down selection for 3-step CBRA only, differentiation between CBRA and CFRA and further details about RACH procedure.

# Discussion

## 2.1 Down selection

In the WID of A-IoT, there was a chance for unified RACH Solution 3 to be supported as followings:

- A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

In our understanding, 2-step CBRA is an optimization that can reduce RACH latency, e.g., in case of a low probability of A-IoT Msg1 collision. Although the reader can choose one suitable RACH type based on paging information and its implementation, supporting both 3-step and 2-step RACH will increase a little complexity from the perspective of device and spec effort. In order to complete the WID smoothly and timely, we can accept that only the 3-step CBRA is supported in Rel-19.

However, if 2-step CBRA would be supported in a later release, e.g., Rel-20, we prefer not to introduce a capability reporting procedure for that. It may be better for CN/reader to judge the device version by deployment or pre-configuration.

- Device capability differentiation may be needed, e.g., by deployment or pre-configuration, if 2-step CBRA is only supported in later releases.

- In Rel-19, only 3-step CBRA is supported.

## 2.2 Differentiation between CBRA and CFRA

According to the A-IoT WID and RAN2 agreements [2-3], paging message may include one of the following three options: a device ID, a group ID or none of ID.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

However, paging message should be considered as upper layer information from the perspective of MAC layer, e.g., unrecognizable or securely protected. The MAC layer of reader side can differentiate the above three scenarios via explicit interface signaling since the MAC layer needs to schedule CBRA or CFRA procedure accordingly. In CBRA case, Msg0 will indicate multiple access occasions, e.g., 2Q where Q>0. In CFRA case, i.e., only one device ID contained in paging message, Msg0 needs to indicate single access occasion, e.g., 2Q and Q=0. The number of access occasion, e.g., the value of Q, may implicitly indicate whether this procedure is CFRA or CBRA.

However, it is a question whether we need to strongly limit single access occasion equals CFRA. For example, a group containing only several devices is paged and the distribution of devices are wide. In the coverage of one reader, the number of devices in that group may be zero or one with a very high probability and more than one device with a very low probability. In this case, a Msg0 with only one access occasion and for CBRA may be possible. Or in later releases, multiple access occasions will be needed for CFRA case, e.g. 3 devices 1-to-1 orderly mapping to 3 access occasions. Considering these special cases and leaving enough freedom for implementation, an explicit single bit in Msg0 to differentiate CFRA and CBRA is useful.

In our understanding, both only one access occasion and explicit single bit in Msg0 can differentiate CBRA and CFRA in Rel-19. An explicit single bit in Msg0 can provide more freedom of reader implementation and backward compatibility with a little bit overhead. Which one is chosen can be decided in MAC PDU format design for Msg0 bit-by-bit.

- Using a MAC indicator in Msg0, e.g. only one access occasion or explicit single bit, to differentiate CBRA and CFRA.

## 2.3 3-step CBRA

In the following figures, we give our understanding of 3-step CBRA procedures and some failure cases.

- Basic CBRA with 3-step RA type

| - Failure case detected by device | - Failure case detected by device |
| --- | --- |
| - Failure case indicated by reader | - Failure case indicated by reader |

In 3-step CBRA, there are two types of failure: contention resolution failure and transmission failure. Failure of Msg1 and Msg2 are contention resolution failure. These two steps are by default considered failure if the expected ACK confirmation is not received timely. For example, after a device transmits its Msg1 with RN16, it may receive a common NACK of Msg1 if reader cannot decode Msg1 successfully due to poor link quality or collision or nothing detection. From device perspective, both common NACK of Msg1 and not receiving its RN16 within a time window or a predetermined time mean contention resolution failure, which failures can be detected by device itself.

Of course, after determination of contention resolution failure, the device should maintain continuous R2D monitoring since it is possible for reader to re-try Msg2 transmission again immediately or re-trigger a new paging round at any time.

- Failure of contention resolution is determined in the device side when:

- Receiving common NACK of Msg1, or;

- The device does not receive its RN16 same as Msg1 within a time window or a predetermined time.

After the device receives Msg2 successfully with confirmation of its own RN16, which means contention resolution is successful, all subsequent D2R transmissions will be considered successful by default except for explicit indication by the reader. If the reader cannot receive Msg3, Msg5, or any other D2R messages, it may trigger an explicit NACK with device specific RN16 to indicate failure. This new MAC NACK will trigger device re-access. Or the reader can transmit the R2D message again to trigger the corresponding D2R message re-attempt, which is transparent to the device, and it can be considered as new message transmission and left to reader implementation.

- Introduce a MAC NACK with device’s RN16 for D2R transmission failure indication and then device triggers re-access.

There may be a recovery procedure when Msg2 reception or Msg3 reception fails. When the reader cannot detect the expected Msg3 successfully in the Msg3 resource location, the reader can consider either Msg2 or Msg3 reception failure. Besides for NACK indication and re-access triggering, the reader can re-try Msg2 transmission to recover Msg2 or Msg3 reception failure. The corresponding device behaviours may include when second reception of its Msg2, the device transmits Msg3 again. The content of new Msg2 may be re-organized, e.g. only including information of failure device.

- The reader can re-try a new Msg2 transmission to recover from Msg2 or Msg3 reception failure.

If Msg1 or Msg3 or other D2R failure in 3-step CBRA is indicated by reader, re-access will be triggered by device. Re-access immediately may still fail with some probability when device is in link deterioration period. Furthermore, re-access immediately may cause additional complexity and increased delay for other devices. The overall access procedure design will also be more complex. Hence, we prefer to trigger re-access in the next access/paging round, whose Msg0 contains at least new RACH parameters, e.g. the number of access occasions, and optionally the whole paging message or paging ID.

- Device will re-access in the next access/paging round indicated by reader via Msg0 containing at least new RACH parameters.

## 2.4 CFRA

In the following figures, we give our understanding of basic CFRA procedure and some special cases.

| - Basic CFRA with inventory only | - CFRA with subsequent command |
| --- | --- |

In basic CFRA procedure, failure indication of Msg1 is not needed since a new CFRA procedure is more preferable when the previous Msg1 is not successfully received by reader.  Failure indication of CFRA Msg1 has ambiguity of function and unclear for subsequent device behaviors. A new CFRA trigger is more useful for re-attempt and potentially with updated resource or parameters allocation, which increase the probability of success in the second retry.

- Failure indication for CFRA Msg1 is not supported since the device may receive a new CFRA Msg0 triggering when the previous Msg1 is not received successfully by reader.

The second issue of CFRA is AS ID allocation. Msg0 of RACH is a broadcast mode and not suitable to carry unicast subsequent upper layer command. If there are subsequent commands for CFRA case, an AS ID allocation and specific scheduling is more efficient from this device and whole system perspective. As previous discussions in RAN2, there are 3 options for AS ID allocation in CFRA case:

- Option 1: AS ID carried in Msg0/paging;

- Option 2: Carried in Msg1 always or according to paging type;

- Option 3: Msg2 allocates AS ID by reader, if needed;

All these 3 options are feasible. However, from the perspective of overhead and efficiency, option 1 will waste and occupy resource & AS ID in other non-serving readers and option 2 may waste Msg1 resource if not needed or cannot be accurately indicated in Msg0/paging. Option 3 is more preferable since AS ID allocation only occurs in necessary cases and directly done by reader, i.e. with optimal latency and avoiding collision processing.

- AS ID can be directly allocated by reader after CFRA Msg1, if needed.

## 2.5 Msg2 Multiplexing

RAN1 had discussed Msg1 TDMA and FDMA scenarios. If Msg1 transmission uses TDMA and/or FDMA scheme, it should be further studied how to deliver Msg2 efficiently.

Agreement Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

RAN1 [4] had considered different solutions for Msg2 transmission as followings:

In Alt-1, Msg2 transmission is respectively per device. Msg2 and Msg3 of device 1 are transmitted first, and then Msg2 and Msg3 of device 2 are transmitted and so on. The benefit from FDMA scheme of Msg3 cannot be achieved.

In Alt-2, Msg2 transmission is also per device. The difference with Alt-1 is all Msg2 of success devices are continuously transmitted before Msg3 transmission. Then all Msg3 of success devices can use TDMA or FDMA scheme. The transmission efficiency of Msg3 will be increased greatly. However, the distance between each pair of Msg2 and Msg3 varies, and the resource indication may be complicated.

In Alt-3, Msg2 multiplexing mechanism is introduced to replace separate Msg2 transmission in Alt-2. Alt-3 seems to have more streamline processes and greater efficiency than other alternatives, e.g. beneficial for access latency reduction and efficiency improvement.

- Msg2 multiplexing seems to be beneficial for access latency reduction and resource efficiency improvement in the cases of Msg1 TDMA and/or FDMA.

Of course, the detailed solution should be evaluated and concluded in RAN1 first.  However, format and multiplexing of Msg2 are in RAN2 scope. RAN2 can concurrently evaluate the feasibility of Msg2 multiplexing and with RAN1 together perform the final selection of Msg2 transmission mechanism.

- RAN2 to evaluate the feasibility of Msg2 multiplexing.

# Conclusion

In this contribution, we discuss further details about 3-step CBRA, CFRA and Msg2 multiplexing and have the following observations:

- Device capability differentiation may be needed, e.g., by deployment or pre-configuration, if 2-step CBRA is only supported in later releases.

- Msg2 multiplexing seems to be beneficial for access latency reduction and resource efficiency improvement in the cases of Msg1 TDMA and/or FDMA.

Based the above discussions and observations, we have the following proposals:

Down selection

- In Rel-19, only 3-step CBRA is supported.

Differentiation between CBRA and CFRA

- Using a MAC indicator in Msg0, e.g. only one access occasion or explicit single bit, to differentiate CBRA and CFRA.

3-step CBRA

- Failure of contention resolution is determined in the device side when:

- Receiving common NACK of Msg1, or;

- The device does not receive its RN16 same as Msg1 within a time window or a predetermined time.

- Introduce a MAC NACK with device’s RN16 for D2R transmission failure indication and then device triggers re-access.

- The reader can re-try a new Msg2 transmission to recover from Msg2 or Msg3 reception failure.

- Device will re-access in the next access/paging round indicated by reader via Msg0 containing at least new RACH parameters.

CFRA

- Failure indication for CFRA Msg1 is not supported since the device may receive a new CFRA Msg0 triggering when the previous Msg1 is not received successfully by reader.

- AS ID can be directly allocated by reader after CFRA Msg1, if needed.

Msg2 Multiplexing

- RAN2 to evaluate the feasibility of Msg2 multiplexing.

- Reference

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, V19.0.0, 2024-12;

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR;

- RAN2#128, Chairman report;

- RAN1#119, Chairman report;