3GPP TSG-RAN WG2 Meeting #129	R2-2500132

Athens, Greece, Feb 17th - Feb 21st, 2025

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Leftover issues on Inter-CU LTM

Document for:	Discussion, Decision

Introduction

This contribution further discusses the feasibility of LTM fast recovery and some other leftover issues for inter-CU SA LTM.

Discussion

LTM fast recovery

In the last meeting, the security for standalone inter-CU LTM was discussed, and RAN2 sent an LS to SA3 for further discussion on solutions other than RRC configuration between LTM cell switches.

Regarding fast recovery, RAN2 decided to postpone the discussion until the security solution is finalized. The agreements can be summarized as follows:

Reply to SA3 that RAN2 think avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection. RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

The main reason RAN2 postponed the inter-CU fast recovery discussion is that it is highly related to the security solutions. According to the LS sent from RAN2 to SA3, there are at least two alternative solutions:

- Pre-configure the security key (by related information, e.g., NCC value) for each candidate cell in the LTM-config and provide it to the UE during the LTM preparation stage.

- Indicate the NCC value in the MAC CE when each LTM cell switch is performed.

We understand that if the MAC-CE solution is finally adopted, it will be nearly impossible to support inter-CU LTM fast recovery, as the UE cannot receive MAC-CE when an LTM failure happens. Consequently, the UE will have no opportunity to obtain new security keys of any other candidate cells.

Observation 1: The LTM fast recovery solution for inter-CU LTM is related to the solution of security update. If MAC-CE solution is finally adopted, the UE cannot obtain the security key of other candidate cells when an LTM failure occurs.

The support of fast recovery is beneficial to the interruption reduction when the failure occurs. We see one aspect that the LTM candidate list also includes intra-CU candidate cells, as shown in Figure 1. For these intra-CU candidate cells, there is no security update issue for fast recovery, and the legacy rules of intra-CU LTM fast recovery still apply.

Figure 1 The CUs of candidate cells of inter-CU LTM configuration

The only thing is that the UE needs to know which cell belongs to which CU. In the last meeting, RAN2 agreed to introduce a Rel-19 No-reset-ID in RRC to indicate whether to perform PDCP re-establishment. If the ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including a security key update. Otherwise, it implies that PDCP re-establishment and security update are not performed.

Observation 2: In inter-CU LTM configuration, for the candidate cells within the same CU as the source cell, PDCP re-establishment and security update are not performed. The Rel-19 No-reset-ID can be used to identify these candidate cells.

Therefore, fast recovery can be supported for those candidate cells within the same CU as the source cell. Although the concept of "the same CU" may not be equal to "No PDCP re-establishment and security update" (the network may indicate PDCP re-establishment and security update for cell change even within the same CU), it is still workable to use the Rel-19 No-reset-ID to determine the candidate cells that do not require PDCP re-establishment and security update, so that the LTM fast recovery can be supported.

Proposal 1: For inter-CU LTM scenario, the LTM fast recovery is supported for those candidate cells within the same Rel-19 No-reset-ID as the source cell.

The support of RACH-based LTM

In the RAN2 #126 meeting, both CG-based RACH-less and DG-based RACH-less procedures were agreed.

Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM

However, one point that may have been missed is the support of RACH-based LTM in inter-CU scenario. This is also raised by other companies but was not discussed in the last meeting.

From RAN2 perspective, we see no issue to support RACH-based LTM in inter-CU scenario, as it does not bring extra complexity or strict latency requirement for inter-node coordination. This includes both CBRA and CFRA cases.

Therefore, we propose:

Proposal 2:	RACH-based inter-CU LTM is supported.

Triggering order in LTM execution

Regarding the order of triggering cell switch command and interaction between the source gNB and target gNB, In Rel-18 LTM, RAN3 clarified that the serving DU may trigger the cell switch command MAC CE first, or inform the target DU before sending the cell switch command to the UE. In the reply, RAN2 agreed that the former approach (serving DU triggering LTM command first and then informing the target DU) should be the baseline to reduce potential latency for LTM execution.

Observation 3: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

In an intra-CU scenario, the typical latency for the F1 interface is around 10ms for round trip time. This results in approximately 10ms from the source DU informing the target DU in the same CU. The UE requires more than 10ms to perform TLTM_RRC-processing and TLTM_processing. This time frame allows the network to prepare well before the UE attaches to the target cell.

However, in an inter-CU scenario, it is worth discussing the latency for the source DU to inform the target DU for the initiation of the LTM cell switch. Typically, the latency over F1 interface is 5ms for one shot, and the latency over Xn interface is 10ms. In the last meeting, RAN2 agreed that it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. So the signalling is send from Source DU->Source CU->Taget CU->Target DU, totally around 20ms.

For UE, the time for Uu interface and MAC-CE decoding is smaller than 4ms. The latency of UE to decode ASN.1 TLTM_RRC-processing can be saved (reduced to zero) by UE’s capability ltm-FastProcessingConfig-r18 (FG 39-6). If UE does not support this capability, the latency is 10ms. The latency for UE to perform LTM processing (TLTM_processing) is also depends on UE’s capability ltm-FastUE-Processing-r18 (FG 39-7), which can be 10~20ms for intra frequency and 20~40ms for inter-frequency cell switch.

Therefore, If the UE supports FG 39-6 and FG 39-7, the UE's total processing time (4+0+10ms) may be shorter than the latency from source DU to target DU (5+10+5ms), potentially leading to the target cell not being well-prepared when the UE finishes the LTM processing. This could lead to more interruptions (rather than latency) during LTM.
Observation 4: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target DU may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

If we take RACH-less LTM as example:

Figure 1 Potential impact in Intra/inter CU network latency for LTM execution

In such cases, the network may need to inform the target DU earlier than sending LTM CSC MAC-CE to UE to align with the UE's processing time, as latency is anyway better than interruption. This behaviour can be dynamically adjusted according to UE’s capability and implemented by network.

Proposal 3: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

Conclusion

In this contribution, the following observations and proposals are made:

LTM fast recovery

Observation 1: The LTM fast recovery solution for inter-CU LTM is related to the solution of security update. If MAC-CE solution is finally adopted, the UE cannot obtain the security key of other candidate cells when an LTM failure occurs.

Observation 2: In inter-CU LTM configuration, for the candidate cells within the same CU as the source cell, PDCP re-establishment and security update are not performed. The Rel-19 No-reset-ID can be used to identify these candidate cells.

Proposal 1: For inter-CU LTM scenario, the LTM fast recovery is supported for those candidate cells within the same Rel-19 No-reset-ID as the source cell.

The support of RACH-based LTM

Proposal 2:	RACH-based inter-CU LTM is supported.

Triggering order in LTM execution

Observation 3: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

Observation 4: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target cell may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

Proposal 3: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

References

- R2-2409534      Reply LS on security handling for inter-CU LTM in non-DC cases (S3-244316; contact: Samsung)

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements