3GPP TSG-RAN WG2 Meeting #129	R2-2500134

Athens, Greece, Feb. 17th – 21st , 2025

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Further discussion on Conditional LTM

Document for:	Discussion, Decision

Introduction

In the last meeting, several agreements were made on triggering event evaluation, TA acquisition, measurement reporting, and candidate configuration for conditional LTM. However, the detailed procedures for implementing these functions still require further discussion.

This paper further discussed the details of these procedures. Besides, this paper also discusses other topics such as TCI state acquisition (beam selection), UL grant and CFRA acquisition, and the scope of conditional LTM.

Discussion

Triggering event evaluation

In the last two meetings, the agreements on triggering event evaluation can be summarized as follow:

Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. The L1 execution condition of a candidate cell is associated to only one triggering event. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

There remain discussions on the timing for these triggering event evaluations and the beams used for evaluation.

The time to start evaluating triggering condition

If we temporarily focus on UE-initiated LTM execution, based on company contributions, the timing for the UE to start the event evaluation can be summarized as follows options:

- Option1: UE start to evaluate triggering condition after LTM configuration.

- Option2: UE start to evaluate triggering condition after early TCI state activation/deactivation.

- Option3: UE start to evaluate triggering condition after early UL synchronization.

Since we already support both RACH-based and RACH-less conditional LTM, it means UL synchronization can be performed before or after the LTM execution condition is triggered. The event evaluation should start at least before early UL synchronization to also include RACH-based conditional LTM.

Between Option 1 and Option 2, Option 1 is preferred as it also covers CHO behaviour. The UE starts the evaluation right after receiving the candidate configuration and reverts back to CHO if no early TCI state activation/deactivation MAC CE is received before LTM cell switch condition is triggered. With option1, there is no need to configure separate CHO and CLTM configuration. This avoids the discussion on co-existence with CHO and reduce extra configuration/signalling overhead.

Proposal 1: The UE starts to evaluate triggering condition after receiving LTM configuration.

The time to stop evaluating triggering condition

To make subsequent conditional LTM work, UE shall continue to evaluate the triggering condition for the next LTM cell switch after one LTM cell switch is performed. If network want to stop UE from evaluating for the triggering condition, extra signalling should be sent to UE, like conditionals handover. The signalling may be used to remove part of conditional LTM configuration/condition (to ban part of candidate cells) or remove all of them, which based on network implementation.

Proposal 2: UE continue to evaluate the triggering condition for the next conditional LTM cell switch after one LTM cell switch is performed.

Proposal 3: Introduce conditional LTM removal signalling to remove part of/all CLTM configuration and conditions (similar to CHO). UE stop evaluating the conditions for one candidate cell if it is removed by network.

The clarification on execution condition per candidate cell

In the meeting, RAN2 has already agreed that each candidate cell can provide its own execution condition.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

However, it is unclear whether this means:

- Understanding 1: Each candidate cell has its own condition as target cell, regardless of which serving cell the UE is in.

- Understanding 2: The condition applies when one candidate cell is the current serving cell, and the same condition is used for all target cells.

- Understanding 3: The conditions apply when one candidate cell is the current serving cell, with different conditions for different candidate cells.

For example:

- Understanding 1

  - Condition A: Any cell switch to candidate cell A

  - Condition B: Any cell switch to candidate cell B

  - Condition C: Any cell switch to candidate cell C

  - …

- Understanding 2

- Condition A: Candidate cell A (as serving cell) switch to any target cell

- Condition B: Candidate cell A switch to any target cell

- Condition C: Candidate cell C switch to any target cell

- …

- Understanding 3

- Condition A1: Candidate cell A switch to candidate cell B

- Condition A2: Candidate cell A switch to candidate cell C

- …

- Condition B1: Candidate cell B switch to candidate cell A

- Condition B2: Candidate cell B switch to candidate cell C

- …

- Condition C1: Candidate cell C switch to candidate cell A

- Condition C2: Candidate cell C switch to candidate cell B

- …

Understanding 3 offers the most flexible configuration, as it can configure different conditions for different source-to-target cell pair, but it comes with more signaling overhead as a two-dimensional matrix. I.e., The size of the condition configurations increase double quickly if the candidate list increases. RAN2 should discuss these three understandings and clarify one of them, considering the trade-off between signaling overhead and configuration flexibility.

Proposal 4: RAN2 to clarify the own execution condition for each candidate cell among three understanding:

- Understanding 1: Each candidate cell has its own condition as target cell, regardless of which serving cell the UE is in.

- Understanding 2: The condition applies when one candidate cell is the current serving cell, and the same condition is used for all target cells.

- Understanding 3: The conditions apply when one candidate cell is the current serving cell, with different conditions for different candidate cells.

TA acquisition and maintenance

In the last meeting, the new TA MAC CE and related agreements were made:

The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. Candidate cell TA is maintained by a new timer.

However, the detail behavior for this new MAC CE and the maintenance of candidate TA requires further discussion and clarification.

Baseline behavior

If there is no valid TA when the LTM cell switch is triggered, it is natural for the UE to initiate RACH-based conditional LTM and perform random access toward the target cell. On the other hand, if the candidate cell TA is valid when the LTM cell switch is triggered, the UE should skip the RACH procedure during conditional LTM execution.

For the TA timer, since the conditional LTM is triggered by the UE, there is no TA indicated by the network as there is no "cell switch command." If the UE has a valid TA, it should be maintained, and the TA timer should not be refreshed. If the UE does not have a valid TA and performs random access during the cell switch, the TA value is newly obtained by the UE, and the TA timer should be refreshed.

Proposal 5: If the early TA for a candidate cell is valid when the conditional LTM cell switch is triggered toward that cell, UE skips the RACH procedure during LTM execution, and the TA timer is not restarted. If the TA is not valid, UE performs random access procedure toward that cell during CLTM execution, and the TA timer is restarted.

TA maintenance and refresh
As early UL synchronization is critical for latency reduction during LTM, we may further consider enhancement to refresh the candidate TA if the TA timer expires. A typical approach is to send another TA MAC CE to update the TA value. However, the TA value is used for the synchronization of uplink transmission to adjust the timing from UE so that the signalling arrives at the gNB at the correct instance. The expiration of the TA may indicate that the UE's position has changed and the network is unaware. Therefore, another preamble may be needed to inform the network about the change.

Since both the UE and the network are synchronized with the TA timer, they should be aware of the TA expiration and can feasibly trigger the update procedure. For example, the network may send a PDCCH order to trigger the UE to send a preamble, or the UE may send a preamble on its own.

Proposal 6: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch is triggered, the network can trigger TA refresh by sending another PDCCH order.

TCI state acquisition (Target beam selection)

TCI state is used to indicate the antenna QCL relationship of the target cell.

- In legacy handover, the UE determine the default antenna QCL assumption by RACH procedure, then receives a finer TCI state activation indication after the cell switch.

- In Rel-18 LTM, the TCI states are pre-configured to UE and can be activated before LTM cell switch (i.e., early TCI state activation). UE tracks the QCL RS of this candidate TCI state until receiving the LTM cell switch command MAC CE. If the same TCI state is indicated in LTM CSC, the interruption can be reduced.

In the last two meeting, the following agreements are made to reuse Rel-18 Early candidate TCI State activation/deactivation:

Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

The Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states that the UE may use for LTM cell switching. The UE tracks the indicated candidate TCI states until it receives the LTM cell switch command MAC CE, which specifies the precise TCI state ID for the UE.

Observation 1: In Rel-18 LTM, the Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states, while the LTM cell switch command MAC CE specifies the precise TCI state for target cell.

However, in the case of conditional LTM, UE initiate the LTM cell switch and the LTM cell switch command MAC CE is not provided. It is unclear how the UE acquires the precise TCI state/select target beam for LTM cell switch.

Observation 2: For conditional LTM, it is not clear how does UE acquire the precise TCI state/select target beam for LTM cell switch by the absence of LTM cell switch command MAC CE.

Case analysis and potential solutions

RACH-less conditional LTM

If the conditional LTM is RACH-less, there is no RACH during LTM execution. It remains unclear how the UE acquires the TCI state for the target cell and indicates it to the network. This lack of TCI state alignment may cause the network to be unaware of the UE's selected beam and unable to receive the UL message correctly.

There may be different solutions for this issue:

- The TA MAC CE is introduced to carry the TA value of the candidate cell. It would be beneficial if the selected TCI state is also indicated with the TA value, so the UE can use the precise TCI state when the conditional LTM cell switch is triggered. However, the downside of this method is that it cannot support the case when UE-based TA measurement is performed.

- Introduce a separate TCI MAC CE before the LTM cell switch is triggered, which is used to indicate the precise beam (similar to LTM CSC MAC CE) for the UE during LTM execution. This TCI MAC CE is decoupled from the TA MAC CE, which applies to both PDCCH order TA acquisition and UE-based TA measurement. However, it may introduce too much signalling interactions and resulting in two separate TCI state activation MAC CEs before the conditional LTM cell switch is triggered.

- Another alternative way is to consider using the early TCI state activation/deactivation MAC CE. The UE chooses one of the TCI states provided in the early TCI state activation MAC CE, and the network should reserve (monitor) all TCI states provided to the UE. That means network sends RSs for each TCI in downlink and also monitor the uplink resources for the 1st UL transmission. The network can adjust the TCI states provided in the early TCI state MAC CE according to its resource utilization. This reduces extra workload and avoids multiple TCI indications to prevents redundant signalling interactions.

Three different solutions can be summarized as follow table:

Table 1. Different solutions to select/determine target beam

|  | Description | Cons |
| --- | --- | --- |
| Solution 1 | For PDCCH ordered early TA, include the TCI state for LTM cell switch in the TA MAC CE. | This solution does not cover UE-based TA measurement case. |
| Solution 2 | Introduce a separate TCI MAC CE (In addition to early TCI state activation/deactivation MAC CE) before the conditional LTM cell switch is triggered. | Two similar early TCI state indication mechanisms may be overcomplicated. |
| Solution 3 | The UE selects one of the TCI states which activated in early TCI state activation/deactivation MAC CE and network reserve all resources until LTM cell switch complete or the resources are deactivated by the network. | It may slightly increase the resource pre-allocation for the network, but the amount of indicated TCI states can be adjusted by network implementation. |

Based on the analysis of these three solutions, solutions 1 and 2 are either not applicable for every scenario, or too complicated in signalling design. we prefer solution 3 to have minimum spec impact and avoid overcomplicated design for TCI state alignment.

Proposal 7: For RACH-less conditional LTM, the UE selects one of the activated TCI states for LTM cell switch according to the early TCI state activation/deactivation MAC CE. The network sends RSs for each indicated TCI in downlink and monitor the uplink resources until LTM cell switch complete or the resources are deactivated by the network.

RACH-based conditional LTM

In RACH-based conditional LTM, the UE performs a random access procedure during LTM execution. The UE can determine the initial beam and indicate it to the network via the RACH preamble. This allows the network to know the UE-selected beam used for the first uplink message transmission.

However, as in legacy handover, the initial beam determined by the random access procedure is typically a default/coarse beam, and UE may receive another TCI state activation indication after the LTM cell switch. For LTM, this may result in the loss of the benefit of fine beam reporting by L1 MR and an increase in throughput degradation or even interruption, particularly if CSI-RS-based measurement is used in conditional LTM.

Observation 3: For RACH-based conditional LTM, if the UE selects the initial SSB via the preamble, the UE may receive another TCI state activation indication after the LTM cell switch. The benefit of LTM fine beam reporting by L1 MR may be lost, leading to increased throughput degradation or interruption.

Therefore, even for RACH-based conditional LTM, the indication of the TCI state may still be helpful for the UE and network to maintain high throughput during the LTM cell switch. If the TCI state solution for RACH-less conditional LTM is agreed upon, it can also apply to RACH-based LTM.

Proposal 8: If proposal 7 is agreed, the same rule can also be applied to RACH-based conditional LTM.

Beam selection by the UE

Regardless of whether it is RACH-less or RACH-based LTM, it remains unclear which target beam is selected by the UE. From the triggering event perspective, it is natural to assume that the selected beam is the one that fulfills the triggering condition. However, this may vary and become complicated in different scenarios.

- L1 vs L3 measurement: The beam that fulfills the triggering condition is only satisfied if L1 measurement is configured. If the triggering condition is defined by L3 measurement results, it may be difficult to determine the target beam due to beam consolidation and L3 filtering based on the measurement results.

- The scope of early TCI state list: If the proposal 7 and 8 are agreed upon, the UE selects one of the TCI states in the early TCI activation MAC CE. The situation may differ depending on whether the beam that fulfilled the triggering condition is included in the early TCI states list or not.

  - If the UE measured beam that fulfilled the triggering condition is included in the early TCI state list indicated by the network, the UE may directly select that beam as the target beam.

  - If the UE measured beam that fulfilled the triggering condition is not included in the early TCI state list, the UE may select the beam within the early TCI state list with the best quality.

- Multiple Beams fulfills the triggering condition: If multiple beams fulfills the triggering condition, it remains unclear which target beam should be selected.

Observation 4: It may be challenging for the UE to always select the beam that fulfilled the triggering condition as the target beam, especially in cases that L3 measurements is configured, or with the limitation of the indicated early TCI activation TCI state list.

Considering these different scenarios, it is difficult to assert that the UE will select the beam meeting the triggering condition as the target beam. Therefore, we prefer to leave this decision to UE implementation.

Proposal 9: It is up to UE implementation to select the target beam from among the beams indicated and activated in the early TCI state activation procedure.

UL grant

In LTM completion stage, CG and DG based UL grant acquisition is supported in Rel-18 LTM. Either the NW can configure configured grant (CG) via RRC LTM configuration, or provide regular dynamic grant via DCI (DG) after the cell switch is triggered.

In RAN2 #127bis meeting, the agreement on UL grant has been made as follow:

For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

For the CG based RACH-less conditional LTM, if the TCI state is selected by the UE as section 2.1.3 discussed, and the UL grant is configurated via RRC, there should be no issue for UE to send the UL message.

Observation 5: There should be no issue to support the first UL transmission if the target beam is selected (as section above) and the UL grant is provided to UE via CG.

On the other hand, in the last meeting, the support for DG-based RACH-less conditional LTM remained unclear. Based on the contributions, there is broad support for not introducing DG-based RACH-less conditional LTM, as the target cell may not know the target beam selected by the UE, making it difficult to use PDCCH for UL resource scheduling. However, considering the benefits of resource utilization, the feasibility of DG-based RACH-less conditional LTM is worth discussing in different cases.

UE based TA measurement

If UE-based TA measurement is used for target TA acquisition, the UE measures the SSBs between the source and candidate cells, then calculates the TA for the candidate cell. There is no opportunity to indicate the UL grant to the UE after LTM candidate configuration, as the network does not know the target beam selected by the UE (as discussed in the TCI acquisition agenda). Therefore, DG-based RACH-less LTM cannot be supported in the UE-based TA measurement case.

Proposal 10: The DG based RACH-less LTM is not supported in UE based TA measurement case.

PDCCH ordered early TA

From the agreement made, the following structure is used if PDCCH ordered early TA acquisition is performed: PDCCH order -> RACH Preamble -> TA MAC CE. After the UE selects the initial SSB of the target cell and sends the RACH preamble, the TA MAC CE is introduced to deliver the TA related information and is transmitted via the source cell to the UE. From a feasibility perspective, the UL grant can also be included in the TA MAC CE. This is not the regular “DG” but another kind of dynamic UL grant solution.

However, the beam of the RACH preamble may differ from the target beam selected by the UE when the LTM cell switch is triggered, so it may be hard to pre-schedule the UL resource between different beam of target cell, and it may not be “dynamic” enough. Also, this requires extra coordination between the source and target cells, as the TA MAC CE is delivered from the source cell to the UE.

Observation 6: Indicating the UL grant in the TA MAC CE is feasible, but it may not be dynamic enough and may introduce extra complexity in network scheduling and inter-node coordination.

Considering the possible benefits of dynamic scheduling and the potential impact and complexity, RAN2 can discuss whether to support the UL grant in the TA MAC CE as a dynamic solution.

Proposal 11: Based on the possible benefits and potential complexity, RAN2 can discuss whether to include the UL grant in the TA MAC-CE as a DG solution.

CFRA information

In Rel-18 LTM, the LTM CSC MAC-CE also includes CFRA related information to UE to perform RACH-based LTM. For conditional LTM, it is still unclear how the UE performs contention-free RACH toward the target cell in the absence of this information.

Since the UE determines the target cell by itself, the simplest approach is to include the CFRA information in the candidate configuration in advance, similar to the legacy solution in conditional handover. This avoids extra specification impact and reuses the current signalling structure. It is hard to provide this information in other signalling since there may be no other signalling indicated from the network to the UE. The same rule should apply to both initial and subsequent conditional LTM. If there are resource constraints from the network, the network may configure RACH-less conditional LTM based on UE capability or use CBRA instead.

Proposal 12: For RACH-based conditional LTM, the CFRA information for candidate cells can be provided in candidate configuration. This applies to both initial and subsequent conditional LTM. If CFRA information is not provided, CBRA is performed.

The motivation and the scope of conditional LTM

Let’s review the historical mobility mechanisms to see the benefits they achieved and identify additional benefits that can be gained from conditional LTM:

Conditional Handover

In Rel-16, Conditional Handover was introduced for UE initiated handover for multiple candidate cells. Compared with legacy handover, two main benefits can be found in Conditional Handover:

- Improve Reliability: UE decides to perform handover when condition is met to react quickly to deteriorating conditions to prevent condition degrade to a point where handover might fail. Besides, the handover command is omitted to prevent HOF caused by this signalling failure.

- UE autonomy and flexibility: Allows the UE to proactively decide which candidate cell to perform handover by certain predefined conditions and pre-configured candidate cells configurations. The decision made by UE may be more accurate and reasonable due to more detailed measurement context of its radio environment, which allows UE to detect more changes that might not be included in the measurement report.

Rel-18 LTM

In Rel-18, LTM was introduced for handover based on beam level measurement report. Compare with legacy handover, two main benefits can be found in LTM to optimize the performance for mobility:

- Interruption reduction. The early sync procedure is introduced to allow UE to establish synchronization toward candidate cells before handover execution, which shortens the interruption for each handover process. This is more important in the scenario when cell switch is performed frequently (e.g., FR2, high speed or ping-pong scenarios)

- Latency reduction: LTM decision is made from L1 beam level measurement. This approach allows for a finer and faster report of the radio environment to let network to make a quick decision to get a timely and detailed cell switch indication. MAC-CE is used instead of RRC message to further fasten the transmission of CSC to prevent channel deterioration.

Observation 7: The benefits of conditional handover are to increase reliability and provides UE autonomy. The benefits of LTM are to reduce latency and interruption.

In Rel-19 conditional LTM, to integrate “conditional” within LTM, several level of improvements can be proposed:

- UE initiate LTM execution for Reliability. LTM currently uses MAC-CE for cell switch commands, which lacks RLC-AM protection, leading to potential signal loss especially at cell edges where channel condition is poor. Therefore, the 1st objective of Rel-19 Conditional LTM is to enable UE initiated LTM cell switch execution without network indication. It can also reduce the latency of signalling interaction when condition is met to react quickly towards deteriorating channel conditions.

- UE initiate early sync for UE autonomy: To improve flexibility and accuracy, the UE can be allowed to proactively select a candidate cell based on its detailed measurements context, but it also needs UE initiated early synchronization and may need synchronization with network. Considering the complexity of early UL synchronization to acquire TA, which is based on UL signaling (besides UE-based TA), it would be easier to implement UE-initiated early DL sync with the candidate cell first.

Currently, the discussion topics are based on UE-initiated LTM execution. Given the benefits it brings, RAN2 may discuss whether to also introduce UE-initiated early DL sync for conditional LTM.

Proposal 13: RAN2 to discuss whether to introduce UE initiated early DL sync for conditional LTM.

Conclusion

In this contribution, the following observations and proposals are made:

Triggering event evaluation

Proposal 1: The UE starts to evaluate triggering condition after receiving LTM configuration.

Proposal 2: UE continue to evaluate the triggering condition for the next conditional LTM cell switch after one LTM cell switch is performed.

Proposal 3: Introduce conditional LTM removal signalling to remove part of/all CLTM configuration and conditions (similar to CHO). UE stop evaluating the conditions for one candidate cell if it is removed by network.

Proposal 4: RAN2 to clarify the own execution condition for each candidate cell among three understanding:

- Understanding 1: Each candidate cell has its own condition as target cell, regardless of which serving cell the UE is in.

- Understanding 2: The condition applies when one candidate cell is the current serving cell, and the same condition is used for all target cells.

- Understanding 3: The conditions apply when one candidate cell is the current serving cell, with different conditions for different candidate cells.

TA acquisition and maintenance

Proposal 5: If the early TA for a candidate cell is valid when the conditional LTM cell switch is triggered toward that cell, UE skips the RACH procedure during LTM execution, and the TA timer is not restarted. If the TA is not valid, UE performs random access procedure toward that cell during CLTM execution, and the TA timer is restarted.

Proposal 6: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch is triggered, the network can trigger TA refresh by sending another PDCCH order.

TCI state acquisition (Target beam selection)

Observation 1: In Rel-18 LTM, the Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states, while the LTM cell switch command MAC CE specifies the precise TCI state for target cell.

Observation 2: For conditional LTM, it is not clear how does UE acquire the precise TCI state/select target beam for LTM cell switch by the absence of LTM cell switch command MAC CE.

Proposal 7: For RACH-less conditional LTM, the UE selects one of the activated TCI states for LTM cell switch according to the early TCI state activation/deactivation MAC CE. The network sends RSs for each indicated TCI in downlink and monitor the uplink resources until LTM cell switch complete or the resources are deactivated by the network.

Observation 3: For RACH-based conditional LTM, if the UE selects the initial SSB via the preamble, the UE may receive another TCI state activation indication after the LTM cell switch. The benefit of LTM fine beam reporting by L1 MR may be lost, leading to increased throughput degradation or interruption.

Proposal 8: If proposal 7 is agreed, the same rule can also be applied to RACH-based conditional LTM.

Observation 4: It may be challenging for the UE to always select the beam that fulfilled the triggering condition as the target beam, especially in cases that L3 measurements is configured, or with the limitation of the indicated early TCI activation TCI state list.

Proposal 9: It is up to UE implementation to select the target beam from among the beams indicated and activated in the early TCI state activation procedure.

UL grant

Observation 5: There should be no issue to support the first UL transmission if the target beam is selected (as section above) and the UL grant is provided to UE via CG.

Proposal 10: The DG based RACH-less LTM is not supported in UE based TA measurement case.

Observation 6: Indicating the UL grant in the TA MAC CE is feasible, but it may not be dynamic enough and may introduce extra complexity in network scheduling and inter-node coordination.

Proposal 11: Based on the possible benefits and potential complexity, RAN2 can discuss whether to include the UL grant in the TA MAC-CE as a DG solution.

CFRA information

Proposal 12: For RACH-based conditional LTM, the CFRA information for candidate cells can be provided in candidate configuration. This applies to both initial and subsequent conditional LTM. If CFRA information is not provided, CBRA is performed.

The motivation and the scope of conditional LTM

Observation 7: The benefits of conditional handover are to increase reliability and provides UE autonomy. The benefits of LTM are to reduce latency and interruption.

Proposal 13: RAN2 to discuss whether to introduce UE initiated early DL sync for conditional LTM.

References

- R2_128_R16R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- RP-242356-Revised-WID-NR-Mob-Enh