3GPP TSG-RAN WG2 Meeting #129	R2-2500142

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item	: 8.1.4

Source	: LG Electronics

Title	: Discussion on UE side data collection

Document for	: Discussion and Decision

Introduction

During the recent RAN2#125bis[1], RAN2#126[2], and RAN2#127[3] meetings, progress has been made in RAN2 regarding mechanisms identified for data collection for UE side model training during Rel-18. The table below has been proposed to be captured in TS 38.843[4] based on the agreement in the RAN2#127[3] meeting.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data content.Opt B) Partial visibility for partially standardized data content. (Note 6) Opt C) No standardized visibility (Note 6). | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included tranperantly) - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5 RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups - Note 6: RAN2 has not concluded on the need for partial and no visibility option - NOTE 7: RAN2 couldn’t reach consensus on the feasibility of UP solution |  |  |  |  |

In this contribution, we will further analyze and discuss the data collection options (Option 1a, Option 1b, Option 2, Option 3) for UE-side model training.

Discussion

CP solution

The table below can be a baseline example for the data transfer process through the CP tunnel for each option, where the server for UE data collection is located within or outside the MNO.

| Location of the server Option | Server inside the MNO | Server outside the MNO |
| --- | --- | --- |
| Option 1a | N/A | N/A |
| Option 1b | N/A | N/A |
| Option 2 |  |  |
| Option 3 |  |  |

From the RAN2 perspective, the data transfer through the CP tunnel can be classified into two categories based on the table summarized above.

- CP solution via NAS from UE to AMF (i.e., Option 2): When transferring data via NAS, there is no need to perform RRC segmentation or specify a separate field.

- CP solution via AS from UE to gNB to OAM (i.e., Option 3): This approach is expected to be similar to the MDT. Therefore, it shares the advantages of MDT, such as cost efficiency, extensive coverage, substantial data collection capability, real-time data acquisition, and user-centric analysis for precise network optimization.

Both AS protocol and NAS protocol are not well-suited for transferring large amounts of data, which is common when transferring data collected for AIML purposes. This limitation is due to the fact that both AS and NAS are ultimately transmitted through RRC messages, which have a size constraint of up to 9000 bytes. Consequently, transferring large amount of data necessitates multiple RRC transmissions, leading to increased signaling overhead. Additionally, it is essential to specify the data type for each use case during transmissions via both AS and NAS. This requirement introduces scalability constraints issues. Whenever new use cases are introduced from the development of AI/ML technology, the UE needs to be updated to support the new use case-specific data types. Also, the network needs to be fully deployed/updated to support the new use case-specific data types.

In this context, discussions about the CP solution via both AS and NAS should be deprioritized within RAN2.

Proposal 1: RAN2 may exclude the CP solution from discussions regarding data collection options for UE-side model training.

UP solution

According to the agreement from the RAN2#127 meeting[3], data transfer through the UP tunnel is considered in all options(Option 1a, Option 1b, Option 2 and Option 3). The table below can be a baseline example for the data transfer process through the UP tunnel for each option, both when the server for UE data collection is inside and outside the MNO.

| Location of the server Option | Server inside the MNO | Server outside the MNO |
| --- | --- | --- |
| Option 1a | N/A |  |
| Option 1b |  |  |
| Option 2 |  |  |
| Option 3 |  |  |

Looking at the table above, when the server is located inside the MNO, the data transfer paths and the scope of the MNO for Option 1b and Option 2 are the same. Similarly, when the server is located outside the MNO, the data transfer paths and the scope of the MNO for Option 1a, Option 1b, and Option 2 are identical. From the RAN2 perspective, the UP solution for Option1a, Option1b and Option2 remains the same regardless of whether the server for data collection is inside or outside the MNO.

On the other hand, Option 3 should be considered separately in RAN2 as the UP solution via OAM, distinct from the other options.

Observation 1. From the RAN2 perspective, regardless of whether the server for data collection is within the MNO or not, the UP solutions for Option1a, Option 1b and Option 2 are identical.

Therefore, from the RAN2 perspective, the data transfer through the UP tunnel can be classified into two categories based on the analysis above.

- data transfer through UP tunnel via UPF (i.e., Option 1a, Option 1b, Option 2)

- data transfer through UP tunnel via OAM (i.e., Option 3)

From the RAN2 perspective, the data transfer through the UP tunnel via UPF already exists between the UE and the CN function in the legacy system. This legacy data transfer operation has already been performed through a well-defined standardized PDU session. Therefore, it would be appropriate to utilize this solution for AIML data. While the feasibility check of this method for AIML data may be necessary by SA2, the existing solution is likely to remain suitable for use.

Consequently, data transfer through the UP tunnel via UPF should be preferred as the baseline over data transfer through the UP tunnel via OAM.

Proposal 2. RAN2 should consider data transfer through the UP tunnel via the UPF as the baseline for data collection options for UE-side model training.

Controllability and Visibility aspects

Based on the agreement from the RAN#105 meeting[5], each option should fulfil the specific requirements. Assuming that data transfer via UPF is the baseline, the discussion below will delve into the controllability and visibility aspects of the UP solution via UPF.

[Agreements by RP105 meeting] - RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - 1. The data collected is secured and data integrity and confidentiality for that data is ensured. - 2. User data privacy, anonymity and user consent is respected. (SA3) - 3. The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - 4. MNO has full visibility for standardized data. - 5. The design is futureproof and extendable. (!!!!!!!!) - FFS/study if and how to handle non-standardized data (i.e. partial visibility). - FFS controllability on data collection - Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

Controllability

For the UP solution via UPF, some companies may think that UPF is just a pass-through, leading to the assumption that MNO lacks full control. However, this is not quite true. Within the core network, the UP solution that facilitates data transfer via UPF can be fully controlled under the control of the SMF.

Furthermore, if AIML and non-AIML data are separated to form different PDU sessions with distinct LCH, DRB, and QoS requirements, the MNO may achieve full control over the standardized data collection transfer process.

Therefore, MNO can have full controllability for the UP solution via UPF, regardless of whether the server for data collection is located within the MNO or not.

Proposal 3. The MNO can have full controllability over the data collection process through the UP solution via UPF.

Visibility

Within the core network, UP solution via UPF can have full visibility for each PDU session under the control of the SMF. Full visibility for standardized data collection is already achievable in legacy operation, as outlined in the standard documents by the relevant working groups. For instance, the TS 26.531 document indicates that when the server for data collection is located within MNO, full visibility for standardized data can be guaranteed for UE-side model training. In this context, full visibility for AIML data is also anticipated to be achievable.

Therefore, MNO can have full visibility for the UP solution via UPF, regardless of whether the server for data collection is located within the MNO or not.

Proposal 4. The MNO can have full visibility over the data collection process through the UP solution via UPF.

Conclusion

Based on the above discussion, we made the following observations and proposals.

[Observations]

Observation 1. From the RAN2 perspective, regardless of whether the server for data collection is within the MNO or not, the UP solutions for Option1a, Option 1b and Option 2 are identical.

[Proposals]

Proposal 1: RAN2 may exclude the CP solution from discussions regarding data collection options for UE-side model training.

Proposal 2. RAN2 should consider data transfer through the UP tunnel via the UPF as the baseline of data collection options for UE-side model training.

Proposal 3. The MNO can have full controllability over the data collection process through the UP solution via UPF.

Proposal 4. The MNO can have full visibility over the data collection process through the UP solution via UPF.

References

- R2_125bis_ChairNotes_24-04-19_final.docx

- R2_126_ChairNotes_24-05-24_final.docx

- R2_127_ChairNotes_24-08-23_17-00_eom.docx

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP#105 meeting report

- RP#106 meeting report