3GPP TSG-RAN WG2 Meeting #129	R2-2500147

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Remaining issues on inter-CU LTM in DC cases

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, the followings were agreed [1]:

Rel-19 Set ID is configured for a candidate configuration. For DRB:

•	Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	UE performs PDCP re-establishment to all radio bearers.

•	Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

o	For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

o	For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

o	If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

This document discusses remaining issues on inter-CU LTM in DC cases.

- Discussions

MN terminated bearer handling at inter-CU SCG LTM

In the last meeting, it was agreed that PDCP recovery is necessary for MN terminated split bearer at inter-CU SCG LTM cell switch execution (sky blue highlighted part). However, PDCP recovery is also necessary for MN terminated SCG bearer. We propose that RAN2 should agree that UE performs PDCP recovery for MN terminated SCG bearer as well as MN terminated split bearer at inter-CU SCG LTM cell switch execution.

Proposal 1: RAN2 to agree that UE performs PDCP recovery for MN terminated SCG bearer as well as MN terminated split bearer at inter-CU SCG LTM cell switch execution.

Bearer type change at intra-CU LTM

In the last meeting, it was agreed that bearer type change is supported at inter-CU MCG/SCG LTM cell switch execution (yellow highlighted parts). However, it has not been discussed whether bearer type change will be supported at intra-CU MCG and/or SCG LTM cases.

In Rel-18 intra-CU LTM, PDCP termination point change at LTM cell switch execution cannot be supported because PDCP re-establishment is not supported. However, PDCP re-establishment is supported in Rel-19 LTM, it will be possible to support PDCP termination point change at intra-CU LTM as well as inter-CU LTM cell switch.

Observation 1: As PDCP re-establishment is supported in Rel-19, it will be possible to support PDCP termination point change at intra-CU LTM as well as inter-CU LTM cell switch.

Also, in Rel-18 intra-CU LTM, some cases of RLC bearer type change, e.g., the cases where associated RLC bearer is released at LTM cell switch, are not supported because the L2 handling due to such cases for AM DRBs is not specified in the RRC specification. This was pointed out in the last meeting [2] and concluded as “not pursued”, but some companies raised their opinion that RAN2 might consider it for Rel-19 [1].

Observation 2: L2 handling for the case associated RLC bearer is released at LTM cell switch is not specified in Rel-18 RRC specification, but it can be considered in Rel-19.

As Rel-18 SCPAC already supports all cases of bearer type change at CPAC execution, it can be reused as a baseline of bearer type change of intra-CU or inter-CU LTM cell switch. To support flexible cell switch operation, we propose that all cases of bearer type change at intra-CU LTM cell switch as well as inter-CU LTM cell switch should be supported in Rel-19. We expect it does not require a significant workload to specify bearer type change at intra-CU LTM case.

Proposal 2: All cases of bearer type change at intra-CU LTM cell switch as well as inter-CU LTM cell switch should be supported in Rel-19.

If proposal 2 is agreeable, L2 handling for bearer type change at intra-CU LTM cell switch should be discussed.

L2 handling upon bearer type change with PDCP termination point change

At first, UE has to distinguish between intra-CU LTM and inter-CU LTM for proper L2 handling. For this purpose, Rel-19 Set ID can be reused. If the NW configures the Rel-19 Set ID and the Rel-19 Set ID of candidate cell and serving cell are same, UE preforms L2 handling for intra-CU LTM.

Both in intra-CU inter-DU and intra-CU intra-DU LTM, if there is a DRB with PDCP termination point change, UE is required to perform PDCP re-establishment and RLC re-establishment of the DRB due to security key association change (change of keyToUse) regardless of whether MCG LTM or SCG LTM.

Proposal 3: When Rel-19 Set ID of candidate cell and serving cell are same, for a DRB with keyToUse changes:

  - UE performs PDCP re-establishment and RLC re-establishment.

L2 handling upon bearer type change without PDCP termination point change

For a DRB without PDCP termination point change, L2 handling is different depending on the LTM cell switch is intra-DU or inter-DU. Therefore, UE has to further distinguish between intra-DU LTM and inter-DU LTM. For this purpose, ltm-NoResetId can be reused. As in Rel-18 intra-CU LTM, if ltm-NoResetID is configured and ltm-NoResetID of candidate cell and serving cell are different, UE can know it is inter-DU LTM and otherwise intra-DU LTM.

In intra-CU inter-DU MCG LTM, for a DRB without PDCP termination point change, UE is required to perform MCG RLC re-establishment as MN DU changes. Similarly, in intra-CU inter-DU SCG LTM, for a DRB without PDCP termination point change, UE is required to perform SCG RLC re-establishment as SN DU changes. If the RLC entity to be re-established is already released in the RLC bearer type change or not associated with the CG, it cannot be re-established. However, if this case, i.e., the case RLC entity cannot be re-established is captured as procedural texts in the specification, the procedure will be complicated. Hence, we propose that this case is not captured or captured as a note in the specification, if needed.

In intra-CU intra-DU LTM, for a DRB without PDCP termination point change, UE does not need to perform RLC re-establishment in all cases of RLC bearer type change without PDCP termination point change.

If AM RLC is re-establishment or released, UE should perform PDCP data recovery to prevent data loss unless the PDCP entity is re-established. Both in intra-CU inter-DU and intra-CU intra-DU LTM, UE releases an associated RLC entity when the bearer changes between MCG and SCG bearer, and from split bearer to MCG or SCG bearer.

All the L2 handling at bearer type change without termination point change described above is same for MN-terminated bearer and SN-terminated bearer.

Proposal 4: When Rel-19 Set ID of candidate cell and serving cell are same, for a DRB with keyToUse maintained:

  - If ltm-NoResetId of candidate cell is different from serving cell or ltm-NoResetId is not configured,

    - UE performs RLC entity re-establishment for the cell group on which the LTM cell switch is triggered.

  - If an associated RLC entity is re-establishment or released, and if the bearer is AM DRB, UE performs PDCP data recovery.

Proposal 5: In proposal 4, if an associated RLC entity is already released, it cannot be re-established. However, this case should not be captured or should be captured as a note in the specification, if needed.

Conclusion

We have the following observations and proposals:

MN terminated bearer handling at inter-CU SCG LTM

Proposal 1: RAN2 to agree that UE performs PDCP recovery for MN terminated SCG bearer as well as MN terminated split bearer at inter-CU SCG LTM cell switch execution.

Bearer type change at intra-CU LTM

Observation 1: As PDCP re-establishment is supported in Rel-19, it will be possible to support PDCP termination point change at intra-CU LTM as well as inter-CU LTM cell switch.

Observation 2: L2 handling for the case associated RLC bearer is released at LTM cell switch is not specified in Rel-18 RRC specification, but it can be considered in Rel-19.

Proposal 2: All cases of bearer type change at intra-CU LTM cell switch as well as inter-CU LTM cell switch should be supported in Rel-19.

L2 handling upon bearer type change with PDCP termination point change

Proposal 3: When Rel-19 Set ID of candidate cell and serving cell are same, for a DRB with keyToUse changes:

  - UE performs PDCP re-establishment and RLC re-establishment.

L2 handling upon bearer type change without PDCP termination point change

Proposal 4: When Rel-19 Set ID of candidate cell and serving cell are same, for a DRB with keyToUse maintained:

  - If ltm-NoResetId of candidate cell is different from serving cell or ltm-NoResetId is not configured,

    - UE performs RLC entity re-establishment for the cell group on which the LTM cell switch is triggered.

  - If an associated RLC entity is re-establishment or released, and if the bearer is AM DRB, UE performs PDCP data recovery.

Proposal 5: In proposal 4, if an associated RLC entity is already released, it cannot be re-established. However, this case should not be captured or should be captured as a note in the specification, if needed.

References

[1] R2-2410911, “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung)

[2] R2-2410063, “Remaining issues on L2 reset for LTM”, NEC