3GPP TSG-RAN WG2 Meeting #128			        R2-2500149

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda Item:	8.5.4 (Netw_Energy_NR-enh-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on common signal and channel adaptation

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

Discussion

- 2.1 Adaptation of paging occasions in the time domain

Regarding the adaptation of paging occasions, RAN2 has made the following agreements so far:

RAN2#128:

1.	Introduce value for Ns=8. FFS on 16.

2.	Introduce value for N= T/32. FFS on T/64, T/128, T/256.

RAN2#127bis:

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Based on the agreements, we will discuss the remaining issues.

Ns value 16

## 7.1 Discontinuous Reception for paging When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

In the RAN2#128 meeting, RAN2 agreed to introduce a value for Ns=8. And the value for Ns=16 is still FFS. There are concerns that extended Ns value could increase signaling overhead in SIB1, as the increasing number of POs in a PF also raises the number of bits required for PO offsets. However, PO offset can be optionally configured. If the PO offset is not configured, POs are consecutively located in a PF. Therefore, the network can support a larger NS value with less signaling overhead in SIB1.

Observation 1	Supporting Ns 16 is not an issue in terms of SIB1 capacity, since the PO offset config is optional.

However, there may be concerns about whether the current PEI configuration can support the extended Ns value in a PF. If the Ns value is 16, PEIs should be associated with 16 POs in a PF. Since the PEI message is transmitted via DCI-format 2-7, which has a limited payload of 41 bits, the number of POs in a PEI is constrained. However, the maximum number of PEIs in a PF is 4, and each PEI can support 4 POs, allowing a total of 16 POs in a PF.

For example, if each PO has 8 subgroups (the lowest false alarm case), a single PEI can be associated with 4POs. The number of bits required for the PEI is calculated as (number of subgroups) *(number of POs associated with PEI). In this case, 8 subgroups * 4 POs = 32 bits, which is less than the 41-bit limit for the DCI-format 2-7 payload. As the current spec supports a maximum of 4 PEIs in a PF, Ns value of up to 16 can be supported without increasing signaling overhead related to PEI-O configuration, and without increasing number of POs in a PEI.

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

maxNrofPagingSubgroups-r17              INTEGER ::= 8       -- Maximum number of paging subgroups per paging occasion

Observation 2	The existing DCI-format 2-7 can support association between PEIs and 16 POs in a single PF.

Proposal 1	Extend Ns up to 16.

N value T/64, T/128, T/256

It is also still FFS how much to extend N. N is the number of total paging frames in a paging cycle and the maximum extendable value depends on the paging cycle. Longer interval between PFs by configuring smaller N provide more NES gain, as it allows more time for gNB in sleep mode between each PF. For the maximum paging cycle rf256, if N is set to T/256, only one paging frame occurs within a paging cycle and gNB can sleep at most for 255frames. If Ns is extended only up to 8, a single PF within a paging cycle may not be sufficient to transmit all paging. However, if the paging load is very low, supporting the N=1 can provide the largest NES gain, so there is no reason to block support. If more POs are needed, the network can configure bigger N.

Proposal 2	Extend N up to T/256.

Issue related to PEI

RAN2 has discussed the PEI issues, and the following remaining issues are leftover. We will discuss the remaining issues from the RAN2 perspective.

- Whether the same PEI can be associated with legacy POs and R19 POs.

- Whether the current PEI-O configuration can support R19 POs.

Before discussing the PEIs association with legacy POs and R19 POs, we clarify the legacy POs and R19 POs. Since there may be confusion between POs from the UEs side and from the network side. As the legacy POs can be shared with legacy UEs and R19 UEs. From the agreement, as R19 UEs only monitor POs configured by R19 PO configuration, R19 POs are POs configured by R19 paging configuration. Therefore, even if some of the POs are shared by legacy UEs and R19 UEs, the POs are configured by R19 PO configuration to R19 UEs as R19 POs. For the legacy UEs, the POs are configured by legacy PO configuration to legacy UEs as legacy POs. The picture (a) below explains the case where the POs are shared by legacy UE and R19 UEs, legacy POs and R19 POs are overlapped. And the picture (b) explains the case where the POs are not shared by legacy UE and R19 UEs, legacy POs and R19 POs are not overlapped.

Figure (a). Legacy POs are shared between legacy UEs and R19 UEs.

Figure (b). Legacy POs are not shared between legacy UEs and R19 UEs.

A PEI field of DCI format 2_7 is defined as below [1]. PEI field includes NPOPEI segments of K bits, where NPOPEIis number of POs associated with a PEI and K is number of subgroups per PO.

From the highlight part, we can deduce that PO index and Ns value determine which POs are associated with the bits in PEI.

From the highlight part, if multiple PEIs are associated with a PF, the POs corresponding to a PEI are determined by the number of PO per PEI and index of PO.

## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by pei-SearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by pei-FrameOffset, from the start of a frame to the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI-2-7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by po-NumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+i_smodNPOPEI is a paging occasion index, and N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the i_sNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O. ## 7.1 Discontinuous Reception for paging Where the index i_s is indicating the index of PO determined by: i_s = floor (UE_ID/N) mod Ns

Above this, we understand that the relationship between PEIs and POs is related to Ns value and PO index. However, the legacy POs and R19 POs are defined with separate Ns value and N value, resulting in different ranges of PO index. Figure (c) describes the case where single PEI is associated with legacy POs and R19 POs, which have different PO index.

Figure(c) Ex. Single PEI is associated with legacy POs and R19 POs

Therefore, if a single PEI is associated with both legacy POs and R19 POs, which have different PO index and PO index ranges, a collision may occur in the PEI bit allocation between the legacy POs and R19 POs. For simplicity and avoiding confusion between PO indexes from different types of POs, separate PEI should be implemented between legacy POs and R19 POs.

Proposal 3	Use sperate PEI for the legacy POs and R19 POs.

Since RAN2 has agreed that R19 UEs only monitor the PO(s) according to R19 paging configuration, and separate PEI should be used between legacy POs and R19 POs. Rel-19 UEs should monitor only the PEIs according to Rel-19 PEI configuration.

Proposal 4	R19 UE only monitors the PEI-O according to Rel-19 PEI configuration.

Also, the maximum number of PEI per PF is 4. The 4 PEIs can each be associated with 4 POs, allowing a total of 16 POs to be associated in a PF. The current PEI-O configuration can support the maximum number of R19 POs per PF. And if network implement separate PEI between legacy POs and R19 POs, there is no need to enhance current PEI-O configuration.

Proposal 5	RAN2 re-uses the current PEI configuration for the R19 PEI-O.

- 2.2 Adaptation of PRACH in time domain

In RAN1 meetings, it is agreed to define additional PRACH resource for NES-capable UEs, for PRACH adaptation in time domain. Further, it is agreed to use DCI for PRACH adaptation for NES-capable UE , e.g., to indicate activation/deactivation of NES PRACH resource.

[RAN1#116bis] Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode [RAN1#118bis] Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details [RAN1#119] Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

Although some details on PRACH adaptation should be further discussed in RAN1, RAN2 may start the high-level discussion on RAN2 aspects for adaptation of PRACH in time-domain.

Regarding configuration of additional PRACH resources, it should be provided via cell specific configuration (e.g., SIB1), given that additional PRACH resource can used by NES capable UEs in RRC_IDLE/INACTIVE state. Since the legacy PRACH resource is provided via SIB1, it would be better to configure the additional PRACH resources for NES-capable UE by SIB1 as well.

Proposal 6. Additional PRACH resource for NES capable UE is configured by SIB1.

In RAN1 discussion, it was mainly discussed 1) how to configure the additional RO and 2) how to indicate whether the additional RO is available or not (e.g., by DCI with P-RNTI).

However, there are still lots of essential discussion points to complete PRACH adaptation which may be discussed in RAN1 or RAN2, including following aspects:

- Detail signalling on additional RO configuration, e.g., whether to

  - 1) introduce additional parameter for additional RO (e.g., separated msg1-FrequencyStart and/or prach-ConfigurationIndex for additional RO) within the legacy RACH configuration; or

  - 2) introduce separated RACH configuration (e.g., RACH-ConfigCommonNES);

- How to select between additional RO and legacy RO for each RA attempt; and

- Whether/how to support the additional PRACH resources for CFRA and Msg1-based SI request

In addition, if the additional PRACH resource can be applied for 2-step RA as well, further discussion is needed, e.g., various aspects on how to configure/determine a separated PUSCH resource for additional RO. Then, further RAN1 discussion may be needed to determine how to identify valid PUSCH occasion associated with the additional RO.

Therefore, due to the limited time to finish the NES WI, additional work for PRACH adaptation should be avoided as much as possible. In particular, further RAN1 impacts should be avoided since there are only three meetings left for Rel-19 NES.

In this sense, in order to focus on essential part of PRACH adaptation in time domain which can be commonly applied for UEs in RRC IDLE/INACTIVE state and RRC_CONNNECTED state, the discussion on PRACH adaptation for 4-step CBRA should be prioritized.

Proposal 7. Focus on 4-step CBRA for PRACH adaptation in time domain.

Conclusion

Regarding adaptation of paging occasions in the time domain

Observation 1	Supporting Ns 16 is not an issue in terms of SIB1 capacity, since the PO offset config is optional.

Observation 2	The existing DCI-format 2-7 can support association between PEIs and 16 POs in a single PF.

Proposal 1	Extend Ns up to 16.

Proposal 2	Extend N up to T/256.

Proposal 3	Use sperate PEI for the legacy POs and R19 POs.

Proposal 4	R19 UE only monitors the PEI-O according to Rel-19 PEI configuration.

Proposal 5	RAN2 re-uses the current PEI configuration for the R19 PEI-O.

Regarding adaptation of PRACH in time domain

Proposal 6. Additional PRACH resource for NES capable UE is configured by SIB1.

Proposal 7. Focus on 4-step CBRA for PRACH adaptation in time domain.

- 4. References

[1] TS 38.213-v18.5.0, Physical layer procedures for control, 2024-12.