3GPP TSG RAN WG2 #129		                                              R2-2500150

Athens, Greece 17th – 21st February 2025

Agenda item:	8.4.1

Source: 	Vodafone, Huawei, HiSilicon, Vivo

Title: 	LR and MR operating frequencies

Document for:	Discussion

- Introduction

The LP-WUS operation on different bands was discussed during RAN Plenary 106, and the way forward was agreed upon as documented in RP-243266, “Way forward on Rel-19 LP-WUS separate band issues.” Vodafone also presented a document, RP-242656, where possible options were analyzed. Based on this document, RAN Plenary 106 defined a way forward summarized in RP-243266:

- For idle/inactive mode, consider a standardized solution for the intra-frequency case (e.g., solutions 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g., solution 4) will not be considered.

- Detailed discussions and final solution selection are left to the working groups (WGs).

The aim of this document is to showcase how the intra-frequency solution could work. For simplicity, we will consider a UE that supports LP-WUS on, for example, 700 MHz, with the main receiver capable of 700 MHz and 3.5 GHz.

- Discussion

The solutions captured in RP-242656 for the intra-frequency case are described as follows:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3:  MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signaling depending on LR band load (up to gNB implementation)

Procedurally, the following steps intend to describe steps needed to realize intra frequency case:

      - The LP-WUS capable UE is attached to operator NW and is camping on an e.g. 3.5 GHz cell.

      - The network broadcasts that a particular frequency (700 MHz) is of a highest priority for LP-WUS capable UEs. Alternatively, or in addition, the priority of a particular frequency (700 MHz) can be provided by dedicated signaling once the UE is getting connected in the cell of 3.5 GHz during the release of this connection. These 2 approaches are described in solutions 1 and 3 in RP-242656.

      - Based on legacy reselection mechanisms the LP-WUS capable UE reselects to 700 MHz cell. Once doing so, the UE may store the information (SIB1 and the like) of the cell it was camping on (3.5 GHz).

      - The UE goes into deep sleep mode and monitor LP-WUS signal with its LP-WUR only on the 700 MHz cell.

      - Once the UE is addressed over the LP-WUS signal, it switches on the main receiver, camps on 700 MHz cell and determines the paging occasions on 700 MHz Cell

      - The UE perform measurements of the serving cell to received paging on the 700 MHz cell by the gNB.

      - Upon being paged, the UE perform inter-frequency measurements target cell (where target cell is at least the 3.5 GHz cell the UE was previously camped on or any other on this spectrum), and if reselection criteria are satisfied for this cell. Once doing so, it can use the previous stored information from the 3.5 GHz cell.

      - Based on this, if the reselection criteria are satisfied, the UE reselects to 3.5 GHz cell and establish RACH there.  If not, the UE performs RACH on 700 MHz cell.

From all the steps above only Step 2 and Step 7 would require specification work considering the addition of LP-WUS procedures. Nonetheless, all the steps could leverage existing specifications, and the idle/inactive/connected transitions outlined in TS 38.304 in Figure 5.2.2-1: RRC_IDLE Cell Selection and Reselection.

Regarding Step 2, it is needed as the UEs having LP-WUS capabilities should be redirected to 700 MHz cell in our example to receive LP-WUS and to make use of its associated battery saving procedures.

Anyhow, as a standalone step, this would not solve the problem as it would either decrease the utilisation of LP-WUS or lead to the overload of low band spectrum as all the connections from LP-WUS capable devices would be making use of the 700 MHz cell to perform RACH.  Therefore steps (3-7) are needed to increase the utilization of LP-WUS feature.

Step 7 is based on a normal cell reselection towards the cell of the higher priority. From TS 38.133 (see table 4.2.2.3-1 and 4.2.2.4-1), with the UE having detected already the 3.5 GHz cell and storing its information, shall be able evaluate whether a cell meets a reselection criteria within at most a maximum of 5.12 s for both intra and inter-frequency (considering there is only one inter-frequency carrier) for a DRX cycle length of 0.32s and 0.64s; and at most 6.4s for a DRX cycle of 1.28s.

After performing drive tests in the same scenario as described above with an idle DRX cycle length of 0.32s, we observed that the reselection takes a fraction of the time compared to the standardized values, proving that the RAN4 requirements are more than conservative. The additional delay, which might be introduced due to the need to measure the high-priority neighbor, would not exceed approximately 2 seconds according to our  observations in the drive tests and could be further reduced through implementation.

Moreover, the CN controls paging for different UEs differently, allowing repetitions based on CN timers that consider the capabilities and the service requirements. Once the CN knows that an LP-WUS capable UE is going to be paged, it can adapt the repetition timers accordingly and avoid paging repetitions.

Regarding NAS aspects; one could argue that the cell in 700 MHz and 3.5 MHz might be on different TACs which would require the tracking area update once executing step 3 and 7. It is anyway important to consider that operator can allocate multiple TACs to the UE where once moving between the UE does not need to execute TAU. The implementation could also address this issue, by not executing selection/reselection to the higher frequency in step 7, but just establishing RACH on 700 MHZ if 3.5 MHz and 700 MHz cells are on different TACs.

- Observations

We have the following observations regarding the steps we see would require specification work:

Observation 1:

On Step 2:

- A mechanism is needed to redirect UEs from bands not supporting LP-WUS (e.g., 3.5 GHz) to bands where LP-WUS is supported (e.g., 700 MHz) for LP-WUS capable UEs.

  - As a standalone step, this would not solve the problem and could lead to either a decrease in the utilization of LP-WUS or an overload of the low band spectrum, as all connections from LP-WUS capable devices would use the 700 MHz cell to perform RACH.

Observation 2:

On Step 7:

- The additional delay associated with reselection from the lower band (e.g., 700 MHz) where LP-WUS and paging are performed is much smaller than the specified RRM requirements.

  - According to drive tests executed in the same scenario described above (2 different cells with 700 MHz and 3.5 GHz) and with a DRX cycle length of 0.32s,  UEs would need around 2 seconds to perform reselection to the known high priority cell.

  - While the delay could be reduced, it does not cause any operational problems as paging repetition timers have network-dependent values (see 24.501, timer T3513 in Table 10.2.1: Timers of 5GS mobility management – UE side, note 4).

  - The values of the paging repetition timers might vary between devices (e.g., smartphones and IoT devices) and also take into account UE capability. Therefore, accommodating an additional small delay caused by LP-WUS devices should not cause any paging repetitions or have any impact on the service.

- Proposals

Proposal 1: Standardize solution 2 from RAN Plenary WF in release 19.

- References

- RP-243266, “Way forward on Rel-19 LP-WUS separate band issues