3GPP TSG-RAN WG2 Meeting #129 	R2-2500163

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item:	8.2.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on paging procedure of A-IoT

Document for:	Discussion and Decision

Introduction

In the RAN #106 meeting, a new WID on solutions for Ambient IoT in NR has been approved [1]. This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system. For the new IoT technoligyes, RAN2 related objectives are as follows.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, the message content of A-IoT paging and paging response issues will be analyzed. And we provide our opinions.

Discussion on paging

2.1 Message content of A-IoT paging

In RAN2, we need to discuss what message content is needed in A-IoT paging message, the following is a detailed analysis.

- Identifier of A-IoT device(s)

The following objectives are set, within the General Scope: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

As the scope of WID, the paging message contains one identifier or no identifier. For one identifier case, the SI phase agrees to support two cases: an A-IoT device ID and a group of A-IoT devices. In our opinion, both cases should be supported because there is a services to inventory an A-IoT device or a group of A-IoT devices.

Proposal 1: Paging message contains one identifier including two cases: an A-IoT device or a group of A-IoT devices

The paging identifier can be placed in the MAC SDU as a container, or transmitted in the MAC CE, depending on how AIoTF transmits this identifier.

- Resource allocation information

In SI phase, RAN2 has agreed that A-IoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

For contention-based random access, A-IoT paging message can indicate the shared resource. In RACH section, we has agreed that as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It can be further discussed if this is applicable to FDMA case. And in RAN1 meeting, they agreed that a R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). For FDMA, some frequency resource can be indicated in R2D trigger message, for example Y frequency resource in figure 1. But there is no agreement about how to determine the access occasion.

Figure 1 the resource allocation of Msg1

In our opinion, A-IoT paging message can indicate the total number of “slot” or access occasion set for the D2R response message, for example A-IoT paging message includes a parameter Q (like RFID mechanism, the total number of access occasion set is 2Q), or a parameter N indicating the total number of access occasion set. The only difference between the two parameter is that the indication methods are different and the occupied bits are different. The parameter Q can only indicate available access occasion set resources in multiples of 2 by using fewer bits. And the parameter N can indicate any number of slots but takes up more bits.

The access occasion set means X*Y resources indicated by R2D trigger message. This method can decouple the spec impact of RAN1 and RAN2. RAN2 standardizes the number of access occasion set and RAN1 is to indicate the location of time-frequency resources within each access occasion set. Multiple R2D trigger messages are required to indicate X*Y access occasions because large SFO of A-IoT device limits the value of X, and the frequency-shit capability of A-IoT device limits the value of Y. So A-IoT paging only needs to indicate the number of access occasion set. The location indication of access occasion within one access occasion set depends on RAN1.

For contention-free random access, A-IoT paging message can indicates one access occasion for the D2R response message.

Proposal 2: A-IoT paging message includes the total number of access occasion sets for the D2R response message. E.g., a parameter Q (like RFID mechanism), or a parameter N indicating the number of access occasion sets. The location indication of access occasion within an access occasion set depends on RAN1.

- Subsequent paging for the same service

SI phase, RAN2 #127-bis agreement: The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase.

In SI phase, we agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. FFS how to indicate. There are three options according to the current contributions:

Option 1: ID generated by CN;

Option 2: ID generated by reader;

Option 3: NDI-like information in A-IoT paging message.

CN or reader can generate the information to identify the service request. For option 1, CN can coordinate this information between multiple readers in the service request from CN. This can avoid the case that CN receiving redundant responses from an A-IoT device in case the device covered by multiple readers. Otherwise, for option 2, there may be a need for coordination between readers or additional information may need to be provided to identify the readers. Or for the case that devices need to redundantly respond to different readers for the same service, RAN2 does not guarantee or optimize this case. In addition, the length of the IDs of the two options is different. Option 1 is longer and option 2 is short. Both options have advantages and disadvantages, depending on how we do the trade-off. In our opinion, for option 2, if no coordination between readers, the redundant response may not be avoided. So Option 1 is prefer.

For option 3, considering forward compatibility, if there is multiple different parallel service, the NDI indication information cannot enable the device to correctly identify the subsequent paging of different services.

Proposal 3: A-IoT paging message includes the ID generated by CN without introducing additional ID generated by reader.

- Msg1 with/without data indication for unified solution 3

The SI phase, RAN2 discussed unified solution for 2-step and 3-step CBRA. However, within the scope of WID, only solution 1 is currently included. But in our opinion, solution 3 is necessary because 3-step CBRA increases latency, and device 1 may run out of power before completing 3-step random access due to limited battery.

In SI phase, RAN2 agreed that for all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. For implicitly indication, CBRA and CFRA may be distinguished by whether the reader allocates dedicated resources or shared resources. For unified solution, it is controlled by the reader by using A-IoT paging message on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data.

Proposal 4: For unified solution for CBRA, A-IoT paging message should indicate whether upper layer data is needed for Msg1.

As for other information, such as the expected size of the D2R response message, we need to wait for the progress of data transmission session of RAN2 A-IoT. We not analyse it here.

2.2 Format of A-IoT paging message

TR 23.700-13 v1.3.0: The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: …… 4. A-IoT Device NAS protocol is supported between the A-IoT Device and the A-IoTF. The A-IoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response. Editor's note: It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer. ### 8.3.4 Principles on the procedures to support AIoT services Editor’s note: If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

TR 38.769 v2.0.0 ## 6.5 Impacts on CN-RAN interface ### 6.5.1 Information exchanged between A-IoT CN and A-IoT RAN #### 6.5.1.1 Inventory Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process A-IoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report. NOTE 3: For topology 1, whether device ID received from the device is sent transparently to the A-IoT CN by the A-IoT RAN depends on RAN2 and SA2. NOTE 4: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2. #### 6.5.1.2 Command Command can be sent by the A-IoT CN for a single device. For topology 1, A-IoT RAN node should be able to differentiate between command and inventory. NOTE 1: Whether command can be performed for a group of devices depends on RAN2, SA2 and SA3. NOTE 2: Whether the A-IoT-enabled-gNB should be able to differentiate between command and inventory needs further discussion, which depends on SA2 and RAN2. NOTE 3: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2. ## 6.3 Protocol stack and signalling procedures ### 6.3.1 General aspects and overall procedure …… The following information are considered useful to be visible to the reader from CN: NOTE 2: It needs to be further discussed on whether following information is mandatory or optional. - The A-IoT service type (e.g., inventory, command) NOTE 3: It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available. - Whether the service is targeted for one or more than one A-IoT devices - The approximate number of target A-IoT devices of this service

As description in message content of A-IoT paging message, paging message for inventory request includes the following information: identification of A-IoT device(s), information to identify the same service request, resource allocation information and indication of whether Msg1 needs to transmit data.

According to above SA2, RAN3 and RAN2 SI agreements [1] [2], the A-IoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response. For the inventory request, the following information can be visible to the reader from CN:

-	The A-IoT service type (e.g., inventory, command)

-	Whether the service is targeted for one or more than one A-IoT devices

-	The approximate number of target A-IoT devices of this service

-	information to identify the same service request to trigger subsequent paging.

According to SA2, inventory request is not placed in the AIoT NAS message. But how to transmit the paging identifier between CN and AIoT RAN depends on RAN3. If paging identifier is transmitted through the container between AIoT RAN and CN, the MAC layer can use MAC SDU to carry it, otherwise, MAC CE can be used.

For command request, SA2 has agreed to include it in the A-IoT NAS message. In this case, A-IoT NAS messages are transmitted via MAC SDU in MAC layer.

Proposal 5: If there is A-IoT NAS content (e.g., paging identifier) in service request between CN and AIoT RAN, it should be transmitted via MAC SDU in A-IoT interface.

Proposal 6: The MAC control information (e.g., resource allocation information) of A-IoT paging should be transmitted via MAC CE.

Conclusions

In this contribution, we discuss the Rel-19 paging procedure and functionality. And the proposals are given as follows:

Proposal 1: Paging message contains one identifier including two cases: an A-IoT device or a group of A-IoT devices

Proposal 2: A-IoT paging message includes the total number of access occasion sets for the D2R response message. E.g., a parameter Q (like RFID mechanism), or a parameter N indicating the number of access occasion sets. The location indication of access occasion within an access occasion set depends on RAN1.

Proposal 3: A-IoT paging message includes the ID generated by CN without introducing additional ID generated by reader.

Proposal 4: For unified solution for CBRA, A-IoT paging message should indicate whether upper layer data is needed for Msg1.

Proposal 5: If there is A-IoT NAS content (e.g., paging identifier) in service request between CN and AIoT RAN, it should be transmitted via MAC SDU in A-IoT interface.

Proposal 6: The MAC control information (e.g., resource allocation information) of A-IoT paging should be transmitted via MAC CE.

References

[1] TR 23.700-13, SA2 Technical report of Study on architecture support of ambient power-enabled IoT.

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things).