3GPP TSG-RAN WG2 Meeting #129	R2-2500164

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT random access

Document for:	Discussion and Decision

1 Introduction

In RAN#106, a new WI was approved for A-IoT in order to standardize RAN aspects of A-IoT. To be specific, the WI will specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission. The objectives are illustrated below [1]:

RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we provide our view on the random access of Ambient IoT.

2 Discussion

Background

3-step CBRA:

For initial inventory to identify multiple A-IoT devices, when a response is expected from multiple devices, an A-IoT contention-based access (CBRA) procedure initiated by the reader is used. For CBRA procedure, slotted-ALOHA based access is baseline. 3-step CBRA random access for A-IoT is shown in Figure 1.

Msg0: Access trigger. The reader sends signal to trigger A-IoT device to perform access. The trigger signal may be paging or other R2D message. The resource for device subsequent transmission may also include in the message.

Msg1: The device sends a random ID to the reader. When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader.

Figure 1 3-step CBRA for A-IoT

Msg2: The reader responds with the successfully received random ID. If reader successful reception the RN16, it sends an ACK which at least carries the same RN16 from the A-IoT device.

Msg3: Device sends Device ID and any other upper layer data (depending on upper layer request). If the A-IoT device receives the same RN16 that it sent in Msg1, it may perform the upper layer data transmission with the reader, which can be the device ID and/or any other upper layer data.

After Msg3 transmission, reader may optional send explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.

2-step CBRA:

For some cases (e.g. Low collision probability of access), such as a small number of devices are triggered, Msg1 and Msg2 in 3-step CBRA may not be needed.

Figure 2 2-step CBRA for A-IoT

Msg0: Access trigger. The reader sends signal to trigger A-IoT device to perform access. The trigger signal may be paging or other R2D message. The resource for device subsequent transmission may also include in the message.

Msg 1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader.

Msg 2: The reader may respond with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader.

CFRA:

For dedicated inventory or command for a identified A-IoT device, the specific A-IoT device ID will included in trigger signal in Msg0. Only the device ID of A-IoT device match can initiate access, so it can be a contention-free random access (CFRA). CFRA based access procedure is shown in Figure 3.

Figure 3 CFRA for A-IoT

Msg 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging or command message which the device ID and resource for RACH are include in.

Msg 1: A-IoT device sends device ID and any other upper layer data to reader.

## 2.1 Re-access for failure handling

It is supported for the A-IoT device to re-access in another opportunity controlled/provided by the reader (i.e., to retry the random access above), in case of D2R data transmission failure and contention resolution failure of contention-based random access.

The A-IoT device is not expected to autonomously re-access. The re-access is always controlled by reader. The R2D message is used by the reader to provide access occasion(s), which can be used for re-access purpose. There are two options for re-access, option 1 is A-IoT paging message is this R2D message (e.g. the "subsequent A-IoT paging message"), option 2 is that it can be some R2D messages between A-IoT paging (e.g. the other "R2D message").

In other words, option 1 is the access occasions following the subsequent A-IoT paging can be used for re-access, and option 2 is reader can assign some access occasions for re-access purpose between two A-IoT paging.

Figure 3 the overall framework example of slotted ALOHA random access

We prefer option 2, same as RFID, consecutive access rounds triggered by one piece of paging message are for the same group of A-IoT devices. A-IoT devices fail to access in the previous access round could try to re-access in the next access round trigger by R2D round message.

Proposal 1: Reader can assign some access occasions for re-access purpose between two A-IoT paging (e.g. in the next access round).

## 2.2 Unified solution for CBRA

In A-IoT SI, two contention-based random access types (i.e. 2-step CBRA, 3-step CBRA) have been studied. Different random access types are suitable for different scenarios. For example, 3-step CBRA is applied to the initial group inventory which the number of devices is not known and reader cannot allocate appropriate random access resources. 3-step CBRA can detect collisions much faster, it is beneficial to reader to adjust the resource allocation according to the collisions and improve the inventory efficiency.

2-step CBRA is suitable for lower latency inventory, it can reduce one step for data transmission interaction in case of success, the signalling overhead is smaller. Comparatively speaking, 3-step CBRA can support relatively large coverage for A-IoT Msg1 than 2-step CBRA. 2-step CBRA has the message number efficiency in case of low probability of A-IoT Msg1 collision.

In addition, for the subsequent paging case, it is used to paging the devices that have previously lost power, or the new devices in the group, or the devices that have failed to access the previous access round. The number of devices in subsequent paging is small, 2-step CBRA is more suitable.

We prefer to support both 3-step and 2-step CBRA (i.e. the unified solution, A-IoT Msg1 optionally includes data). When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data. Then, the reader responds with the successfully received random ID in Msg2.

Proposal 2: Support the unified solution (A-IoT Msg1 optionally includes data) for CBRA.

## 2.3 Random access type determination

The A-IoT device determines the random access type (CBRA or CFRA) from the A-IoT paging message. The reader can configure either CBRA or CFRA. It needs to be further discussed on whether it is explicitly or implicitly.

If reader allocates a set of shared access occasions for a group device, it indicates that device uses CBRA. If reader allocates a dedicated access occasion for a device, it indicates that device uses CFRA.

Proposal 3: The reader implicitly indicates the random access type (CBRA and CFRA) through the allocated access resources.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: Reader can assign some access occasions for re-access purpose between two A-IoT paging (e.g. in the next access round).

Proposal 2: Support the unified solution (A-IoT Msg1 optionally includes data) for CBRA.

Proposal 3: The reader implicitly indicates the random access type (CBRA and CFRA) through the allocated access resources.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei.