3GPP TSG-RAN WG2 Meeting #129		R2-2500180

Athens, Greece, 17th – 21st Feb. 2025

Source:	China Telecom

Title:	Discussion on UE initiated beam reporting

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

- Q is RRC configured with subjective to UE capability signalling

- UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

According to RAN1’s progress as above, we will discuss the RAN2 aspects related to UEIBM, including RRC impact and MAC impact.

- Discussion

  - RRC impact

The impact of UEIBM on the RRC layer primarily involves RRC configuration parameters, including RS configuration, event types, event-related parameters, reporting transmission mode, report configuration etc. According to RAN1 progress, Event-1, Event-2, Event-7 are supported. Therefore, parameter eventType-r19 is needed to inform an event type for UE-initiated/event-driven beam reporting and the relevant parameters of each event is as follow.

Event-1: Quality of the current beam is worse than a certain threshold.

{

eventThresholdEvent1-r19

}

Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

{

EventThresholdEvent2-r19,

EventDetectionTimeWindowLength-r19,

EventInstanceCount-r19

}

Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

{

ValueOfQ

}

By comparing the parameters associated with Event-1 and Event-2, it was observed that Event 1 lacks the EventInstanceCount and EventDetectionTimeWindowLength parameters. It may be necessary to confirm with RAN1 whether these two parameters are required for Event-1. If Event-1 also requires these two parameters, it is necessary to determine whether all events share a common set of parameter configuration or if each event has its own parameter configuration.

Observation 1: According to LS from RAN1, it was observed that Event 1 lacks the EventInstanceCount and EventDetectionTimeWindowLength parameters by comparing the parameters associated with Event-1 and Event-2.

Proposal 1: Send an LS to RAN1 to confirm the parameters associated with events.

  - MAC impact

Take event-2 as an example, the evaluation and determination of the event can split into RS measurement, event instance evaluation and event determination. RS measurement is executed by PHY layer. For event instance evaluation,  two options are available:

- option 1: The PHY layer reports the measurement results to MAC layer, and MAC layer perform event evaluation.

- option 2: PHY layer evaluates whether the measured result fulfill the event or not and indicates to MAC layer once the event instance occurs.

Event Determination is performed by MAC layer. When the event instance count reaches the configured threshold, a report is triggered.

- Proposal 2: Event determination is performed by MAC layer and the event evaluation needs to wait for RAN1’s progress.

Refer to the beam failure detection. The MAC layer utilizes BFI_COUNTER, beamFailureInstanceMaxCount, and beamFailureDetectionTimer to determine beam failure and trigger BFR.

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if the Serving Cell is configured with two BFD-RS sets:

2>  if beam failure instance indication for a BFD-RS set has been received from lower layers:

3>  start or restart the beamFailureDetectionTimer of the BFD-RS set;

3>  increment BFI_COUNTER of the BFD-RS set by 1;

3>  if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount:

4>trigger a BFR for this BFD-RS set of the Serving Cell;

For event detection of UEIBM, a similar approach can be adopted. When an instance is detected and if the timer is not running, start the timer, and the value of the timer can be EventDetectionTimeWindowLength-r19 configured by RRC and increase the Event Instance Counter. If the Event Instance Counter reaches EventInstanceCount-r19 configured by RRC before the timer expiry, trigger a report.

- Proposal 3: Refer to beam failure detection, introduce a timer and a counter for event detection of UEIBM.

- Conclusion

Observation 1: According to LS from RAN1, it was observed that Event 1 lacks the EventInstanceCount and EventDetectionTimeWindowLength parameters by comparing the parameters associated with Event-1 and Event-2.

Proposal 1: Send an LS to RAN1 to confirm the parameters associated with events.

- Proposal 2: Event determination is performed by MAC layer and the event evaluation needs to wait for RAN1’s progress.

- Proposal 3: Refer to beam failure detection, introduce a timer and a counter for event detection of UEIBM.