3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500181

Athens, GR, 17th – 21st February 2025

Source:	CATT

Title:	Further Discussion on the SN Gap Report

Agenda Item:	7.0.2.16

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, the following contribution was discussed and postponed:

R2-2408003 Clarification on the SN Gap Report CATT discussion Rel-18 NR_XR_enh-Core Proposal 1: Suggest RAN2 to discuss when setting the PDCP SN gap report whether it should clarify the discarded PDCP SDUs only refer to the PDCP SDU(s) which has not been submitted by RLC to lower layers. - LG and Ericsson think that it is already clear in the three conditions for triggering SN gap reporting - Vivo doesn’t think the CR is correct but the issue is correct and the wording in existing spec may need some clarification. Samsung thinks the last change is not correct The CR is postponed

In RAN2#128 meeting, this issue was re-discussed but no conclusion was reached. In this contribution, this issue will be further discussed.

Discussion

The discussion is divided into two sections: the first section is aiming to clarify the issue, and the second section tries to find an acceptable solution to solve the issue.

Issue description

In subclause 5.16.1 of TS38.323, regarding to the transmission operation for SN gap report, the following was captured:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

Based on the above description, it is clearly that the description in subclause 5.16.1 includes two parts:

- Part 1: The trigger condition for SN gap report (marked with yellow);

- Part 2: How to set the PDCP SN gap report (marked with blue).

For Part 1, when discussing the trigger condition, the SDU should satisfy the following three conditions:

- PDCP SDU(s) are discarded;

  - Discarded may be caused by any of the following three cases: 1) due to its own discardTimer expiry; 2) due to the discardTimer  expiry of SDU belonging to the same PDU Set; 3) confirmed by PDCP status report.

- There is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s);

- The discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

For example, in the following figure, PDCP SDU1/2/3 all belongs to PDU Set 1, the PDCP SDU1 has been delivered to MAC by RLC, PDCP SDU2 has been delivered to RLC but has not been delivered to MAC, and PDCP SDU3 is still in PDCP. At T4, the discardTimer of PDCP SDU1 expiries. Assuming pdu-SetDiscard is configured for PDU Set 1, all of PDCP SDU1/2/3 should be discarded at the same time. But considering PDCP SDU1 has been delivered to MAC, it should not trigger SN gap report. If there is other PDCP SDUs which associated COUNT larger than the COUNT of PDCP SDU3, PDCP SDU2 and PDCP SDU3 can trigger SN gap report.

Figure-1 Illustration of the SN gap report trigger

Observation 1: It is possible that more than one PDCP SDUs are discarded simultaneously, but only partial of them can trigger PDCP SN gap report according to the current specification.

In case of more than one PDCP SDUs are discarded simultaneously, but only partial of them trigger PDCP SN gap report, it should further discuss whether the COUNT of the discarded PDCP SDU which with COUNT value smaller than the COUNT value of PDCP SDU which trigger the PDCP SN gap but have been delivered by RLC to lower layers should be included in the PDCP SN gap or not. Take Figure-1 as example, what we need to discuss is that whether the COUNT of PDCP SDU1 should be included in the PDCP SN gap report.

Proposal 1: Suggest RAN2 to discuss whether the discarded PDCP SDU(s) which have been delivered by RLC to lower layers should be included in the PDCP SN gap report or not.

Potential solutions

Regarding to the issue mentioned in section 2.1, there are two options:

- Option 1: The discarded PDCP SDU in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered by RLC to lower layers.

Take Figure-1 as example, the COUNT value of PDCP SDU1 should not be included in the PDCP SN gap report.

- Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include the PDCP SDU(s) which have been delivered by RLC to lower layers.

Take Figure-1 as example, the COUNT value of PDCP SDU1 can be included in the PDCP SN gap report.

Regarding to Option 1 and Option 2, the comparisons are performed from the following two aspects:

- Tx side handling

If option1 is the correct understanding, according to the current specification, SDU1 of PDU Set 1 should not be included in the PDCP SN gap report. The FDC field will be set to the COUNT value of SDU2 of PDU Set1.

If option2 is the correct understanding, that is to say, the discarded PDCP SDU(s) which have been delivered by RLC to lower layers can be included in the PDCP SN gap report, then the issue is how to set the FDC filed. In the following Figure-2, besides the SDU1 of PDU Set1, there may be lots of discarded SDUs which had been delivered from RLC to low layer (In the box in the top left corner of Figure-2). The scope of determining the smallest COUNT value of the discarded PDCP SDU which had been delivered from RLC to low layer is not captured in the current specification, hence it is hard to decide how to set the FDC field. For example, SDU1 of PDU Set x had been discarded and delivered from RLC to low layer for a long time ago, it is doubt that set the COUNT value of SDU1 of PDU Set x into the FDC field.

Figure-2 Illustration on how to set the FDC filed in PDCP SN gap report

Observation 2: Compared with Option 1, for Option 2, it is not clear for the Tx UE to set the FDC field in the SN gap report.

- Rx side handling

For the Rx side, the parameter update procedure is as below [1]:

### 5.16.2 Receive operation At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall: - consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded; - if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs: - if RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report: - update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1; - if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s): - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before: - all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV; - if t-Reordering is running, and if RX_DELIV >= RX_REORD: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering.

Based on the above description, it is obvious that the RX_NEXT is updated based on the largest COUNT value in the PDCP SN gap report. How to set the FDC filed in the PDCP SN gap report will not impact the RX_NEXT. But for the RX_DELIVE update, it should judge if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s). Hence, how to set the FDC filed will impact the update of RX_DELIV. Take Figure-1 as an example, if the COUNT value of SDU1 of PDU Set 1 equals to RX_DELIV, if Option 1 is used, the RX_DELIV will not be updated upon receiving the PDCP SN gap report, and it is possible for the Rx side to deliver SDU1 of PDCP Set 1 to upper layer. However, if Option 2 is used, the RX_DELIV will be updated. Even if SDU1 of PDCP Set 1 is successfully transmitted to Rx side, since it is out of the Rx window, it will be discarded.

Observation 3: Compared with Option 1, for Option 2, it may discard the PDCP SDU which has been successfully received in Rx side since the RX_DELIV is updated based on PDCP SN gap report. This will cause different Rx UE behavior compared to legacy and may result resource waste.

Based on observation 2 and observation 3, we slightly prefer Option 1.

Proposal 2: RAN2 agrees that the discarded PDCP SDU(s) which have been delivered by RLC to lower layers should not be included in the PDCP SN gap report (Option1).

On the other hand, if majority companies can’t agree with proposal2, some further clarification on how to solve the issue in observation 2 should be added. Considering Rel-18 is frozen, it is preferred to leave it to UE implementation and add one NOTE to handle it.

Proposal 3: In case Option2 is finally agreed, RAN2 should add a NOTE to further clarify how to determine the smallest COUNT value associated with the discarded PDCP SDU(s).

Conclusion

According to the analysis in section 2, it is suggested that:

Observation 1: It is possible that more than one PDCP SDUs are discarded simultaneously, but only partial of them can trigger PDCP SN gap report according to the current specification.

Proposal 1: Suggest RAN2 to discuss whether the discarded PDCP SDU(s) which have been delivered by RLC to lower layers should be included in the PDCP SN gap report or not.

For the issue mentioned in Proposal 1, there are two observations:

Observation 2: Compared with Option 1, for Option 2, it is not clear for the Tx UE to set the FDC field in the SN gap report.

Observation 3: Compared with Option 1, for Option 2, it may discard the PDCP SDU which has been successfully received in Rx side since the RX_DELIV is updated based on PDCP SN gap report. This will cause different Rx UE behavior compared to legacy and may result resource waste.

Based on the above two observations, it is further proposed:

Proposal 2: RAN2 agrees that the discarded PDCP SDU(s) which have been delivered by RLC to lower layers should not be included in the PDCP SN gap report (Option1).

Proposal 3: In case Option2 is finally agreed, RAN2 should add a NOTE to further clarify how to determine the smallest COUNT value associated with the discarded PDCP SDU(s).

The corresponding TP of Proposal 2 and Proposal 3 are attached in Annex.

Reference

- TS38.323  NR; Medium Access Control (MAC) protocol specification (Release-18)

Annex

------------------------------------------TP for Proposal 2-------------------------------------------------------------

- 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s);

-	if more than one PDCP SDUs are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

NOTE：When compiling a PDCP SN gap report, the discarded PDCP SDU(s) should also satisfy the conditions for triggering the PDCP SN gap report.

------------------------------------------TP for Proposal 3-------------------------------------------------------------

- 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s);

-	if more than one PDCP SDUs are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting

NOTE:	It is up to UE implementation how to determine the smallest COUNT value associated with the discarded PDCP SDU(s).