3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500182

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Discussion on Enabling TX/RX for XR during RRM Measurements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, the following agreements were achieved on RRM measurement gaps/restrictions related enhancements.

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

In this contribution, we give our further considerations on other leftover issues.

Discussion

Specification impacts for DCI based solution

According to the RAN1/4 progress, specification impacts for DCI based solution include:

- For RRC: RRC parameters and new UE capability for DCI based solution should be introduced.

For example:

In RAN1#118bis meeting, it was agreed:

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer.

In RAN4#113 meeting, RAN4 replied the LS [1] with the following conclusion:

RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability.

- For MAC: The behavior of MAC entity during an activated measurement gap is captured in TS 38.321 [2] as follows. When explicit indication by DCI to skip a particular gap is received, handling of measurement gaps in MAC entity is not valid for the skipped gap. MAC specification needs to be updated with dynamic indication solution, e.g. the MAC entity doesn’t perform the handling of measurement gaps if an explicit indication by DCI to skip a particular gap is received.

- 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Considering DCI based solution is still under discussion, RAN2 can wait for RAN1/RAN4 further conclusions.

Proposal 1: With DCI based solution, RAN2 impacts include:

- For RRC: RRC parameters and new UE capability for DCI based solution should be introduced.

- For MAC: The handling of measurement gaps needs to be updated to skip a particular gap/restriction indicated by DCI based solution.

Need of semi-static solution

In RAN4 replied LS [1] it confirms DCI based solution is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability, where X is the minimum time offset between indication to skip and skipped measurement occasion.

Measurement gaps are periodic with integer periodicity, while XR traffic is also periodic but has non-integer periodicity. Therefore, the overlapping between measurement gaps and XR traffic can be determined in advance but inevitable, as shown in Figure 1.

Figure 1 An example of the overlapping between MG and data burst

In Figure 1, there are three overlapping occasions between measurement gaps and data burst. For the first two overlapping occasions, the UE is in inactive time of DRX. For the third overlapping occasion, the UE is in active time of DRX but the time offset between the start of PDCCH monitoring and measurement gap is 3.2ms which may be less than the minimum time offset X between indication to skip and skipped measurement occasion as mentioned in RAN4 LS [1]. Hence, for these three overlapping occasions, the network cannot use DCI based solution to skip measurement gap within active time of current DRX cycle. One may argue the network can send DCI based indication within active time of previous DRX cycle. But in this case measurement gaps are skipped in advanced. With DCI based solution, three and more indications via DCI are needed. While with semi-static solution only one indication is needed. Hence, it is not efficient to use DCI based solution in advance for this case.

Observation 1: The overlapping between measurement gaps and XR traffic can be determined in advance but inevitable if measurement gaps are integer periodic while XR traffic is non-integer periodic. And for the overlapping case, it is not feasible or not efficient to use DCI based solution to skip measurement gaps considering the network cannot send dynamic DCI based indication timely.

According to RAN4 discussion, DCI based solution is not supported for EN-DC and NE-DC scenarios [1] but it is FFS for NR-DC [3]. With DCI based solution, it is time limited to align the understanding of skipped measurement gap between MN and SN. With semi-static solution, the understanding of skipped measurement gap can be aligned between MN and SN via exchanging some info of semi-static solution. Hence, compared with DCI based solution, semi-static solution can be extended to DC scenarios smoothly.

Observation 2: Compared with DCI based solution, semi-static solution can be extended to DC scenario smoothly.

Furthermore, semi-static solution can ease UE implementation complexity as the UE can have the knowledge if the measurement gap is skipped or not and arrange the measurement timing in advance.

Observation 3: Semi-static solution can ease UE implementation complexity.

Semi-static solution may include RRC solution or MAC CE solution. MAC CE solution is a semi-static solution which can provide more dynamic indication than RRC solution. But DCI based solution has already been supported to skip RRM operation dynamically. It is enough to use RRC solution to skip RRM operation semi-statically.

Proposal 2: RAN2 to consider RRC solution to skip measurement gap occasions.

UE assistant information

In RAN2#117 meeting, RAN1 discussed UE assistance information for enabling TX/RX during RRM measurements. And the following agreement was made:

Agreement RAN1 agrees to send an LS to RAN4 (CC: RAN2) to convey the following information about UE assistance information (including the conclusion on UAI): 1. Overall Description: RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. 2. Actions: To RAN4: ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.. Final LS is approved in R1-2405736.

Considering UE assistance information related to measurements occasions may impact on RRM performance, RAN1 kindly asked RAN4 to decide whether or not to introduce any UE assistance information related measurement occasions [4]. In RAN4#112 meeting, RAN4 started the discussion on the objective and no agreement was made on UE assistance information related to measurements occasions. RAN2 can wait for RAN4’s conclusions on UE assistance information related to measurements occasions.

Proposal 3: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: With DCI based solution, RAN2 impacts include:

- For RRC: RRC parameters and new UE capability for DCI based solution should be introduced.

- For MAC: The handling of measurement gaps needs to be updated to skip a particular gap/restriction indicated by DCI based solution.

Observation 1: The overlapping between measurement gaps and XR traffic can be determined in advance but inevitable if measurement gaps are integer periodic while XR traffic is non-integer periodic. And for the overlapping case, it is not feasible or not efficient to use DCI based solution to skip measurement gaps considering the network cannot send dynamic DCI based indication timely.

Observation 2: Compared with DCI based solution, semi-static solution can be extended to DC scenario smoothly.

Observation 3: Semi-static solution can ease UE implementation complexity.

Proposal 2: RAN2 to consider RRC solution to skip measurement gap occasions.

Proposal 3: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Reference

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, RAN4#113.

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- R4-2420103	, WF on RRM requirements for NR_XR_Ph3, RAN4#113.

- R1-2405736, LS on UE assistance information, RAN1#117