3GPP TSG-RAN WG2 Meeting #129                                                        	R2-2500183

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Consideration on LCP enhancement

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on LCP enhancement including LCP additional priority configuration, the PBR issue in resource allocation and intra-UE prioritization, our views are also given in this paper.

Discussion

Agreements on LCP enhancements - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

Based on above agreements in RAN2#128 meeting[1], it has agreed that the additional priority of LCH will be applied to both the first round and the second round of the LCP procedure as baseline, but the other issues are still under discussion, more details are discussed here:

How to configure the additional priority?

Generally, there are two options to configure the additional priority:

- Option 1: Pre-configuration.

- Option 2: Configured by RRC signalling.

For Option 1, the priority to LCH with LCH priority-adjusted data could be preconfigured as a fix value, such as the highest priority. This option is not flexible. Furthermore, in some cases, it is not reasonable. For example, if there is important MAC CE or other data which is more important than the delay-critical data, it is not reasonable to always prioritize the LCH priority-adjusted data. Hence, it is preferable to let it configurable as in Option 2. The network can configure the additional priority for the LCH by RRC signaling, the specific value of additional priority could be considered as network implementation.

Proposal 1: The additional priority and the corresponding remaining time threshold for one LCH can be configured by NW through dedicated RRC signaling.

Besides, only one additional priority and its remaining time threshold is sufficient for LCP. Otherwise, if two or more additional priority values and its remaining thresholds are introduced, multiple additional priorities triggering to a LCH may be performed. In some cases, the additional priority triggering for the high remaining time threshold may be not used before resource allocation, the triggering procedure will be wasteful. The network may configure a reasonable additional priority and the remaining time threshold to avoid the unnecessary early additional priority triggering.

Proposal 2: One additional logical channel priority and its corresponding remaining time threshold will be configured by network.

Whether to enhance the Bj/PBR for LCH using additional priority

According to legacy procedure as below, the logical channels selected for the UL grant are allocated resources in a decreasing priority order with Bj >0 (step 1), and Bj will be updated (step 2), then logical channels selected are allocated remaining resources in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first (step 3).

5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

Based above legacy procedure, the logical channel with Bj > 0 will be selected in the first round of resource allocation. However, considering that the additional priority of LCH will give priority-adjusted data more opportunity to be scheduled, after using of the last UL grant, it is very possible that the there is priority-adjusted data with Bj <=0 before the first round of resource allocation, to avoid the priority-adjusted data missing this UL grant and to be obsolete, it is better not to consider the Bj limitation and make the priority-adjusted data to be transmitted as soon as possible.

Generally, the MAC entity will stop allocating resource to a LCH if its PBR could be met in the first round resource allocation. However, there may be still UL priority-adjusted data without resource after the PBR is met, if this type of data is not allocated resource, maybe it will become obsolete data. One option is to increase the PBR, such as setting it as infinity. If going to this option, based on leagcy procedure, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel, which will also inlcude the non priority-adjusted data. Hence, this option is not fair to the other LCH with higher priority than the legacy priority of the LCH. Thus, even if there is remained priority-adjusted data in the first round of resource allocation, it is better not to set PRB as infinity, the specific data size allocated resource could be left to UE implementation, after the first round resource allocation, Bj is already allowed to be negative based on legacy procedure. The remained delay-critical data may also be allocated resource in the second round.

Hence, considering the resource allocation procedure, it is proposed:

Proposal 3: When allocating resource for LCH with priority-adjusted data in the first round, it is not retricted by Bj>0.

Intra-UE prioritization

According to the current specification, the resource overlapping will occur among different types of transmissions, such as, the PUSCH duration of the uplink grant overlaps with a PUSCH duration of another uplink grant, the PUCCH resource for the SR transmission occasion overlaps with the PUSCH duration of an uplink grant received. For UL grant, for the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU. If the additional priority of LCH with delay critical data is highest priority among the LCH multiplexed in MAC PDU in UL grant, it is reasonable that the priority of UL grant should be the additional priority.  For SR, the priority may be determined by the priority of logical channel triggering SR, hence, the additional priority could be appplied to SR if there is LCH priority-adjusted data in the corresponding logical channel. Then, when the overlapping of SR resource and the other resource is happened, the priority of SR should also consider as the additional priority of the LCH.

Proposal 4: The additional priority of LCH could be considered for the priority of the UL grant in intra-UE prioritization.

Proposal 5: The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

In legacy MAC procedure, the priority of UL resource for a MAC PDU could be determined by the highest priority among priorities of logical channels in the MAC PDU. Considering the the UE could fallback to legacy prority in the secnod round of resrouce allocation, then two pirority values for a logical channel are applied in a MAC PDU. In our view, for a MAC PDU, the priority of a logical channel could be determined by higher priority between the legacy pirority and the additional priority to meet the XR QoS requirement.

Proposal 6：If the data with additional priority and legacy priority for a logical channel are both included in a MAC PDU, the priority of  a logical channel in a MAC PDU is determined by the higher priority between the legacy pirority and the additional priority.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The additional priority and the corresponding remaining time threshold for one LCH can be configured by NW through dedicated RRC signaling.

Proposal 2: One additional logical channel priority and its corresponding remaining time threshold will be configured by network.

Proposal 3: When allocating resource for LCH with priority-adjusted data in the first round, it is not retricted by Bj>0.

Proposal 4: The additional priority of LCH could be considered for the priority of the UL grant in intra-UE prioritization.

Proposal 5: The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

Proposal 6：If the data with additional priority and legacy priority for a logical channel are both included in a MAC PDU, the priority of  a logical channel in a MAC PDU is determined by the higher priority between the legacy pirority and the additional priority.

Reference

- RAN2#128 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei