3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500184

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Consideration on DSR Enhancement

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss and give our views on the issues on DSR enhancement including DSR MAC CE format, the co-existing of new DSR and legacy DSR, and the handling of DSR when the PUSCH resource is limited.

Discussion

The enhanced DSR agreements in RAN2#128 [1] are as follows.

RAN2#128 agreements on DSR enhancements: Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In the following, more details will be discussed:

The DSR MAC CE format

The legacy DSR MAC CE is as follows, it is not easy to extend legacy DSR to include buffer size information for different remaining time since the number of remaining time of each LCG may be various, it has been agreed to introduce a new MAC CE to enhance DSR.

Figure-1: DSR MAC CE

RAN2#127bis agreement: A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

In RAN2#127bis meeting, as above, one-bit indication in legacy Reserved bit is used to indicate whether a certain/further pair of remaining time information and buffer size information is present for the associated LCG. In data buffer, considering the data associated to a reporting threshold may be empty, it is preferable to introduce an indication to indicate whether a further pair of remaining time information and buffer size information is present, which could make the enhanced DSR report more flexible from the view of bits saving. Another way is , new bits will be introduced in new DSR MAC CE to indicate whether the remaining time and buffer size for each reporting threshold of a LCG is included or not, which could give a fixed bit size for enhanced DSR MAC CE, but more bits will be introduced for this MAC CE, it is a complex design and not beneficial from the view of bit size.

Proposal 1: The reserved bit in Rel-18 DSR MAC CE is used to indicate whether a further pair of remaining time information and buffer size information is present in the enhanced DSR MAC CE.

The co-existing of new DSR and legacy DSR

In RAN2#127bis meeting, one FFS issue as follows needs to be further discussed:

RAN2#127bis agreement: 确饿哦e FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

The legacy DSR is reported per LCG based on the remainingTimeThreshold configured per LCG, hence the new DSR is also configured per LCG. It is possible that network could configure some LCG with legacy DSR including only one remaining time threshold and configure other LCG with new DSR including multiple reporting thresholds, which depends on the network requirement to the delay granularity for each LCG.

Proposal 2: It is network implementation to configure either legacy remainingTimeThreshold for legacy DSR or new reporting threshold(s) for new DSR to the same LCG.

For UE side, if only the LCG(s) configured with legacy remainingTimeThreshold triggers the delay status reporting, the legacy DSR MAC CE will be used. If the LCG(s) configured with legacy remainingTimeThreshold and the LCG(s) configured with the new reporting threshold(s) trigger the delay status report simultaneously, considering the new DSR could cover the report of only one pair of remaining time information and buffer size information for legacy DSR, it is preferable to use the new DSR for all the triggered LCH. In the new DSR, for the LCG configured with remainingTimeThreshold, the introduced indication will be set based on new DSR format, then network could interpret the new DSR with same options and get the delay status information for the LCG without any mistake.

Proposal 3: The UE will use new DSR in case that at least one LCG configured with new reporting threshold(s) needs to report the delay status.

Handling of DSR when the PUSCH resource is limited

For legacy procedure, if PUSCH resource is not big enough to include the BSR, a truncated BSR will be included. For enhanced DSR, three options as follows could be considered for further discussion. For option 2, a new LCID value will be introduced for truncated DSR. For option 3, the UE needs to re-construct a R18 DSR, the buffer size information for non-delay critical data in Rel-19 DSR may be missed. Anyway, RAN2 needs to perform down selection among these options.

- Option 1, there is no spec impact, UE will transmit Rel-19 DSR in the following UL grant.

- Option 2, a truncated DSR will be introduced for Rel-19 DSR

- Option 3, a new procedure is introduced, such as fallback to Rel-18 DSR.

Proposal 4: RAN2 to discuss and perform the down selection among the below three options for limited PUSCH resource case:

- Option 1, there is no spec impact,  UE will transmit Rel-19 DSR in the following UL grant;

- Option 2, a truncated DSR will be introduced for Rel-19 DSR;

- Option 3, new procedure is introduced, such as fallback to Rel-18 DSR.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The reserved bit in Rel-18 DSR MAC CE is used to indicate whether a further pair of remaining time information and buffer size information is present in the enhanced DSR MAC CE.

Proposal 2: It is network implementation to configure either legacy remainingTimeThreshold for legacy DSR or new reporting threshold(s) for new DSR to the same LCG.

Proposal 3: The UE will use new DSR in case that at least one LCG configured with new reporting threshold(s) needs to report the delay status.

Proposal 4: RAN2 to discuss and perform the down selection among the below three options for limited PUSCH resource case:

- Option 1, there is no spec impact,  UE will transmit Rel-19 DSR in the following UL grant;

- Option 2, a truncated DSR will be introduced for Rel-19 DSR;

- Option 3, new procedure is introduced, such as fallback to Rel-18 DSR.

Reference

- RAN2#128 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei