3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500186

Athens, GR, 17th – 21st February 2025

Source:	CATT

Title:	Discussion on XR Rate Control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, regarding to XR rate control, the below agreements were agreed [1]:

Agreements on XR rate control

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level.

- gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

- Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

- RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

- RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE.

- FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In the current contribution, we further discuss the below leftover issues on XR rate control:

- Details on per QoS flow level.

- Bit rate indication enhancement.

- Assistance information to gNB.

- FFS on UL MAC CE rate query.

Discussion

Details on per QoS flow level

There are two options recorded in the chairman notes of the last RAN2 meeting. One is to introduce per-QoS flow granularity in the recommended bitrate MAC CE, and the other is to reuse the legacy per DRB granularity in the recommended bitrate MAC CE with introducing RRC signalling to show which QoS flows are subject to bitrate adaptation. Both options are feasible, here we are focused on analysing the potential impacts to support the related options according to online comments. For the solution to introduce per-QoS flow granularity in the recommended bitrate MAC CE, the overhead of the new MAC CE will be increased. Since in the RAN AS layer, the QoS flow is invisibility after QoS to DRB mapping. To identify specific QoS flow in the new MAC CE, besides the QoS flow ID, the PDU session ID should also be included which means the overhead compared to legacy will increase dramatically. The other concern is whether the CU-DU split case can be supported. On the other hand, for the RRC indication with per-DRB MAC CE solution, the issue that how to guarantee the sequence of RRC signalling and MAC CE should be further discussed. Considering there was one related LS[2] had been sent to SA4 and RAN3 in RAN2#128 meeting, it is preferred to postpone this issue until the LS reply is received.

Proposal 1: RAN2 postpones discussing the details on per QoS flow level solution until LS reply received.

Bit rate indication enhancement

In RAN2#127bis meeting, one LS was sent to SA4 asking about range/granularity which is required[3]. Before RAN2#128 meeting, the reply LS are excerpted below [4] :

The SA4 reply is as follows:

The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps.

The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

With the green marked part above, it is cleared that the appropriate range for XR service is from 100kbps to 40Mbps, and the granularity of uplink bit rates is generally preferred as 1%. Hence, 40Mbps*1%=400kbps is the preferred interval. The below new table (7 bits) is designed to handle the Rel-19 XR rate control case:

| Index | Recommended Bit Rate value [kbit/s] | Index | Recommended Bit Rate value [kbit/s] | Index | Recommended Bit Rate value [kbit/s] | Index | Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Note | 32 | 12500 | 64 | 25300 | 96 | 38100 |
| 1 | 100 | 33 | 12900 | 65 | 25700 | 97 | 38500 |
| 2 | 500 | 34 | 13300 | 66 | 26100 | 98 | 38900 |
| 3 | 900 | 35 | 13700 | 67 | 26500 | 99 | 39300 |
| 4 | 1300 | 36 | 14100 | 68 | 26900 | 100 | 39700 |
| 5 | 1700 | 37 | 14500 | 69 | 27300 | 101 | 40100 |
| 6 | 2100 | 38 | 14900 | 70 | 27700 | 102 | Reserved |
| 7 | 2500 | 39 | 15300 | 71 | 28100 | 103 | Reserved |
| 8 | 2900 | 40 | 15700 | 72 | 28500 | 104 | Reserved |
| 9 | 3300 | 41 | 16100 | 73 | 28900 | 105 | Reserved |
| 10 | 3700 | 42 | 16500 | 74 | 29300 | 106 | Reserved |
| 11 | 4100 | 43 | 16900 | 75 | 29700 | 107 | Reserved |
| 12 | 4500 | 44 | 17300 | 76 | 30100 | 108 | Reserved |
| 13 | 4900 | 45 | 17700 | 77 | 30500 | 109 | Reserved |
| 14 | 5300 | 46 | 18100 | 78 | 30900 | 110 | Reserved |
| 15 | 5700 | 47 | 18500 | 79 | 31300 | 111 | Reserved |
| 16 | 6100 | 48 | 18900 | 80 | 31700 | 112 | Reserved |
| 17 | 6500 | 49 | 19300 | 81 | 32100 | 113 | Reserved |
| 18 | 6900 | 50 | 19700 | 82 | 32500 | 114 | Reserved |
| 19 | 7300 | 51 | 20100 | 83 | 32900 | 115 | Reserved |
| 20 | 7700 | 52 | 20500 | 84 | 33300 | 116 | Reserved |
| 21 | 8100 | 53 | 20900 | 85 | 33700 | 117 | Reserved |
| 22 | 8500 | 54 | 21300 | 86 | 34100 | 118 | Reserved |
| 23 | 8900 | 55 | 21700 | 87 | 34500 | 119 | Reserved |
| 24 | 9300 | 56 | 22100 | 88 | 34900 | 120 | Reserved |
| 25 | 9700 | 57 | 22500 | 89 | 35300 | 121 | Reserved |
| 26 | 10100 | 58 | 22900 | 90 | 35700 | 122 | Reserved |
| 27 | 10500 | 59 | 23300 | 91 | 36100 | 123 | Reserved |
| 28 | 10900 | 60 | 23700 | 92 | 36500 | 124 | Reserved |
| 29 | 11300 | 61 | 24100 | 93 | 36900 | 125 | Reserved |
| 30 | 11700 | 62 | 24500 | 94 | 37300 | 126 | Reserved |
| 31 | 12100 | 63 | 24900 | 95 | 37700 | 127 | Reserved |

Proposal 2: RAN2 adapts the new values for Bit Rate field table (7 bits) for Rel-19 XR rate control.

Assistance information to gNB

For XR service, not all the related QoS flows have different codec choice. For example, speech and video flows may have adaptive codec options. But for rendering, there may be not. Before the gNB sends new introduced recommended bit rate MAC CE to UE, the gNB should know which QoS flow/DRB are subject to adaptive bit rate. In RAN2#128 meeting, the below agreement and FFS are recorded:

Agreements on XR rate control

- gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

For the FFS part, there are two solutions raised: one is UE based and the other is CN based. The relationship between the two is complementary. In case the CN doesn’t support the bit rate exposure, the UE based approach can be used. From this point of view, The UE based approach that UE using one UL RRC signalling to let the gNB get the information of which QoS flow(s) can be throttled should be supported in Rel-19 XR.

Proposal 3: RAN2 agrees that UE using one UL RRC signalling to let the gNB get the information of which QoS flow(s) can be throttled in Rel-19 XR.

FFS on UL MAC CE rate query

In the last RAN2#128 meeting, some companies raised to introduce the UL MAC CE rate query mechanism for different scenario. One company raised that before the UL congestion happens, UE can initiate a bit rate query. Then the gNB can identify which flows or logical channels are subject to rate adaptation. This part is related to what had discussed in “Assistance information to gNB” sector, in case that one UL RRC signalling can be used instead, that is to say, there is no need to introduce the query MAC CE in the uplink Uu interface before the UL congestion happens. Another company raised that after the UL congestion had been solved, the bitrate should be recovered to normal status. While the peer UE may send a query message to its local gNB to check if the recommended bit rate can be provided. This is to avoid ping pong in tuning where the increase of the bitrate in one end of the connection is immediately followed by a request from the other end to reduce the bit rate.  Furthermore, to limit UE sending frequent query messages, a prohibit timers can be configured by the gNB. The query procedure is to handle the Video and VoIP service, in the current Rel-19 XR service, it is not clear the benefit to introduce this query procedure. Hence, the query message and related prohibit timer is not supported in Rel-19 XR UL rate control.

Proposal 4: Query message and related prohibit timer is not supported in Rel-19 XR rate control.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: RAN2 postpones discussing the details on per QoS flow level solution until LS reply received.

Proposal 2: RAN2 adapts the new values for Bit Rate field table (7 bits) for Rel-19 XR rate control.

Proposal 3: RAN2 agrees that UE using one UL RRC signalling to let the gNB get the information of which QoS flow(s) can be throttled in Rel-19 XR.

Proposal 4: Query message and related prohibit timer is not supported in Rel-19 XR rate control.

Reference

- R2-2410915	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	report

- R2-2411218 LS on XR UL bit rate control

- R2-2409273	LS on appropriate range and granularity of bit rate adaptation for XR applications	RAN2	LS out	NR_XR_Ph3-Core	To:SA4

- S4-242182 Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications