3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500188

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Discussion on the Control Plane Procedures

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting and RAN2#128 meeting, the control plane of multi-hop Relay were discussed. In this contribution, we will further discuss the following issues based on approach 1:

- How to establish the E2E connection between the Remote UE and NW?

- Whether assistant information is needed for QoS split？

- How to perform the system information forwarding?

- How to perform the paging forwarding?

- How to handle RLF?

Discussion

E2E connection setup in approach 1

In RAN2#128 meeting, regarding to approach 1, it was agreed the figure and description under P1 of [5] serves as baseline. The figure in [5] is shown as below:

Figure-1 Signaling flows of approach 1

Based on the above signaling flow, there are still some FFS issues in step 1, step 2 and step 3. In the following, the FFS issues will be discussed one by one.

Step 1

In step 1, according to the description in [5], there is one FFS:

[FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.]

This FFS is tightly depends on the reply of LS [4], the question in the LS is as below:

- Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

In [6], SA2 replied that:

- Question from RAN2: Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message. - Answer from SA2: Yes. …… SA2 also discussed behaviour of Intermediate UE-to-Network Relay related to Model B discovery that Intermediate UE-to-Network Relay is not required to setup PC5 link with the relay from which the discovery response message is received before forwarding the discovery response message.

From the LS reply, it is obvious that for Model A, PC5-RRC connection has already been established between Relay UEs during discovery procedure. There is no need to support PC5-RRC connection establishment between adjacent Relay UEs after transmission of the first RRC message in step 2. For Model B, there may be no PC5-RRC connection established during discovery procedure. But in [7], it stated that when Remote UE sends DCR to the selected first Relay UE, the DCR message includes the path information. Hence, when to trigger the PC5-RRC connection setup can be handled by upper layer. RAN2 does not need to discuss this timing issue.

- 5G ProSe Intermediate UE-to-Network Relay sends a unicast Direct Communication Request message to the next 5G ProSe Intermediate UE-to-Network Relay or the UE-to-Network Relay according to the path information in the received Direct Communication Request message. The Direct Communication Request message additionally includes: - Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate UE-to-Network Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure. - QoS Info: End to End QoS Info and the remaining QoS Info of hops from the Intermediate UE-to-Network Relay to the network as specified in clause 5.6.2.

Proposal 1:  In approach 1, when to establish the PC5-RRC connection between adjacent UEs can be left to upper layer, and this FFS can be closed.

Step 2

In step 2, it is obvious that the Remote UE sends RRCSetupRequest to the first Relay UE using a specified PC5 Relay RLC channel configuration. And then, the Relay UE can send SUI to NW. But it is still FFS which Relay UE sends the SUI.

[FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.]

Regarding to the above FFS, there are two alternatives:

- Alternative 1: Only the first Relay UE sends the SUI to NW;

- Alternative 2: Only the last Relay UE sends the SUI to NW.

No matter which alternative is used, NW should aware all Relay UEs in the multi-hop path in order to provide the proper SRAP configuration for each Relay UE. If Alternative 1 is used, the NW can aware all the Relay UEs in the multi-hop path since the first Relay UE has already enter CONNECTED state before sending the SUI. Hence, upon receiving the SUI sent by the first Relay UE, the NW aware all Relay UEs in the multi-hop path. But if Alternative 2 is used, the NW can only aware the last Relay UE, it cannot aware the Intermediate Relay UEs in the multi-hop path. In order to let NW can aware all Relay UEs in the multi-hop path, enhancement is needed, e.g., the last Relay should get the list of Relay UEs in the multi-hop path and report it to NW. This will introduce additional specification effort. Hence, Alternative 1 is slightly preferred.

Proposal 2: In approach 1, only the first Relay UE needs to send SUI to NW for the Remote UE local ID allocation.

Proposal 3: In approach 1, upon receiving SUI sent from the first Relay UE, the NW can allocate Local L2 ID for Remote UE and configure the mapping between Remote UE’s SRB0 and its egress RLC channel for each Relay UE in the multi-hop path via RRCReconfiguration message.

Step 3

Regarding to step 3, there is one FFS in [5]:

[FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2].

In step 2, if the NW provides the ingress/egress channel mapping for SRB1 to each Relay UE, the Relay UE can establish the RLC channel for SRB1. If the NW does not provide the channel mapping, the Relay can apply the default configuration of SL-RLC1.

Proposal 4: In approach 1, upon receiving RRCReconfiguration message from NW, if it contains the SRB1 related RLC channel configuration, Relay UE should apply it; otherwise, default configuration of SL-RLC1 can be used.

QoS split for Multi-hop relay

Regarding to the QoS split, in RAN2#128 meeting, the following agreement was reached for approach 1:

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

Based on the above agreement, it should further discuss whether the assistance information from the UE(s) is needed in order to assistant the NW to perform QoS split.

In Rel-17 U2N Relay, the NW performs the QoS split based on its implementation, no UE assistant information is reported. In case of multi-hop, although the number of hops is higher than Rel-17 U2N relay, it is still unnecessary to report assistance information. The main reason is that considering both the remote UE and the Relay UE may be moving, the PC5 link quality between two UEs is not stable and can be changed dynamically. However, the QoS split is semi-static. Hence, the PC5 link channel quality is not very trustable when performing QoS split.

Proposal 5:  In approach 1, UE assistance information is not needed for QoS split.

System information forwarding

In legacy Rel-17 U2N Relay, system information forwarding procedure is as below：

Figure-2 System information forwarding for Rel-17 U2N Relay

In RAN2#128 meeting, regarding to the system information forwarding in case of multi-hop, the following agreements were reached:

Agreement:

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

With the introduction of multi-hop, the similar system information forwarding procedure in Rel-17 U2N relay can be reused. The basic procedure can be as below: the Remote UE can send the request SIB(s) to the first Relay UE, and then, the first Relay UE can forward the request to its parent Relay UE, and its parent Relay UE can further forward it until the last Relay UE receives it.

Based on the above basic procedure, the following enhancement can be considered in multi-hop scenario:

- When each Intermediate Relay UE (including the first Relay UE) performs the SIB request forwarding, it can include the SIB(s) requested by both its child UE(s) and itself.

- If the parent Relay UE has stored the valid SIB(s) requested by the child Relay UE or the Remote UE, it can forward it to its child Relay UE or the Remote UE directly and no need to continue the forwarding of SIB request to its parent Relay.

Proposal 6: In approach 1, upon receiving the request SIB(s) from its child UE(s), the Intermediate Relay UE (including the first Relay UE) can forward the requested SIB(s) from its child UE(s) and the requested SIB(s) of itself to its parent Relay UE if it has no valid corresponding SIB(s).

Proposal 7:  In approach 1, the Intermediate Relay UE is allowed to forward the valid SIB(s) requested by its child UE(s) directly (rather than retrieving it from the last Relay UE) if it has stored the valid SIB(s).

Furthermore, in Rel-17, unsolicited SIB1 forwarding was supported in Rel-17 U2N Relay. Hence, in Rel-19 multi-hop, it should also discuss whether unsolicited SIB1 forwarding still needs to be supported. In our understanding, each Relay UE can support unsolicited SIB1 forwarding.

Proposal 8:  In approach 1, unsolicited SIB1 forwarding can be supported for each Relay UE.

Paging forwarding

In legacy ReL-17, IDLE/INACTIVE Remote UE can notify its UE ID (e.g., 5G-S-TMSI or I-RNTI) to the U2N Relay UE. And the U2N Relay UE will monitor the corresponding PO and forwarding the paging to the Remote UE if the paging message contains the Remote UE ID.

In Rel-19, with the introduction of multi-hop, the similar procedure can be reused. Remote UE should inform the paging related information (which including the Remote UE paging identity and paging cycle) to the first Relay UE. Each Relay UE can forward the Remote UE paging related information and its own paging related information to its parent Relay UE.

Proposal 9: In approach 1, the Remote UE can inform its paging related information (which includes paging identity and paging cycle) to the first Relay UE.

Proposal 10: In approach 1, upon receiving the paging related information from its child UE(s), the Intermediate Relay UE(including the first Relay UE) can forward the received paging related information from its child UE(s) and the paging related information of itself to its parent Relay.

RLF handling

With the introduction of multi-hop, there are three kinds of links as below:

- Sidelink 1: The sidelink between Remote UE and first Relay UE;

- Sidelink 2: The sidelink between Relay UEs;

- Uu link: The Uu link between the last Relay UE and gNB.

Figure-3  Multi-hop relay

When analyzing the RLF handling, we should discuss based on the above three kinds of links:

Table-1 RLF handling for multi-hop Relay

| Failed link | Legacy Rel-17 behaviors | Rel-19 behaviors |
| --- | --- | --- |
| Sidelink 1 | Remote UE detected PC5 RLF, key behaviors: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; - If Remote UE is in CONNECTED, initiate RRC connection re-establishment. Relay UE detected PC5 RLF: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; If Relay UE is in CONNECTED, send SUI to gNB. | Remote UE detected PC5 RLF: same as Rel-17 U2N Remote UE. First Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Sidelink 2 | N/A | First Relay UE detected PC5 RLF: enhancement is possible. Send NotificationMessageSidelink message to Remote UE to indicate the failure between first Relay UE and Intermediate Relay (in case of more than 2 Relays) or between first Relay UE and last Relay UE (in case of only 2 Relays). Last Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Uu link | Relay UE detected Uu RLF: - Send NotificationMessageSidelink message to remote UE. | Relay UE detected Uu RLF: enhancement is possible. - Send NotificationMessageSidelink message to its child Relay UE and its child relay UE should forward it the Remote UE. |

Based on the analysis in the above table, the following are proposed:

Proposal 11: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 12: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay .

Proposal 13: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 14: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

Proposal 15: In approach 1, when the last Relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child Relay UE, and its child Relay UE can forward it to the Remote UE.

Conclusion

According to the analysis in section 2, for Approach 1, it is proposed:

RRC connection establishment procedure

Proposal 1:  In approach 1, when to establish the PC5-RRC connection between adjacent UEs can be left to upper layer, and this FFS can be closed.

Proposal 2: In approach 1, only the first Relay UE needs to send SUI to NW for the Remote UE local ID allocation.

Proposal 3: In approach 1, upon receiving SUI sent from the first Relay UE, the NW can allocate Local L2 ID for Remote UE and configure the mapping between Remote UE’s SRB0 and its egress RLC channel for each Relay UE in the multi-hop path via RRCReconfiguration message.

Proposal 4: In approach 1, upon receiving RRCReconfiguration message from NW, if it contains the SRB1 related RLC channel configuration, Relay UE should apply it; otherwise, default configuration of SL-RLC1 can be used.

QoS split

Proposal 5:  In approach 1, UE assistance information is not needed for QoS split.

System information forwarding

Proposal 6: In approach 1, upon receiving the request SIB(s) from its child UE(s), the Intermediate Relay UE (including the first Relay UE) can forward the requested SIB(s) from its child UE(s) and the requested SIB(s) of itself to its parent Relay UE if it has no valid corresponding SIB(s).

Proposal 7:  In approach 1, the Intermediate Relay UE is allowed to forward the valid SIB(s) requested by its child UE(s) directly (rather than retrieving it from the last Relay UE) if it has stored the valid SIB(s).

Proposal 8:  In approach 1, unsolicited SIB1 forwarding can be supported for each Relay UE.

Paging forwarding

Proposal 9: In approach 1, the Remote UE can inform its paging related information (which includes paging identity and paging cycle) to the first Relay UE.

Proposal 10: In approach 1, upon receiving the paging related information from its child UE(s), the Intermediate Relay UE(including the first Relay UE) can forward the received paging related information from its child UE(s) and the paging related information of itself to its parent Relay.

RLF handling

Proposal 11: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 12: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay .

Proposal 13: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 14: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

Proposal 15: In approach 1, when the last Relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child Relay UE, and its child Relay UE can forward it to the Remote UE.

Reference

- RP-242349  New WID on NR sidelink multi-hop relay

- TR23700-03   Study on system enhancement for Proximity based Service (ProSe) in the 5G System (5GS) Phase 3

- S2-2407207   LS on security considerations for 5G ProSe multi-hop relays

- R2-2411184  LS to SA2 on relay discovery announcement

- R2-2410006  Report of [POST127][402][Relay] Multi-hop relay control plane

- S2-2501334  Reply LS on relay discovery announcement (S2-2500045/R2-2411184)

- TS23.304  Proximity based Services (ProSe) in the 5G System (5GS)