3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500189

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Intra-gNB Service Continuity for Multi-hop U2N Relay

Agenda Item:	8.13.4

Document for:	Discussion and Decision

Introduction

In this contribution, based on the agreements at RAN2#128 meeting as in [1], the details on intra-gNB service continuity for multi-hop U2N relay will be further discussed for Scenario A/B/C/D.

Discussion

Remaining issue on First prority scenarios

In RAN2#128 meeting, the following agreements are achieved for scenario A/B:

Agreements: - For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused. - For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching. Spec impact will be minimised. - For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE. Agreements for baseline control plane model: - The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching. - Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay gUE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. - The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure. - The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching: - Event Y2 - Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. Agreements: - Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300. - Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

One remaining issue is about the data lossless in scenarios A/B.

For data lossless issue, it is necessary to review the history of discussion and legacy procedure for Rel-17 intra-gNB path switch with one U2N serving relay node firstly.

Figure-1 The PDCP SDU transmission before intra-gNB I2D path switch

According to the history discussion, the main issue was the potential data loss from source relay UE to network during intra-gNB I2D path switch. For example, as shown in Figure-1, before path switching, the remote UE may send the SDU 1to Relay UE and receive ACK for SDU1 from Relay UE, then the remote UE will believe that SDU 1 has been transmitted successfully. However, the relay UE may receive NACK for SDU1 from gNB, then the issue is how to keep data lossless transmission during intra-gNB I2D path switching procedure.

For DL, UE will trigger the PDCP status report after performing the PDCP re-establishment or data recovery, then, the network could retransmit the data based on the network implementation. Hence, it was agreed that there is no impact to spec in Rel-17 path switching procedure for DL in RAN2#116e meeting.

For UL, in RAN2#116e/116bis-e meeting, it was agreed that the legacy PDCP re-establishment or data recovery in UL should be performed by the Remote UE during path switch if gNB configures it, and no spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation).

RAN2#116-e agreements: Proposal 24 (modified): The legacy PDCP re-establishment or data recovery in UL should be performed by the Remote UE during path switch if gNB configures it. Proposal 25: No spec impact is required for DL lossless transmission during path switch. RAN2#116bis agreements: No spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation).

Observation 1: For data lossless, there is no spec impact introduced for Rel-17 intra-gNB I2D or D2I path switching in DL/UL.

For Rel-19 Intra-gNB multi-hop indirect to direct path switching, multi-hop source relay nodes are introduced, the issue on data loss caused by the source multi-hop relay to network also exists. For DL, the legacy procedure based on PDCP status report could be reused to keep data lossless transmission. For UL, considering the multi-hop links could be maintained by network, and the source relay node is in “direct/indirect” RRC Connected mode and could have RRC connection with network, so it is also feasible to leave it to network implementation to keep UL data lossless transmission.

Proposal 1: For DL data lossless, the legacy procedure based on PDCP status report could be reused for scenario A/B, there is no specification impact foreseen.

Proposal 2: For UL, the data loss in multi-hop relay node is corner case during path switching, the data lossless could be guaranteed by network implementation, there is no specification impact foreseen.

Second prority Scenarios

In this section, the approach 1 in multi-hop relay CP discussion is considered as baseline.

Firstly, the signaling flows and the details for selected steps of scenario C and scenario D are discussed; and then, the data lossless issue related to both scenario C and D is discussed.

Signaling flow and details of Scenario C

Similar as the signaling flow of Rel-17 Intra-gNB U2N Relay D2I path switching, the signaling flow of Scenario C is as below:

Figure-2: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from direct to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details depend on CP discussion.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop relay path.

9.	The data path is switched from direct path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the associated PC5 link for RRCreconfiguration message transmission if needed.

Proposal 3: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Based on the above Figure-2, most of the steps are same as the Intra-gNB U2N Relay D2I path switching. Hence, in the following, mainly step 1/3/4/5 will be further discussed.

Step 1

For scenario C, the WID already limit the path switching to a target multi-hop indirect path consisting of the Last Relay UE in “direct” RRC Connected mode and all the other Intermediate Relay(s) in “indirect” RRC Connected mode to the same cell, which means that the network needs to ensure the target Last/Intermediate/First Relay UE will be under the control of network, then the RRCReconfiguration message could be transmitted to each relay UE in the target multi-hop path.

Maybe, UE could report more information on the neighbor multi-hop relay path, such as the collected neighbor multi-hop relay path information and whether the relay UE is in RRC connected mode or not to help network to find a suitable target path, furthermore, the final decision on selecting target path is better to be made by gNB other than UE, gNB will determine multi-hop relay path, single-hop relay path or direct path is used based on the information reported from remote UE and relay UE.

Proposal 4: UE could report the collected multi-hop relay information to network to help network select a suitable target path.

Proposal 5: The network will determine and configure the suitable target path (maybe multi-hop relay path) to UE.

In legacy Rel-17 intra-gNB D2I path switching, the measurement event Y1/Y2 could be reused to let network understand the channel status of remote UE and determine whether a path switching is needed. Besides some potential enhancement, such as above proposal, UE reporting the neighbor multi-hop relay information to help network select the suitable path. For Rel-19 intra-gNB direct to multi-hop indirect path switching, the legacy measurement event could be reused except the description on candidate L2 U2N Relay UE will be revised to include the candidate First Relay UE.

Event Y1: PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) Event Y2: Candidate L2 U2N Relay UE becomes better than threshold

Proposal 6: The UE will perform the legacy measurement event Y1/Y2 except the description on candidate L2 U2N Relay UE in event Y1/Y2 should be revised to include the candidate First Relay UE.

Step 3/4/5

Different from legacy intra-gNB direct to one-hop indirect path, the target path includes extra First Relay UE and Intermediate Relay UE(s). Since the First/Intermediate Relay UE already has indirect RRC connection to the network, it is natural for network to send RRC reconfiguration message as in step 4/5 in Figure-2, which include at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Then, the First/Intermediate Relay UE could transfer the data from remote UE to Last Relay UE.

Proposal 7: For Scenario C, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Signaling flow and details of Scenario D

Clarification to Scenario D

Besides the general case with the target First/Intermediate/Last Relay UE(s) different from the source U2N Relay UE, the source Relay UE may be the First Relay UE as in case 1, the Intermediate relay as in case 2 or the Last relay UE in case 3 as below.

- Case 1: Remote-Relay1-gNB(cell1) TO Remote-Relay1-Relay2-Relay3-gNB(cell1/2)

- Case 2: Remote-Relay1-gNB(cell1) TO Remote-Relay2-Relay1-Relay3-gNB(cell1/2)

- Case 3: Remote-Relay1-gNB(cell1) TO Remote-Relay3-Relay2-Relay1-gNB(cell1/2)

For Case 1, similarly to the discussion in Scenario B, it is not needed to support the path switching at group level. This case can be handled in the baseline control plane model, as implementation, the network could trigger the sequential path switches for the remote UE and the source relay UE.

For Case 2, the source U2N Relay UE becomes the target Intermediate Relay, this case can be handled in the baseline control plane model, the network could trigger the sequential path switches for the Remote UE and the source Relay UE.

Proposal 8: For Scenario D, RAN2 does not specify anything to support the case that the source single-hop Relay UE is the target First/Intermediate Relay UE using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the source Relay UE.

For Case 3, the source Relay UE becomes the target Last Relay UE, similar to the general case, the gNB could send the RRCReconfiguration message to the First/Intermediate Relay UE via the source Relay UE(i.e., the target Last Relay UE) for remote UE, then send the RRCReconfiguration message to the Remote UE via the source Relay UE, the Remote UE will establish the PC5 link to the First Relay UE. Compared to the general case of scenario D, there is no additional impact to spec.

Proposal 9: For scenario D, RAN2 will support the case that the source single-hop relay UE is the last relay UE on the target path using the Rel-19 i2M path switching with different target last relay UEs as baseline. Spec impact will be minimised.

The signaling flow for Scenario D

Similar as the signaling flow of Rel-18 U2N Relay I2I path switching, the signaling flow of Scenario D is as below:

Figure-3: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from single-hop indirect to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details could be FFS and depend on CP discussion

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop Relay path.

9.  The gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the source L2 U2N Relay UE can be sent any time after step 6 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

10.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 6 or step7.

11.	The data path is switched from single-hop indirect path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the PC5 link if needed.

Proposal 10: Suggest RAN2 to adopt Figure-3 as the baseline signaling flow for intra-gNB single-hop indirect to multi-hop indirect path switching.

Based on the above figure, most of the steps are same as the Rel-18 Intra-gNB I2I path switching. Hence, in the following, mainly step 1/3/4/5 will be further discussed:

Step 1

For scenario D, similar to the analysis to scenario C, besides the enhanced measurement result reporting on candidate multi-hop relay path information, the legacy measurement event X2/Y2/Z1, as below could be reused. The term of candidate L2 U2N Relay should be revised to include the candidate First Relay UE.

Event X2: Serving L2 U2N Relay UE becomes worse than threshold Event Y2: Candidate L2 U2N Relay UE becomes better than threshold Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

Proposal 11: The UE will perform the legacy measurement event X2/Y2/Z1 except the description on candidate L2 U2N Relay UE in event Y2/Z1 should be revised to include the candidate First Relay UE.

Step 3/4/5

Different from legacy intra-gNB one-hop indirect to one-hop indirect path, in scenario D, the target path includes one extra First Relay UE and Intermediate Relay UE. Since the First/Intermediate Relay UE already has indirect RRC connection to the network, similar to scenario C, it is natural for network to send RRC reconfiguration message in step 3/4/5, which include at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Then, the First/Intermediate Relay UE could transfer the data from the remote UE to the Last Relay UE. Details could be FFS and depend on CP discussion.

Proposal 12: For Scenario D, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Data lossless in Scenario C/D

In scenario C, there is no relay node in the source path, so there is no extra data lossless issue during path switching. The data lossless could be guaranteed by legacy procedure.

In scenario D, there is one U2N relay node in the source path, so the data lossless issue caused by this relay has been discussed and guaranteed by the legacy procedure.

Proposal 13: For Scenario C/D, the legacy procedure will be reused to keep DL/UL lossless delivery during the path switch, there is no specification impact foreseen.

Others

In Rel-17 and Rel-18 relay discussion, it didn’t support the CHO and DAPS procedure, the same rule could be applied to Rel-19 multi-hop relay scenarios to reduce the workload in the limited time scheduling.

Proposal 14: CHO-like and DAPS-like path switch procedure is not considered in Rel-19 multi-hop U2N relay.

In legacy discussion on relay scenarios, RAN2 agreed that S-measure criteria is not used by the Remote UE for direct-indirect path switch at RAN2#115-e meeting, and the S-measure criterion based on SL/SD-RSRP of serving relay during indirect to direct path switching is not introduced at RAN2#116bis-e meeting. Thus, similarly to the analysis in history discussion, we do not see the significant benefits to introduce S-measure criteria for Rel-19 multi-hop relay scenarios, so the S-measure criteria is not used in Rel-19 multi-hop relay scenarios.

Proposal 15: S-measure criterion is not considered in Rel-19 multi-hop U2N relay.

Conclusion

According to the analysis in section 2, it is proposed:

Data Lossless for Scenario A/B

Observation 1: For data lossless, there is no spec impact introduced for Rel-17 intra-gNB I2D or D2I path switching in DL/UL.

Proposal 1: For DL data lossless, the legacy procedure based on PDCP status report could be reused for scenario A/B, there is no specification impact foreseen.

Proposal 2: For UL, the data loss in multi-hop relay node is corner case during path switching, the data lossless could be guaranteed by network implementation, there is no specification impact foreseen.

For Scenario C

Proposal 3: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Proposal 4: UE could report the collected multi-hop relay information to network to help network select a suitable target path.

Proposal 5: The network will determine and configure the suitable target path (maybe multi-hop relay path) to UE.

Proposal 6: The UE will perform the legacy measurement event Y1/Y2 except the description on candidate L2 U2N Relay UE in event Y1/Y2 should be revised to include the candidate First Relay UE.

Proposal 7: For Scenario C, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

For Scenario D

Proposal 8: For Scenario D, RAN2 does not specify anything to support the case that the source single-hop Relay UE is the target First/Intermediate Relay UE using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the source Relay UE.

Proposal 9: For scenario D, RAN2 will support the case that the source single-hop relay UE is the last relay UE on the target path using the Rel-19 i2M path switching with different target last relay UEs as baseline. Spec impact will be minimised.

Proposal 10: Suggest RAN2 to adopt Figure-3 as the baseline signaling flow for intra-gNB single-hop indirect to multi-hop indirect path switching.

Proposal 11: The UE will perform the legacy measurement event X2/Y2/Z1 except the description on candidate L2 U2N Relay UE in event Y2/Z1 should be revised to include the candidate First Relay UE.

Proposal 12: For Scenario D, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Data Lossless for Scenario C/D

Proposal 13: For Scenario C/D, the legacy procedure will be reused to keep DL/UL lossless delivery during the path switch, there is no specification impact foreseen.

Others

Proposal 14: CHO-like and DAPS-like path switch procedure is not considered in Rel-19 multi-hop U2N relay.

Proposal 15: S-measure criterion is not considered in Rel-19 multi-hop U2N relay.

Reference

- Report from session on positioning and sidelink relay, MediaTek, RAN2#128 meeting.