3GPP TSG-RAN WG2 #129	R2-2500193

Athens, Greece, February 2025

Agenda Item:	8.13.4

Source:	OPPO

Title:	Service continuity of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the service continuity related issues of multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

Discussion

Case A/B: multi-hop indirect to single-hop indirect path switching

For case A/B, it was confirmed in the last RAN2 meeting that the legacy measurement event and path switch procedure can be reused. The content of measurement report is not confirmed due to lack of time. In the current specification, the measurement report content for both the serving relay and the candidate relay is identical. It includes cellIdentity, sl-RelayUE-Identity, and sl-MeasResult.

1> if the UE is acting as L2 U2N Remote UE: 2> set the sl-MeasResultServingRelay in accordance with the following: 3> set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the serving L2 U2N Relay UE; 3> set the sl-RelayUE-Identity to include the Source L2 ID of the serving L2 U2N Relay; 3> if the measurement of serving L2 U2N Relay UE is based on SL-RSRP: 4> set the sl-MeasResult to include the SL-RSRP of the serving L2 U2N Relay UE; 4> set the sl-MeasQuantity to sl-rsrp, if supported by the UE; 3> else: 4> set the sl-MeasResult to include the SD-RSRP of the serving L2 U2N Relay UE; 4> set the sl-MeasQuantity to sd-rsrp, if supported by the UE; NOTE 1: In case of no data transmission from L2 U2N Relay UE to L2 U2N Remote UE, it is left to UE implementation whether to use SL-RSRP or SD-RSRP when setting the sl-MeasResultServingRelay of the serving L2 U2N Relay UE. 1> if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report: 2> if the reportType is set to eventTriggered or periodical: 3> if the measurement report concerns the candidate L2 U2N Relay UE: 4> set the sl-MeasResultsCandRelay in measResultNeighCells to include the best candidate L2 U2N Relay UEs up to maxNrofRelayMeas in accordance with the following: 5> if the reportType is set to eventTriggered: 6> include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset; 5> for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay: 6> set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE; 6> set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE; 6> set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE; 5> for each included L2 U2N Relay UE, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows: 6> set the sl-MeasResult to include the quantity(ies) indicated in the reportQuantityRelay within the concerned reportConfigRelay in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best L2 U2N Relay UE is included first; 6> if the UE supports multipathRemoteUE-PC5L2 and idle/inactive relay UE reporting, and if the sl-RelayIndicationMP is contained in the discovery message received from the concerned L2 U2N Relay UE: 7> set the sl-RelayIndicationMP in the sl-MeasResultsCandRelay;

In the current measurement report for relay UE, cellIdentity, sl-RelayUE-Identity, and sl-MeasResult are included for both serving and candidate relay report.

Therefore, in multi-hop U2N Relay case, the measurement report for the serving first relay should include the cellIdentity, sl-RelayUE-Identity, and sl-MeasResult.

For Case A and B, the measurement report for the serving first relay includes cellIdentity, sl-RelayUE-Identity, and sl-MeasResult as in R17/R18 U2N Relay.

In the last RAN2 meeting, the following agreements for the cases to be supported/not supported in scenario B were made:

Agreements: For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused. For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching. Spec impact will be minimised. For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

For scenario B, it is agreed to reuse the R18 i2i path switch procedure as baseline.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

Based on the current path switching procedure, the remote UE measures the serving and candidate relay, and reports measurement results based on the measurement event. The path switch decision is made by the gNB.

As agreed for the baseline procedure, the remote UE just measures the serving/candidate relays and trigger measurement report based on network configuration.

Regarding the measurement of serving/candidate relay and measurement report at the UE side, there is no special handling regarding whether it is the old last/intermediate relay at the source side.

RAN2 to confirm for scenario B, no spec impact is needed for the remote UE to identify whether the U2N Relay at the target side is the intermediate or last relay at the source side.

Case C/D: path switch to multi-hop indirect path

For the path switch to multi-hop indirect path, it is restricted in the WID that all relays at the target multi-hop relay link should be in RRC connected state to the same cell.

Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

With this restriction, the target multi-hop indirect path consisting of all the relays in RRC Connected mode to the same cell is identical to the target single-hop indirect path since:

- From UE perspective, it measures and reports the measurement results only for the direct detected first relay UE which is similar to single hop case.

- From network perspective, since all the relay UEs are RRC connected and multi-path multi-hop relay is not supported, the network can identify the target path via measurement report from the remote UE as in single hop case.

With the restriction in the WID that all the relay UEs on the target multi-hop path, from both UE and network perspective, there is no much difference for multi-hop target path and single-hop target path.

For the path switching preparation procedure, Y1/Y2/Z1 are defined for the path switching to indirect path:

- Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

- Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

- Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

Like the discussion for Case A and B, with the understanding that the “first relay” in multi-hop relay is the serving/candidate relay to be reported, Y1/Y2/Z1 can be reused.

With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

RAN2 to discuss event Y1/Y2 can be reused in direct path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

RAN2 to discuss event Y2/Z1 can be reused in single-hop indirect path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “serving” or “candidate” relay to be reported.

For the measurement report, in the current path switching to single-hop indirect path procedure, the L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) (L2 ID, Cell ID, SD-RSRP) that fulfil both AS and upper layer criteria. The principle should be reused. And the candidate L2 U2N Relay UE(s) in multi-hop relay case should be the first relay, since only the RSRP towards the first relay can be measured by the remote UE. It is also sufficient to report L2 ID, Cell ID, and the RSRP of the first relay since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB, network can identify the target link via the same information as legacy.

It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as per legacy, is sufficient for the path switching to multi-hop indirect path.

Then for the path switching execution, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

For scenario D, similar to Case-B, UE just needs to perform the measurement and report based on gNB configuration. It doesn’t need to consider if the candidate relay UE is the U2N Relay at the source side.

RAN2 to confirm for scenario D, no specific handling is needed for the case that the either intermediated relay or last relay at the target side is the U2N relay UE at the source side compared with the normal case.

Conclusion

We have the following observations:

Observation 1	In the current measurement report for relay UE, cellIdentity, sl-RelayUE-Identity, and sl-MeasResult are included for both serving and candidate relay report.

Observation 2	As agreed for the baseline procedure, the remote UE just measures the serving/candidate relays and trigger measurement report based on network configuration.

Observation 3	With the restriction in the WID that all the relay UEs on the target multi-hop path, from both UE and network perspective, there is no much difference for multi-hop target path and single-hop target path.

Observation 4	With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

Observation 5	It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

Observation 6	For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

We have the following proposals:

Proposal 1	For Case A and B, the measurement report for the serving first relay includes cellIdentity, sl-RelayUE-Identity, and sl-MeasResult as in R17/R18 U2N Relay.

Proposal 2	RAN2 to confirm for scenario B, no spec impact is needed for the remote UE to identify whether the U2N Relay at the target side is the intermediate or last relay at the source side.

Proposal 3	RAN2 to discuss event Y1/Y2 can be reused in direct path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

Proposal 4	RAN2 to discuss event Y2/Z1 can be reused in single-hop indirect path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “serving” or “candidate” relay to be reported.

Proposal 5	RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as per legacy, is sufficient for the path switching to multi-hop indirect path.

Proposal 6	RAN2 to confirm for scenario D, no specific handling is needed for the case that the either intermediated relay or last relay at the target side is the U2N relay UE at the source side compared with the normal case.