3GPP TSG-RAN WG2 #129	R2-2500196

Athens, Greece, February 2025

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on Measurement Gap Enhancements

Document for:	Discussion, Agreement

Introduction

In previous meeting, RAN2 discussed potential L2 impacts due to skipping of measurement and the need for semi-static/UE-initiated solution besides the DCI-based solution to enable measurement gap skipping. In this paper, the RAN2 impacts will be further discussed.

Discussion

Issue-1: UAI

Firstly, on how to enable the transmission and reception in MG/scheduling restriction occasions, RAN1 has agreed on the following solution based on dynamic indication from the network.

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

As shown in the above agreements, it is the network that determines whether to enable Tx/Rx during a specific gap or restriction caused by RRM measurements, and there was a discussion in RAN1 on whether the assistance information from the UE is needed for network’s decision. There was no consensus in RAN1, one LS[1] was sent to RAN4 and CC RAN2. In the LS, the candidate assistance information was listed as follows.

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

Whether the assistance report from the UE is needed is still under discussion in RAN4, and several options were proposed in RAN4

Issue 5-1: General on UAI - Proposals related to measurement occasions - Option 1: Define UAI related to measurement occasions (CMCC, Huawei, Qualcomm, Apple, vivo, MTK) - Option 1a (CMCC, Huawei, Qualcomm): number of occasions which can be skipped - Option 1b (CMCC): number of measurement occasions that are needed for measurement - Option 1c (Apple): UE indicates NW a status whether it can tolerate measurement cancellation. - Option 1d (Huawei, vivo): patterns of gaps positions of MG that can be skipped - Option 1e (vivo): study how UE assistance information/UE capability is reported when DRX is configured. - Option 1f (Qualcomm): Maximum number of consecutive MGs that can be skipped - Option 1g (Qualcomm): MG type that can be skipped - Option 2: (Nokia, E///) - Option 2a (Nokia): Do not define UAI related to measurement occasions. - Option 2c (Ericsson): RAN4 will not specify any XR-related requirement, based on XR UAI. - Draft LS response in R4-2419707. - Option 3: The information about gaps not used for measurement are reported as UE capability. - Option 4 (Ericsson): The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. - Proposals related to channel conditions - Option 1 (Nokia): For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

RAN4 is discussing whether/how the assistance information (may be in form of UE capability) from the UE for RRM skipping is useful.

For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

If RAN4 confirms that assistance information is needed, for the detailed information to be carried, the following candidates are discussed in RAN1

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

Among the above information, the first FFS on assistance information related to measurement occasions is more of RAN4 scope, and the last three FFS related to channel quality and traffic are more of RAN2 scope which can be discussed in RAN2.

Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions and is in RAN4 scope, while the others are more in RAN2 scope.

But anyway, the discussion at RAN2 can happen after seeing RAN4 LS reply.

Issue-2: Semi-static/UE initiated approach

For approach to enable measurement gap skipping, RAN4 has replied the LS as follows

RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability. It is also RAN4 understanding that the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331. Furthermore, from RAN4 RRM requirements perspective, Alt 1-1 is not supported for EN-DC and NE-DC scenarios. Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

As shown in the above RAN4 LS, the feasibility of Alt 1-1 is confirmed by RAN4, while it is also indicated that RAN4 is also discussing the need to introduce semi-static approach.

The feasibility of Alt 1-1 (i.e., dynamic approach) is confirmed by RAN4. On the other hand, it is also indicated by RAN4 that they are also discussing the need to introduce semi-static approach.

To avoid duplex discussion and conflict conclusion, RAN2 should wait for further RAN4/RAN1 conclusions of the need for other solutions for measurement gap skipping.

RAN2 wait for further RAN4/RAN1 conclusion of the need for other solutions for measurement gap skipping.

Issue-3: Capability

As agreed by RAN1 and RAN4, for DCI-based measurement gap skipping, there is a minimum time offset between indication to skip and the skipped measurement occasion.

RAN1 Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). RAN4 conclusion: RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability.

As shown in the above RAN1/4 agreements, the RRC configuration for the minimum time offset is needed. Further details on can wait for RAN1/4 progress.

As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

In addition, as indicated in RAN4 LS, at least for X=3ms, the optional UE capability is needed.

For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.

- Conclusion

We have the following observations:

Observation 1	RAN4 is discussing whether/how the assistance information (may be in form of UE capability) from the UE for RRM skipping is useful.

Observation 2	Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions and is in RAN4 scope, while the others are more in RAN2 scope.

Observation 3	The feasibility of Alt 1-1 (i.e., dynamic approach) is confirmed by RAN4. On the other hand, it is also indicated by RAN4 that they are also discussing the need to introduce semi-static approach.

Observation 4	As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

We have the following proposals:

Proposal 1	For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

Proposal 2	RAN2 wait for further RAN4/RAN1 conclusion of the need for other solutions for measurement gap skipping.

Proposal 3	For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.

Reference

- R1-2405736, LS on UE assistance information

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements