3GPP TSG-RAN WG2 Meeting #129	R2-2500199

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

# Introduction

In Rel-19 NES WID [1], the objective on adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on this objective of Rel-19 NES.

# Discussion

# Adaptation of SSB in time domain

The SSB configuration in time domain includes SSB periodicity and time domain positions of the transmitted SSB in an SSB burst. For legacy RRC_IDLE/ RRC_INACITVE UE, CD-SSB is configured by the parameters ssb-periodicityServingCell and ssb-PositionsInBurst in ServingCellConfigCommonSIB in SIB1. Network may adapt SSB in time domain relying on system information modification, and a modification period is used as effective time. For legacy RRC_CONNECTED UE, both CD-SSB and NCD-SSB could be configured and adapted by network via dedicated RRC signalling.

Regarding the applicable scenarios of SSB adaptation in time domain, R1 has made the following agreements.

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

R1#118bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

R1#119 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

For RRC_IDLE UEs and PCell of RRC_CONNECTED UEs, the adaptation of SSB in time domain has been ruled out in Rel-19 due to no consensus in R1. The only applicable scenario of SSB adaptation in time domain is for SCells of RRC_CONNECTED UEs. R1 only touches on RRC_IDLE and RRC_CONNECTED, so it is necessary to clarify for RRC_INACTIVE from R2 perspective. We understand the situation of RRC_INACTIVE UEs is similar to RRC_IDLE, and it is necessary to confirm that SSB adaptation in time domain for RRC_INACTIVE UEs is also not supported.

- R1 has ruled out the support of SSB adaptation in time domain in Rel-19 for 1) RRC_IDLE UEs, and 2) PCell of RRC_CONNECTED UEs.

- Only SSB adaptation for SCells of RRC_CONNECTED UEs is supported as concluded by R1.

- R2 further confirms that SSB adaptation in time domain for RRC_INACTIVE UEs is not supported in Rel-19 (i.e., same as RRC_IDLE as concluded by R1).

In case that the applicable scenarios of SSB adaptation are narrowed down to SCell of RRC_CONNECTED UEs, now OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3) are both for SCells of RRC_CONNECTED UEs, therefore relying on MAC CE to indicate SSB adaptation in time domain would be sufficient. Note that in NES O1, a new MAC CE has already been introduced to indicate OD-SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode. Therefore, it would be possible to have a unified MAC CE design for both OD-SSB and SSB adaptation.

- For NES O1, a new MAC CE will be introduced to indicate OD-SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode.

- R2 adopts a unified MAC CE design for both OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3).

Another issue is L3 measurement impact due to SSB adaptation, which is also raised by R4 LS R4-2416911. In legacy L3 RRM measurement, UE performs SSB measurement for SCell in activated state based on SMTC window, and performs SSB measurement for SCell in deactivated state based on measCycleSCell.

In NES O1, L3 measurement impact due to OD-SSB of SCell is also being discussed. For OD-SSB, a hybrid L3 measurement framework including fast and slow mode SCell measurement is agreed in R4, as shown in the following figure, i.e., UE will perform a hybrid fast + slow mode measurement for SCell in deactivated state, where

- in the fast mode window, the measurement requirement is defined based on SSB periodicity, which allows networks to activate SCell for UE faster,

- after the fast mode window (i.e., in the slow mode window), if the OD-SSB is still activated, the measurement requirement is defined as in legacy, i.e., based on measCycleSCell and DRX cycle.

Considering now

1/ scenario-wise: OD-SSB and SSB adaptation are both for SCells of RRC_CONNECTED UEs, and the actual effect due to SSB adaptation of AO-SSB is almost same as OD-SSB case#2 (i.e., AO-SSB + OD-SSB),

2/ control-wise: both rely on MAC-CE,

3/ use-case wise: for SCell, the typical use case is the fast SCell activation, where both allows SSB periodicity changed from sparse to dense before SCell activation, and may further fallback to be sparse afterwards,

NES O1/3 can both rely on one unified framework for L3 measurement, as shown in the following figure, i.e.,

1/ before SCell activation, relying on fast + slow mode L3 SCell measurement, which can achieve the effect of fast SCell activation;

2/ after SCell activation (before SCell deactivation), relying on legacy SMTC-based SCell measurement. For SSB adaptation, since the measurement anyway relies on periodicity of SMTC window which is statically configured, no issue is foreseen if relying on legacy SMTC-based SCell measurement.

- R2 aims at a unified L3 RRM measurement design for both OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3):
1) For deactivated SCell, adopt the fast + slow mode measurement framework as agreed by R4 for OD-SSB already;
2) For activated SCell, rely on SMTC window periodicity as in legacy.

# Adaptation of RACH in time domain

In legacy, the time domain configuration of RACH occasion (RO) is configured using

- An index via SIB or dedicated RRC signalling, i.e., by prach-ConfigurationIndex, where the periodicity and time domain positions are mapped in TS 38.211.

- Besides, the parameters PRACH mask indexes (i.e., ra-ssb-OccasionMaskIndex) are also used to define RO associated with an SSB in which the MAC entity may transmit a Random-Access Preamble, and the mapping table is defined in TS 38.321.

Based on R1 minutes shown, R1 has made some agreements on this.

R1#118 Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation, which is fully up to R1 to further specify the detailed design. No R2 work is foreseen.

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

Another leftover issue raised by companies in the email discussion is RACH bundling, i.e., given the PO bundles, whether to also adapt PRACH resources so that RO can be bundled with PO bundles. In this case, from network perspective, network can perform both paging transmission and RACH reception in a shorter time range to obtain additional power saving gain.

Currently, it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not. Further check and confirmation are needed. In our understanding, if there is a need to go for RACH bundling, it is preferred that R1 lead the work.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

Most work on the RACH enhancement is within R1 scope. From R2 perspective, what we need to discuss includes whether the RACH adaptation mechanism applies to different types of RACH resources (e.g., CBRA/CFRA, 2/4-step RA type, feature/feature combination-specific RACH resources).

For CBRA/CFRA, we understand that it is not justified on the motivation to apply time-domain RACH adaptation to CFRA, since CFRA can rely on dedicated signalling of networks to assign UE-specific RACH resources. So we propose to only apply the RACH adaptation mechanism to CBRA.

- It is not motivated to apply time-domain RACH adaptation to CFRA, since CFRA can rely on dedicated signalling of networks to assign UE-specific RACH resources.

- From R2 perspective, apply time-domain RACH adaptation only to CBRA (i.e., not apply to CFRA).

In the current RACH framework, networks can configure cell-common CBRA resources via BWP-UplinkCommon in SIB1, as highlighted below, including

- Rel-15 4-step RACH resources (i.e., rach-configCommon),

- Rel-16 2-step RACH resources (i.e., msgA-ConfigCommon-r16),

- IAB RACH resources (i.e., rach-ConfigCommonIAB-r16) introduced for IAB-MT in Rel-16 IAB,

- Additional RACH resources (i.e., additionalRACH-ConfigList-r17) introduced in Rel-17 for feature/feature combination-specific RACH partitioning framework.

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                            OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                           OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                           OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                            OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }             OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                          OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }     OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                         OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17      OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                   OPTIONAL   -- Cond Msg3Rep

]],

(omit the irrelevant IEs)

}

Besides, networks can configure cell-common RACH resources for Msg1-based SI request, with/without Msg1 repetition, on NUL or SUL, for normal UE or (e)RedCap UE, via SI-RequestConfig/SI-RequestConfigRepetition in SI-SchedulingInfo in SIB1.

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5,s10,s20,s40,s80,s160,s320,s640,s1280,s2560-v1710,s5120-v1710},

si-RequestConfig                    SI-RequestConfig                                    OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                    OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                              OPTIONAL,  -- Need R

...

}

(omit the irrelevant IEs)

SI-SchedulingInfo-v1740 ::=         SEQUENCE {

si-RequestConfigRedCap-r17          SI-RequestConfig                                    OPTIONAL   -- Cond REDCAP-MSG-1

}

SI-SchedulingInfo-v1800 ::=         SEQUENCE {

si-RequestConfigMSG1-Repetition-r18           SI-RequestConfigRepetition-r18            OPTIONAL,  -- Cond MSG-1

si-RequestConfigRedCap-MSG1-Repetition-r18    SI-RequestConfigRepetition-r18            OPTIONAL,  -- Cond REDCAP-MSG-1

si-RequestConfigSUL-MSG1-Repetition-r18       SI-RequestConfigRepetition-r18            OPTIONAL   -- Cond SUL-MSG-1

}

- The current cell-common CBRA resource configuration includes: 1) Rel-15 4-step RACH resources, 2) Rel-16 2-step RACH resources, 3) IAB RACH resources, 4)	additional RACH resources for RACH partitioning, and 5) RACH resources for Msg1-based SI request.

As baseline, at least 4-step RA type needs to be supported. And we are open to also support 2-step RA type from R2 perspective, since no additional workload in R2 is foreseen. Considering that the support of 2-step RA may have impact on DCI design in R1, e.g., indicating 2-step and/or 4-step RACH type in DCI. Therefore, it is good to send LS to R1 to further check the support of 2-stpe RA.

- From R2 perspective, at least apply time-domain RACH adaptation to 4-step RA. Send LS (as in Annex) to R1 to check whether to apply to 2-step RA or not.

For RACH resources for MSG1-based SI request, in which case the requested Other SI is associated to a subset of RACH resources, we understand that it is also not justified on the motivation to apply additional RACH resources in time domain. Although RACH resources for MSG1-based SI request are contention-based, for the purpose of SI-request actually no need to perform contention resolution, thus it is meaningless to require more RACH resources.

- From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. Send LS (as in Annex) to R1 to ask if any concern.

For IAB RACH resources, and additional RACH resources for feature/feature combination-specific RACH partitioning, we understand the simplest way forward is that we do not consider applying the RACH adaptation mechanism for them. Otherwise, the impact and complexity on R1-designed DCI-based solution needs to be evaluated by R1. So it would be safer to also check with R1.

Another point is that within the Rel-15/16 4/2-step RACH resource configurations, it can also allow feature/feature combination-specific RACH partitioning, since featureCombinationPreamblesList is optionally present in RACH-ConfigCommon/RACH-ConfigCommonTwoStepRA-r16 for Rel-15/16 4/2-step RACH resource configurations respectively. We also hold the same position on this, i.e., not to consider it for the RACH adaptation mechanism.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. Send LS (as in Annex) to R1 to ask if any concern.

- From R2 perspective, not apply time-domain RACH adaptation to feature(-combination)-specific RACH partitioning. Send LS (as in Annex) to R1 to ask if any concern.

For Rel-19 UEs, both legacy RACH resources and Rel-19 additional RACH resources are available. Another issue is that whether to specify UE behaviours for Rel-19 UEs to choose between additional RACH resources and legacy RACH resources. For example, Rel-19 UEs prioritize to choose additional RACH resources, considering legacy RACH resources can only be used by legacy UE. We understand that which resources to choose from UE perspective depends on different purposes, e.g., RACH collision avoidance, RACH latency minimization, so it would be various rules and hard to specify the related UE behaviour. We propose to leave it to UE implementation.

- Up to UE implementation on the selection between R19 DCI-indicated additional RACH resources and legacy RACH resources, i.e., no need to specify the UE behaviours on this.

# Adaptation of Paging

In legacy, the PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N), Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns,

where T denotes DRX cycle of the UE, N denotes the number of total paging frames in T, Ns denotes the number of paging occasions for a PF, PF_offset is the offset used for PF determination, and UE_ID is derived based on 5G-S-TMSI. The legacy PF/PO determination mechanism has already supported to confine the PF/PO in the time domain based on the adjustment of parameters N/Ns.

In the last R2 meeting, R1 has made some agreements on paging adaptation.

R2#128 Agreements Introduce value for Ns=8. FFS on 16. Introduce value for N= T/32. FFS on T/64, T/128, T/256. RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

Here we further discuss on the leftover issues.

For PEI configuration and PEI transmission, as summrarized in offline discussion summary [2] in the last meeting, the issues include a) PEI covering both legacy PO and R19-only PO, and b) PEI-O location configuration.

For issue-a), the key issue is whether to allow or disallow configuration of po-numberPerPEI so that a same PEI associates with the two types of PO:s, i.e., 1) PO:s monitored by legacy UE (may or may not shared with R19 UE, up to network configuration), and 2) PO:s monitored only by R19 UE.

For issue-b), the key issue is whether we 1) simply leave it to network implementation to set po-NumPerPEI and the number of PEI-O location per PF properly, in order to be compatible with Ns=8/16 w/o further spec effort, or 2) RAN2 further look into the issue when the setting of po-NumPerPEI and the number of PEI-O location per PF are incompatible with Ns=8/16.

Based on the WFs in offline discussion summary [2], as shown below, further consideration is needed.

Proposal-3: R2 further discuss to whether 1) based on a single PEI-O configuration, and rely on network implementation to have separate PEI:s for the two types of PO:s, or 2) based on separate PEI-O configurations, and thus automatically separate PEIs for the two types of PO:s. Where the two types of POs are 1) PO:s monitored by legacy UE (may or may not shared with R19 UE, up to network configuration), and 2) PO:s monitored only by R19 UE Proposal-4: R2 assume to leave it to network implementation to set po-NumPerPEI and maxPEI-perPF properly, in order to be compatible with Ns=8 without further spec effort

For P3 in WFs, two options, as highlighted above, are put on the table, where Option-2) is straightforward, however for Option-1), we need to further consider that how network implementation based on a single PEI-O configuration can achieve separate PEI:s for two types of PO:s. We understand that it is somehow not feasible.

As shown in current spec, the PEI-O locations are configured via firstPDCCH-MonitoringOccasionOfPEI-O as a sequence, and the association between PEI-O locations and PO:s is determined based on po-NumPerPEI and Ns, as highlighted in the field description below:

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

As shown in current spec, po-NumPerPEI, i.e., PO number per PEI, can be configured via with value 1, 2, 4, 8:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

Considering for Rel-19 NES UE and legacy UE, there are separate PO configurations, including N, Ns and PO locations, the PO number and PO locations in a PF for Rel-19 NES UE and legacy UE would be very flexible, e.g.,

a) for PO number, whether PO number per PF (i.e., Ns) for Rel-19 NES UE and legacy UE is equal or not, and

b) for PO locations, whether PO:s monitored by legacy UE may be shared with R19 NES UE or not, and

c) for PO locations, the order of PO:s for Rel-19 NES UE and legacy UE,

which is fully up to network configuration.

For Option-1), networks provide with only a single PEI-O configuration for both Rel-19 NES UE and legacy UE, including po-NumPerPEI and PEI-O location list. In most of network configurations of PO:s, based on the current association between PEI-O locations and PO:s, it is not feasible to have two separate PEI-O:s. The only feasible case is that shared PO:s are in front of either legacy PO:s or Rel-19 PO:s, which has significant restriction for network configuration.

- For Option-1), the only feasible network configuration of PO:s is that shared PO:s are in front of either legacy PO:s or Rel-19 PO:s, which has significant restriction for network configuration.

Therefore, we propose to go for Option-2).

- Adopt separate PEI-O configurations for 1) PO:s monitored by legacy UE, and 2) PO:s monitored only by R19 UE.

- Leave it to network implementation to set po-NumPerPEI and the number of PEI-O location per PF properly, in order to be compatible with Ns=8 without further spec effort.

Besides, for N/Ns values, in order to avoid further spec effort, we understand that no additional values should be introduced. For Ns, the compatibility with Ns=16 would lead to higher signalling overhead, i.e., with the introduction of Ns=16, the value range of firstPDCCH-MonitoringOccasionOfPO also needs to be extended which require more additional bits for each PO. For N, if additional value is added, there is a case of less than 1 PF in a paging cycle, since the minimum value of paging cycle is 32. Thus, the FFS point on N/Ns values should be closed.

- No additional value for Ns and N, except for Ns=8 and N=T/32.

Another issue needed to be started to discuss is the capability for paging adaptation enhancement. The paging adaptation enhancement introduced in Rel-19 NES should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

However, it is unclear from R2 perspective whether the capabilities of paging adaptation enhancement for each frequency band of UE are coincident or not. It is worth referring that for PEI enhancement in Rel-17, per-band capability (i.e., the band list supporting PEI) is introduced. R2 may further discuss the granularity of this UE capability (e.g., per UE, per band, etc.). In case that R2 is not confident to make the decision, R1/4 may need to be involved on this issue.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Adaptation of SSB in time domain

- R1 has ruled out the support of SSB adaptation in time domain in Rel-19 for 1) RRC_IDLE UEs, and 2) PCell of RRC_CONNECTED UEs.

- Only SSB adaptation for SCells of RRC_CONNECTED UEs is supported as concluded by R1.

- R2 further confirms that SSB adaptation in time domain for RRC_INACTIVE UEs is not supported in Rel-19 (i.e., same as RRC_IDLE as concluded by R1).

- For NES O1, a new MAC CE will be introduced to indicate OD-SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode.

- R2 adopts a unified MAC CE design for both OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3).

- R2 aims at a unified L3 RRM measurement design for both OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3):
1) For deactivated SCell, adopt the fast + slow mode measurement framework as agreed by R4 for OD-SSB already;
2) For activated SCell, rely on SMTC window periodicity as in legacy.

Adaptation of RACH in time domain

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

- It is not motivated to apply time-domain RACH adaptation to CFRA, since CFRA can rely on dedicated signalling of networks to assign UE-specific RACH resources.

- From R2 perspective, apply time-domain RACH adaptation only to CBRA (i.e., not apply to CFRA).

- The current cell-common CBRA resource configuration includes: 1) Rel-15 4-step RACH resources, 2) Rel-16 2-step RACH resources, 3) IAB RACH resources, 4)	additional RACH resources for RACH partitioning, and 5) RACH resources for Msg1-based SI request.

- From R2 perspective, at least apply time-domain RACH adaptation to 4-step RA. Send LS (as in Annex) to R1 to check whether to apply to 2-step RA or not.

- From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. Send LS (as in Annex) to R1 to ask if any concern.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. Send LS (as in Annex) to R1 to ask if any concern.

- From R2 perspective, not apply time-domain RACH adaptation to feature(-combination)-specific RACH partitioning. Send LS (as in Annex) to R1 to ask if any concern.

- Up to UE implementation on the selection between R19 DCI-indicated additional RACH resources and legacy RACH resources, i.e., no need to specify the UE behaviours on this.

Adaptation of Paging

- For Option-1), the only feasible network configuration of PO:s is that shared PO:s are in front of either legacy PO:s or Rel-19 PO:s, which has significant restriction for network configuration.

- Adopt separate PEI-O configurations for 1) PO:s monitored by legacy UE, and 2) PO:s monitored only by R19 UE.

- Leave it to network implementation to set po-NumPerPEI and the number of PEI-O location per PF properly, in order to be compatible with Ns=8 without further spec effort.

- No additional value for Ns and N, except for Ns=8 and N=T/32.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

[2] R2-2410930, Offline discussion summary [AT128][111][NES], OPPO

# 5. Annex: LS to R1 on RACH Adaptation

3GPP TSG-RAN WG2 Meeting #129	R2-25xxxxx

Athens, Greece, 17th – 21st Feb, 2025

Title:	[Draft] LS on Applicability of Time-Domain RACH Adaptation

Response to: 	-

Release:	Rel-19

Work Item:	Netw_Energy_NR_enh-Core

Source:	[to be RAN2]

To:	RAN1

Cc:	-

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 had discussed the issue of applicability of time-domain RACH adaptation w.r.t the following RACH related features:

- Rel-15 4-step RACH resources (i.e., rach-configCommon), with and without feature(-combination)-based RACH partition,

- Rel-16 2-step RACH resources (i.e., msgA-ConfigCommon-r16), with and without feature(-combination)-based RACH partition,

- IAB RACH resources (i.e., rach-ConfigCommonIAB-r16) introduced for IAB-MT in Rel-16 IAB,

- Additional RACH resources (i.e., additionalRACH-ConfigList-r17) introduced in Rel-17 for feature/feature combination-specific RACH partitioning framework, and

- RACH resources for Msg1-based SI request, with and without Msg1 repetition, on NUL or SUL, for normal UE or (e)RedCap UE.

And reached the following conclusion from RAN2 perspective:

[…]

2. Actions:

To RAN1:

ACTION: 	RAN2 kindly asks RAN1 to provide feedback if any concern on the R2 conclusion above.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis		7th – 11st April 2025					China, CN

RAN2#130			19th – 23rd May 2025					Malta, MT