3GPP TSG-RAN WG2 Meeting #129	R2-2500200

Athens, Greece, 17 - 21 Feb, 2025

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon, Turkcell

Title:	Further consideration on UL capacity enhancement

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, RAN2 has made the following agreements:

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this contribution, we will further discuss the details of SA and DSA mechanisms.

# 2	Discussion

## 2.1 SA

### 2.1.1 CB-msg3 EDT Configuration

During the last meeting, it was agreed at least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. However, this is only applied to 3.75kHz SCS NB-IoT and eMTC CE mode A, which confirmed by RAN1 that don not require TA validation.

For 15kHz SCS NB-IoT and eMTC CE mode B, RAN2 should assume that UE still needs to be configured with TA validation parameters through RRC dedicated signalling, such as using RRCRelease. In case TA is valid, these UEs can share the resource configured in the system information.

Proposal 1: For 15kHz SCS NB-IoT UEs and eMTC CE mode B UEs, TA validation parameters need to be provided via RRC dedicated signalling.

### 2.1.2 Contention resolution of CB-msg3 EDT

Since the contention-based MSG3 resource is shared by multiple UEs, contention can occur when different UEs choose the same resource, similar to the potential contention during the RACH procedure. As a result, contention resolution is required via Msg4 for the contention-based MSG3 transmission. To achieve this, a mechanism similar to the one used contention resolution during RACH can be applied. Specifically, the network retrieves the UE ID in Msg3 and identifies the winner of the contention. To inform the UE of the contention resolution result, the NW can send a MAC CE containing the UE ID via Msg4, allowing the UE to verify the result.

Proposal 2: A MAC CE containing UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Msg3 transmission may fail again if different UEs select the same resource again. In the RACH procedure, a Backoff Indicator is introduced to address this issue. The UE selects a random backoff time according to a Backoff Indicator included in Random Access Response, delaying the subsequent Random Access transmission by the backoff time. A similar mechanism can be applied to handle Msg3 transmission failures in contention-based Msg3 EDT transmission. Specifically the backoff parameters can be configured for UE via system information, allowing the UE to randomly selects a backoff time based on these parameters.

Proposal 3: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

### 2.1.3 Fallback

Another issue is whether the UE can fall back to 4-step random access EDT after a certain number of CB-msg3 EDT attempts. Generally, having this backup mechanism is beneficial in case the CB-msg3 EDT attempts are successful.

Proposal 4: After a specified number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

### 2.1.4 RNTI for Msg4 reception

To complete the contention resolution, the UE needs the RNTI to monitor the PDCCH for MSG4 scheduling. In the last meeting, RAN2 has agreed that the RNTI used for Msg4 transmission will be derived based on the resource associated to the PUSCH occasion for CB-Msg3 EDT transmission. The detail is FFS.

In legacy RACH procedure, UE determines RA-RNTI according to selected PRACH resources.

36.321 5.1.4 Random Access Response reception The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI= 1 + t_id + 10*f_id where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10)) where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

RNTI used for Msg4 in CB-MSG3 EDT can use the similar mechanism as RA-RNTI.

Proposal 5: RNTI used for Msg4 in CB-MSG3 EDT can follow RA-RNTI calculation formula as baseline. FFS on the exact parameters.

During legacy RACH procedure, Msg2 contains Temporary C-RNTI. When UE receives Msg4 and considers this Contention Resolution successful, UE sets the C-RNTI to the value of the Temporary C-RNTI. However, Msg2 is omitted for CB-msg3 EDT. Which RNTI is used by the UE for further scheduling after MSG4 needs to be decided by RAN2. A straightforward approach would be for the network to is that NW allocates a C-RNTI in Msg4 to the UE so that UE can use it subsequently.

Proposal 6: NW should allocate a C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

## 2.2 DSA

### 2.2.1 DSA transmission window

In the last meeting, RAN2 has agreed to introduce a transmission window for DSA . UE is required to randomly select multiple occasions within this transmission window to transmit the multiple replicas. As far as we can see, there are two options for the modelling of this transmission window.

Option 1：Fixed transmission window.

In this option, the start time and duration of the transmission window are configured by the NW. However the drawback of this option is that, if the UE initiates DSA procedure in the second half of a transmission window, there may be not enough occasions for the UE to select, e.g., when the number of available CB-Msg3 occasions within the second half of the select window is less than the configured number of replicas for DSA procedure. As a result, the UE cannot initiate the DSA procedure in this transmission window and must delay to the next window, which causes unnecessary delay. What’s worse, this will increase the contention probability of the first half of each window.

Option 2：Dynamic transmission window.

In this option, the duration of the transmission window is still configured by the NW while the start time of the transmission window is when the UE sends the first replica for DSA. In this case, UE can always have enough occasions (within the whole duration of the transmission window) to select for the transmission of DSA replicas no matter when the UE initiates DSA.

Proposal 7: In DSA, the transmission window starts at the transmission time of the first replica. And the duration of the transmission window is configured by the NW.

### 2.2.2 RNTI monitoring for DSA

In the last meeting, there is no consensus on whether the UE needs to start one or multiple monitoring windows for MSG4 reception. In our opinion, there is not much difference between these two directions since multiple monitoring windows can be combined as one window in time domain. On the other hand, the modelling of multiple windows is much simple than one window requiring no extra spec effort. For one window solution, we need to define a new starting time and a new duration. What’s more, multiple windows can avoid the unnecessary monitoring time introduced by the one window solution, in case the gap between two replicas are large. So obviously starting multiple reception windows is a better choice.

Proposal 8: In DSA, multiple PDCCH monitoring windows are started for multiple replicas.

Besides, for either one window or multiple windows, UE may need to monitor the feedback for different replicas at the same time. In this case, if the RNTI is different for different replicas, The IoT UE will have to monitor multiple RNTIs at the same time. The maximum number of replicas configured by the NW is 4. So the UE needs to monitor a maximum of 4 RNTIs at the same time which will add complexity to especially low capability NB-IoT UEs. A better way is to define a common RNTI for all the replicas in DSA. Besides, if one common RNTI is used for monitoring MSG4s, NW can multiplex multiple MSG4s into one MAC PDU to reduce overhead.

Proposal 9: In DSA, one common RNTI is used for multiple MSG4 monitoring which corresponds to multiple MSG3 copies.

2.2.3 Indication of the position of the replicas

As far as we consider, the indication for the position of other replicas is only needed for CRDSA but not for DSA. It will bring extra specification efforts without clear benefit. For instance, we need to decide where and how to indicate the positions of other replica, which will bring additional overhead. Since the goal here is for capacity enhancement, it is not a good choice to introduce more overhead.

Proposal 10: Indication of the replica position is not pursued.

# 3	Conclusion

This contribution analyses the TA issue for regenerative payload, and proposes the following:

SA

CB-msg3 EDT Configuration

Proposal 1: For 15kHz SCS NB-IoT UEs and eMTC CE mode B UEs, TA validation parameters need to be provided via RRC dedicated signalling.

Contention resolution of CB-msg3 EDT

Proposal 2: A MAC CE containing UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Proposal 3: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

Fallback

Proposal 4: After a specified number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

RNTI for Msg4 reception

Proposal 5: RNTI used for Msg4 in CB-MSG3 EDT can follow RA-RNTI calculation formula as baseline. FFS on the exact parameters.

Proposal 6: NW should allocate a C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

DSA

DSA transmission window

Proposal 7: In DSA, the transmission window starts at the transmission time of the first replica. And the duration of the transmission window is configured by the NW.

RNTI monitoring for DSA

Proposal 8: In DSA, multiple PDCCH monitoring windows are started for multiple replicas.

Proposal 9: In DSA, one common RNTI is used for multiple MSG4 monitoring which corresponds to multiple MSG3 copies.

Indication of the position of the replicas

Proposal 10: Indication of the replica position is not pursued.

# 4	References

- RP-242397, WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3