3GPP TSG-RAN WG2 Meeting #129                              	  R2-2500210

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.12.3

Source: 	OPPO

Title:  	Discussion on UEI beam reporting

Document for:	Discussion and Decision

Introduction

In previous meeting，RAN1 has studied the UE initiated beam reporting mechanism and the following events are agreed for triggering beam reporting.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

- Q is RRC configured with subjective to UE capability signalling

- UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Furthermore, RAN1 has reached consensus on the evaluation details for event 2:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

In this contribution, we discuss the FFS in RAN1 agreement, i.e. the modelling of event-driven beam reporting.

Discussion

In earlier meeting, RAN2 has initially discussed the modeling of UE initiated beam reporting, while no consensus has been reached.

In general, there are two alternative for capturing the event evaluation procedure:

- Alt 1: event evaluation is captured in PHY

For event-driven beam reporting, RAN1 has agreed that the beam report is carried in UCI. And currently UCI related procedure is captured in PHY. Furthermore, the event evaluation is based on the latest L1 measurement result which is also performed in L1. Based on this, it is straightforward to capture the event evaluation in PHY, which can avoid unnecessary inter-layer interaction and reduce the reporting latency.

- Alt 2: event evaluation is captured in MAC

As shown in RAN1 agreement, the evaluation procedure for event-2 relies on timer and counter to determine the triggering event, which is usually captured in MAC spec. For example, the beam failure detection procedure is captured in MAC and PHY is responsible to provide the beam failure instance to MAC for counting. However, this require additional interaction between MAC and PHY, which may postpone the beam report.

According the analysis above, it is preferred to go Alt 1, i.e. event evaluation for UE initiated beam reporting is specified in PHY.

The event evaluation for UE initiated beam reporting is specified in PHY.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The event evaluation for UE initiated beam reporting is specified in PHY.

Reference

[1] RP-242394 WID revision: NR MIMO Phase 5