3GPP TSG-RAN WG2 Meeting #129	R2-2500212

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT paging

Document for: Discussion and Decision

# 1	Introduction

In the previous RAN2 meetings, some related agreements were achieved in the following:

RAN2#128 agreements on paging 1. An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size. 2. Capture the scenario that different readers may page the same device for same service request. We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs 3. Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

In RAN WID, RAN2 scope is concluded:

R19 RAN WID - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Based on the current RAN2 progress, RAN WID, and focusing on the issues without conclusions, this contribution discusses subsequent paging and paging message design/content.

# 2	Discussion

## 2.1	Subsequent paging

### 2.1.1	Transaction ID size and maintenance

Some companies proposed to support parallel services which may interleave in the time domain. To support the devices to handle parallel services triggered by a reader, and the procedure can be shown in the Figure 2.1-1.

Figure 2.1-1	Multiple in-parallel services from one reader

To make the procedures work, it requires additional signalling overheads and device complexity. For instance, service ID needs to be included in every R2D message to allow a device to differentiate the associated service. And the devices need to maintain the association between service ID and the radio resource allocated for the service ID, as well as to remember the access/data transmission status of each service. Meanwhile, the interleaving services also require a device to continue monitoring for a longer time during the gap of its services, which may lead to higher failure probability due to lack of energy.

Observation 1:	Supporting parallel services under a single reader will increase the complexity of the device. It will also lead to significantly higher memory consumption, as multiple sets of AS configuration and context associated with multiple services need to be maintained. Moreover, it will add more R2D signalling overhead, as the service ID has to be included in each R2D message.

Proposal 1a:	Do not support parallel/interleaved services from a single BS reader in the A-IoT radio interface (i.e., reader must complete the procedure associated with one service, before starting a new service).

As to the multiple readers deployment case, how the network side to avoid the device to receive the parallel/interleaved services from the neighbour readers is further discussed.

- For the neighbour readers belonging to the same BS, the BS implementation should trigger its readers one-by-one, i.e. the services are serial in time-domain without parallel/interleaved services among those readers.

- The parallel/interleaved services from the neighbour readers require all the R2D message to include the reader ID/service ID, which means unacceptable overhead in the AS layer. Otherwise, the device has no idea on the associated service for each R2D message. Then, the network has to avoid the parallel/interleaved services from the neighbour readers belonging to different BSs. For example, CN implementation should avoid the parallel/interleaved services from the neighbour readers belonging to different BSs.

Observation 2a:	BS implementation can avoid parallel/interleaved services from the neighbour readers that belong to the same BS.

Observation 2b:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 2c:	Parallel services from the neighbour readers require that all the R2D messages include the reader ID/service ID, which implies an unacceptable overhead at the AS layer.

In RAN2#128 meeting, the following agreement on multiple/subsequent paging were achieved:

Multiple/Subsequent Paging from the same Reader - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

The “ID” in the agreements above can be realized by “transaction ID”. The device needs to maintain the transaction ID and the status (successful or not yet) of the current service. If the received transaction ID in the paging message is the same as the one maintained by the device, the device will determine whether to respond to the paging message based on the associated maintained status (successful or not). For example, when a device maintains a “transaction ID” and a “success status”, the device will not respond to the following (subsequent) paging messages with the same transaction ID.

Proposal 1b:	A device only maintains the status (successful or not) of the current single service.

On the other hand, if the received transaction ID in the paging message is different from the one maintained by the device, the device updates its maintained “current transaction ID” as the received one. If the device is selected to access by the paging identifier in the paging message, the device will respond to this paging message corresponding to the new transaction ID and store its status (successful or not).

Proposal 1c:	If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device will update its currently maintained “transaction ID” to the received value.

According to the discussion in RAN3/SA2, there may be a correlation ID transmitted over XX/NG-AP interface. The reader can use correlation ID to associate the response(s) from devices to a specific service request from CN. If using this correlation ID as the transaction ID, the length of the transaction ID could be large since the correlation ID is used to identify a service among all the services from different CN nodes. But, considering the transmission overhead and the buffer size required by the device storage for transaction ID, the transaction ID in the paging message should not/does not have to be the same as the correlation ID from CN. It could be the BS generated ID which can be shorter than the correlation ID.

Observation 3a:	The “transaction ID” in the paging message does not have to be identical to the session/correlation ID from CN, if any. In fact, it can be an ID generated by the BS, and this ID can be shorter than the session/correlation ID.

As discussed above, the device needs to store the current transaction ID and the status (successful or not) corresponding to the transaction ID. On top of the device cost, maintaining/buffering information (e.g., the transaction ID) also increases the power consumption, e.g., requirements of larger AS buffer/register sizes. The larger the buffer size is, the larger the power consumption will be. Therefore, in order to reduce the required buffer size, we propose to use a shorter size of the transaction ID.

Observation 3b:	To minimize the size of the AS buffer and register requirements, the shorter the “transaction ID” is, the more favorable it is.

If the parallel/interleaved services within one BS reader is not supported, as proposed, the reader only starts a new service-1 after it completes the previous service-0. The device only needs to distinguish whether it is for a new service request or a re-transmission for the old service request. Therefore, a 1-bit transaction ID is sufficient to implement this function.

Observation 4:	If reader starts a new service-1 after successfully finishing the previous service-0, a 1-bit transaction ID is sufficient for a single reader case.

Considering there may be a case that several readers are deployed adjacently, as shown in Figure 2.1-2, a 1-bit transaction ID may be insufficient to distinguish old and new services from different BSs. In detail, if the transaction ID is too short, there could be two different services from 2 different neighbour BSs that use the same transaction ID. Then the device will miss a service from one of the 2 readers. Therefore, more than one bit transaction ID may be needed. Besides, multiple-bit transaction ID can also reduce the possibility of wrap round issue. If the device failed to receive the paging messages for the new (the second) service, the device can still process the third service instead of regarding it as an old service (the first one).

Figure 2.1-2		Device in an overlapped coverage of two readers

Observation 5:	If the transaction ID is of an insufficient length, there is a risk that two different services, originating from different neighbour BSs, might end up using the same transaction ID. This potential issue provides the motivation for using a transaction ID with more than one bit in scenarios involving multiple BSs.

To balance the purpose of reducing the required buffer size mentioned above, we propose a 2-bit transaction ID in the paging message is sufficient to differentiate the current service.

Proposal 2:	A 2-bit “transaction ID” within the paging message is adequate to identify the current service for the subsequent paging purpose.

### 2.1.2	Case of response to different readers for the same service

It was discussed whether there is a scenario that a device responds to paging messages for the same service but from different readers. As described in TR 38.769, so far, it is not clear whether such scenario is included in the WI scope.

TR 38.769 It needs to be further discussed for the scenario that different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response. If this scenario is in the scope, it needs to be further discussed, by considering the progress from all the WGs.

Some companies consider to support a device to respond to paging messages of the same service request from different readers, for some use cases, e.g., locating use cases. To support such use cases, a separate ID (e.g., like reader ID) in paging message is needed, in order to indicate the paging needs to be redundantly responded even if being triggered by the same service request. Considering the services are anyway triggered by the CN, it is up to CN to decide whether the device needs to redundantly respond to different readers or not.

Observation 6a:	Whether the device needs to respond to different readers for one service or not may be up to CN’s decision, if considering the multiple-reader case.

If a multiple-reader case is considered, there are two options addressing this:

- Option 1: CN allocates different session/correlation ID for different readers for the same service if it intends devices to respond to different readers.

- Option 2: CN indicates a reader whether to include additional identification information in paging messages. Based on CN instruction, the reader decides whether to include reader ID in the paging message.

Both options have impacts on RAN3 and SA2, e.g., correlation ID design/allocation in SA2, XX/NG-AP indication in RAN3.

Observation 6b:	Controlling the device’s response to different readers requires the impacts to RAN3 and/or SA2. The CN has two options: manage the session/correlation ID on a per-reader basis, or indicate the reader whether to include the reader ID in a paging message.

The session/correlation ID (in option 1) or reader ID (in option 2) should be different among readers. So this requires the size of reader ID or session/correlation ID needs to be large enough to avoid collision among readers. This causes a higher AS transmission overhead. Besides, the devices should also keep the session/correlation ID per reader (or per reader ID), which is used to be compared with the ID in subsequent paging messages. This also causes a higher requirement of the buffer size.

Observation 6c:	To avoid collisions, the size of the reader ID/session ID/correlation ID needs to be relatively large. However, this results in increased overheads and device buffer requirement.

It is proposed not to specify any solution to ensure and control the devices need to redundantly respond to different readers for the same service. But implementation solution is still allowed. One implementation example can be that the transaction ID is the truncated part of correlation ID from CN associated with the service. If the CN intends to ask the device to redundantly respond to different readers for the same service, the CN can send different correlation IDs to different readers for such service. Otherwise, the same correlation ID can be sent to multiple readers for such service. In summary, it is not worthy to spend too much specification efforts to guarantee the reader-edge devices behaviour, while the CN implementation manner can be the best efforts solution.

Observation 6d:	An example implementation for controlling responses to different readers could be: In the CN implementation, it can send/allocate appropriate correlation ID to BSs/reader. On the reader side, its implementation can use a truncated portion of correlation ID (e.g., the rightmost 4 bits) as the transaction ID within paging message.

Proposal 3:	In the case where devices need to respond redundantly to different readers for the same service, RAN2 agrees to reuse the transaction ID while not to introduce the reader ID in the paging message. The control of the transaction ID is left to the network implementation.

## 2.2	Paging message design/content

### 2.2.1	Paging message design/content

As captured in SA2 TR 23.700-13, the format of device ID is given:

SA2 TR 23.700-13 The permanent Ambient IoT Device Identifier includes the following information: 1) Part1information: - The ID type, including - information indicating whether the network identifier is included or not. - information indicating whether information used to identify a 3rd party is included or not. - the Part2 type indicating EPC or the other format. - A network identifier (i.e. MCC+MNC and/or NID), when the ID includes the network identifier. - Information used to identify a 3rd party when the ID includes the information used to identify a 3rd party. 2) Part2 information: - The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

For the part2 information, EPC-like information can be included in device identifier. The EPC is a unique identifier for an item that can be used in supply chain management systems to track and manage products, particularly in RFID (Radio Frequency Identification) systems where EPCs are often used. The EPC length can vary from 96 to 496 bits according to EPC Tag Data standard [1], and directly impacts the information capacity about the product. For example, longer EPC can include more details like manufacturer and product type, and can also be used to associate a larger number of products.

Besides, RAN2 agreed to consider multiple cases of paging identifier in an A-IoT paging message, i.e., device ID or group ID. Considering the length of device identifier, i.e., device ID or group ID, may vary, the A-IoT paging message should include a field to indicate the length of paging identifier field. The size and details are FFS, e.g., can be decided further after SA2 concludes the range of the size of the device identifier.

Proposal 4:	In a A-IoT paging message, there is a field dedicated to indicating the length of the paging identifier field. Size and details are FFS.

From RAN2 perspective, paging is the first received R2D message, after which device can perform random access. To achieve the slotted-ALOHA random access, the device should be able to first determine its own access occasion, which is selected among all the provided ones. Then, the total number of access occasions or at least the total number of access occasion sets (can include multiple access occasion) should be provided in the beginning, which will be used by device to randomly select one access occasion.

Proposal 5:	In a A-IoT paging message, there is a field dedicated to indicating the total number of access occasions or access occasion sets. Size and details are FFS.

Specifically, if a paging message indicates the number of access occasions to be one, which can be considered as a dedicated access occasion allocated by a reader. By this way, the device is implicitly indicated to perform CFRA. Hence, there is no need to identify CFRA or CBRA by an explicit indication.

Proposal 6:	Whether the random access is contention-based or contention-free can be implicitly indicated by the number of access occasion(s), e.g., a single access occasion implies contention-free access, i.e., there is no need for a dedicated explicit indication of the access type.

In RAN WID:

R19 RAN WID: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. - Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

As clarified in WID, considering future proof, some reserved bit/value(s) of message type in the (sub)header in a paging message may be needed. However, it is not urgent to discuss the future proof/extension in the beginning of WI phase, which can be checked in later WI phase after R20 scope becomes clear.

Proposal 7:	RAN2 assumes that there could be a reserved bit or reserved value(s) of the message type field in paging message to accommodate potential future extension.

### 2.2.2	Paging message size

It is confirmed by RAN1 from previous LS that the maximum R2D TBS can be 1000bits. Given that only one paging identifier can be included in the paging message, e.g., about few hundred bits device ID, the paging message content will not exceed the 1000bits limitation. It is also complicated to support the segmentation of broadcast R2D messages.

Proposal 8a:	RAN2 assumes that there will be no MAC layer segmentation for the A-IoT paging message.

In practical performance as to the R2D coverage, the actual used paging message size may impact the R2D coverage. It is beneficial if RAN2 can control/reduce the paging message size, when possible.

Observation 7:	A shorter paging message size enables better R2D coverage.

On one hand, the device ID size is one major part in paging payload. Shorter device ID is then beneficial, if it can still achieve the global uniqueness, while the design is up to SA and CT.

On the other hand, the possible physical layer parameters also determine the size of paging message. In case RAN2 considers that the whole information/parameters in paging message would results in one big paging message, some of those physical layer parameters can be moved from paging message into another R2D message following the paging message.

Proposal 8b:	RAN2 waits for further progress from RAN1 regarding which physical layer parameters need to be carried in the paging message.

Proposal 8c:	Depending on exact size of all the information/parameters within the paging message, RAN2 can further discuss whether to move some of this content from the paging message to another R2D message.

## 2.3	Others

In previous discussion, some company papers propose to use paging to indicate whether there is subsequent R2D messages. Based on this, devices can determine when to stop R2D monitoring e.g., after sending Msg3. For instance, in inventory-only case, the device can stop monitoring R2D messages once it sends Msg3, while in inventory + command case, it should continue receiving R2D command after sending Msg3.

However, as agreed in previous RAN2 meetings, a R2D transmission after Msg3 is needed to indicate whether Msg3 transmission is successful, i.e., a failure/success feedback indication for re-access purpose, or Msg2 for Msg3 re-transmission purpose. Hence, the device always needs to continuously monitor the subsequent R2D message to determine whether Msg3 transmission is successful after sending Msg3, in any case.

RAN2#127bis agreement: 2. Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. RAN2#128 agreement: 4. (msg3 re-transmission) RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

In addition, the default device behaviour should be continuously monitoring, considering a possible coming paging message for a new service. Therefore, no addition indication on whether there is a subsequent R2D message in paging is needed.

Proposal 9:	Devices do not need to be informed about subsequent messages via the paging message.

# 3	Conclusion

This contribution makes the following observations:

Observation 1:	Supporting parallel services under a single reader will increase the complexity of the device. It will also lead to significantly higher memory consumption, as multiple sets of AS configuration and context associated with multiple services need to be maintained. Moreover, it will add more R2D signalling overhead, as the service ID has to be included in each R2D message.

Observation 2a:	BS implementation can avoid parallel/interleaved services from the neighbour readers that belong to the same BS.

Observation 2b:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 2c:	Parallel services from the neighbour readers require that all the R2D messages include the reader ID/service ID, which implies an unacceptable overhead at the AS layer.

Observation 3a:	The “transaction ID” in the paging message does not have to be identical to the session/correlation ID from CN, if any. In fact, it can be an ID generated by the BS, and this ID can be shorter than the session/correlation ID.

Observation 3b:	To minimize the size of the AS buffer and register requirements, the shorter the “transaction ID” is, the more favorable it is.

Observation 4:	If reader starts a new service-1 after successfully finishing the previous service-0, a 1-bit transaction ID is sufficient for a single reader case.

Observation 5:	If the transaction ID is of an insufficient length, there is a risk that two different services, originating from different neighbour BSs, might end up using the same transaction ID. This potential issue provides the motivation for using a transaction ID with more than one bit in scenarios involving multiple BSs.

Observation 6a:	Whether the device needs to respond to different readers for one service or not may be up to CN’s decision, if considering the multiple-reader case.

Observation 6b:	Controlling the device’s response to different readers requires the impacts to RAN3 and/or SA2. The CN has two options: manage the session/correlation ID on a per-reader basis, or indicate the reader whether to include the reader ID in a paging message.

Observation 6c:	To avoid collisions, the size of the reader ID/session ID/correlation ID needs to be relatively large. However, this results in increased overheads and device buffer requirement.

Observation 6d:	An example implementation for controlling responses to different readers could be: In the CN implementation, it can send/allocate appropriate correlation ID to BSs/reader. On the reader side, its implementation can use a truncated portion of correlation ID (e.g., the rightmost 4 bits) as the transaction ID within paging message.

Observation 7:	Shorter paging message size enables better R2D coverage.

This contribution makes the following proposals:

Transaction ID size and maintenance

Proposal 1a:	Do not support parallel/interleaved services from a single BS reader in the A-IoT radio interface (i.e., reader must complete the procedure associated with one service, before starting a new service).

Proposal 1b:	A device only maintains the status (successful or not) of the current single service.

Proposal 1c:	If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device will update its currently maintained “transaction ID” to the received value.

Proposal 2:	A 2-bit “transaction ID” within the paging message is adequate to identify the current service for the subsequent paging purpose.

Reader ID/response to different readers

Proposal 3:	In the case where devices need to respond redundantly to different readers for the same service, RAN2 agrees to reuse the transaction ID while not to introduce the reader ID in the paging message. The control of the transaction ID is left to the network implementation.

Paging message design/content

Proposal 4:	In a A-IoT paging message, there is a field dedicated to indicating the length of the paging identifier field. Size and details are FFS.

Proposal 5:	In a A-IoT paging message, there is a field dedicated to indicating the total number of access occasions or access occasion sets. Size and details are FFS.

Proposal 6:	Whether the random access is contention-based or contention-free can be implicitly indicated by the number of access occasion(s), e.g., a single access occasion implies contention-free access, i.e., there is no need for a dedicated explicit indication of the access type.

Proposal 7:	RAN2 assumes there could be a reserved bit or reserved value(s) of the message type field in paging message to accommodate potential future extension.

Paging message size

Proposal 8a:	RAN2 assumes that there will be no MAC layer segmentation for the A-IoT paging message.

Proposal 8b:	RAN2 waits for further progress from RAN1 regarding which physical layer parameters need to be carried in the paging message.

Proposal 8c:	Depending on exact size of all the information/parameters within the paging message, RAN2 can further discuss whether to move some of this content from the paging message to another R2D message.

Others

Proposal 9:	Devices do not need to be informed about subsequent messages via the paging message.

# 4	References

- EPC Tag Data Standard, version 2.0.0, available at https://ref.gs1.org/standards/tds/2.0.0