3GPP TSG-RAN WG2 Meeting #129	R2-2500213

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon

Title:	A-IoT data transmission and general aspects

Document for: Discussion and Decision

# 1	Introduction

In the previous RAN2 meetings, some related agreements were achieved in the following:

RAN2#128 agreements on visibility: - The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available. RAN2#128 agreements on segmentation: - Segmentation indication will be discussed in WI phase. - If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment. FFS how this is done. RAN2#128 agreements on AS ID: - Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID. FFS what message is used - Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

In RAN WID, RAN2 scope is concluded:

R19 RAN WID: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Based on the current RAN2 progress, R19 RAN WID, and focusing on the issues without conclusions, this contribution discusses data transmission, segmentation, AS ID, MAC PDU format design, and other aspects.

# 2	Discussion

## 2.1	Data transmission design

### 2.1.1	Message size information

As captured in TR 38.769, two options on how a reader knows D2R message size are listed:

RAN TR 38.769: - It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential. Two options are studied: - Option 1: The estimate of expected following D2R message size from the CN to the reader; - Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed. - Option 2: The simple message size indication from the device to the reader. - Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

Comparison between CN indication and device reporting is as follows:

- CN indication: CN can indicate the expected data size of the following D2R transmission to the reader when sending DL command. Then the reader can schedule the corresponding D2R message according to the CN’s indication, e.g., indicating TBS together with transmitting the DL command. Compared with device reporting-based approach, this method reduces the transmission overhead over A-IoT interface.

- Device reporting: The device can indicate the size of the next D2R data to be transmitted once being triggered by reader. For example, a device can know the data size to be transmitted only after receiving a command message. Then the device reports the message size information in the following D2R message. This increases the additional transmission overheads over A-IOT interface and also increases the complexity of devices. Furthermore, size granularity of message size reporting further impacts the D2R transmission overhead. For example, a more accurate message size is better for scheduling by reader, but takes more overheads.

By comparison between the above two methods, message size information indicated by CN will be more efficient.

Observation 1:	CN providing the D2R message size information is more efficient than the device reporting.

Proposal 1a:	RAN2 should adopt the solution that CN provides the D2R message size, as long as SA2 can confirm that CN can always provide message size information (even if such confirmation may be late).

RAN2 has sent LS to SA2 in R2-2409412, to ask whether CN can provide D2R message size to the reader. As described in SA2 LS reply S2-2413035 and conclusions captured in the up-to-date SA2 TR 23.700-13, there is no final conclusion on whether CN (e.g., AIOTF) can provide D2R message size if AF does not provide it.

SA2 LS reply S2-2413035: Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request? SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

SA2 TR 13.700-13: The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: Editor's note: Final conclusions are assumed to be taken in coordination with RAN WGs. 1. A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. e. The AIOTF may provide the following assistance information to RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request.

According to the reply LS, whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2. If SA2 later confirms that CN cannot always provide D2R message size, RAN2 should support reporting the D2R message size from device side to assist reader scheduling.

Proposal 1b:	RAN2 will support D2R message size information from device side, if SA2 confirms the message size information cannot be always available (e.g. after SA2 Feb. meeting).

The way reader getting D2R message size information also has impacts on segmentation. If the CN can always provide message size information, a reader can directly schedule a segmented/complete transmission without any additional indication from device, e.g., indicating message size information or whether the message is last segment (or segmented). Otherwise, the device needs to transmit this segmentation indication. Therefore, RAN2 can study the segmentation solution under the two directions, i.e. message size information either from CN or from device.

Proposal 1c:	During RAN2#129 meeting, RAN2 can design the segmentation in two different possible directions related to the message size information (originating from CN or device).

If message size information is not available from CN, one alternative solution is to initially allocate a rough TBS for the first D2R transmission by reader. The rough TBS can be determined by reader implementation. There will be two possible cases of transmitting the first scheduled D2R message:

- Case 1: If the allocated TBS is not sufficient for D2R message size, the device transmits a segment of D2R data together with information of the remaining D2R message size. Then reader can schedule the next segment according to the remaining D2R message size. An example is shown in Figure 2.1.1-1.

- Case 2: If the allocated TBS is sufficient for D2R message size, the device directly transmits the whole D2R message.

In case 1, there are some detailed issues which need to be further discussed. For example, RAN2 needs to further discuss whether remaining size information or D2R data is prioritized to be put into D2R MAC PDU if the allocated TBS is not sufficient (e.g., TBS is just sufficient for message size report).

For command use case, command type information can help a reader to estimate the D2R response size from device after a R2D command transmission. Subsequently, the reader can allocate an approximately accurate TBS for this D2R response. For example, to respond a read command, the device should transmit data indicated by read command. A write command response or disable command response  may just include a much shorter acknowledgment information. The D2R message sizes of command responses are significantly different for different command types. Hence, if reader knows the command type information from CN, the reader can allocate a more accurate TBS for the command response.

Figure 2.1.1-1	An example of transmitting remaining size information together with the first segment

Proposal 2:	If the message size information is not always available from CN (still depending on SA2):

In case the reader does not have the D2R message size information:

  - The reader implementation can initially allocate one rough TBS for device’s first transmission (e.g., it may result in D2R segmentation);

  - Then, the device can indicate the “remaining D2R data size” together with the transmission of first segment, if the allocated TBS is not sufficient.

CN should at least indicate the command type information to reader.

### 2.1.2	Segmentation

In this sub-clause, the issues of segmentation are discussed. In previous RAN2 meetings, some agreements on segmentation were achieved:

RAN2#127-bis agreements on segmentations 5. Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). 6. RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

Actually, a reader can know whether data is segmented or whether it is last segment if it knows the D2R message size. Specifically, reader can determine this by comparing the already transmitted data size with D2R message size information from device or CN. Hence, the segmentation indication is not needed.

Proposal 3:	The “segment indication” is not needed anymore, since reader already has the relatively accurate D2R message size information, either from CN or device, to determine the end of one D2R message.

According to the agreements, a device can rely on data stored in the upper layer(s) for segmentation transmission. In our understanding, the device additionally needs some position information to determine which part of upper layer(s) data needs to be transmitted when constructing one segmentation.

When the device transmits the first D2R segment, the segment’s initial start position addressed to the upper layer memory is the start position of the whole D2R message. For example, if receiving Msg2, the device can by default consider the initial start position is first bit/byte of memory of device ID. If a device receives a read command, the command will indicate the initial start position of memory range to be read.

When the device transmits the next D2R segment, the device needs to update its new start position. Assuming a device knows information of initial start position (e.g., be stored by device or indicated by R2D message), to determine the next segment’s start position, there are different potential options:

- Option 1: a) The device remembers the next segment’s position, i.e., addressed to start position of the upper layer memory for the next segment. This position can be obtained by adding the last transmitted bits to last segment's start position, i.e., being gradually updated from the initial start position. b) The device remembers the already transmitted bits, which is used together with the initial start position to decide the next segment’s position. Whether 1a) or 1b) is used depends on device implementation.

- Option 2: The reader indicates the size of already received bits in the R2D message which is used to schedule the next segment.

For the first and subsequent D2R segment transmissions, the end position can be determined by the allocated TBS after the next segment’s position is determined. As shown in Figure 2.1.2-1, the next segment is from “next segment’s position” to “next segment’s position + TBS”. The reader can allocate the sufficient TBS and check whether the whole D2R data is finished according to the D2R size and already transmitted data size.

Figure 2.1.2-1 Updating start and end positions of a segment

Specifically, Figure 2.1.2-2 shows an example of the complete procedure of how segmentation is performed. First, a reader schedules a TBS=50 bits which is not enough for complete 150-bit D2R message transmission. So, the device can only transmit the first 50-bit D2R segment together with remaining size information. After obtaining remaining size of D2R transmission=100 bits, the reader schedules the remaining segments. During the procedure, the device needs to update the next segment’s position information addressed to the NVM before transmitting the next segment, and reader determines the end of segmentation.

Figure 2.1.2-2	An example of procedures of Option 1

Then, we analyse the impacts of the above options:

Logical/calculation complexity impact: For both options, the calculation complexity seems to be similar, because the device anyway needs to calculate the next segment’s position by using position information (either initial start position or last start position) together with offset information (i.e., already or last transmitted bits).

Memory impact: For the above calculation, each option requires the device to temporarily store some information in registers. Even though some information is indicated by reader, devices also need to first store the indicated information for a short time and then use that for calculation. Hence, the required peak memory size is almost the same for both options. In our understanding, the higher peak size of temporarily storing information requires higher register size, so the required register size is almost the same for all options. The only difference is that the maintaining time of these information varies.

Transmission overhead impact: The overhead of option 1 is less than that of option 2.

Proposal 4:	To determine the next bit/position to transmit for one segment of the D2R message:

Option 1: The device remembers the current position;

Option 2: The reader indicates the size of already-received bits.

Another issue of segmentation is whether D2R segment re-transmission is supported:

RAN2#128 agreement on segmentations 5. If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment. FFS how this is done.

If segment re-transmission is supported, only the information of current bit/position of segment is not enough:

- For option 1, given the calculated next segment’s position, the device additionally needs to remember the last segment’s position, or remember the size of last segment. The purpose is to enable devices to transmit data starting from the position of the last segment if the last transmission fails. Besides, the device needs to process one more calculation to determine the position of the last segment. It requires higher register sizes, and adds more calculation complexity.

- For option 2, the segment re-transmission can be performed by reader implementation. For example, reader transmits the same size of already received bits as the previous segment for the next one. However, as analysed above, this option requires more transmission overheads.

Observation 2:	To support the D2R segment re-transmission, device may need to know both the next bit/position to transmit and the bit/position of previous transmission.

On the other hand, supporting R2D segment re-transmission will be much more complex, if R2D segment is supported:

- First, a D2R feedback, e.g., ACK/NACK, is needed by reader to indicate whether the segment is received. This is used to avoid that the reader transmits the next segment while the current segment transmission is not successful.

- Second, if ACK feedback is transmitted by device but not received by reader, the reader will erroneously consider that the transmission is failed. The reason is that the reader considers that the transmission is not successful when the time between a R2D and corresponding D2R transmission, e.g., TR2D, expires. If the reader re-transmits a R2D message which has been received by device, the reassembled packet will contain duplicates (AS segmentation) or the device will execute the duplicate commands (upper layer segmentation). To solve this issue, the SN is needed to ensure the in-order and reliable delivery of data packets.

Observation 3:	R2D segment re-transmission requires to introduce the SN, since the only NACK feedback does not work if missed.

However, SN for segmentation is not supported in RAN2#127bis agreement.

RAN2#127bis agreement 3. Do not support sequence number for A-IoT segmentation functionalities. 4. Do not support segment number and the number of segments for A-IoT segmentation functionalities.

As analysed above, segment re-transmission is not supported because it requires higher register sizes/transmission overheads, higher calculation complexity, and is much more complex for R2D segment.

Proposal 5:	Segment re-transmission is not supported.

As for R2D segment, as described in RAN2#127bis agreements, RAN2 can postpone to discuss R2D segmentation after agreements on clear solutions of D2R segmentation.

RAN2#127bis agreement 2. Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

Proposal 6:	RAN2 postpones the R2D segmentation discussion (i.e., whether and how to support), until the D2R segmentation solution becomes clear.

Due to the limited AS buffer size and device capability (e.g., very low writing speed), the allowed size of R2D write command is limited. This is because the device may need to buffer the received write command before the device can actually perform the write operation into NVM after CRC checking. For example, the required maximum write command size is just 16 bits for the device.

Observation 4:	The device capability and AS buffer size limitation restrict the allowed maximum R2D message size for write command case.

### 2.1.3	AS ID

In the previous RAN2 meetings, AS ID was studied:

RAN2#127bis: - RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined.

From RAN2 perspective, AS ID is needed at least for the device identification, i.e., can be associated with a R2D message. When X>1 Msg1 time-domain resources are triggered by one R2D trigger, or FDMed Msg1s are triggered, multiple devices have chances to successfully access in the same RA procedure and optionally perform subsequent data transmission. If there is subsequent data transmission, the reader will transmit a unicast R2D message to carry the upper-layer data or schedule a D2R transmission, and AS ID is needed to identify which device can see the content.

Proposal 7a:	RAN2 confirms that the AS ID is needed at least for the device identification purposes, including:

to identify the device supposed to receive the R2D message, and

to identify the device supposed to use the D2R scheduling.

To associate AS ID with a R2D message, there are two options: 1) carry AS ID in R2D MAC PDU, and 2) AS ID is associated with a CRC mask in layer 1, which is used to get CRC attachment. Compared to option1, option 2 can save additional MAC PDU overheads. But whether to support option 2 is up to RAN1. Whether AS ID is carried in R2D MAC PDU can be decided after RAN1 achieves the final conclusions on CRC discussion.

Proposal 7b:	It is up to RAN1 whether to support the CRC mask associated with this AS ID. Subsequently, it can be further decided whether the AS ID is carried in R2D MAC PDU.

For a broadcast R2D message, e.g., paging/QueryRep-like and Msg2 corresponding to multiple Msg1, it is expected to be received by multiple/all devices. There is no need to associate a broadcast R2D message with AS ID.

Proposal 7c:	For R2D transmission, the AS ID only applies to the unicast R2D messages, and not to the broadcast R2D messages (e.g., A-IoT paging and Msg2).

If the above option 2 is supported, the relationship between the size of AS ID and the number of bits of CRC mask can be further decided by RAN1. CRC is still under RAN1 discussion, and there are different options of the length of CRC lengths.

If this AS ID is going to be included in MAC PDU, 16-bit seems to be long from overhead perspective. RAN2 may need to discuss how to reduce the length and overhead.

Proposal 7d:	The size of the AS ID can be decided later after RAN1 concludes the usage of this AS ID.

Another issue is AS ID collision. If one AS ID is considered to be used in multiple random access rounds, i.e., triggered by multiple paging/QueryRep-like messages, there is a probability of facing an AS ID collision issue. To avoid this issue, the reader should use an AS ID to schedule a device only in one set of access occasions triggered by one paging/QueryRep-like message. The device which still does not successfully complete data transmission should perform re-access, and a new AS ID is generated in the next re-access round. Hence, one way is to only consider AS ID is valid only in one access round, so that there is no need to handle the AS ID collision across multiple access rounds. This can also reduce the power consumption of the device and also the size of AS ID.

Observation 5:	One way to reduce the size of AS ID is to consider the AS ID as valid only within one access round.

As to AS ID generation, three options are captured in TR 38.769:

TR 38.769: From higher layer perspective, following options are possible for this "AS ID" (it is aimed to define one common design for all access procedures in clause 6.3.4, if technically possible): - Option 1: a random ID (if used in first D2R message) can be reused; - Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message. - Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message. - NOTE 1: The further down-selection for "AS ID" needs to consider the involved message and whether the reader needs to handle the collision.

In the case where the same paging/QueryRep-like triggers multiple Msg1 transmissions, it is also possible that the random ID in Msg1 may collide across access occasions. Hence, the collision probability will have impact on whether the random ID can be reused as AS ID.

Observation 6:	Whether the random ID after contention resolution can be reused as the AS ID depends on the collision probability among those devices (e.g., whose Msg1 are triggered by the same R2D message and successfully echoed by Msg2).

Furthermore, we give an estimate of the above collision probability, which is the probability that there are devices in different occasions selecting the same random ID as the AS ID. Denote the number of devices which successfully access in different occasions by N. If the first device randomly selects a random ID, the probability that the second device randomly selects a different ID is (216-1)/216. By analogy, the probability that the N-th device selects an ID different with the other N-1 devices’ ID is (216-(N-1))/216. Hence, the probability that N device’s random ID is totally different from each other is:

P0=216×216-1…216-N-1216N.

The probability that at least two devices generate the same 16-bit random ID as AS ID is:

P=1-P0=1-216×216-1…216-N-1216N.

According to analysis in R1-2409419 [3], the total number of access occasions triggered by one R2D message is limited, e.g., no more than 5 and 8 time- and frequency-domain resources for msg1 transmission, respectively. So we consider that the peak value of the number of occasions is about 40 as one example.

Table 2.2.3-1	Calculation results of AS ID collision probability P with different number of successful devices N

| N | P | N | P | N | P |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00000% | 6 | 0.02289% | 11 | 0.08389% |
| 2 | 0.00153% | 7 | 0.03204% | 12 | 0.10066% |
| 3 | 0.00458% | 8 | 0.04272% | 13 | 0.11896% |
| 4 | 0.00916% | 9 | 0.05492% | 14 | 0.13877% |
| 5 | 0.01526% | 10 | 0.06864% | 15 | 0.16010% |

Then, we substitute N=1~15 into the above formula, and the results are shown in Table 2.2.3-1. Considering the optimal successful access probability in ALOHA [5] as 36.8%, under the above condition, the number of successful devices is about 15 (i.e., N=15). Even for this case, according to the Table 2.2.3-1, the corresponding collision probability P is about 0.16%.

Observation 7a:	If the random ID reused as the AS ID, the AS ID collision probability within the same Msg2 is below 0.16%.

As in our Proposal 7a, the AS ID is applied to R2D reception and D2R scheduling, which can be affected by AS ID collision. For example, if the AS ID of two devices in Msg4 collide, their Msg5s will be collided due to both of them receive the same scheduling information, e.g., time-/frequency-domain resources for Msg5 transmission. However, the Msg4 scheduling Msg5 is only included in command use cases.

As analysed above, the collision issue of AS ID seems more critical in the case of D2R scheduling than in R2D reception, because the device’s corresponding D2R transmission may collide if D2R scheduling is associated with the same AS ID. Specifically, the D2R scheduling is included in Msg4, which is used to schedule Msg5 and later messages.

To solve the above issue, the reader can by implementation filter the received Msg1s which have no collided random ID. The reader does not echo the random ID which is the same in different access occasions. That means, the devices who select the same random ID will not receive correct Msg2 and consider a failure of contention resolution. Then, they can perform re-access.

Observation 7b:	Assuming random ID is reused as the AS ID, in case of AS ID collision, inventory case is not impacted. While for command case, the reader can decide not to echo the two devices with the same random ID, which avoids the situation that the two devices cannot differentiate its own Msg4 and Msg5 scheduling.

Jointly considering the above observations, we consider that reusing random ID as AS ID may be feasible.

Observation 7c:	It may be feasible to reuse the random ID as the AS ID, if RAN2 assumes the AS ID collision issue/valid issue is only considered with a short period (e.g., in one access round).

As an alternative to avoid the AS ID collision, the location of the access occasions can be used instead of the random ID. For instance, the AS ID only needs to be different among different access occasions triggered by the same paging/QueryRep-like message. According to analysis in R1-2409419[3], no more than 40 access occasions are considered, so 6 bits mapping to access occasions is sufficient. Hence, AS ID can also be defined to be the time-frequency occasion index of Msg1 transmission, and the length is shorter than the size of 16-bit random ID.

Observation 8:	It is also feasible to use the association between the Msg1 and its time-frequency resource to form a shorter AS ID, assuming the AS ID is only valid in the time duration controlled by the R2D message allocating the time-frequency resource, e.g., just 6 bits.

However, if there is no such assumption on the AS ID validity duration, the solution of deriving AS ID based on Msg1 time-frequency resource info may not be able to work well. In this case, we can consider the alternative to always allocate a new AS ID. Nevertheless, in this case the AS ID does not need to be 16-bit long. For instance, if the new AS ID is 6-bit, the reader can already handle 64 devices (i.e. 64 devices are monitoring their Msg4) at the same time, which should be sufficient. Additionally, for this case,	to identify the Msg2 that corresponds to the transmitted Msg1 of device(s), the information/index derived from the time/frequency resource of Msg1 needs to be carried in Msg2.

Observation 9:	The solution that the Msg2 always allocates a new shorter AS ID (e.g. 6 bits) can also address the AS collision issue. However, it comes at the cost of AS signalling overhead due to a larger Msg2 size.

We understand the above three alternatives are all feasible (under some conditions), but with different Pros and Cons. Therefore, we think it would be useful that companies can first align the understanding on the solutions, and then discuss which one to go for.

Proposal 8:	For AS ID allocation, RAN2 discusses the following alternatives:

- Alt. 1: The 16-bit random ID in Msg1 is reused as the AS ID, and it is up to the reader implementation to handle the AS ID collision;

- Alt. 2: Msg2 always re-allocates a shorter AS ID (e.g. 6 bits), and the reader implementation controls the AS ID valid duration;

- Alt. 3: A shorter AS ID is derived from the used time-frequency resource of Msg1 (e.g., less than 6 bits), assuming that the AS ID is only valid within the duration before the reader triggers the next set of Msg1 resources for other devices.

CFRA can be triggered by paging, which includes the device ID. According to RAN WID:

R19 RAN WID: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In the WID, a paging is considered to only include one identifier, and multiple identifiers is considered only for forward compatibility. In R19 WI, RAN2 can assume no more than one identifier can be contained in a paging. That means, only one device is supposed to be triggered to perform CFRA. In this case, R2D messages can only be sent to this paged device. So there is no need of AS ID for CFRA cases to differentiate the devices.

Proposal 9:	RAN2 assumes there is no need of AS ID for CFRA case (i.e., only single device is handled during the current procedure).

## 2.2	MAC PDU format design principle

In SI, some agreements related to AS protocol stacks and functionalities were made as below:

- Only one L2 AS layer is supported, i.e. A-IoT MAC layer, and there is no RRC layer/PDCP layer/SDAP layer between MAC layer and upper layer (e.g. A-IoT NAS layer).

- There is no CP/UP protocol stack differentiation on AIoT interfaces.

It can be easily understood that all the HL AS functionality, i.e. paging, RACH, segmentation, are to be defined in MAC layer. Therefore, the MAC PDUs need to be defined to carry all essential AS parameters and A-IoT upper layer data/message during those AS procedures.

In the following table, we summarize the potential MAC PDU and parameters based on the RAN2 SI outcome.

Table 2.2-1 MAC procedures and corresponding MAC PDUs

| MAC procedures | Message/MAC PDU | Message content | PHY channel | Virtual cast type |
| --- | --- | --- | --- | --- |
| Paging procedure | Paging message | - Paging identifier, - Retransmission indication, - RA configuration | PRDCH | Broadcast message |
| Random Access procedure | Msg1 | - Random ID, - FFS device ID (pending to discussion on unified solution) | PDRCH | Device-specific message |
|  | Msg2 | - Echo of random ID, - Resource allocation/scheduling info for Msg3 transmission | PRDCH | Broadcast message |
| R2D data transfer procedure | R2D command message | - A-IoT upper layer data (i.e. command), - FFS AS ID | PRDCH | Device-specific message |
| D2R data transfer procedure | Msg3 | - A-IoT upper layer data (i.e. Device ID), - FFS D2R padding, - FFS AS ID | PDRCH | Device-specific message |
|  | D2R command response message | - A-IoT upper layer data (i.e. command response), - FFS D2R padding, - FFS AS ID | PDRCH | Device-specific message |
| D2R segmentation procedure | R2D message for D2R scheduling | - Resource allocation/scheduling info for next D2R segment | PRDCH | Device-specific message |
|  | D2R message for one segment | - One segment of A-IoT upper layer data, - Segmentation indication/message size info (if any), - FFS D2R padding | PDRCH | Device-specific message |
| Failure/success indication | R2D ACK/NACK indication | - AS ID, - ACK/NACK indication | PRDCH | Broadcast message (TBD) |
| Others if any | … | … | … | … |

For the A-IoT MAC PDU format (i.e. how to carry all above parameters in A-IoT MAC PDUs), the most fundamental principle should be to make it as simple as possible, taking into account device processing complexity, AS signalling overhead, as well as specification efforts.

Proposal 10:	RAN2 should strive for the simple A-IoT MAC PDU format, considering device processing complexity, AS signalling overhead, and specification efforts.

For the detailed design, there are two directions.

1. RFID-like MAC PDU format, which means each message (e.g. Interrogator Command) consists of multiple specified fields, e.g. Command Code (acting like a header), Length. The description of each field is clearly specified (e.g. captured in a table). For instance, the format of Select command can be found in the following table in EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard.

Excerpt from EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard

2. NR-like MAC PDU format, which means one MAC PDU consists of one or more MAC subPDU, where each MAC subPDU consists of a MAC subheader and payload containing either a MAC CE or a MAC SDU (i.e. PDU from layer above MAC).

Excerpt from TS 38.321 Figure 6.1.2-4: Example of a DL MAC PDU

In NR uplink, each MAC CE has its own trigger conditions, and once the UE has UL resources, among the triggered MAC CE(s) and the available UL data, the UE shall perform LCP and then multiplex MAC CEs and MAC SDUs in a MAC PDU. In NR downlink, upon reception of a MAC PDU, the UE decodes the first subheader and knows the format of the first subPDU, and then decodes the next subPDU, until the end of this PDU. In this way, for both of uplink and downlink, whether a subPDU is present is indicated by the presence of the subheader, thus it’s quite flexible to assemble which/how many MAC CE(s) and/or MAC SDU(s).

Both formats are feasible to clearly specify the information delivered in the MAC PDU, so that the recipient can decode the MAC PDU easily and interpret the received information correctly. However, either one may not fit for A-IoT system perfectly. For instance,

- RFID-like format is quite simple and clear, but all the parameters are listed for each message, which would create duplications when those parameters present in multiple messages. As shown in Table 2.1.2-1, in A-IoT system, the resource allocation/scheduling info is to be included in the paging message, Msg2, as well as R2D command message. If we follow RFID-like format, those parameters will be specified several times for each message. Moreover, in the future, when there is any enhancement/correction to those parameters, all related messages will be impacted. This leads to more standard/maintenance work.

- The MAC CE-like style in NR MAC format seems to be a good way to organize the parameters for the same functionality (e.g. resource allocation/scheduling info), but the flexibility of multiplexing may not be all useful for A-IoT, so it can be simplified to reduce the device complexity. For instance, LCP is not needed, and some of the subheaders can be skipped when the message content (i.e. the combination of the subPDUs) is fixed. On the other hand, if some parameters are optional in a message, subheader is still useful to indicate the presence/absence of the subheader/subPDU.

Please note, here we do not differentiate the resource allocation/scheduling info in paging message when targeting multiple devices or one device, or in Msg2 or R2D command message when dedicated scheduling info is provided, because those details are quite related to RAN1 discussion. If in the end, the specific time/frequency of radio resource can be derived by some rules instead of explicit configuration, and only limited parameters need to be provided via AS signalling, then there may be no difference between the common scheduling info and dedicated scheduling info. Otherwise, RAN2 may need to define two scheduling info, one is for common resource, and the other is for dedicated resource. Nevertheless, the situation still exist that the common resource or dedicated resource need to be present in more than one messages. Thus, for now we just put one scheduling info to simplify the discussion, and further adjustment could be done after seeing more RAN1 inputs.

Observation 10:	If we follow RFID format style, some parameters (e.g. D2R scheduling info) are likely to be repeatedly specified in different messages (e.g. paging message, Msg2, R2D command message) in A-IoT MAC. This leads to redundant standard/maintenance efforts.

Observation 11a:	If we follow NR MAC format style, only the first subheader is necessary, while the subsequent subheaders may be redundant, unless the parameters are optional.

Observation 11b:	If we follow NR MAC format style, the LCP is not needed.

Based on the above analysis, we should take the advantages of the two solutions into consideration, in order to design a more suitable format for A-IoT MAC PDU. To reduce the device complexity, all the possible message content/the combination of the MAC CEs/SDU should be clearly defined in the specification (e.g. via procedure text, in a table, or in other manner). The LCID field is not needed in each subheader, as those information can be replaced by the message type indication in a PDU header.

Figure 2.3-1: Example of an A-IoT R2D/D2R MAC general PDU format

Proposal 11a:	The concept of MAC CE can be reused, in order to aggregate some parameter(s)/configuration(s) into a better-organized unit.

Proposal 11b:	An A-IoT MAC PDU consists of a message type indication (as the common header) and one or more subPDUs. Each subPDU can include either an A-IoT MAC CE or an A-IoT MAC SDU (i.e. upper layer data).

Proposal 11c:	All possible A-IoT MAC PDU contents (i.e., the combination of the A-IoT MAC CEs and/or MAC SDU) should be explicitly captured in the specification (e.g., via procedure text, in a table, or some other manner).

If the TB size and MAC PDU are not fully matched, padding may be needed. But it's not clear yet whether the padding should be done in PHY layer or MAC layer, and whether both of R2D and D2R directions will require padding. The final decision can be made after the TBS and message size discussion has progressed more.

Proposal 12:	Whether to support D2R and/or R2D padding in MAC layer can be decided later (e.g., after further conclusions on MAC format, TBS and message size).

Upon receiving a R2D/D2R MAC PDU, the device/reader first decodes the header of the MAC PDU, and figure out the message type and content by the message type indication. Since the Rel-19 A-IoT system only supports several messages, it seems that 3 or 4 bits should be sufficient for message type indication. If the MAC CE in a subPDU has variable size, the L field should be present in the subheader. It’s the same for MAC SDU, as the data length is not fixed. For the subPDU including fixe-sized MAC CE, the subheader can be omitted. It should be mentioned is that in RFID system, there is no header defined for RN16. This is because only RN16 is expected from devices after a paging message, so no ambiguity with other D2R messages. From signalling overhead perspective, in A-IoT system, the Msg1 could skip header as well.

Proposal 13:	FFS whether 3 or 4 bits are sufficient for the message type indication.

Proposal 14:	For an A-IoT MAC CE with variable size or a SDU, an L field in its subheader is present to indicate the length; for an A-IoT MAC CE with fixed size, the L field is absent. The length of the L field should be short, and FFS how many bits are needed for this field.

Proposal 15:	FFS whether the MAC PDU header of Msg1 can be omitted (i.e., the Msg1 MAC PDU only includes the random ID).

Another aspect is future proofing purpose. For instance, in the future more messages/parameters may be introduced on top of the Rel-19 design. One way to design a future proof format is to reserve some values of the message type indication. For instance, type #000 is not used in Rel-19, and it is assumed to be reserved for future releases. Another way is to specify a reserved bit in the MAC PDU header.

Proposal 16:	One bit in the MAC PDU header or some values of message type indication can be reserved for future-proofing purpose.

## 2.3	Command only

RAN2#126 discussed the “Command only” use case and agreed the following:

RAN2#126 agreement: From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it: Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command)

For the option of including DL command in the initial trigger message, i.e., the A-IoT Paging Message (Step A in TR 38.769 [1]), there will be a security issue, especially for Write and Disable command. For example, once the unprotected Write command is transmitted in a clear text, an attacker may tamper a Write command and send a fake Write command to the A-IoT device. It may change all the data stored in the memory, even including the device ID.

The SA2 Ad-hoc meeting concluded to remove the EN about “command-only” (See TR 23.700-13-130).

Proposal 17:	RAN2 confirms not to support “command-only” case by including command in the paging message, as concluded by last SA2 meeting.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	CN providing the D2R message size information is more efficient than the device reporting.

Observation 2:	To support the D2R segment re-transmission, device may need to know both the next bit/position to transmit and the bit/position of previous transmission.

Observation 3:	R2D segment re-transmission requires to introduce the SN, since the only NACK feedback does not work if missed.

Observation 4:	The device capability and AS buffer size limitation restrict the allowed maximum R2D message size for write command case.

Observation 5:	One way to reduce the size of AS ID is to consider the AS ID as valid only within one access round.

Observation 6:	Whether the random ID after contention resolution can be reused as the AS ID depends on the collision probability among those devices (e.g., whose Msg1 are triggered by the same R2D message and successfully echoed by Msg2).

Observation 7a:	If the random ID reused as the AS ID, the AS ID collision probability within the same Msg2 is below 0.16%.

Observation 7b:	Assuming random ID is reused as the AS ID, in case of AS ID collision, inventory case is not impacted. While for command case, the reader can decide not to echo the two devices with the same random ID, which avoids the situation that the two devices cannot differentiate its own Msg4 and Msg5 scheduling.

Observation 7c:	It may be feasible to reuse the random ID as the AS ID, if RAN2 assumes the AS ID collision issue/valid issue is only considered with a short period (e.g., in one access round).

Observation 8:	It is also feasible to use the association between the Msg1 and its time-frequency resource to form a shorter AS ID, assuming the AS ID is only valid in the time duration controlled by the R2D message allocating the time-frequency resource, e.g., just 6 bits.

Observation 9:	The solution that the Msg2 always allocates a new shorter AS ID (e.g. 6 bits) can also address the AS collision issue. However, it comes at the cost of AS signalling overhead due to a larger Msg2 size.

Observation 10:	If we follow RFID format style, some parameters (e.g. D2R scheduling info) are likely to be repeatedly specified in different messages (e.g. paging message, Msg2, R2D command message) in A-IoT MAC. This leads to redundant standard/maintenance efforts.

Observation 11a:	If we follow NR MAC format style, only the first subheader is necessary, while the subsequent subheaders may be redundant, unless the parameters are optional.

Observation 11b:	If we follow NR MAC format style, the LCP is not needed.

Message size information

Proposal 1a:	RAN2 should adopt the solution that CN provides the D2R message size, as long as SA2 can confirm that CN can always provide message size information (even if such confirmation may be late).

Proposal 1b:	RAN2 will support D2R message size information from device side, if SA2 confirms the message size information cannot be always available (e.g. after SA2 Feb. meeting).

Proposal 1c:	During RAN2#129 meeting, RAN2 can design the segmentation in two different possible directions related to the message size information (originating from CN or device).

Proposal 2:	If the message size information is not always available from CN (still depending on SA2):

In case the reader does not have the D2R message size information:

  - The reader implementation can initially allocate one rough TBS for device’s first transmission (e.g., it may result in D2R segmentation);

  - Then, the device can indicate the “remaining D2R data size” together with the transmission of first segment, if the allocated TBS is not sufficient.

CN should at least indicate the command type information to reader.

Segmentation

Proposal 3:	The “segment indication” is not needed anymore, since reader already has the relatively accurate D2R message size information, either from CN or device, to determine the end of one D2R message.

Proposal 4:	To determine the next bit/position to transmit for one segment of the D2R message:

Option 1: The device remembers the current position;

Option 2: The reader indicates the size of already-received bits.

Proposal 5:	Segment re-transmission is not supported.

Proposal 6:	RAN2 postpones the R2D segmentation discussion (i.e. whether and how to support), until the D2R segmentation solution becomes clear.

AS ID

Proposal 7a:	RAN2 confirms that the AS ID is needed at least for the device identification purposes, including:

to identify the device supposed to receive the R2D message, and

to identify the device supposed to use the D2R scheduling.

Proposal 7b:	It is up to RAN1 whether to support the CRC mask associated with this AS ID. Subsequently, it can be further decided whether the AS ID is carried in R2D MAC PDU.

Proposal 7c:	For R2D transmission, the AS ID only applies to the unicast R2D messages, and not to the broadcast R2D messages (e.g., A-IoT paging and Msg2).

Proposal 7d:	The size of the AS ID can be decided later after RAN1 concludes the usage of this AS ID.

Proposal 8:	For AS ID allocation, RAN2 discusses the following alternatives:

- Alt. 1: The 16-bit random ID in Msg1 is reused as the AS ID, and it is up to the reader implementation to handle the AS ID collision;

- Alt. 2: Msg2 always re-allocates a shorter AS ID (e.g. 6 bits), and the reader implementation controls the AS ID valid duration;

- Alt. 3: A shorter AS ID is derived from the used time-frequency resource of Msg1 (e.g., less than 6 bits), assuming that the AS ID is only valid within the duration before the reader triggers the next set of Msg1 resources for other devices.

Proposal 9:	RAN2 assumes there is no need of AS ID for CFRA case (i.e., only single device is handled during the current procedure).

MAC PDU format design principle

Proposal 10:	RAN2 should strive for the simple A-IoT MAC PDU format, considering device processing complexity, AS signalling overhead, and specification efforts.

Proposal 11a:	The concept of MAC CE can be reused, in order to aggregate some parameter(s)/configuration(s) into a better-organized unit.

Proposal 11b:	An A-IoT MAC PDU consists of a message type indication (as the common header) and one or more subPDUs. Each subPDU can include either an A-IoT MAC CE or an A-IoT MAC SDU (i.e. upper layer data).

Proposal 11c:	All possible A-IoT MAC PDU contents (i.e., the combination of the A-IoT MAC CEs and/or MAC SDU) should be explicitly captured in the specification (e.g., via procedure text, in a table, or some other manner).

Proposal 12:	Whether to support D2R and/or R2D padding in MAC layer can be decided later (e.g., after further conclusions on MAC format, TBS and message size).

Proposal 13:	FFS whether 3 or 4 bits are sufficient for the message type indication.

Proposal 14:	For an A-IoT MAC CE with variable size or a SDU, an L field in its subheader is present to indicate the length; for an A-IoT MAC CE with fixed size, the L field is absent. The length of the L field should be short, and FFS how many bits are needed for this field.

Proposal 15:	FFS whether the MAC PDU header of Msg1 can be omitted (i.e., the Msg1 MAC PDU only includes the random ID).

Proposal 16:	One bit in the MAC PDU header or some values of message type indication can be reserved for future-proofing purpose.

Command-only

Proposal 17:	RAN2 confirms not to support “command-only” case by including command in the paging message, as concluded by last SA2 meeting.

# 4	References

- 3GPP TR 23.700-13 V1.2.1, Study on Architecture support of Ambient power-enabled Internet of Things, 2024-11

- 3GPP TR 38.769 V2.0.0, Study on solutions for ambient IoT (Internet of Things), 2024-12

- R1-2409419, On frame structure and timing aspects of Ambient IoT, Huawei, HiSilicon, CBN, China Broadnet, Nov. 2024

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.

- EPC Radio-Frequency Identity Generation-2 UHF RFID, Standard Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, Jan. 2024