3GPP TSG-RAN WG2 Meeting #129	R2-2500214

Athens, Greece, Feb. 17th – 21st, 2025

Source:	vivo

Title:	Simulation results of RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#128 meeting, the following agreements were made regarding the RRM measurement prediction:

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

Agreements/Observations to be captured in TR

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.  Companies may report the adopted skipping pattern when providing simulation results.   Companies are not required to run new simulations but can clarify in the spreadsheet.    We can capture in the TR how the skipping patter affects the performance.

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.   FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input.    For temporal domain, the gains are unclear.   RAN2 will focus on frequency domain for cluster based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

In this contribution, we will provide updated simulation results on the agreed scenarios and agreed assumptions.

- Discussion

  - FR1 to FR1 intra-frequency (temporal domain Case B)

    - Evaluation of L1 beam-level measurement as input

For sub-use case 1, L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated. At the previous meeting, it was agreed that for the L1 beam-level measurement, it is up to company’s implementation whether L1 filtering is applied.

To evaluate whether L1 samples (i.e., L1 beam-level measurement without L1 filtering) can be the model input/output, we provide the corresponding simulation results in this section. Some key simulation parameters are shown in the following table:

Table 2.1-1: Key parameters of temporal domain prediction Case B

| Parameter | Values |
| --- | --- |
| Sub-use case | 1 |
| UE speed | 30/60/90 km/h |
| OW length | N*40 ms |
| PW length | 40 ms |
| MRRT | 50% |
| Non-AI benchmark | Sample-and-hold |
| Pattern |  |

When both input and output are L1 samples (i.e., L1 beam-level measurement without L1 filtering), the simulation results of L1 beam-level measurement prediction accuracy are shown in Table 2.1-2.

Table 2.1-2 Simulation results when both input and output are L1 samples

(L1 beam-level Prediction error in dB)

| OW length (ms) |  | 6*40 | 8*40 | 10*40 | 20*40 |
| --- | --- | --- | --- | --- | --- |
| Number of samples within OW |  | 6 | 8 | 10 | 20 |
| Number of samples as input |  | 3 | 4 | 5 | 10 |
| UE speed @ 30km/h | AI | 0.2 | 0.19 | 0.19 | 0.18 |
|  | Non-AI | 0.19 | 0.19 | 0.19 | 0.19 |
| UE speed @ 60km/h | AI | 0.324 | 0.316 | 0.314 | 0.302 |
|  | Non-AI | 0.347 | 0.347 | 0.347 | 0.347 |
| UE speed @ 90km/h | AI | 0.483 | 0.470 | 0.471 | 0.467 |
|  | Non-AI | 0.525 | 0.525 | 0.525 | 0.525 |

It can be observed that the L1 beam-level prediction accuracy of the AI-based approach and non-AI approach is quite close.

Observation 1:	For temporal domain prediction Case B with sub-use case 1, if the model input is L1 samples, the prediction accuracy is quite close to the non-AI approach.

From our understanding, this is because the L1 sample values are unstable and the corresponding rule of RSRP change is not clear, so a longer observation window does not provide more information to improve prediction accuracy. That is, the AI model primarily relies on the last RSRP measurement in the observation window to generate the prediction since the prediction window is short in this case. At the RAN2#128 meeting, RAN2 agreed that “For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant”, which further confirms the rationality of this conjecture. Based on the same principle, we think that the above Observation 1 is also applicable to sub-use case 3 and temporal domain prediction Case A. Therefore, we suggest:

Proposal 1:	The L1 sample is not expected to be the model input and output for temporal domain prediction.

    - Clarification on L1 filtering option

At the RAN#127 meeting, RAN2 discussed and agreed on two L1 filtering options, i.e., sliding and non-sliding L1 filtering. However, we found that certain combinations of L1/L3 filtering options and the sub-use cases for measurement reduction Case B may achieve no gain of measurement reduction.

Sliding L1/L3 filtering

Figure 2.1-1 Sub-use case 1 (input & output: L1 sample)

In sub-use case 1, if both input and output are L1 samples, the sliding L1/L3 filtering can achieve the expected effect of measurement reduction as illustrated in Figure 2.1-1.

Figure 2.1-2 Sub-use case 1 (input & output: L1 filtered RSRP)

The pattern of sub-use case 1 when both input and output are L1 filtered RSRP is illustrated in Figure 2.1-2. The L1 filtered RSRP at Point 05 can be skipped, which means the measurement of L1 samples 01-05 can be skipped. However, if all the L1 samples within the sliding window are needed to derive the L1 filtered RSRP, then the L1 filtered RSRP at Point 04 and Point 06 requires the measurement of L1 samples 00-06. Then all the L1 samples need to be measured and the goal of measurement reduction can not be achieved.

Figure 2.1-3 Sub-use case 2

Figure 2.1-4 Sub-use case 3 (input: L1 filtered RSRP)

Similarly, in sub-use cases 2 and 3, the sliding L1/L3 filtering can not achieve the goal of measurement reduction in the examples of Figure 2.1-3 and Figure 2.1-4.

Non-sliding L1/L3 filtering

Figure 2.1-5 Sub-use case 1 (input & output: L1 sample)

Figure 2.1-6 Sub-use case 1 (input & output: L1 filtered RSRP)

Figure 2.1-7 Sub-use case 2 (input: L3 filtered RSRP)

Figure 2.1-8 Sub-use case 3 (input: L1 filtered RSRP)

In all sub-use cases, the non-sliding L1/L3 filtering can achieve the expected effect of measurement reduction for measurement reduction Case B, and the patterns are shown in Figure 2.1-5 to Figure 2.1-8.

Observation 2:	For intra-frequency temporal domain case B with sliding L1/L3 filtering, the goal of measurement reduction can not be achieved if all the actual measurements of L1 samples within the sliding window are needed to derive the L1 filtered RSRP, except the sub-use case 1 with input & output being L1 sample.

Observation 3:	For intra-frequency temporal domain case B with non-sliding L1/L3 filtering, the goal of measurement reduction can be achieved for all the sub-use cases.

Although the sub-use case 1 with input & output being L1 sample can achieve the gain of measurement reduction, the prediction accuracy is quite close to the non-AI approach as discussed in Section 2.1.1. Therefore, sliding L1/L3 filtering is not considered in the following simulations.

    - Performance Comparison of Sub-use Cases 1/2/3

In the section, the simulation results are provided to compare the prediction performance of sub-use cases 1/2/3 for FR1 case B.

At the RAN2#128 meeting, RAN2 agreed that “Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3”. Within the above three options, Option 1 is similar to the current L3 filtering mechanism, with the difference being that it considers predictive results when calculating the L3 result. Therefore, L3 filtering Option 1 is selected in our simulation.

Some key simulation parameters are shown in the following table:

Table 2.1-3: Key parameters of temporal domain prediction Case B

| Parameter | Values |
| --- | --- |
| Sub-use case | 1, 2, 3 |
| L1/L3 filtering | Non-sliding |
| L3 filtering | Option 1 |
| Input of Sub-use case 1/3 | L1 filtered RSRP |
| OW length | 10*200=2000 ms |
| PW length | 200 ms |
| MRRT | 50% |
| Pattern |  |

The simulation results of sub-use case 1/2/3 are shown in Table 2.1-4.

Table 2.1-4 Simulation results of sub-use case 1/2/3

(L3 cell-level Prediction error in dB)

|  |  | Sub-use case 1 | Sub-use case 2 | Sub-use case 3 |
| --- | --- | --- | --- | --- |
| UE speed @ 30km/h | AI | 0.446 | 1.730 | 0.453 |
|  | Non-AI | 0.538 | 0.538 | 0.538 |
| UE speed @ 60km/h | AI | 0.678 | 2.743 | 0.796 |
|  | Non-AI | 0.77 | 0.77 | 0.77 |
| UE speed @ 90km/h | AI | 0.881 | 3.915 | 1.178 |
|  | Non-AI | 0.955 | 0.955 | 0.955 |

From the above Table 2.1-4, it can be observed that the AI-based approach is better than that of the non-AI approach for sub-use cases 1. We think the gain is due to L1-filtered RSRP being selected as input.

Observation 4:	For temporal domain prediction Case B with sub-use case 1, if the model input is L1 filtered RSRP, the prediction accuracy is better than the non-AI approach.

Besides, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3. For Sub-use case 2, as the input is L3 filtered RSRP which is derived based on earlier pure predictive L3 filtered RSRP, inaccurate predictive results can introduce errors into the next inference, thereby affecting the accuracy of the next inference, i.e., the errors will accumulate. From our understanding, the poor performance of sub-use case 2 is the prediction error accumulation issue. In addition, the performance of sub-use cases 2 and 3 is worse than the Non-AI approach, especially for the UE at high speed. Therefore, for measurement reduction Case B, sub-use case 1 should be prioritized.

Observation 5:	For temporal domain prediction Case B, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3.

Proposal 2:	For measurement reduction Case B, sub-use case 1 should be prioritized.

    - Performance Comparison of Skipping Pattern Examples 1/2

Two skipping pattern examples are captured in the current TR 38.744 [1] for temporal domain case B:

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B

At the RAN2#128 meeting, RAN2 agreed that “For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance”. In the section, the simulation results are provided to compare the prediction performance for the above two skipping pattern examples.

Some key simulation parameters are shown in the following Table 2.1-5. Note that the pattern of example 1 is updated to align the OW length.

Table 2.1-5: Key parameters of temporal domain prediction Case B

| Parameter | Values (skipping pattern example 1) | Values (skipping pattern example 2) |
| --- | --- | --- |
| Sub-use case | 1 |  |
| L1/L3 filtering | Non-sliding |  |
| UE speed | 30/60/90 km/h |  |
| OW length | 1600 ms |  |
| PW length | 400 ms | 200ms |
| MRRT | 50% |  |
| Pattern |  |  |

The simulation results of Cell-level RSRP prediction accuracy for the two skipping pattern examples are shown in Table 2.1-6, it can be seen that the performance of skipping pattern example 2 is better than that of example 1.

Table 2.1-6 Simulation results for the two skipping pattern examples

(L3 cell-level Prediction error in dB)

|  | 30km/h | 60km/h | 90km/h |
| --- | --- | --- | --- |
| Skipping pattern example 1 | 0.72 | 1.08 | 1.38 |
| Skipping pattern example 2 | 0.45 | 0.68 | 0.88 |

Observation 6:	For temporal domain case B, with the same MRRT and OW length, Skipping pattern Example 2 can provide better prediction performance than Skipping pattern Example 1.

  - FR1 to FR1 inter-frequency (frequency domain)

    - Pattern of inter-frequency prediction

For frequency domain prediction with measurement reduction purposes, the pattern/ timing relationship between the measurement (on frequency 1) and prediction (on frequency 2) should be clarified.

From our understanding, the following two options can be considered:

-	Option 1: The input is historical measurement values on Frequency 1(e.g., instances 1-10) and the output is the value on Frequency 2 at the last time instance (e.g., instances 10) of measurement on Frequency 1.

-	Option 2: The input is historical measurement values on frequency 1 (e.g., instances 1-10) and the output is the values on Frequency 2 at the same time instances (e.g., instances 1-10) of measurement on Frequency 1.

Figure 2.2-1(a) Pattern of inter-frequency prediction of Option 1

Figure 2.2-1(b) Pattern of inter-frequency prediction of Option 2

The advantage of Option 2 is that multiple instances on Frequency 2 can be out for one single model inference. On the contrary, multiple model inferences are needed for Option 1 to obtain multiple predicted instances on Frequency 2. However, the typical usage of inter-frequency prediction is inter-frequency handover/redirection, in which Option 2 is not applicable. For instance, extra latency of historical prediction at Frequency 2 may lead to too-late handover.

Therefore, the pattern of Option 1 should be the baseline for inter-frequency prediction.

Proposal 3:	For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

For the inter-frequency simulation, we adopt the pattern of inputs and outputs shown in Figure 2.2-1(a).

    - Simulation results for cluster approach

At the RAN2#128 meeting, RAN2 reached the following agreements:

In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset. For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. In cluster approach the model takes measurements from more than one cell as inputs.

In this section, we provide some simulation results to show which cells among the cluster of input cells are critical for prediction accuracy.

The results of prediction accuracy for different inputs are shown in Table 2.2-1.

Table 2.2-1 Simulation results of inter-frequency prediction from 2GHz to 4GHz

| Input | Prediction error (dB) |
| --- | --- |
| Single Cell (co-located in the same sector) | 0.55 |
| All Cells (21 Cells) | 0.36 |
| 3 Cells of co-located station | 0.35 |
| 18 Cells (exclude 3 Cells of co-located station) | 1.42 |
| 3 Cells with the highest RSRP | 0.24 |

It can be observed that:

- The performance of cluster-based approach with all cells is better than that of single cell as input.

- The performance of 3 co-located cells as input has a close performance with that of 21 cells as input.

- When kicking out the 3 co-located cells from the 21 cells, the prediction accuracy dropped dramatically. It means that cells of the co-located station as input have a significant contribution to the prediction accuracy.

- The performance of the 3 strongest cells as input has a better performance than that of 21 cells as input. It means that a few strongest cells as input have a significant contribution to the prediction accuracy.

Observation 7:	For cluster-based inter-frequency prediction, cells of the co-located station as input have a significant contribution to the prediction accuracy.

Observation 8:	For cluster-based inter-frequency prediction, a few strongest cells as input have a significant contribution to the prediction accuracy.

  - FR2 to FR2 intra-frequency (temporal domain Case A)

In this section, the simulation is utilized to illustrate the prediction accuracy of the instances within the PW window.

At the RAN2#127bis meeting, it was observed that among sub-use cases 1, 2, and 3, at least with a shorter prediction window sub-use case 2 demonstrates the highest prediction accuracy. Some key simulation parameters are shown in the following table:

Table 2.3-1: Key parameters of temporal domain prediction Case A

| Parameter | Values |
| --- | --- |
| Sub-use case | 2 |
| L1/L3 filtering | Sliding |
| L3 filtering | Option 1 |
| OW length | 5*80=400 ms |
| PW length | 5*80=400 ms |
| Pattern |  |

The simulation results of prediction accuracy of the instances within the PW window are illustrated in the following Table 2.3-2:

Table 2.3-2 Simulation results of the predicted instances within the PW window

(L3 cell-level Prediction error in dB)

| UE speed @ 60km/h | AI | [0.232, 0.586, 1.065, 1.621, 2.225] |
| --- | --- | --- |
|  | Non-AI | [0.732, 1.446, 2.133, 2.78, 3.388] |
| UE speed @ 90km/h | AI | [0.328, 0.812, 1.434, 2.156, 2.923] |
|  | Non-AI | [0.918, 1.804, 2.628, 3.396, 4.098] |
| UE speed @ 120km/h | AI | [0.375, 0.922, 1.626, 2.426, 3.267] |
|  | Non-AI | [1.013, 1.969, 2.862, 3.685, 4.429] |

The results show that within the prediction window, the accuracy of the later prediction instances is worse. The specific impact can be analyzed in the simulation of measurement event prediction and/or system-level performance.

Observation 9: For temporal domain case A, the prediction accuracy of multiple prediction instances within the PW decreases in sequence.

  - New scenario: Beam-level RRM measurement prediction

At the RAN2#127 meeting, in addition to cell-level prediction, 3 sub-use cases are considered for beam-level RRM measurement prediction:

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A

We assume that some of the observations of cell-level prediction on FR2 intra-frequency temporal domain case A are also valid for Beam-level, e.g.,

-	For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

-	Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits.

Therefore, both OW and PW length are set as 400 ms in the simulation and the simulation results are shown in Table 2.3-1.

Table 2.3-1 Simulation results of beam-level RRM measurement prediction

(L3 cell-level Prediction error in dB)

|  |  | Sub-use case 4 | Sub-use case 5 | Sub-use case 6 |
| --- | --- | --- | --- | --- |
| UE speed @ 60km/h (Average prediction error within PW) | AI | 1.307 | 1.299 | 1.271 |
|  | Non-AI | 1.993 | 2.287 | 1.993 |
| UE speed @ 60km/h (Multiple predicted instance within PW) | AI | [0.614, 0.811, 1.19, 1.68, 2.241] | [0.46, 0.742, 1.2, 1.745, 2.353] | [0.606, 0.783, 1.145, 1.623, 2.196] |
|  | Non-AI | [0.904, 1.331, 1.929, 2.575, 3.224] | [0.798, 1.577, 2.329, 3.036, 3.699] | [0.904, 1.331, 1.929, 2.575, 3.224] |
| UE speed @ 90km/h (Average prediction error within PW) | AI | 1.786 | 1.71 | 1.68 |
|  | Non-AI | 2.536 | 2.792 | 2.536 |
| UE speed @ 90km/h (Multiple predicted instance within PW) | AI | [0.842, 1.143, 1.647, 2.289, 3.008] | [0.588, 0.974, 1.585, 2.312, 3.095] | [0.717, 1.025, 1.54, 2.191, 2.929] |
|  | Non-AI | [1.186, 1.738, 2.471, 3.257, 4.025] | [0.995, 1.958, 2.856, 3.695, 4.46] | [1.186, 1.738, 2.471, 3.257, 4.025] |
| UE speed @ 90km/h (Average prediction error within PW) | AI | 2.09 | 1.947 | 1.909 |
|  | Non-AI | 2.831 | 3.033 | 2.831 |
| UE speed @ 120km/h (Multiple predicted instance within PW) | AI | [0.981, 1.354, 1.956, 2.681, 3.475] | [0.631, 1.138, 1.826, 2.643, 3.501] | [0.814, 1.175, 1.763, 2.492, 3.304] |
|  | Non-AI | [1.364, 1.964, 2.772, 3.62, 4.437] | [1.095, 2.134, 3.109, 4.008, 4.819] | [1.364, 1.964, 2.772, 3.62, 4.437] |

It can be observed in Table 2.3-1 that the performance of prediction accuracy of sub-use cases 4/5/6 is comparable.

Observation 10:	For beam-level temporal domain prediction, the prediction accuracy of sub-use cases 4/5/6 is comparable.

Note that the above prediction accuracy is calculated based on the prediction for all the beams. However, the intention for beam-level prediction is for RACH resource allocation. Thus only the prediction of the best K (K>= 1) beams are needed and the performance analysis should focus on them, which is quite similar to beam management.

The KPI for beam management is copied as follows:

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams. - Top-1 genie-aided Tx-Rx beam pair considers the following definitions: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options not precluded and can be reported - Average L1-RSRP difference of Top-1 predicted beam: - The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam - Beam prediction accuracy (%): - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - Where K >1 and values can be reported - CDF of L1-RSRP difference for Top-1 predicted beam - Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam" - Other beam prediction accuracy related KPIs are not precluded and can be reported

Within them, we think the average L1-RSRP difference of the Top-1 predicted beam and beam prediction accuracy (%) with 1dB margin for the Top-1 beam can be reused for beam-level temporal domain prediction.

Proposal 4:	For beam-level temporal domain prediction, reuse the following KPIs of AI/ML-based beam management:

- Average RSRP difference of Top-1 predicted beam: The RSRP difference between the Top-1 predicted beam and the actual Top-1 beam.

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam: the percentage of the Top-1 predicted beam whose prediction accuracy is within 1dB.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

FR1 to FR1 intra-frequency (temporal domain Case B)

Observation 1:	For temporal domain prediction Case B with sub-use case 1, if the model input is L1 samples, the prediction accuracy is quite close to the non-AI approach.

Observation 2:	For intra-frequency temporal domain case B with sliding L1/L3 filtering, the goal of measurement reduction can not be achieved if all the actual measurements of L1 samples within the sliding window are needed to derive the L1 filtered RSRP, except the sub-use case 1 with input & output being L1 sample.

Observation 3:	For intra-frequency temporal domain case B with non-sliding L1/L3 filtering, the goal of measurement reduction can be achieved for all the sub-use cases.

Observation 4:	For temporal domain prediction Case B with sub-use case 1, if the model input is L1 filtered RSRP, the prediction accuracy is better than the non-AI approach.

Observation 5:	For temporal domain prediction Case B, the performance of sub-use case 1 is better than that of sub-use case 2 and sub-use case 3.

Observation 6:	For temporal domain case B, with the same MRRT and OW length, Skipping pattern Example 2 can provide better prediction performance than Skipping pattern Example 1.

Proposal 1:	The L1 sample is not expected to be the model input and output for temporal domain prediction.

Proposal 2:	For measurement reduction Case B, sub-use case 1 should be prioritized.

FR1 to FR1 inter-frequency (frequency domain)

Observation 7:	For cluster-based inter-frequency prediction, cells of the co-located station as input have a significant contribution to the prediction accuracy.

Observation 8:	For cluster-based inter-frequency prediction, a few strongest cells as input have a significant contribution to the prediction accuracy.

Proposal 3:	For frequency domain prediction for measurement reduction, the input is a set of historical measurement instances on Frequency 1 and the output is the predicted value on Frequency 2 at the time of the last measurement instance on Frequency 1.

FR2 to FR2 intra-frequency (temporal domain Case A)

Observation 9: For temporal domain case A, the prediction accuracy of multiple prediction instances within the PW decreases in sequence.

New scenario: Beam-level RRM measurement prediction

Observation 10:	For beam-level temporal domain prediction, the prediction accuracy of sub-use cases 4/5/6 is comparable.

Proposal 4:	For beam-level temporal domain prediction, reuse the following KPIs of AI/ML-based beam management:

- Average RSRP difference of Top-1 predicted beam: The RSRP difference between the Top-1 predicted beam and the actual Top-1 beam.

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam: the percentage of the Top-1 predicted beam whose prediction accuracy is within 1dB.

- Reference

- 3GPP TR38.744 V0.0.4