3GPP TSG-RAN WG2 Meeting #129                                                                                   R2-2500221

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM [1]. The contribution is organized as the following. In section 2, we discuss the following aspects,

- Remaining issues on inter-CU LTM

- Further consideration on the co-existence in DC

Our proposals are summarized in Section 3.

Discussion

Remaining issues on inter-CU LTM

PDCP re-establish for SRB

During the last meeting discussion, it was agreed that for inter-CU SCG LTM,

For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

That means the PDCP reestablishment of SRB3 is based on the newly introduced R19 set ID (i.e., security key update). For SRB1/SRB2, it was confirmed to not perform PDCP reestablishment, and the indication is based on the NW configuration where the candidate configuration does not include the PDCP re-establishment flag.

However, according to the current RRC running CR, it captures that if the R19 set ID is different between the candidate cell and serving cell, UE will perform PDCP re-establishment for all SRBs,

1> if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 1> if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigSCG indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID: … 2> for each srb-Identity value that is part of the current UE configuration: 3> if the UE has selected a new sk-Counter value due to this LTM cell switch procedure: … 4> re-establish the PDCP entity of this SRB as specified in TS 38.323 [5]; 3> else: 4> trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5]; Editor’s Note: How to indicate and perform security key change is pending SA3 outcome. Editor’s Note: FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18).

So it should be aligned with the current agreement.

Proposal 1a: Upon inter-CU SCG LTM execution, the PDCP re-establishment/SDU discard is not performed for SRB1/2 by UE as per NW configuration (i.e., by not including the re-establishment/SDU discard flag in the candidate configuration).

Then the next issue is how to indicate the L2 behavior for SRBs upon inter-CU MCG LTM.

For SRB1/2, upon the preparation phase, the candidate cell does not know whether UE will enter this cell from an intra-CU cell or inter-CU cell, so it cannot determine whether to include the PDCP re-establish flag in the candidate configuration. So the PDCP re-establishment for SRB1/2 should be based on the R19 set ID upon inter-CU MCG LTM.

For SRB3, upon the execution of inter-CU MCG LTM, the SN key will change based on MN key update. But at the preparation phase, the candidate cell does not know whether UE will enter this cell from intra-CU cell or inter-CU cell, so it cannot determine whether MN key will be updated. Therefore, the PDCP re-establishment of SRB3 should be based on the R19 set ID for MN security key update.

Proposal 1b: Upon inter-CU MCG LTM execution, UE determines whether to perform PDCP re-establishment for SRB1/2/3 based on the R19 set ID.

The provision of reference configuration for inter-CU SCG LTM

In the current RRC running CR, a new field is introduced to provide the configuration for inter-CU SCG LTM by MCG RRCReconfiguration, i.e., ltm-ConfigSCG defined by reusing the Rel-18 IE of LTM-Config-r18.

RRCReconfiguration-v19xy-IEs ::= SEQUENCE { ltm-ConfigSCG-r19 SetupRelease {LTM-ConfigSCG-r19} OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } ... LTM-ConfigSCG-r19 ::= SEQUENCE { ltm-ConfigurationSCG-r19 LTM-Config-r18 OPTIONAL -- Need R }

Regarding how to provide the reference configuration for inter-CU SCG LTM, it still remains the following FFS,

LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M Editor’s Note: FFS is the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it containes both MCG and SCG configurations. … ..., [[ ltm-ServingCellNoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-SK-Counters-r19 SetupRelease {SK-CounterList-r19} OPTIONAL, -- Need M cltm-ServingCellExecutionConditions-r19 SetupRelease {CLTM-ExecutionConditions-r19} OPTIONAL -- Need M ]] }

As per the previous agreement,

RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

That means if the mixture of intra-CU and inter-CU LTM candidates is configured, all LTM candidate configurations should be provided within ltm-ConfigSCG. Regarding the reference configuration,

Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs.

As highlighted above, the MN is responsible for the generation of the reference configuration for intra-CU/inter-CU candidates within the ltm-ConfigSCG. Thus the MCG RRCReconfiguration should be able to provide the reference configuration (for the candidate configuration that is not complete configuration) by the ltm-ConfigSCG. Then based on above principle, it’s up to NW implementation to ensure the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration.

Another issue is whether the ltm-ReferenceConfiguration-r18 configured in ltm-Config for MCG (if it includes both MCG and SCG reference configuration) could be used for ltm-ConfigSCG.

Firstly, if both the intra-CU MCG LTM and inter-CU SCG LTM are configured to UE, for intra-CU MCG LTM, it is expected that the SCG part of the reference configuration is as much common as possible. However, for inter-CU SCG LTM, it may be quite opposite as the SCG part configuration need to be changed much more than MCG configuration. From this perspective, using one reference configuration for both intra-CU MCG LTM and inter-CU SCG LTM does not make much contribution on the signaling reduction.

Secondly, it may have much spec impact on the procedure description as there may be the situation where only inter-CU SCG LTM is configured by ltm-ConfigSCG but the reference configuration is from ltm-Config configured for intra-CU MCG LTM.

In summary, it is proposed that,

Proposal 2: For inter-CU SCG LTM, the reference configuration can only be provided to UE within the ltm-ConfigSCG by reusing the ltm-ReferenceConfiguration-r18.

MCG handling upon inter-CU SCG LTM

The inter-CU LTM execution is similar as inter-SN SCPAC, the UE behavior upon inter-CU SCG LTM execution could take SCPAC execution as baseline.

Upon the execution of SCPAC stored in MCG VarConditionalReconfig (i.e., MN format), it is captured in TS 38.331 as following,

Upon the conditional reconfiguration execution for subsequent CPAC, the UE shall: 1> if the selected subsequent CPAC candidate configuration is stored in MCG VarConditionalReconfig: 2> for each SRB/DRB in current UE configuration: - keep the associated RLC, PDCP and SDAP entities, their state variables, buffers and timers; - release all fields related to the SRB/DRB configuration except for srb-Identity, drb-Identity, and securityConfig; 2> release/clear all current dedicated radio configuration except for the following: - the MCG C-RNTI; - the AS security configurations associated with the master key and the secondary key; - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers; - the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers; - the UE variables VarConditionalReconfig and VarServingSecurityCellSetID; - the logged measurement configuration. 2> release/clear all current common radio configuration, except for the ServingCellConfigCommon of the PCell; 2> apply the default MAC Cell Group configuration for MCG MAC and SCG MAC as specified in 9.2.2; 2> use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG; 2> apply the default L1 parameter values as specified in corresponding physical layer specifications for the MCG and SCG;

As above, the UE applies the default MAC cell group configuration for both MCG and SCG, but not performs MAC reset for MCG. The intention is to avoid the potential impact on MCG transmission upon the execution of SCPAC.

Besides, the UE will clear all current common configurations except for the ServingCellConfigCommon of the PCell. This is because the ServingCellConfigCommon of the PCell is provided within the field ReconfigurationWithSync. If the ServingCellConfigCommon of the PCell is cleared by UE upon the execution of SCPAC, it needs to be provided by the reference or candidate configuration. Therefore, upon the execution of SCPAC, it will cause UE to execute the unexpected behaviors related to ReconfigurationWithSync.

Since the candidate configuration for inter-CU SCG LTM also includes the MCG configuration, the UE behavior on handling of MCG configuration can take the SCPAC as baseline upon the execution of inter-CU SCG LTM.

Therefore, it is proposed that,

Proposal 3: For the MCG configuration handling upon inter-CU SCG LTM execution, the similar behavior upon the SCPAC execution should be taken as baseline, e.g., MAC reset and release of ServingCellConfigCommon of the PCell are not performed at MCG side.

On the configuration of SK counter list for inter-CU SCG LTM

As per the current running CR, to provide the SK counter configuration for inter-CU SCG LTM, it is configured in the LTM-Config-r18 with the SK-CounterList-r19.

As agreed during RAN2 discussion, for inter-CU SCG LTM, the security update follow the SCPAC as baseline, and the corresponding agreement had been sent to SA3. In SCPAC, per candidate SN is provided with a SK counter list so that UE selects the first one upon performing inter-SN SCPAC to this SN. Therefore, for inter-CU SCG LTM, we should also provide the SK counter list per candidate SN.

Besides, if a single SK-counter list is used for all candidate SNs, the target SN has to wait for the SK-counter forwarded by source MN upon the execution of inter-CU SCG LTM. It will cause the delay on the decoding of RRCReconfigComplete. If we follow the similar way in SCPAC, each candidate SN can acquire the corresponding SK-counter list upon the preparation phase, so no further delay is observed.

Based on above, it is suggested that,

Proposal 4: For inter-CU SCG LTM, the SK counter list is configured to UE per candidate SN, similar as SCAPC.

Whether to have a new field or re-use the existing selectedSK-Counter

Upon the generation of RRCReconfigurationComplete due to the execution of inter-CU SCG LTM, UE will include the selected SK counter in the RRCReconfigurationComplete. Then it remains the following FFS in the current running CR,

Editor’s Note: Whether to have a new field or re-use the existing selectedSK-Counter is FFS.

In Rel-18 inter-SN SCPAC, the selectedSK-Counter-r18 was introduced in the RRCReconfigurationComplete so that the MN can forward this SK counter to the target SN to align the SN security key.

When considering inter-CU SCG LTM, we think the selectedSK-Counter-r18 can be reused. Even the inter-SN SCPAC and inter-CU SCG LTM is configured to UE simultaneously, the source MN can know the application of the RRCReconfiguration is due to the execution of inter-SN SCPAC or inter-CU SCG LTM by the field selectedCondRRCReconfig-r17 or appliedLTM-CandidateId-r18 included in the RRCReconfigurationComplete. So we see no further issue if we reuse the selectedSK-Counter-r18 for inter-CU SCG LTM.

Proposal 5: Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

On the association of CSI-RS with CG resource

In MAC running CR, there is a FFS on Whether the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE.

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall: 1> if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the TCI state ID field in LTM Cell Switch Command MAC CE, as specified in clause 5.18.35: Editor’s NOTE: Whether the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE is FFS. 2> select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

RAN1 has concluded that the LTM TCI state could be associated to CSI-RS as follows,

Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA

Therefore, it is quite reasonable to confirm that CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE.

Proposal 6a: CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE.

In Rel-18 LTM, for the CG-based RACH-less cell switch, the UE considers the CG resource associated with the SSB indicated by the TCI state in the LTM cell switch command as the valid resource. When considering Rel-19 inter-CU LTM, it is reasonable to follow the mechanism in R18 LTM, i.e., the association between the CSI-RS and the CG resource for RACH-less cell switch is configured to UE. UE determines the valid CG resource associated with the corresponding CSI-RS associated with the TCI state indicated in the LTM cell switch command.

Proposal 6b: Support the association between the CSI-RS and the CG resource for RACH-less cell switch.

Further consideration on the co-existence in DC

As per the current running CR, a new field is introduced in the inter-node RRC to indicate the SN to configure inter-CU SCG LTM or not. The intention is due to the fact that if the inter-CU SCG LTM candidate is configured, the intra-CU SCG LTM candidate should be configured with MN format too.

[[ configureInterCU-LTM-r19 ENUMERATED{allowed} OPTIONAL ]] Editor’s Note:: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling. }

However, in our understanding, the indication is not needed. In SCPAC, it leaves to OAM to configure whether MN format SCPAC or SN format SCPAC is used.

For inter-CU SCG LTM, only the SN could trigger the SN LTM. So the SN needs to know whether inter-CU SCG LTM is allowed by MN to be configured to UE. There may be two cases that MN does not want SN to configure inter-CU SCG LTM, e.g., the MN wants to configure UE with inter-CU MCG LTM or the MN does not support configuring inter-CU SCG LTM with MN format RRCReconfiguration (e.g., MN is R18 gNB). Similar as SCPAC, the OAM could indicate the SN whether inter-CU SCG LTM is allowed.

Proposal 7: It is up to OAM configuration to indicate the SN whether inter-CU SCG LTM is allowed, i.e., no inter-node or Xn message is needed.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Remaining issues on inter-CU LTM

Proposal 1a: Upon inter-CU SCG LTM execution, the PDCP re-establishment/SDU discard is not performed for SRB1/2 by UE as per NW configuration (i.e., by not including the re-establishment/SDU discard flag in the candidate configuration).

Proposal 1b: Upon inter-CU MCG LTM execution, UE determines whether to perform PDCP re-establishment for SRB1/2/3 based on the R19 set ID.

Proposal 2: For inter-CU SCG LTM, the reference configuration can only be provided to UE within the ltm-ConfigSCG by reusing the ltm-ReferenceConfiguration-r18.

Proposal 3: For the MCG configuration handling upon inter-CU SCG LTM execution, the similar behavior upon the SCPAC execution should be taken as baseline, e.g., MAC reset and release of ServingCellConfigCommon of the PCell are not performed at MCG side.

Proposal 4: For inter-CU SCG LTM, the SK counter list is configured to UE per candidate SN, similar as SCAPC.

Proposal 5: Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

Proposal 6a: CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE.

Proposal 6b: Support the association between the CSI-RS and the CG resource for RACH-less cell switch.

Further consideration on the co-existence in DC

Proposal 7: It is up to OAM configuration to indicate the SN whether inter-CU SCG LTM is allowed, i.e., no inter-node or Xn message is needed.

Reference

[1] Draft_RAN2_128_Meeting_Report_v1