3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500225

Athens, Greece, 17th – 21st Feb, 2025

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In the RAN2#127bis meeting [1], on-demand SSB was discussed, with the following agreements:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we share our view on L3 measurement and MAC CE design for OD-SSB in section 2. Our proposals are summarized in Section 3.

Discussion

OD-SSB measurement of SCell

- L3 RRM Framework

In the RAN2#127bis meeting, the details on L3 RRM framework were discussed and the following two options were summarized.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Here we compare the two options, considering the following aspect.

- Impact on RAN2

For option1, UE measures OD-SSB within SMTC after receiving OD-SSB transmission indication. Currently one cell can only be configured with one SMTC. If the SCell is configured with always-on SSB and OD-SSB, the legacy SMTC is not applicable to OD-SSB. Especially when multiple OD-SSB periodicities are configured, introducing one additional SMTC for OD-SSB may not be sufficient. And if more than one additional SMTC for OD-SSB are introduced for RRM measurement on OD-SSB, the relationship between additional SMTC for OD-SSB and the received new MAC CE for OD-SSB transmission needs to be introduced.

For option2, UE measures the OD-SSB within the transmission duration based on the configuration and transmission indication of the OD-SSB for each SCell, which conflicts with existing spec that UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion. Option2 does not affect L3 RRM framework, except that the OD-SSB measurement method is changed to be independent of SMTC, the existing measurement configuration and reporting configuration can be reused. Compared to option 1, option 2 does not change the existing MO configuration.

- Impact on RAN4

During RAN4#113 meeting, the issue of OD-SSB based deactivated SCell measurement was discussed, and RAN4 have the following agreements:

Agreement: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

It is clear that RAN4 defines a new measurement requirement for OD-SSB based deactivated SCell measurement. Therefore, whether supporting option 1 or option 2, it has a spec impact on RAN4, i.e., updating the measurement periodicity of SCell upon receiving MAC CE indication. And if option1 is supported, an additional SMTC needs to be indicated to the UE, which is more complex compared to option 2.

- Power consumption

For option1, if multiple SMTCs are introduced and the activated SMTC of OD-SSB is adjusted with the variation of OD-SSB period, the duration of SMTC and OD-SSB period appears to be the same. Hence, the UE power consumption of option1 and option2 appear to be the same.

In Table 1, we summarize the two options, considering the above aspect.

Table 1 Comparison of two options

|  | Option1 | Option2 |
| --- | --- | --- |
| Impact on RAN2 | 1) Additional configurations of multiple SMTCs are introduced 2)The relationship between additional SMTC for OD-SSB and the received new MAC CE for OD-SSB transmission needs to be introduced | 1)Modified the restriction for SSB measurement within SMTC |
| Impact on RAN4 | Yes | Yes |
| UE Power consumption | Same | Same |

According the above analyses, we slightly prefer option 2.

Proposal 1: The RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

In general, both option1 and option2 will have an impact on RAN4. RAN2 needs to send relevant LS to RAN4 after determining the specific options.

Proposal 2: Send LS to inform RAN4 that the RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC.

- Measurement of Case #2

Here we provide further analysis and proposals of the following options for RRM measurements of SCell configured with on demand SSB.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

The RRM measurement of always-on SSB and/or OD-SSB is affected by the relation of OD-SSB and always-on SSB. RAN4 identified that the relation (including periodicity, time location, frequency and spatial) between always-on SSB and on-demand SSB has impacts on UE’s behavior and requirements in SCell. RAN4 has already sent an LS [2] to RAN1 for clarification, and the reply of RAN1 is as follows [3].

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Considering RAN4 will continue to discuss UE’s behavior and requirements in SCell, we think the issue of measurement of Case #2 can be decided by RAN4.

Proposal 3: The issue on how to measure SSB in Case #2 can be decided by RAN4. RAN2 sends LS to RAN4 to inform this.

- L3 measurement reporting

- Scenario #2

According to RAN4 [4], if the SCell is an unknown SCell, after UE receives SCell activation command, UE has to perform AGC, synchronization and L1 measurement/report based on SSB, and then gNB activate TCI state for data transmission according to UE’s report. If the SCell is a known SCell, gNB can directly activate the TCI state for data transmission. Therefore, larger activation delay is required for the unknown SCell.

Observation 1: The latency of activating a SCell is different between a known cell and an unknown cell.

For Case #1, at RAN2#127 meeting, it was agreed that the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. Therefore, the measurement reporting is triggered only after receiving the OD-SSB transmission indication.

Observation 2: In Scenario #2 with Case #1, the SCell may be an unknown cell because the measurement reporting is triggered only after the indication of the OD-SSB transmission.

According to RAN4 [4], for known cells, there is a limit to the time between the measurement reporting and UE receiving the SCell activation command.

8.3.2	SCell Activation Delay Requirement for Deactivated Scell

SCell in FR1 is known if it has been meeting the following conditions:

- During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified

in clause 9.2 and 9.3.

- the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time

when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

…

For Case #2, UE can measure the always-on SSB, and UE can measure the OD-SSB after OD-SSB is activated. If the measurement reporting of always-on SSB is triggered too early, and the time between the measurement reporting and UE receiving the SCell activation command exceeds the limit, it is beneficial to quickly measure and report the latest measurement results after receiving the OD-SSB transmission indication, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell. However, according to the existing specification, if the event triggered reporting is configured, UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication, e.g. the measurement reporting for the cell is triggered long along and no new applicable cell is fulfilled the event triggered condition.

Observation 3: For Scenario #2 with Case #2, the SCell may be an unknown cell if UE does not trigger the measurement reporting after receiving the OD-SSB transmission indication.

In order for UE to quickly measure and report measurement results after receiving the OD-SSB transmission indication, turn an unknown cell into a known cell, a new trigger conditions of measurement reporting should be introduced, i.e., UE triggers measurement reporting after the OD-SSB is activated via MAC CE.

Proposal 4: For Scenario #2, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

- Scenario #2A

Enhancement for multiple unknown SCells’ activation was discussed in R18. The RRC CR [5] for introduction of FR2 SCell enhancements is shown below.

| Reason for change: | This CR is to introduce the FR2 unknown SCell activation enhancement as requested by RAN4 in their LS in R2-2304636 and R2-2307039. In the enhancement of network triggering report (of L3 measurement result) during FR2 unknown SCell activation, when SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unkown SCell, the RRC measurement report will be intiated which includes the SCell with beam level measurement result for that SCell. RAN2 agreed to reuse the existing RRM measurement framework with a new report type to support this enhancement. In RAN2#123bis meeting, RAN2 made the following agreements on multiple SCell activation case: - If the network activates multiple Sells within same MAC CE the UE may send only one measurement report. |
| --- | --- |

Based on the CR, after receiving the SCell activation command, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated. This can assist SCell activation.

Observation 4: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

For Scenario #2A with Case #1, RAN2 can discuss whether the similar functions can be reused, i.e. after receiving the SCell activation command and OD-SSB transmission indication, the L3 measurement report will be initiated if UE has valid L3 measurement results available. For Scenario #2A with Case #2, measurement results based on alway-on SSB before Scell activation may not be the latest measurement results, so RAN2 can also discuss whether a similar function can be introduced.

Proposal 5: For Scenario #2A, RAN2 discusses whether to trigger the L3 measurement reporting after receiving the SCell activation command and OD-SSB transmission indication.

OD-SSB measurement of neighbor cell

According to the agreement of “the UE expects to measure SSB when on-demand SSB transmission is activated”, there is a question about whether UE can measure the OD-SSB of a neighbor cell when the OD-SSB transmission of the neighbor cell is activated. According to the WID [6], OD-SSB transmission is only applicable to SCell. Indicating the OD-SSB transmission of the neighbor cell to the UE is not supported.

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In addition, during last RAN4 # 112bis meeting [7], following agreements have been achieved on neighbour cell measurement.

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. This agreement can be revisited if it is not aligned with further agreement in other WGs.

Based on the agreement, RAN4 will not update the neighbour cell L3 measurement requirement due to the OD-SSB transmission. This means that UE will continue to measure SSB using the always-on SSB measurement configuration, even if OD-SSB transmission is activated. Therefore, OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Proposal 6: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

In the RAN2#127bis meeting, it was agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. In the existing specification [8], the SCell Activation/Deactivation MAC CE support 1 octet (7 SCells) or 4 octets (31 SCells). Therefore, the new MAC CE should also support sending OD-SSB indication for 7 or 31 SCells simultaneously.

Proposal 7: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

In the last RAN1#119 meeting, following agreements have been achieved on OD-SSB deactivation:

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Based on the above agreement, explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication has been approved by RAN1. In the previous RAN2 meeting, a new MAC-CE for OD-SSB transmission indication is introduced. For simplicity, the new MAC CE for OD-SSB transmission indication should indicate OD-SSB activation and deactivation.

Observation 5: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission has been approved by RAN1.

Proposal 8: The new MAC CE for OD-SSB transmission indication can be used to indicate OD-SSB activation or deactivation.

During RAN1#118 meeting, it was agreed that multiple candidate values of periodicity of the on-demand SSB can be configured by RRC and the applicable value can be indicated by MAC CE as follows.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB. - FFS: Any other relevant parameters.

RAN1 had agreed in the last RAN1#119 meeting that the periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. Hence, the following two options can be used to indicate the periodicity of on-demand SSB:

- Option1: 3bit in the new MAC CE is used to indicate the activated periodicity of OD-SSB (5ms, 10ms, 20ms, 40ms, 80ms and 160ms) explicitly.

- Option2: OD-SSB configuration by RRC includes the periodicity of the on-demand SSB and other parameters. Several OD-SSB configurations for a SCell may be configured via RRC in advance. When an OD-SSB is activated, the corresponding OD-SSB configuration index is included in the new MAC CE.

In RAN1#118bis meeting, RAN1 has already agreed to provide several parameters for OD-SSB configuration by RRC as follows.

Agreement - For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

And in the last RAN1 meeting, time domain location of on-demand SSB burst was discussed, and RAN1 have the following agreements:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1.

Based on the above agreement, the time domain location of on-demand SSB is configured per on-demand SSB periodicity by RRC, and there are multiple candidate values for the time domain location of on-demand SSB. If option1 is supported, MAC CE also needs to indicate the time domain location of OD-SSB. So from the perspective of signaling overhead, it is more effective to include the periodicity of on-demand SSB as a parameter of OD-SSB SSB configuration by RRC. Option2 seems more appropriate.

Proposal 9: The different periodicities can be as associated to different OD-SSB configuration index in RRC, and the activated OD-SSB configuration index is indicated in the new OD-SSB indication MAC CE.

From the above, the OD-SSB transmission MAC CE should indicate OD-SSB activation/deactivation and the activated OD-SSB configuration index. A bit map can be used to indicate whether the OD-SSB of an OD-SSB SCell is activated or deactivated. For the indication of OD-SSB configuration index, it can be indicated based on the ascending order of the ServCellIndex for which on-demand SSB transmission is activate. Hence, OD-SSB transmission MAC CE needs to have variable size.

Proposal 10: The size of the OD-SSB transmission MAC CE is variable.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: The latency of activating a SCell is different between a known cell and an unknown cell.

Observation 2: In Scenario #2 with Case #1, the SCell may be an unknown cell because the measurement reporting is triggered only after the indication of the OD-SSB transmission.

Observation 3: For Scenario #2 with Case #2, the SCell may be an unknown cell if UE does not trigger the measurement reporting after receiving the OD-SSB transmission indication.

Observation 4: During SCell activation procedure, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated.

Observation 5: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission has been approved by RAN1.

OD-SSB measurement of SCell

Proposal 1: The RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC when OD-SSB is transmitted.

Proposal 2: Send LS to inform RAN4 that the RRM measurement of OD-SSB is performed based on OD-SSB pattern while ignoring SMTC.

Proposal 3: The issue on how to measure SSB in Case #2 can be decided by RAN4. RAN2 sends LS to RAN4 to inform this.

Proposal 4: For Scenario #2, a L3 measurement reporting is triggered after the OD-SSB is activated via MAC CE.

Proposal 5: For Scenario #2A, RAN2 discusses whether to trigger the L3 measurement reporting after receiving the SCell activation command and OD-SSB transmission indication.

OD-SSB measurement of neighbor cell

Proposal 6: OD-SSB SCell operation has no impact to RRM measurement of neighbor cells.

Contents of OD-SSB MAC CE

Proposal 7: The new OD-SSB MAC CE can indicate up to 7 or up to 31 SCells.

Proposal 8: The new MAC CE for OD-SSB transmission indication can be used to indicate OD-SSB activation or deactivation.

Proposal 9: The different periodicities can be as associated to different OD-SSB configuration index in RRC, and the activated OD-SSB configuration index is indicated in the new OD-SSB indication MAC CE.

Proposal 10: The size of the OD-SSB transmission MAC CE is variable.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 127bis

- R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4#112bis

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1#119

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12)

- R2-2313936, Introduction of FR2 SCell enhancements

- RP-242354 Revised WID: Enhancements of network energy savings for NR

- RAN4 #112bis RRM session meeting report

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification (Release 18)”, V18.2.0 (2024-06)