3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500227

Athens, Greece, 17th – 21st Feb, 2025

Source:	CATT

Title:	Adaptation of Common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, the following agreements on paging adaptation were achieved:

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In this contribution, remaining issues on paging adaptation are discussed. In addition, considerations on PRACH adaptation and SSB adaptation are provided.

Discussion

Adaptation of Paging

- Extension of PMO configuration

The following is the legacy PMO (i.e. firstPDCCH-MonitoringOccasionOfPO) configuration [1].

[[ firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL -- Cond OtherBWP ]], //skip firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE { sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679) } OPTIONAL, -- Need R

In legacy paging configurations, the maximum value of Ns is 4 and the maximum number of bits for PMO is 17, where the maximum offset of the first PDCCH monitoring occasion for paging of each PO of the PF is 16 radio frames. The maximum signaling overhead for PMO configuration is maximum value of Ns*bits for each PMO (4*17=68 bits).

In RAN2#128 meeting, it was agreed to introduce Ns=8 and N=T/32. When the maximum number of Ns is extended to 8, the maximum PO per PF (i.e. maxPO-perPF) needs to be extended correspondingly. Although POs are expected to be clustering together, the possible values of PMO need to be extended correspondingly as well as they may be clustered in any time domain within 32 radio frames. The maximum number of bits for PMO needs to be extended to 18. Then the maximum signaling overhead for extended PMO configuration is 144 bits (8*18) which is acceptable to the limited massage size of SIB1.

Proposal 1: Introduced a new parameter of firstPDCCH-MonitoringOccasionOfPO is to support Ns=8 and N=T/32 for R19 NES UEs.

- Mechanism to activate/deactivate the paging adaptation resources

Assume there are legacy paging mechanism and Rel-19 paging mechanism in a cell. There are two possible solutions to activate/deactivate the paging adaptation resources:

- Solution 1:  Via system information update.

With the solution, paging adaptation parameters are provided in system information if NW wants to activate paging adaptation. The Rel-19 NES UE applies Rel-19 paging mechanism immediately upon receiving corresponding parameters in system information. Paging adaptation parameters are removed from system information if NW wants to deactivate paging adaptation. The UE is notified of this via legacy system information modification mechanism. And when paging adaptation parameters are absent in system information, the Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters.

- Solution 2: Via Short Message in paging DCI

With the solution, parameters for Rel-19 paging mechanism are broadcasted in SI but not activated immediately. The Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters if Rel-19 paging mechanism is not activated. If the network wants to activate Rel-19 paging mechanism, Short Messages with paging adaptation indication is sent in one modification period. Then the UE can apply Rel-19 paging mechanism in the next modification period without acquiring system information for parameters of Rel-19 paging mechanism.

Both solution 1 and solution 2 are adapted Rel-19 paging mechanism in a semi-static way. But solution 2 needs to use the reserved bits in Short Message although it may reduce the UE power saving without acquiring system information for parameters of Rel-19 paging mechanism. There are only 4 bits not used in Short message. Considering the limited reserved bits in Short message and the limit benefit of solution 2, solution 1 is preferred.

Proposal 2: Reuse the legacy System information update procedure to activate/deactivate paging adaptation mechanism.

Proposal 3: R19 NES UEs follows legacy paging mechanism if paging adaptation configuration is absent in the R19 NES cell.

- PEI with paging adaptation

In order to reduce UE power consumption due to paging reception, PEI is introduced in R17. In PEI, the number of PO(s) associated with one PEI monitoring occasion, i.e. po-NumPerPEI, is related Ns, as shown below.

PEI-Config-r17 ::= SEQUENCE { po-NumPerPEI-r17 ENUMERATED {po1, po2, po4, po8}, payloadSizeDCI-2-7-r17 INTEGER (1..maxDCI-2-7-Size-r17), pei-FrameOffset-r17 INTEGER (0..16), subgroupConfig-r17 SubgroupConfig-r17, lastUsedCellOnly-r17 ENUMERATED {true} OPTIONAL, -- Need R ... } //skip po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

Observation 1: pei-Config includes R17 PEI related configurations. The maximum value of po-NumPerPEI is related to the value of Ns and the maximum number of PF associated with one PEI.

As shown below, the maximum number of PEI occasion per PF (i.e. the list size of firstPDCCH-MonitoringOccasionOfPEI-O) in R17 is 4, which is equal to the maximum value of legacy Ns.

//skip pei-ConfigBWP-r17 SEQUENCE { pei-SearchSpace-r17 SearchSpaceId, firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239), sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479), sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959), sCS480KHZquarterT-SCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919), sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679) } //skip maxPEI-perPF-r17 INTEGER ::= 4 -- Maximum number of PEI occasion per paging frame

Observation 2: pei-ConfigBWP includes R17 PEI related configurations. The maximum number of PEI occasion per PF (i.e. the list size of firstPDCCH-MonitoringOccasionOfPEI-O) in R17 is 4, which is equal to the maximum value of legacy Ns.

The following is the definition of PEI format [2]:

7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field - NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter po-NumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO. - Each bit in the field indicates one UE subgroup of a paging occasion. - TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured if configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI-2-7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

Observation 3: Within PEI format, paging indication field is decided by high layer parameters po-NumPerPEI and subgroupsNumPerPO within PEI-Config and TRS availability indication is next to paging indication field.

In R17, the maximum value of NPOPEI (i.e. po-NumPerPEI) is 8 and the maximum value of NSGPO (i.e. subgroupsNumPerPO) is 8. Then the maximum value of NPOPEINSGPO is 64. But the maximum size of payloadSizeDCI-2-7 is 43. Hence, in R17 PEI, it is up to network implementation with smart configurations to ensure the number of information bits in format 2_7 to be equal to or less than the payload size of format 2_7.

Observation 4: In R17 PEI, it is up to network implementation with smart configurations to ensure the number of information bits in format 2_7 to be equal to or less than the payload size of format 2_7.

During the discussion in RAN2#128 meeting, there are two possible options for PEI with paging adaptation [3]:

- Option 1: Based on a single PEI-O configuration, and rely on network implementation to have separate PEI:s for the two types of PO:s.

- Option 2: Based on separate PEI-O configurations, and thus automatically separate PEIs for the two types of PO:s.

In both above options, the two types of POs are 1) PO:s monitored by legacy UE (may or may not be shared with R19 UE, up to network configuration), and 2) PO:s monitored only by R19 UE.

R17 PEI related configurations include pei-Config which contains po-NumPerPEI, and pei-ConfigBWP which contains firstPDCCH-MonitoringOccasionOfPEI-O. For R17 PEI and R19 PEI, it is more meaningful to analyze po-NumPerPEI and firstPDCCH-MonitoringOccasionOfPEI-O. Therefore, possible options for PEI with paging adaptation are.

- Alt1: Legacy po-NumPerPEI and legacy firstPDCCH-MonitoringOccasionOfPEI-O are used for both legacy UEs and R19 UEs.

- Alt2: Legacy po-NumPerPEI is used for both legacy UEs and R19 UEs. Legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for legacy UEs only while a new separate parameter of firstPDCCH-MonitoringOccasionOfPEI-O is introduced for R19 UEs.

- Alt3: Legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for both legacy UEs and R19 UEs. Legacy po-NumPerPEI is used for legacy UEs only while a new separate parameter of po-NumPerPEI is introduced for R19 UEs.

- Alt4: Legacy po-NumPerPEI and Legacy firstPDCCH-MonitoringOccasionOfPEI-O are used for legacy UEs only. And a new separate parameter of po-NumPerPEI with a new separate parameter of firstPDCCH-MonitoringOccasionOfPEI-O are introduced for R19 UEs

With observation 3, if legacy po-NumPerPEI is used for both legacy UEs and R19 UEs, PEI format is the same for both legacy UEs and R19 UEs. If the total number of PEIs per reference frame for R19 UEs is not bigger than 4, legacy firstPDCCH-MonitoringOccasionOfPEI-O can be used to configure for both legacy UEs and R19 UEs. And in this case, same PEI can be used for both legacy UEs and R19 UEs. However, Ns may be configured as 8 for R19 UEs. With observation 2, the legacy firstPDCCH-MonitoringOccasionOfPEI-O is not enough for R19 UEs if one PO is associated with one PEI while Ns=8. Hence, Alt1 is not feasible for the case that one PO is associated with one PEI while Ns=8.

With observation 3, if separate parameters of po-NumPerPEI are configured for legacy UEs and R19 UEs, PEI format for legacy UEs is different from that for R19 UEs. Then if legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for both legacy UEs and R19 UEs, PEIs for legacy UEs collides with PEIs for R19 UEs with different PEI format if a single pei-SearchSpace is used. In this case, UEs cannot decode PEI correctly. Alt3 is not feasible.

Alt 4 provides the maximum flexibility that up to network configuration R17 PEIs and R19 PEIs can be different or same. But larger value of POnumPerPEI can reduce the average PEI overhead per PO, there can also cause potentially larger paging latency and larger UE power consumption due to longer UE wake-up time before PO monitoring. It is not sensible to configure a bigger value of POnumPerPEI for R19 UEs. Hence, it is preferred to use legacy po-NumPerPEI for both legacy UEs and R19 UEs. Alt 2 is preferred.

Table 1 Summary of four alternatives

| Alt1 | The legacy firstPDCCH-MonitoringOccasionOfPEI-O is not enough for R19 UEs if one PO associated with one PEI while Ns=8. |
| --- | --- |
| Alt2 | Provide the flexibility that R17 PEIs and R19 PEIs can be different or same with suitable network configuration. |
| Alt3 | It is not feasible as UEs cannot decode PEI correctly if a single pei-SearchSpace is used as PEIs for legacy UEs collides with PEIs for R19 UEs with different PEI format. |
| Alt4 | Compared with alt2, it provides the maximum flexibility that R17 PEIs and R19 PEIs can be different or same with suitable network configuration. But it is not sensible to configure a bigger value of POnumPerPEI for R19 UEs. |

Therefore, Alt2 is preferred.

Proposal 4a: Legacy po-NumPerPEI is used for both legacy UEs and R19 UEs.

Proposal 4b: Legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for legacy UEs only. Introduce a new parameter of firstPDCCH-MonitoringOccasionOfPEI-O for R19 NES UEs.

Adaptation of PRACH in time domain

RAN1 has already agreed that the adaptation mechanism for additional PRACH resources is applied to both CONNECTED mode UEs and IDLE/INACTIVE mode UEs. In [5], the following events are supported for random access procedure:

The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

Which events for triggering RACH procedure are supported for PRACH adaptation may impact RRC specification and stage 2 specification. Hence, we propose:

Proposal 5: RAN2 discuss which events for triggering RACH procedure are supported for PRACH adaptation.

- Whether to support RACH adaptation to RACH resources of R17 RACH partitions

In Rel-17, RACH partitioning was introduced. A list of RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., SDT) or feature combination (e.g., SDT+redcap).

At first, we want to clarify RACH adaptation is not considered as a new feature of RACH partitions. RACH partitioning is used for early identification. However, adaptation for additional PRACH resource doesn’t require early identification.

Proposal 6: RACH adaptation is not considered as a new feature of RACH partitions.

Then we can discuss whether the RACH resource adaptation is supported for R15 RACH resources configuration only or for both R15 RACH resources configuration and R17 RACH partition resources configuration. In our understanding, it would be simpler if RACH resource adaptation is supported for R15 RACH resources configuration only on R19 NES cell. But RACH adaptation can also introduce additional RACH resources for some features or feature combinations in high load or medium load case.

Therefore, it is proposed:

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it with partition resources configuration.

Adaptation of SSB in time domain

In the LS on SSB adaptation from RAN4 [4], RAN4 asks RAN2 to answer the SMTC impact question due to SSB adaptation.

RAN2 has not discussed SSB adaptation yet. And the following are agreements made in RAN1.

RAN1#116bis: Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger RAN1#117: Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB RAN1#118: Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) RAN1#118bis: Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact RAN1#119: Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

According to RAN1’s agreements, different from on-demand SSB SCell, SSB adaptation only focuses on adaptation of SSB burst periodicity. And RAN1 confirms it is applicable to NCD-SSB on SCell for SSB adaptation. Currently NCD-SSB can be used for L3 measurement. Although there is no agreement if SSB adaptation is applied to L3 measurement in RAN1, it is straightforward that SSB adaptation can be used for L3 measurement as well.

Proposal 8a: SSB adaptation can be used in L3 measurement.

STMC impact due to on-demand SSB SCell is discussing in RAN2, including:

- Option1: Based on different measurement configuration when OD-SSB is transmitted.

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

The signaling for SSB adaptation is unclear. But both two options for SMTC impact with on-demand SSB SCell can be applied to SSB adaptation. If SSB adaptation is applied to L3 measurement, the similar mechanism can be considered as a baseline. And if agreed, send reply LS to inform RAN4 the related agreement.

Proposal 8b: If P8a is agreed, the mechanism in on-demand SSB SCell for SMTC impact with L3 measurement can be considered as a baseline for SMTC impact due to SSB adaptation. Send reply LS to inform RAN4 this.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: pei-Config includes R17 PEI related configurations. The maximum value of po-NumPerPEI is related to the value of Ns and the maximum number of PF associated with one PEI.

Observation 2: pei-ConfigBWP includes R17 PEI related configurations. The maximum number of PEI occasion per PF (i.e. the list size of firstPDCCH-MonitoringOccasionOfPEI-O) in R17 is 4, which is equal to the maximum value of legacy Ns.

Observation 3: Within PEI format, paging indication field is decided by high layer parameters po-NumPerPEI and subgroupsNumPerPO within PEI-Config and TRS availability indication is next to paging indication field.

Observation 4: In R17 PEI, it is up to network implementation with smart configurations to ensure the number of information bits in format 2_7 to be equal to or less than the payload size of format 2_7.

Adaptation of paging

Proposal 1: Introduced a new parameter of firstPDCCH-MonitoringOccasionOfPO is to support Ns=8 and N=T/32 for R19 NES UEs.

Proposal 2: Reuse the legacy System information update procedure to activate/deactivate paging adaptation mechanism.

Proposal 3: R19 NES UEs follows legacy paging mechanism if paging adaptation configuration is absent in the R19 NES cell.

Proposal 4a: Legacy po-NumPerPEI is used for both legacy UEs and R19 UEs.

Proposal 4b: Legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for legacy UEs only. Introduce a new parameter of firstPDCCH-MonitoringOccasionOfPEI-O for R19 NES UEs.

Adaptation of PRACH in time domain

Proposal 5: RAN2 discuss which events for triggering RACH procedure are supported for PRACH adaptation.

Proposal 6: RACH adaptation is not considered as a new feature of RACH partitions.

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it with partition resources configuration.

Adaptation of SSB in time domain

Proposal 8a: SSB adaptation is used in L3 measurement.

Proposal 8b: If P8a is agreed, the mechanism in on-demand SSB SCell for SMTC impact with L3 measurement can be considered as a baseline for SMTC impact due to SSB adaptation. Send reply LS to inform RAN4 this.

Reference

- TS 38.331 v18.4.0

- TS 38.212 v18.4.0

- R2-2410930	, Offline discussion summary [AT128][111][NES], OPPO

- R2-2409520, LS on SSB adaptation (R4-2416911; contact: Apple)

- TS 38.300 v18.4.0