3GPP TSG-RAN WG2 Meeting #129                                          R2-2500229

Athens, Greece, Feb. 17th – 21st, 2025

Source:              CATT, Huawei, HiSilicon, China Telecom, China Unicom

Title:              	Discussion on FDD-FDD inter-band CA simultaneous Rx/Tx (LS R4-2420410)

Agenda Item:	8.19

Document for:	Discussion and Decision

Introduction

According to RAN4 LS [1], RAN4 agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL, and RAN2 is requested to double check whether there is an issue for this FDD-FDD CA case and introduce necessary UE capability signaling. In this paper, our analysis of the feasibility of this CA case and the key point of the corresponding UE capability are provided.

Discussion

2.1 Background

The content of RAN4 LS [1] is as follows:

1. Overall Description: RAN4 has discussed the UE RF requirements for 2UL/2DL inter-band FDD-FDD CA_n5-n8. It is agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL. Because n8 UL frequency range of CA_n5-n8 partially overlaps with n5 DL frequency range, there is no concurrent operation between n8 UL transmitting and n5 DL receiving, which is a special FDD-FDD inter-band CA case. Figure below conceptually illustrates the intended operation of this UL CA combination where the operation will be alternating between the two status under the RRC configuration (UL CA_n5-n8 and DL CA_n5-n8). Usually for FDD-FDD interband CA, a band pair nA+nB can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. For the specific band pair n5+n8, the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not. RAN4 kindly asks RAN2: - To make necessary modifications in RAN2 specification and/or to add UE capability, if needed, to enable the described functionality. - If there’s a capability defined, it’s mandatory for the corresponding band combinations when indicating UL CA support. - Comparing to the TDD-TDD/TDD-FDD CA non-simultaneous Rx/Tx feature, it’s encouraged to double Check whether there is an issue for this FDD-FDD CA case, and report back to RAN4 if those are found. NOTE: This is intended only for special case when UL and DL from different FDD bands partially overlap. This is not intended for FDD bands which do not partially overlap. NOTE: The described functionality is targeted also for cases when higher order CA configuration (e.g CA_n5A-n8A-nXXA) supports 2UL in n5 and n8. Currently there is only one band combination, i.e., n5+n8 has such an issue, however, RAN4 appreciate if RAN2 may consider a generic future-proof solution in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced in RAN4 2. Actions: To RAN WG2: RAN4 respectively asks RAN2 to consider the above information and to make necessary modifications into RAN2 specification and to define UE capability if necessary.

According to above LS, RAN4 has agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL. Actually, 2UL/2DL inter-band FDD-FDD CA_n5-n8 was initially discussed in RAN2#122 meeting, the RAN2 LS R2-2306862 [2] was sent out with some RAN2 questions, and the corresponding reply LS R2-2309466 [3] from RAN4 gave a clarification on the intended scenario as follows. The key clarification is that the intended CA_n5-n8 is with 2UL and 2DL, and the non-concurrent n5 DL and n8 UL is implemented by scheduling restriction.

Clarification on the scenario: The RAN4 discussion for the 2UL scenario for CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is based on the assumption that UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL. Based on that assumption, RAN4 has the following answers for the questions in the LS R2-2306862. Answer to Question 1: - Cross carrier scheduling is transparent from RAN4’s perspective. RAN4 didn’t see any impact to RAN4 specifications. Answer to Question 2: - RAN4 expects measurement of the SCell (n5) in this scenario is needed. - In RAN4 understanding, network would avoid the collision with the DL transmission of SCell (n5) when scheduling the UL in n8. No impact to existing RRM specifications is seen from RAN4 perspective.

But in RAN#101 meeting, the “DL CA_n5-n8 with single UL in n5 only” was supported in R18, and this 2UL CA_n5-n8 case with non-simultaneous Rx/Tx between n5 DL and n8 UL was excluded due to its own potential issues. Therefore there was no further discussion in RAN2#123bis meeting. The detailed chair minutes of RAN#101 meeting are as shown in the Annex A.

Observation 1: DL CA_n5-n8 with single UL in n5 only has been supported in R18.

Now for R19, this 2UL CA case is brought up again by the new LS [1] from RAN4. But we think the previous clarification LS from RAN4 should be considered together with the new one, i.e., the intended CA_n5-n8 is with 2UL and 2DL, and the non-concurrent n5 DL and n8 UL is implemented by scheduling restriction.

Observation 2: For R19 inter-band FDD-FDD CA_n5-n8, 2UL and 2DL are configured, and UE is expected to work with non-concurrent n5 DL and n8 UL. This non – concurrency is achieved by the scheduling restriction implemented at network side.

2.2 Analysis of 2UL and 2DL CA_n5-n8 scenarios

The feasibility analysis of the relevant CA scenarios is given as follows, which involves three cases, i.e., (PCell on n8, SCell on n5), (PCell on n5, SCell on n8) and (both n5 and n8 are for SCells).

Case 1: (PCell on n8, SCell on n5)

For 2UL/2DL inter-band FDD-FDD CA_n5-n8, if band n8 is configured as PCell, considering there is no overlap between n8 DL and n5 UL, UE could perform the normal operations on PCell DL without any impact, such as the RRM measurement, DL synchronization, PDCCH monitoring, DL data receiving and so on. The network could configure/schedule n5 SCell DL to avoid the collision with UL resource on n8 PCell for a UE, e.g., by cross carrier scheduling with PDCCH only in n8 PCell, or all by network implementation (e.g., SSB occasions and active time of DRX in n5 SCell are not overlapped with UL dynamic/semi-static resources in n8 PCell, as shown in Figure-1). So, it is feasible to configure 2UL/2DL inter-band FDD-FDD CA_n5-n8 with PCell on n8 and SCell on n5, and scheduling restriction can be achieved by network implementation.

Figure-1 One example for non-concurrent n5 DL and n8 UL based on scheduling restriction

Case 2: (PCell on n5, SCell on n8)

For 2UL/2DL inter-band FDD-FDD CA_n5-n8, if band n5 is configured as PCell, the UE will monitor the PDCCH, and perform the corresponding measurements for RLF monitoring on band n5. In order to guarantee the normal operations on PCell DL for UE, the network needs to configure/schedule the UL resource for data/SRS/UCI/RACH on n8 UL to avoid the collision with n5 DL. E.g., network may configure DRX for PCell, and schedule n5 UL in inactive time of DRX cycle of PCell to avoid the collision between PDCCH monitoring in n5 PCell and UL transmission in n8 SCell.

Compared to the Case 1, it’s more complicated to achieve this scheduling restriction from network perspective, as more potential collisions have to be avoided when PCell is on n5 (e.g., there may be more DL scheduling in n5 PCell, which leads to very limited time for n8 UL scheduling). But there is no blocking issue for this scenario according to current specification. Hence, it is also feasible to configure 2UL/2DL inter-band FDD-FDD CA_n5-n8 with PCell on n5 and SCell on n8 from specification point of view, and it’s up to network implementation to achieve the necessary scheduling restriction.

Case 3: (both n5 and n8 are for SCells)

Although “Currently there is only one band combination, i.e., n5+n8 has such an issue”, but RAN4 LS also mentioned that “The described functionality is targeted also for cases when higher order CA configuration (e.g CA_n5A-n8A-nXXA) supports 2UL in n5 and n8”, so this Case 3 where both n5 and n8 are for SCells can also be discussed.

When n5 and n8 are both configured as SCells, compared to Case 1 and Case 2, less complexity for scheduling restriction can be foreseen, as there may be less UL transmission in n8 UL and more network control has been supported  (e.g., RACH in SCell is only triggered by PDCCH order). It is more flexible for network to configure/schedule the non-concurrent DL resource on n5 and UL resource on n8. Obviously, it is also feasible to configure n5 and n8 both to be SCells.

In summary, based on above analysis of 2UL/2DL inter-band FDD-FDD CA_n5-n8, it can be seen that it is feasible to support these scenarios from specification point of view, and the required scheduling restriction at network side is also doable (e.g., by cross carrier scheduling, or SSB occasions and active time of DRX in n5 DL are not overlapped with UL dynamic/semi-static resources in n8 UL). Both n8 and n5 could be configured as PCell, and there is no RAN2 spec impact.

Observation 3: The scheduling restriction of non-concurrent n5 DL and n8 UL can be achieved by network implementation without RAN2 spec impact, e.g., by cross carrier scheduling, or SSB occasions and active time of DRX in n5 DL are not overlapped with UL dynamic/semi-static resources in n8 UL.

Proposal 1: RAN2 to confirm that the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction can be all up to network implementation).

2.3 UE Capability

In order to support 2UL/2DL CA_n5-n8 with non-concurrent n5 DL and n8 UL, a new UE capability is needed. Considering that the described functionality is “targeted also for cases when higher order CA configuration (e.g CA_n5A-n8A-nXXA) supports 2UL in n5 and n8”, and a generic future-proof solution is expected “in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced in RAN4”, this new UE capability should be per BC, not per UE.

The UE should also indicate whether the simultaneous transmission and reception of a band pair (X, Y) in a FDD-FDD inter-band NR CA band combination is supported, and the new UE capability should involve two separate cases, i.e., simultaneous transmission of band X and reception of band Y, simultaneous transmission of band Y and reception of band X. The corresponding TPs are provided in Annex B too.

Proposal 2: For a FDD-FDD inter-band NR CA band combination, define per BC UE capabilities to indicate:

- For each band pair (X, Y) in this band combination, whether simultaneous transmission of band X and reception of band Y is supported;

- For each band pair (X, Y) in this band combination, whether simultaneous transmission of band Y and reception of band X is supported.

If the proposals above can be agreed in RAN2, we further propose to send out a reply LS to RAN4 based on RAN2 agreements, and a draft LS is provided in our companion tdoc [4].

Proposal 3: RAN2 to send out a reply LS with RAN2 agreements.

Conclusion

According to above discussion on 2UL/2DL inter-band FDD-FDD CA_n5-n8 scenario, we have the following observations:

Observation 1: DL CA_n5-n8 with single UL in n5 only has been supported in R18.

Observation 2: For R19 inter-band FDD-FDD CA_n5-n8, 2UL and 2DL are configured, and UE is expected to work with non-concurrent n5 DL and n8 UL. This non – concurrency is achieved by the scheduling restriction implemented at network side.

Observation 3: The scheduling restriction of non-concurrent n5 DL and n8 UL can be achieved by network implementation without RAN2 spec impact, e.g., by cross carrier scheduling, or SSB occasions and active time of DRX in n5 DL are not overlapped with UL dynamic/semi-static resources in n8 UL.

And we propose:

Proposal 1: RAN2 to confirm that the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction can be all up to network implementation).

Proposal 2: For a FDD-FDD inter-band NR CA band combination, define per BC UE capabilities to indicate:

- For each band pair (X, Y) in this band combination, whether simultaneous transmission of band X and reception of band Y is supported;

- For each band pair (X, Y) in this band combination, whether simultaneous transmission of band Y and reception of band X is supported.

Proposal 3: RAN2 to send out a reply LS with RAN2 agreements.

Reference

- R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx.

- R2-2306862, Reply LS on non-simultaneous UL and DL from different two bands during UL CA.

- R2-2309466, Reply LS on non-simultaneous UL and DL from different two bands during UL CA.

- R2-2500230, DRAFT Reply LS on UE capability for FDD-FDD inter-band CA simultaneous RxTx

Annex A

RAN#101 meeting: RP-231970 Rescoping CA_n5-n8 in UL CA_n5-n8 support Type: discussion For: Decision Source: Apple Summarized below are the three operation configurations considered in the current WID for CA_n5-n8 for mitigating the n8 UL to n5 DL self-interference issue: - Option 1: DL CA_n5-n8 with single UL in n5 only - Option 2: UL/DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL - Option 3: UL/DL CA_n5-n8 with additional dedicated filter for the restricted frequency ranges for specific operator. Discussion: Proposal 1: For CA_n5-n8, Option 1 (DL CA_n5-n8 with single UL in n5 only) configuration is prioritized. (Note: This is similar to the situation for CA_n28-n105 where only single UL is supported to avoid the potential IMD self-interference issue.) Proposal 2: Option 2 (UL/DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL) configuration can be retained if no RAN1 and RAN2 impact is identified, or postpone both Option 2 and Option 3 (UL/DL CA_n5-n8 with additional dedicated filter for the restricted frequency ranges for specific operator) to later releases as an enhancement feature. AT&T: option 3 approach is not aligned with the study, so it it not feasible Qualcomm: we support option 1 and would like to drop option 2 and 3 Mediatek: agrees that option 3 can be dropped, option 2 could be discussed Oppo: supports Apples analyse CATT: would not like to drop option 2, RAN2 and RAN4 are still working on it RAN chair: we could save some work in RAN2 and RAN4; who wants to have option 2? seems only CATT Huawei: fine to drop option 2, option 3 was from an operator but if they fine to drop it, then we are also fine to drop option 3 conclusion: options 2 & 3 are dropped and only option 1 will be considered; will be covered in RP-232680

Annex B

38.331 TP (6.3.3	UE capability information elements)

<Unnecessary part omitted>

UE-NR-Capability-v1830 ::=               SEQUENCE {

sib19-Support-r18                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v19xy                                       OPTIONAL

}

UE-NR-Capability-v19xy ::=               SEQUENCE {

rf-Parameters-v19xy                      RF-Parameters-v19xy                                          OPTIONAL

}

RF-Parameters-v19xy ::=                            SEQUENCE {

supportedBandCombinationList-v19xy                  BandCombinationList-v19xy                     OPTIONAL

}

BandCombinationList-v19xy ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v19xy

BandCombination-v19xy ::=          SEQUENCE {

ca-ParametersNR-v19xy               CA-ParametersNR-v19xy                                                  OPTIONAL

}

CA-ParametersNR-v19xy ::= SEQUENCE {

simultaneousRxTxInterBandCA-BandX-DL-BandY-UL-r19            SimultaneousRxTxPerBandPair       OPTIONAL,

simultaneousRxTxInterBandCA-BandX-UL-BandY-DL-r19            SimultaneousRxTxPerBandPair       OPTIONAL

}

<Unnecessary part omitted>

38.306 TP (4.2.7.4	CA-ParametersNR)

<Unnecessary part omitted>

| simultaneousRxTxInterBandCA-BandX-DL-BandY-UL-r19 Indicates whether the UE supports simultaneous transmission and reception for DL of band x and UL of band y for each band pair (x,y) in the FDD-FDD inter-band NR CA band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for the DL of band x and UL of band y in a band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. The UE does not include this field if the UE supports simultaneous transmission and reception for DL of band x and UL of band y for all applicable band pairs in the band combination. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | FDD only | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCA-BandX-UL-BandY-DL-r19 Indicates whether the UE supports simultaneous transmission and reception for UL of band x and DL of band y for each band pair (x,y) in the FDD-FDD inter-band NR CA band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for the UL of band x and DL band y in a band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. The UE does not include this field if the UE supports simultaneous transmission and reception for UL of band x and DL of band y for all applicable band pairs in the band combination. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | FDD only | N/A |

<Unnecessary part omitted>