3GPP TSG-RAN WG2 Meeting #129                                                      R2-2500235

Athens, GR, Feb 17th – 21st, 2025

Source:	CATT

Title:	MRO Enhancements for CHO with Candidate SCGs

Agenda Item:	8.10.2.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#128 meeting, MRO enhancements for CHO with candidate SCGs was discussed and the following agreements were achieved [1].

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

In this contribution, we further discuss the issues about MRO enhancements for CHO with candidate SCGs.

2. Discussion

2.1 RLF report enhancement

For CHO with candidate SCG(s) mobility failure, it is with high possibility that the CHO and/or CPAC execution condition configurations are not properly configured, considering CHO candidate cells and associated execution conditions have been included in RLF report for CHO only mobility failure optimization, thus, for CHO with candidate SCG(s), some CPAC configuration information (e.g., CPAC candidate cells and associated execution conditions) should also be included in RLF report together with CHO configuration information for the CHO with candidate SCG(s) failure optimization.

Proposal 1: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Besides the CPAC configuration information, the execution condition fulfilled situation of each candidate PSCell (e.g., first fulfilled event and the time duration between two events fulfilled) should also be included in RLF report to help the network optimization for CHO with candidate SCG(s).

Proposal 2: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Furthermore, the association between CHO and CPAC is important for network to optimize the CHO with candidate SCG(s) configuration, thus, the association between CHO and CPAC should also be recorded and informed to network.

Proposal 3: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

After RAN2#128 meeting, RAN2 receives an LS from RAN3 [2], in [2], regarding MRO for CHO with candidate SCG(s), including the following content.

(1) Regarding MRO for CHO with candidate SCG(s): RAN3 concluded that the following information can be reported by the UE in RLF Report for failure reason detection and optimization: - The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met. - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

For the first information above, RAN2 has agreed to include the first fulfilled execution condition (e.g., CPAC or CHO) in RLF report. For the last three information, RAN2 discusses and agrees to include them in RLF report as agreed by RAN3, FFS explicit or implicit.

Proposal 4: Enhance RLF report for CHO with candidate SCGs to include the following information:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

2.2 SCGFailureInformation enhancement

Based on the discussion in section 2.1, similarly, the CHO configuration information, each candidate PCell information and the association between CHO and CPAC should be supported to be included in SCGFailureInformation for network optimization. Considering that the MN has the configuration information of CHO with candidate SCGs, i.e., the MN has the information of CHO configuration and the association between CHO and CPAC, the UE does not need to report the two information to MN included SCGFailureInformation, but the network has no idea about each candidate PCell information (e.g., first fulfilled event and the time duration between two events fulfilled), thus, we should consider to enhance SCGFailureInformation to include the information for each candidate PCell.

Proposal 5: Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each candidate PCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

2.3 SHR/SPR enhancement

In case of CHO with candidate SCGs, both PCell and PSCell are configured with execution conditions, and only the PCell and PSCell execution conditions are fulfilled at the same time, the UE triggers the CHO with candidate SCGs execution. If one of the execution condition (e.g., the execution condition of PSCell) is not fulfilled in a long time but another execution condition (e.g., the execution condition of PCell) is fulfilled, the UE will also not execute the CHO with candidate SCGs, which will cause the unexpected situation, i.e., the CPAC blocks the CHO execution, and vice versa. The ideal situation is the execution conditions of CHO and CPAC should be fulfilled as closely as possible in terms of time. A new trigger condition can be considered for successful CHO with candidate SCGs execution, e.g., the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

Proposal 6: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

In case of successful CHO with candidate SCG(s) execution, but both SHR and SPR are triggered (e.g., both T310 in PCell and T310 in PSCell exceed a configured threshold), UE can store two successful report (SHR and SPR) separately. Considering that the two reports are related to the same one handover procedure, RAN2 needs to consider how to associate the two reports to network.

Proposal 7: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

- Option 1: one report to include both SHR and SPR information;

- Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

3. Conclusion

According to the discussion in section 2, we propose:

RLF report enhancement

Proposal 1: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Proposal 2: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Proposal 3: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

Proposal 4: Enhance RLF report for CHO with candidate SCGs to include the following information:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

SCGFailureInformation enhancement

Proposal 5: Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each candidate PCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

SHR/SPR enhancement

Proposal 6: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

Proposal 7: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

- Option 1: one report to include both SHR and SPR information;

- Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

4. Reference

- R2-2500002, RAN2#128 Meeting Report

- R3-247908 LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT