3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500236

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Consideration on SONMDT enhancements for intra-NTN mobility

Agenda Item:	8.10.4

Document for:	Discussion and Decision

Introduction

For R19 SONMDT, RAN3 has sent an LS on NTN in last meeting [1]:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

In this contribution, we discuss the intra-NTN mobility related SON/MDT enhancements and give our views.

Discussion

MRO for NTN

For MRO, RAN3 only agree to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

In R17, for RLF report for CHO related Event A3 and A5 configuration we introduced:

[[ choCandidate-r17 ENUMERATED {true} OPTIONAL, choConfig-r17 SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16 OPTIONAL, triggeredEvent-r17 SEQUENCE { timeBetweenEvents-r17 TimeBetweenEvent-r17 OPTIONAL, firstTriggeredEvent-r17 ENUMERATED {condFirstEvent, condSecondEvent} OPTIONAL } OPTIONAL ]],

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

The CondTriggerConfig-r16 could include all the measurement based trigger condition of event A3/A4/A5 and time/location based trigger condition of event T1/D1/D2.

CondTriggerConfig-r16 ::= SEQUENCE { condEventId CHOICE { condEventA3 SEQUENCE { a3-Offset MeasTriggerQuantityOffset, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, condEventA5 SEQUENCE { a5-Threshold1 MeasTriggerQuantity, a5-Threshold2 MeasTriggerQuantity, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, ..., condEventA4-r17 SEQUENCE { a4-Threshold-r17 MeasTriggerQuantity, hysteresis-r17 Hysteresis, timeToTrigger-r17 TimeToTrigger }, condEventD1-r17 SEQUENCE { distanceThreshFromReference1-r17 INTEGER(0.. 65525), distanceThreshFromReference2-r17 INTEGER(0.. 65525), referenceLocation1-r17 ReferenceLocation-r17, referenceLocation2-r17 ReferenceLocation-r17, hysteresisLocation-r17 HysteresisLocation-r17, timeToTrigger-r17 TimeToTrigger }, condEventT1-r17 SEQUENCE { t1-Threshold-r17 INTEGER (0..549755813887), duration-r17 INTEGER (1..6000) }, condEventD2-r18 SEQUENCE { distanceThreshFromReference1-r18 INTEGER(0.. 65535), distanceThreshFromReference2-r18 INTEGER(0.. 65535), hysteresisLocation-r18 HysteresisLocation-r17, timeToTrigger-r18 TimeToTrigger } },

The choConfig related procedure description is as below, and no detailed event is mentioned in this description:

2> for each neighbour cell, if any, included in measResultListNR in measResultNeighCells: 3> if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure: 4> set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig; 4> if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or 4> if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure: 5> set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time; 5> set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

Therefore as RAN3 mentioned, RAN2 could reuse both the existing field/IE(s) and corresponding procedure description, and there is no spec impact on TS38.331.

Proposal 1: RAN2 to reuse both the existing field/IE(s) and corresponding procedure description for MRO for time/location and measurement based CHO.

MDT for NTN

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP.

For mapped cell ID, it is a ID transmitted in NGAP and it is not used in Uu. So in Uu, the gNB could configure the same area scope format of logged MDT for both the geographical area and mapped cell ID format in NGAP.

Proposal 2: The gNB uses the same format in Uu for configuring area scope of logged MDT for both the geographical area and mapped cell ID format in NGAP

For geographical area, RAN3 think the geographical area defined for R19 MBS NTN can be reused for Area Scope of logged MDT over Uu. In RAN2#125bis meeting, MBS NTN agreed 2 geographical area formats:

At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - - Circles (like for TN coverage) - - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

In RAN2#127 meeting, it was agreed the geographical area can be a set of regions:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Therefore, for geographical area definition, we could wait until the MBS NTN running CR version stabilizes, and reuse the geographical area defined for MBS NTN for logged MDT area scope over Uu for NTN.

Proposal 3: Wait until the MBS NTN running CR version stabilizes and then reuse the geographical area defined for MBS NTN for logged MDT area scope over Uu for NTN.

Multiple TA for NTN

In R17 NTN, gNB may broadcast a TA list in SIB1 in NTN scenario instead of single TAC: The trackingAreaList is broadcast in the PLMN-IdentityInfo in SIB1. After UE receives the SIB1, the UE AS will forward the trackingAreaList to upper layers (similar as the trackingAreaCode).

PLMN-IdentityInfo ::= SEQUENCE { plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity, trackingAreaCode TrackingAreaCode OPTIONAL, -- Need R ranac RAN-AreaCode OPTIONAL, -- Need R cellIdentity CellIdentity, cellReservedForOperatorUse ENUMERATED {reserved, notReserved}, ..., [[ iab-Support-r16 ENUMERATED {true} OPTIONAL -- Need S ]], [[ trackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode OPTIONAL, -- Need R gNB-ID-Length-r17 INTEGER (22..32) OPTIONAL -- Cond eventID-TSS ]], [[ mobileIAB-Support-r18 ENUMERATED {true} OPTIONAL -- Need S ]] }

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, network does not configure trackingAreaCode. Total number of different TACs across different PLMN-IdentityInfos shall not exceed maxTAC. This field is only present in an NTN cell.

In SON or MDT related report, the cell is identified by the CGI, i.e. CGI-Info-Logging-r16 or CGI-InfoEUTRALogging. There is only one TAC in the CGI information for logging. Therefore how to set the TA information into the CGI for logging among the TAs in the NTN specific TA list should be considered.

Proposal 4: RAN2 to consider how to set the TA information into the CGI report among the TAs in the NTN specific TA list.

Conclusion

According to the analysis in section 2, we have the following proposals:

Proposal 1: RAN2 to reuse both the existing field/IE(s) and corresponding procedure description for MRO for time/location and measurement based CHO.

Proposal 2: The gNB uses the same format in Uu for configuring area scope of logged MDT for both the geographical area and mapped cell ID format in NGAP

Proposal 3: Wait until the MBS NTN running CR version stabilizes and then reuse the geographical area defined for MBS NTN for logged MDT area scope over Uu for NTN.

Proposal 4: RAN2 to consider how to set the TA information into the CGI report among the TAs in the NTN specific TA list.

Reference

- R3-247908 LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT RAN3