3GPP TSG RAN WG2 #129                                                                                  R2-2500238

Athens, GR, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on LCM for UE-sided model for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, RAN2 discussed the LCM for UE-sided model for beam management use case and achieved the agreements as follows [1].

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

6.	For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

After RAN2#128 meeting, RAN2 receives the reply LS on applicable functionality reporting for beam management UE-sided model from RAN1 [2]. In this contribution, we will discuss the applicable functionality reporting related issues based on RAN1’s reply LS, and other FFS issues.

- Discussion

  - Applicable functionality reporting (RAN1 reply LS related)

In previous RAN2 meeting, RAN2 discussed applicable functionality reporting related issues, some agreements were made and sent to RAN1 in LS [3], some questions also were included in the LS for checking RAN1’s understanding. After RAN2#128 meeting, RAN2 receives the reply LS from RAN1, and in this reply LS, RAN1 provides some replies to these issues of RAN2 in LS [3]. In this section, we continue to discuss these issues based on RAN1’s reply LS, and for the convenience of discussion, the signalling procedure figure of applicable functionality reporting is placed here.

Figure1: Applicable functionality reporting signalling procedure

For Step 1/Step 2

- Granularity of functionality

In RAN2#127 meeting, RAN2 discussed and agreed the following functionality type which was included in the LS [3] sent to RAN1.

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

In LS [3], RAN2 asked the issue of “In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?”, and in reply LS [2], RAN1 gives the understanding of the three functionality types from RAN1’s perspective, as follow.

In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

Observation 1: In RAN1’s understanding, the meaning and the granularity of “functionality” for “supported functionalities”, “applicable functionalities” and “activated functionalities” may or may not be the same.

Furthermore, for the issue, RAN1 replies that “In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.”, thus, for the supported functionalities that UE needs to report in step 2, the reporting content and granularity can wait for RAN1’s further discussion.

Proposal 1: For supported functionalities reporting in step2, the reporting content and granularity can wait for RAN1’s further discussion.

For Step 3

- NW-side additional condition

In [3], RAN2 pointed out that these agreements of applicable functionality reporting agreed in RAN2 is based on the assumption “NW-side additional condition is assumed as associated ID.”, and further asked RAN1 whether the assumption is correct or not. Based on the issue, RAN1 gives the reply as follow in [2].

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Observation 2: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Another issue proposed by RAN2 is that “Is NW-side additional condition functionality specific?”, but RAN1 does not give the clear answer and it is still not clear whether the associated ID can be used within a functionality or cross functionalities.

Based on RAN1’s reply in [2], it can be confirmed that the associated ID can be used to ensure the consistency of NW-side additional condition across training and inference for UE-side model, if the UE receives the same associated ID value for training and inference, the UE can consider that the DL Tx beam or beam set/list for training and inference have the similar properties (i.e., NW-side additional condition is the same). However, some details about NW-side additional condition are still not clear and need to be further discussed in RAN1.

Proposal 2: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

- Configuration information in Step 3

In previous RAN2 meeting, RAN2 has agreed to include the following information in Step 3.

- “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

These FFS issues above were included in LS [3] sent to RAN1 to ask RAN1’s understanding. For the configuration information in Step 3, RAN1 made the following agreement.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

In the agreement above, RAN1 gives two options, i.e., Option A and Option B summarized as follow.

Option A: Include one or more of CSI-ReportConfig for inference configuration in Step 3;

Option B: Include one set or multiple sets of inference related parameters for applicability report only (not for inference);

In Option A, the network configures one or more of CSI-ReportConfig for inference configuration in Step 3, based on the received inference configuration, the UE decides the applicable functionalities and reports the applicable functionalities to network in Step 4. In Option B, the network configures one set or multiple sets of inference related parameters for applicability report only (not for inference), based on the received inference related parameter, the UE decides the applicable functionalities and reports the applicable functionalities to network in Step 4. From our perspective, it is not necessary to include inference configuration in Step 3, the network can include the inference configuration in Step 5 based on the UE reported applicable functionalities, providing the inference configuration in Step 3 increases the air interface signaling overhead as the inference configuration is configured based on supported functionalities instead of applicable functionalities, i.e., the useless inference configuration can be configured in Step 3 which will never be used as e.g., there is no model for it or the UE-side additional condition is not met. Therefore, we prefer Option B.

Proposal 3: In Step 3, include one set or multiple sets of inference related parameters for applicability report only (not for inference).

According to RAN1’s agreement, the set of inference related parameters can be selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

Proposal 4: The set of inference related parameters can be selected from the IEs in/referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

For other FFS issues above, e.g., whether the NW-side additional condition is mandatory or optional, are discussing in RAN1, RAN2 can wait for more progress made in RAN1.

For Step 4

- Applicable functionality reporting in Step 4

In [2], for Step 4, RAN1 proposed that UE reports applicability for all the above Option A, i.e., one or more CSI-ReportConfig and/or Option B set(s) of inference related parameters. FFS whether/what other information along with the applicability is needed.

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Based on Proposal 3 and Proposal 4 above, UE should report applicability for all set(s) of inference related parameters configured in Step 3, the detailed content in Step 4 needs to be further discussed, e.g., whether the UE needs to report all inference related parameters of applicable set of inference related parameters, or only to report the index of the applicable set of inference related parameters, from RAN2’s perspective, reporting index is more simple and less air interface signaling overhead.

Proposal 5: UE reports applicability for all set(s) of inference related parameters configured in Step 3. Detailed content e.g., reporting applicable inference related parameters or only reporting the index of the applicable inference related parameters needs further discussion.

Another issue is that if the inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? For the issue, RAN1 gives the reply that applicable aperiodic CSI report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported; applicable periodic CSI report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Proposal 6: If the inference configuration is provided in Step 3,

- Applicable aperiodic CSI report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

- Applicable periodic CSI report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

For Step 5

In [2], for Step 5, RAN1 has the following agreement.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN1 agreed that in Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration if UE has already been configured with CSI-ReportConfig in Step 3. According to Proposal 3, we understand the set(s) of inference related parameters for applicability report only (not for inference) should be included in Step 3, thus, in Step 5, for performing inference, NW needs to configure CSI-ReportConfig for inference configuration in RRCReconfiguration.

For the content of inference configuration in Step 5, RAN1 will discuss and design later. RAN2 can wait for RAN1’s discussion and progress.

Q6: What is the content of inference configuration in Step 5? Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Proposal 7: In Step 5, if proposal 3 is agreed, NW needs to configure CSI-ReportConfig for inference configuration in RRCReconfiguration. The content of inference configuration in Step 5 is up to RAN1.

Another issue is that if the inference configuration is provided in Step 5, does the inference configuration in Step 5 activate the functionality immediately upon receiving Step 5? For the issue, RAN1 gives the reply that aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete; periodic CSI report is considered as activated after RRCReconfigurationComplete.

Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

Proposal 8: If the inference configuration is provided in Step 5,

- Aperiodic CSI report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- Periodic CSI report is considered as activated after RRCReconfigurationComplete.

In [3], RAN2 proposed an issue that if more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? In [2], RAN1 gives the following agreement, RAN1 agreed that for beam management use case, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Proposal 9: If more than one functionality are configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

In [3], another issue about activation/deactivation is whether L1/L2 signaling for functionality activation/deactivation is needed? In [2], RAN1 gives the reply as follow. In RAN1’s understanding, L1 and L2 signalling can be used for aperiodic CSI report and semi-persistent CSI report.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed? Answer to Q10: Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

Proposal 10: Regarding functionality activation/deactivation, L1 and L2 signalling can be used for aperiodic CSI report and semi-persistent CSI report.

  - Other applicable functionality reporting related issues

In last RAN2 meeting, RAN2 discussed and agreed that when a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. A FFS is left that whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

In RAN2#127 meeting, RAN2 agreed that UE reports applicable functionality upon change of applicable functionality via UAI. Based on the agreement, when an applicable functionality becomes non-applicable, the applicable functionality set change occurs. Thus applicable functionality reporting is triggered to inform network the new applicable functionality set and the UAI is used as the reporting signaling. Based on the reporting, network can get to know which functionality becomes non-applicable from applicable functionality compared with previous reported applicable functionality set implicitly.

Proposal 11: When an applicable functionality becomes non-applicable, it can be reported to network implicitly by triggering the applicable functionality reporting via UAI.

In RAN2#127bis meeting, another issue is left that whether it is regarding an active functionality, i.e., whether only when active functionality becomes non-applicable functionality can be reported to network. We understand any applicable functionality becomes non-applicable can be reported to network, so that the network can get to know which functionality can be configured with inference configuration in Step 5.

Proposal 12: When any applicable functionality becomes non-applicable, it can be reported to network, i.e., not only the active functionality.

After RAN2#126 meeting, during the email discussion [4], an issue was discussed that whether to define available functionalities separately from applicable functionalities, but it is still FFS. In [5], the related issue was discussed that whether UE can include non-applicable functionalities in Step 4 (i.e., along with reporting applicable functionalities), which is that UE can report the functionalities for which a model is available in device, but is not currently applicable. From our perspective, the intention of reporting the non-applicable functionalities is not clear, e.g., it is uncertain that reporting the non-applicable functionalities in step 4 is for the network to provide the configuration (such as the NW-side additional condition) to make the non-applicable functionalities change to applicable functionalities. If this is the purpose, RAN2 needs to check whether it is feasible.

Proposal 13: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

In addition, if there is no model available in device for a functionality, whether the UE can also report the non-applicable functionalities without an available model to network for e.g., model transfer needs to be discussed, which is also related to model transfer discussion.

Proposal 14: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

A related issue is that should it provide the non-applicable functionalities when it doesn’t have applicable functionality that can be reported upon a change in applicable functionality or upon receiving a network request for applicable functionalities, or should the UE report the non-applicable functionalities as long as there are non-applicable functionalities? This can be discussed further based on Proposal 13 and Proposal 14.

Another related issue is what behavior of UE should be when it doesn’t have applicable functionality that can be reported upon a change in applicable functionality or upon receiving a network request for applicable functionalities. Should it report empty content or should some other assistant information (such as non-applicable functionalities) be sent to the network for model transfer or providing NW-side additional condition to UE.

Proposal 15: RAN2 to discuss the UE behaviors when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

  - Data collection for training

In last meeting, RAN2 discussed and agreed that for data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. When the UE is configured to allow initial request data collection configuration, if the UE e.g., has no model for a functionality or has no model for a NW-side additional condition or triggered by OTT server etc., the UE can send the request information to network to request network to provide the data collection configuration. The request information can be, for example,

Option 1: only a single indicator information to indicate request data collection configuration;

Option 2: UE sends the desired configuration to network, e.g., desired NW-side additional condition, desired measurement resource configuration etc.

For Option1, it is simpler and less air interface signaling overhead, but an issue is the network can have no idea what type data does the UE expect to collect, e.g., the UE has no model for NW-side additional condition associated ID=1, NW sends the data collection configuration to UE after receiving the UE’s request, but current NW-side additional condition is associated ID=2, the UE will collect the data under associated ID=2, which is not the UE’s expectation. Another issue is that the granularity of the indicator needs to be considered, e.g., whether it is associated to functionality or is associated to NW-side additional condition or is associated to other aspects, which will introduce more complexity. For option2, the UE can include the desired configuration in the request to network, e.g., desired NW-side additional condition, desired measurement resource configuration etc, the network can get to know what configuration the UE expects and provide the suitable configuration to UE. But we needs to discuss the detailed request information, the detailed request information can be discussed and decided by RAN1.

Proposal 16: RAN2 to discuss the following two options for the UE request contents of data collection for training configuration request:

Option 1: only a single indicator information to indicate request data collection configuration;

Option 2: UE sends the desired configuration to network, e.g., desired NW-side additional condition, desired measurement resource configuration etc.

Another an FFS issue in last meeting is whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration. From our perspective, it seems to be unnecessary that the UE reports the indicator to network to indicate UE can’t perform data collection based on received configuration. On the one hand, the UE can provide the sufficient information in the data collection request procedure, to make sure the network can get the sufficient information to configure the effective data collection configuration. Even if the configuration from network is invalid, the UE can send the request again to network to request the suitable configuration. One the other hand, if the UE can’t perform data collection based on received configuration, e.g., low power or low memory, the UE can stop the data collection, and continue to collect data when the UE has high power or sufficient memory again.

Proposal 17: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

- Conclusion

According to the discussion in section 2, we propose:

Applicable functionality reporting (RAN1 reply LS related)

Observation 1: In RAN1’s understanding, the meaning and the granularity of “functionality” for “supported functionalities”, “applicable functionalities” and “activated functionalities” may or may not be the same.

Proposal 1: For supported functionalities reporting in step2, the reporting content and granularity can wait for RAN1’s further discussion.

Observation 2: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Proposal 2: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

Proposal 3: In Step 3, include one set or multiple sets of inference related parameters for applicability report only (not for inference).

Proposal 4: The set of inference related parameters can be selected from the IEs in/referred by CSI-ReportConfig as a starting point, e.g., the information listed below. The final inference related parameters are discussed and decided by RAN1.

- the associated ID;

- Set A related information;

- Set B related information;

- Report content related information;

- For BM-Case 2,

- Time instances related information for measurements;

- Time instances related information for prediction.

Proposal 5: UE reports applicability for all set(s) of inference related parameters configured in Step 3. Detailed content e.g., reporting applicable inference related parameters or only reporting the index of the applicable inference related parameters needs further discussion.

Proposal 6: If the inference configuration is provided in Step 3,

- Applicable aperiodic CSI report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

- Applicable periodic CSI report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Proposal 7: In Step 5, if proposal 3 is agreed, NW needs to configure CSI-ReportConfig for inference configuration in RRCReconfiguration. The content of inference configuration in Step 5 is up to RAN1.

Proposal 8: If the inference configuration is provided in Step 5,

- Aperiodic CSI report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- Periodic CSI report is considered as activated after RRCReconfigurationComplete.

Proposal 9: If more than one functionality are configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

Proposal 10: Regarding functionality activation/deactivation, L1 and L2 signalling can be used for aperiodic CSI report and semi-persistent CSI report.

Other applicable functionality reporting related issues

Proposal 11: When an applicable functionality becomes non-applicable, it can be reported to network implicitly by triggering the applicable functionality reporting via UAI.

Proposal 12: When any applicable functionality becomes non-applicable, it can be reported to network, i.e., not only the active functionality.

Proposal 13: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

Proposal 14: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

Proposal 15: RAN2 to discuss the UE behaviors when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

Data collection for training

Proposal 16: RAN2 to discuss the following two options for the UE request contents of data collection for training configuration request:

Option 1: only a single indicator information to indicate request data collection configuration;

Option 2: UE sends the desired configuration to network, e.g., desired NW-side additional condition, desired measurement resource configuration etc.

Proposal 17: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

- References

- R2-2500002, RAN2#128 Meeting Report

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

- R2-2407848 LS on applicable functionality reporting for beam management UE-sided model

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase1

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase2