3GPP TSG RAN WG2 #129                                                                         	  R2-2500243

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT, Turkcell

Title:	Simulation results of Model Generalization

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, the following agreements for model generalization were made[1].

Agreements - Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. Study generalization over UE speeds The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz

Based on these agreements, the further simulations are done in the following aspects:

- FR2 temporal domain case A with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h;

- Inter-frequency generalization for 2GHz and 4GHz.

- Discussion

  - FR2 to FR2 temporal domain case A

For FR2 to FR2 temporal domain case A, the following simulation parameters are adopted.

Table 1 Simulation combination for FR2 generalization study

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 |  |  | Yes | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 |  |  | Yes |  | Yes |  |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 |  | yes |  |  |  | Yes |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

In our simulation, we focus on the following speed combination in Table 2. The parameters of AI model and simulation results are provided in Table 3.

Table 2: Generalization cases of 120Km/h for FR2 temporal domain case A

| Case | Training dataset(Km/h) | Inference data set (Km/h) |
| --- | --- | --- |
| Baseline | 120 | 120 |
| GC#1 | 60 | 120 |
| GC#2 | 60，90，120 | 120 |

Table 3: Simulation report table for FR2 temporal domain case A

| Report parameters |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | See Table 2. |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |
|  | Number of TX beams | 32 |  |  |  |
|  | Number of RX beams | 4 |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |  |
|  | Observation window(Note3) | 240ms |  |  |  |
|  | Prediction window (Note3) | 80ms |  |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |
| Data Size (Number of Samples) | Training/validity | ≈116K/29K |  |  |  |
|  | Testing | ≈29K |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |  |
|  | Model complexity in a number of parameters(M) | 780,150 |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 8.963MB |  |  |  |
|  | Computational complexity [FLOPs] | 2.14M |  |  |  |
| Metrics | Prediction points index (m) | Baseline | GC#1 | GC#2(trained with partial data set of each speed) | GC#2(trained with full data set of each speed) |
|  | Average L3 cell level RSRP difference (dB) | 0.604 | 0.641 | 0.628 | 0.520 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |

It can be seen that the RSRP difference in GC#2 trained with partial data set of each speed is higher than the baseline RSRP difference, which is approximate 0.024dB increase for Speed 120Km/h. It means that the predication in GC#2 with partial data set of each speed is worse than that of the baseline. This is because GC#2 with partial data set of each speed is training based on the data set with mixed data of different speed. So the model needs to learn the features of different speeds. Furthermore, the data size of GC#2 with partial data set of each speed is close to the data size of baseline (data set with single speed). Therefore the data size of single speed in GC#2 with partial data set of each speed occupies only 1/3 part of the data set of baseline. This results in that some features of single speed may not be learnt in GC#2 with partial data set of each speed.

Then, we use the data set with the total amount of all the mixed speed. In this case, the data set will be 3 times of the data set of baseline. And the RSRP difference of GC#2 trained with full data set of each speed is lower than that of baseline which shows positive gain with 0.084dB decrease in RSRP difference.

Observation 1: The RSRP difference in GC#2 trained with partial data set of each speed is larger than baseline RSRP difference for FR2 to FR2 temporal domain case A.

Observation 2: The RSRP difference in GC#2 trained with full data set of each speed is smaller than that of baseline RSRP difference for FR2 to FR2 temporal domain case A.

The RSRP difference in GC#1 is larger than that of baseline, which is approximate 0.037dB increase on RSRP difference for Speed 120Km/h. It is the worst case among all the generalization cases. In GC#1, the model is trained using the data set of different speed, which lacks the features of the data set in baseline. The model can’t learn some of the specific features of the data set of 120Km/h which results in higher RSRP difference.

Observation 3: The RSRP difference in GC#1 is the highest among all generalization cases. I.e. the GC#1 shows the worst predication performance among these 3 cases for FR2 to FR2 temporal domain case A.

Even though the RSRP difference in GC#1 and GC#2 is higher than that of the baseline, it does not bring significant decline on performance. I.e. the RSRP difference growth is still under 0.1dB. Hence, the results of generalization with different speed are applicable to AI predication. The simulation results prove that generalization in speed aspect is feasible especially considering the real UE moves with changing speed.

Observation 4: The simulation results prove that generalization in speed aspect is feasible for FR2 to FR2 temporal domain case A.

  - FR1 to FR1 Inter-frequency

For inter-frequency, the following combinations of frequencies in Table 4 were adopted. It should be noted that only partial data set is simulated for inter-frequency. And we use 2 AI models, ResNet and MLP.

Table 4: Generalization cases for inter-frequency (2G)

| Case | Training dataset (InputOutput) | Inference data set inputOutput) |
| --- | --- | --- |
| Baseline | 4GHz2GHz | 4GHz2GHz |
| GC#1 | 2GHz4GHz |  |
| GC#2 | (2GHz, 4GHz4GHz, 2GHz) |  |

Table 5: Generalization cases for inter-frequency (4G)

| Case | Training data (InputOutput) | Inference data set (InputOutput) |
| --- | --- | --- |
| Baseline | 2GHz4GHz | 2GHz4GHz |
| GC#1 | 4GHz2GHz |  |
| GC#2 | (2GHz, 4GHz 4GHz, 2GHz) |  |

And the AI simulation parameters and results are shown in Table 5.

Table 6: Simulation report table for FR1 to FR1 cell-level inter-frequency

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |
|  | UE speed (30,60,90,120 Km/h) | 0 |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | No |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | N/A |  |
|  | Number of TX beams | 8 |  |
|  | Number of RX beams | 2 |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |
|  | Observation window(Note3) | N/A |  |
|  | Prediction window (Note3) | N/A |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | 13000/1625 |  |
|  | Testing | 1625 |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level predicted RSRP of all cells |  |
|  | Model output(Note 6) | Cell-level predicted RSRP of all cells |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet | MLP |
|  | Model complexity in a number of parameters(M) | 0.017M | 0.912M |
|  | Model complexity in model size (e.g. Mbyte) | 0.065MB | 3.497MB |
|  | Computational complexity [FLOPs] | 285.60K | 19.68M |
| Metrics | Average L3 cell level RSRP difference (dB) | See Table 7-10. |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |

Table 7: Simulation results of generalization for FR1 to FR1 inter-frequency of ResNet model

| Case | Training dataset (InputOutput) | Inference data set (InputOutput) | RSRP Diff(dB) | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | 4GHz->2GHz | 4GHz->2GHz | 0.2009 | N/A |
| GC#1 | 2GHz->4GHz |  | 0.3947 | 0.1938 |
| GC#2 | (2GHz, 4GHz 4GHz, 2GHz) |  | 0.2556 | 0.0547 |

Table 8: Simulation results of generalization for FR1 to FR1 inter-frequency of MLP model

| Case | Training dataset (InputOutput) | Inference data set (InputOutput) | RSRP Diff | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | 4GHz->2GHz | 4GHz->2GHz | 0.1317 | N/A |
| GC#1 | 2GHz->4GHz |  | 0.3661 | 0.2344 |
| GC#2 | (2GHz, 4GHz 4GHz, 2GHz) |  | 0.2258 | 0.0941 |

Table 9: Simulation results of generalization for FR1 to FR1 inter-frequency of ResNet model

| Case | Training dataset(Hz) (InputOutput) | Inference data set (Hz) (InputOutput) | RSRP Diff | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | 2GHz->4GHz | 2GHz->4GHz | 0.2131 | N/A |
| GC#1 | 4GHz->2GHz |  | 0.3487 | 0.1356 |
| GC#2 | (2GHz, 4GHz 4GHz, 2GHz) |  | 0.2700 | 0.0569 |

Table 10: Simulation results of generalization for FR1 to FR1 inter-frequency of MLP model

| Case | Training dataset(Hz) (InputOutput) | Inference data set (Hz) (InputOutput) | RSRP Diff | ABS of RSRP Difference between GC#n and Baseline (dB) |
| --- | --- | --- | --- | --- |
| Baseline | 2GHz->4GHz | 2GHz->4GHz | 0.1254 | N/A |
| GC#1 | 4GHz->2GHz |  | 0.3360 | 0.2106 |
| GC#2 | (2GHz, 4GHz 4GHz, 2GHz) |  | 0.2185 | 0.0931 |

Similar to the case of generalization of speed, it can be seen that the RSRP difference in GC#2 for both ResNet and MLP models is higher than the baseline RSRP difference, which is approximate 0.0547dB higher for ResNet model and approximate 0.0941dB higher for MLP model. This is because GC#2 is being trained based on the data set with mixed data of 2GHz and 4GHz. So the model needs to learn the features of different frequencies. Furthermore, the data size of GC#2 (data set with both 2GHz and 4GHz) is close to the data size of baseline (data set with 2GHz or 4GHz). This results in that some features of single frequency in baseline may not be learnt in GC#2.

Observation 5: The RSRP difference in GC#2 trained with partial data set of each frequency is larger than baseline RSRP difference for FR1 to FR1 Inter-frequency.

The GC#1 is the worst case, which has the highest RSRP difference among all the generalization cases. Compared with baseline RSRP difference, the RSRP differences in GC#1 increase near 0.2dB for ResNet model and MLP model. This is because the model is trained using the data set of another frequency in GC#1, which lacks the features of the data set in baseline. The model can’t learn some of the specific features of the data set of different frequency which results in higher RSRP difference.

Observation 6: The RSRP difference in GC#1 is the highest among all generalization cases for FR1 to FR1 Inter-frequency.

Even though the RSRP difference in GC#1 and GC#2 are higher than that of the baseline, it does not bring significant decline on performance. I.e. the increase of RSRP difference is under 0.25dB. Hence, the simulation results show that generalization with different frequencies is feasible for inter-frequency RRM predication.

Observation 7: The simulation results show that generalization with different frequencies is feasible for inter-frequency RRM predication for FR1 to FR1 Inter-frequency.

- Conclusion

In this contribution, we provide our views on the simulation of generalization for 120Km/h and different frequencies (2G and 4G), and the observations are summarized as follows:

FR2 to FR2 temporal domain case A:

Observation 1: The RSRP difference in GC#2 trained with partial data set of each speed is larger than baseline RSRP difference for FR2 to FR2 temporal domain case A.

Observation 2: The RSRP difference in GC#2 trained with full data set of each speed is smaller than that of baseline RSRP difference for FR2 to FR2 temporal domain case A.

Observation 3: The RSRP difference in GC#1 is the highest among all generalization cases. I.e. the GC#1 shows the worst predication performance among these 3 cases for FR2 to FR2 temporal domain case A.

Observation 4: The simulation results prove that generalization in speed aspect is feasible for FR2 to FR2 temporal domain case A.

FR1 to FR1 Inter-frequency:

Observation 5: The RSRP difference in GC#2 trained with partial data set of each frequency is larger than baseline RSRP difference for FR1 to FR1 Inter-frequency.

Observation 6: The RSRP difference in GC#1 is the highest among all generalization cases for FR1 to FR1 Inter-frequency.

Observation 7: The simulation results show that generalization with different frequencies is feasible for inter-frequency RRM predication for FR1 to FR1 Inter-frequency.

- References

- R2-2500002, RAN2#128 Meeting Report