3GPP TSG RAN WG2 #129                                                                         	  R2-2500245

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT, Turkcell

Title:	Simulation results of AI based Handover performance

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, the following agreements were made for event A3 indirect prediction.

1	Agreed values for Case A.  NOTE1 indirect prediction only

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 90 | Open for 60 and 120km/h |
| OW length (ms, note1) | N/A | Up to implementation |
| PW length (ms, note1) | 400 | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |

2	Agreed values for Case B. NOTE1 indirect prediction only

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30 | Open for 60 and 90km/h |
| OW length (ms,note1) | N/A | Up to implementation |
| PW length (ms,note1) | 200 (non-sliding) 40ms (sliding) | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |
| MRRT | 50% | Open for more values |

And for system level simulation, the following procedures were agreed.

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. 3 Handover Preparation time is 40ms and handover execution time: 40ms 4 RAN2 focus on indirect prediction methodology.

Based on the agreements above, we simulate the HO performance for FR1 temporal domain case B, and FR2 temporal domain case A with Option 2 and Option 3.

- Discussion

  - FR1 to FR1 intra-frequency temporal domain Case B

For Case B, it needs to check whether the reduced measurement brings significant impacts to the handover performance. Following the assumption agreed in RAN2#128 meeting, the simulation parameters and results are provided in Table 1.

Table 1: Simulation parameters and results for measurement prediction for temporal domain case B

| Report parameters |  | CATT |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30Km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Procedure A |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50% |
|  | Observation window(Note3) | N/A |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 72000，9000 |
|  | Testing | 9000 |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |
|  | Model output(Note 6) | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | MLP |
|  | Model complexity in a number of parameters(M) | 0.9167M |
|  | Model complexity in model size (e.g. Mbyte) | 0.086MB |
|  | Computational complexity [FLOPs] | 19.680M |
| Metrics | MRRT | 50% |
|  | Average L3 cell level RSRP difference (dB) | 0.313 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |

And the simulation results for HO performance are shown in Table 2.

Table 2: HO performance for FR1 to FR1 intra-frequency temporal domain Case B

| Case | Handover number | HoF rate(%) |
| --- | --- | --- |
| Actual HO procedure | 0.0304 per UE per second | 10.76% |
| HO procedure with AI prediction | 0.0309 per UE per second | 11.19% |

It can been that with MRRT = 50%, the handover number and HoF rate remain almost the same as non-AI case. It can be seen that there is limited impacts on handover performance when MRRT = 50 for FR1 to FR1 intra-frequency temporal domain case B. When MRRT = 50%, referring to [1], it can be seen that F1 score of FR1 to FR1 intra-frequency temporal domain Case B is around 0.7. But this does not bring significant negative impacts to the handover number. One reason is that it’s counted as a successful prediction only when the target cell of the real handover and the target cell of the predicted handover are the same. That is, during AI handover procedure, the UE may be switched to another target cell which is different from the target cell in real handover procedure. But this kind of handover is not considered during our F1 score calculation.

Observation 1: There is limited impacts on handover performance, including handover number and HoF rate when MRRT = 50% for FR1 to FR1 intra-frequency temporal domain case B.

  - FR2 to FR2 intra-frequency temporal domain case A

HoF model

It was agreed in RAN2#127bis meeting that “ The HO model in 36.839 is taken as baseline”.

And based on 36.839, the HoF is defined as below:

Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3. For calculating the handover failures for the two states: - In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met: 1) Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure) or 2) RLF is declared in the state 2 - In state 3: after the UE is attached to the target cell a handover failure is counted if the following criterion is met: - target cell downlink filtered average (the filtering/averaging here is same as that used for starting T310) wideband CQI is less than the threshold Qout (-8 dB) at the end of the handover execution time (Table 5.1.4.1) in state 3.

In our simulation, the HoF in each state is simulated, and whether/how AI improves the aspects of HO performance for HO number and HoF rate are also shown. In order to facilitate the discussion in the document, we define the following terminologies:

- HoF rate 2_1: Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating HO_CMD failure);

- HoF rate 2_2: RLF is declared in the state 2 (late HO).

Regarding AI prediction for Option 2 and Option 3, we use the same AI model and the parameters are provided in Table 3 for temporal domain case A.

Table 3 Simulation parameters and results for measurement prediction for temporal domain case A

| Report parameters |  | CATT |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |
|  | UE speed (30,60,90,120 Km/h) | 90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |
|  | Whether LOSsoft is modeled or not | Yes |
|  | spatial consistency option (A or B) | Procedure A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | 400ms |
|  | Prediction window (Note3) | 400ms |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | ≈194K/48K |
|  | Testing | ≈48K |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |
|  | Model output(Note 6) | Cell level RSRP of all cells |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 5.601M |
|  | Model complexity in model size (e.g. Mbyte) | 64.138MB |
|  | Computational complexity [FLOPs] | 23.44M |
| Metrics | OW vs. PW | [400ms, 400ms] |
|  | Average L3 cell level RSRP difference (dB) | 1.459 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |

Option 2

In Option 2, the source cell sends HO request to the predicated target cell. And the predicated target cell sends HO request ACK to the source cell. Later, if actual A3 event is triggered and the actual target cell is the same as predicated target cell, the handover command can be sent to the UE immediately after actual measurement event is reported to the source cell. Hence, the handover preparation time is saved and the UE can perform handover in advance to avoid the further channel deterioration of the source cell. Considering the duration waiting HO command is reduced by handover preparation duration, the HoF decreases.

However, this improvement on handover performance may not be significant in case the handover preparation time is short. During the simulation of handover performance, the HOF model is defined as T310 is running when handover command is received, while T310 is assumed as 1 second but the handover preparation time is 40ms. The handover is only performed 40/1000 = 4% in advance. Hence, the gain on HoF is marginal.

With assumption agreed in RAN2#128 meeting and the observation window length is 400ms in AI model, the HO number and HoF failure rate is shown in Table 4 and parameters in Table 4.

Table 4: HO performance of Option 2 for FR2 to FR2 intra-frequency temporal domain case A

| Case | Handover number | HoF rate(%) | HoF rate 2_1(%) |
| --- | --- | --- | --- |
| Actual HO procedure | 0.2406 per UE per second | 17.82% | 13.66% |
| HO procedure with AI prediction | 0.2414 per UE per second | 17.13% | 13.08% |

It can be seen that the handover number is almost unchanged considering the handover number mostly depends on the UE speed. And HoF rate is decreased from 17.82% to 17.13%. But the gain is relatively low, which proves the analysis above.

The HoF rate reduction of HoF rate 2_1 contributes most of the gain for HoF rate. That means when handover command is sent with handover preparation in advance, the channel condition of the source cell is still stable to provide connection to send handover command.

Besides, the benefits on reduction of HoF rate also depends on the AI model accuracy. In our simulation, if the predicted target cell is not the same as the actual triggered cell, the UE will perform the handover after handover preparation. Hence, if the AI model accuracy is low, less UE will perform handover over in advance, and the benefits on HoF failure reduction is small.

Observation 2: The handover number does not decrease with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 3: The HoF rate decreases slightly with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A, considering the HO preparation time (40ms) is relatively short compared with the length of the T310 (1s).

Observation 4: HoF rate reduction for the HO_CMD failure case contributes most of the gain for HoF rate reduction with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A

Observation 5: AI model accuracy plays important role on HoF rate reduction, i.e. more accurate of the AI model, more gain will be obtained for HoF rate reduction with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A.

Option 3

In Option 3, the UE will perform the handover based on the predicted A3 event with TTT duration in advance. With assumption agreed in RAN2#128 meeting and the observation window length is 400ms in AI model, the HO number and HoF failure rate are shown in Table 5.

Table 5: HO performance of Option 3 for FR2 to FR2 intra-frequency temporal domain case A

| Case | Handover number | HoF rate(%) | HoF rate 2_1(%) |
| --- | --- | --- | --- |
| Actual HO procedure | 0.2406 per UE per second | 17.82% | 13.66% |
| HO procedure with AI prediction | 0.2504 per UE per second | 12.74% | 8.02% |

It can be seen that with AI prediction and the handover execution is performed with TTT duration in advance based on the prediction, the HoF rate can be reduced by 5.08%, which is almost 1/4 reduction compared with HoF in actual HO procedure. Hence, AI prediction on A3 event in Option 3 brings more benefits on HoF reduction than that of Option 2.

Like Option 2, the HoF rate reduction of HoF rate 2_1 contributes most of the gain for HoF rate, which decreases from 13.66% to 8.02%. Compared with Option 2, the gain on HoF rate reduction is significant. Normally, TTT is used to avoid measurement jilter so that a cell which shows stable signal performance can be selected as a target cell. In Option 3, the target cell is predicated to trigger A3 event, which means the target cell is stable to fulfill A3 event based on AI prediction. Considering the A3 event offset is set as 2dB in the simulation, the RSRP of the target cell is higher than that of the serving cell when A3 event is triggered. In this case, the channel condition of the target cell is better than that of the source cell. The UE should access to the target cell as early as possible to obtain the benefits of better channel condition of the target cell. It can be seen that the HoF rate decreases if the UE accesses to the target cell with TTT duration in advance which proves the analysis above.

Observation 6. The HoF rate decreases with HO model Option 3 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 7: HoF rate reduction for the HO_CMD failure case contributes most of the gain for HoF rate reduction with HO model Option 3 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 8: The HoF rate decreases with significant gain in Option 3 compared with Option2 for FR2 to FR2 intra-frequency temporal domain case A.

- Conclusion

In this contribution, we provide our views on the simulation of handover performance for FR1 to FR1 intra-frequency temporal domain case B and FR2 to FR2 intra-frequency temporal domain case A, and the observations are summarized as follows:

FR1 to FR1 intra-frequency temporal domain case B:

Observation 1: There is limited impacts on handover performance, including handover number and HoF rate when MRRT = 50% for FR1 to FR1 intra-frequency temporal domain case B.

FR2 to FR2 intra-frequency temporal domain case A:

Option 2:

Observation 2: The handover number does not decrease with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 3: The HoF rate decreases slightly with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A, considering the HO preparation time (40ms) is relatively short compared with the length of the T310 (1s).

Observation 4: HoF rate reduction for the HO_CMD failure case contributes most of the gain for HoF rate reduction with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A

Observation 5: AI model accuracy plays important role on HoF rate reduction, i.e. more accurate of the AI model, more gain will be obtained for HoF rate reduction with HO model Option 2 for FR2 to FR2 intra-frequency temporal domain case A.

Option 3:

Observation 6. The HoF rate decreases with HO model Option 3 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 7: HoF rate reduction for the HO_CMD failure case contributes most of the gain for HoF rate reduction with HO model Option 3 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 8: The HoF rate decreases with significant gain in Option 3 compared with Option2 for FR2 to FR2 intra-frequency temporal domain case A.

- References

- R2-2500244, Simulation results of event A3 prediction, CATT