3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500247

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, RRM relaxation was discussed and the following work assumption was achieved:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

In this contribution, we provide our further considerations on RRM relaxation and offloading in RRC_IDLE/INACTIVE.

Discussion

Relationships among conditions

According to RAN4 agreements, the following cases were agreed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Until now, RAN2 has discussed LP-WUS monitoring conditions, MR RRM measurement relaxation (i.e. case #3 Relaxed cased b in RAN4) conditions, and MR serving cell fully offloading (i.e. case #1 Fully offloading case in RAN4) conditions. And we have made the following progress:

- For LP-WUS monitoring conditions, we have agreed:

  - Entry condition: based on MR and optional LR.

  - Exit condition: based on LR.

- For MR serving cell fully offloading conditions, we have agreed:

  - Entry condition: based on MR and optional LR.

  - Exit condition: based on LR.

If conditions for these cases, including LP-WUS monitoring, MR RRM measurement relaxation and MR serving cell fully offloading, are configured in a cell, we would like to clarify their relationships at first. It is straightforward that conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation. During previous RRM relaxation discussion, MR serving cell relaxation was not introduced as it may seriously impact the user’s experience. In R19, MR serving cell relaxation is discussing with LP-WUS capable UEs. If the UE doesn’t monitor LP-WUS, it is dangerous to relax MR serving cell. In worst case the UE maybe out of coverage and paging maybe lost. Therefore, conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring. One example of relationships among these conditions is shown in Figure 1.

Figure 1 One example of relationships among different conditions

Proposal 1: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation.

MR RRM measurement relaxation

- Same or separate criteria for MR serving cell and neighbor cell measurement relaxation

In RAN4#113 meeting, it was agreed:

Agreement: Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

As the same relaxation factor(s) applies, it is straightforward to use the same criteria for both MR serving cell and neighbor cell relaxation. In addition, relaxed RRM results of MR serving cell and neighbor cell are used for cell reselection. In order to reduce UE power consumption, there is no motivation to achieve normal MR serving cell measurement results and relaxed MR neighbor cell measurement results for cell reselection and vice versa .

Currently, the following working assumption for neighbor cell measurement relaxation in RAN2#127bis is aligned with agreements for MR serving cell measurement relaxation.

- Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Hence, we propose:

Proposal 2: Same relaxation criteria are applied to both MR serving cell and neighbor cell measurement for UEs capable of LP-WUS.

- Criteria for MR RRM measurement relaxation

Currently, MR serving cell quality is agreed as condition(s) for both MR serving cell and neighbor cell measurement relaxation. But different from R16 RRM relaxation, MR serving cell may be relaxed as well. MR serving cell measurement results cannot be updated timely during MR RRM measurement relaxation period. This will lead UEs capable of LP-WUS cannot exit MR RRM measurement relaxation timely. In the worst case, cell reselection cannot be performed timely. Therefore, for neighbor cell measurement relaxation for UEs capable of LP-WUS, LR needs to be considered together.

Proposal 3: Apart from MR, LR needs to be considered for the entry/exit condition of MR RRM relaxation for UEs capable of LP-WUS.

There are two possible options for MR RRM measurement relaxation considering both MR and LR need to be considered:

- Option 1: Serving cell quality measured by MR is higher than relaxation threshold, while serving cell quality measured by LR is higher than relaxation threshold. No separate exit condition.

- Option 2: Introduce separate entry condition and exit condition for MR RRM measurement relaxation, including:

Entry condition: based on MR serving cell quality and optional LR serving cell quality.

Exit condition: based on LR serving cell quality.

Option 1 is similar as R16 RRM relaxation criteria while option 2 is similar as criteria for both LP-WUS monitoring and serving cell fully offloading. Both options can work. We propose RAN2 discuss which option is adopted.

Proposal 4: RAN2 discuss which option is adopted for MR RRM relaxation

- Option 1: Serving cell quality measured by MR is higher than relaxation threshold, while serving cell quality measured by LR is higher than relaxation threshold. No separate exit condition.

- Option 2: Entry condition: based on MR serving cell quality and optional LR serving cell quality.

Exit condition: based on LR serving cell quality.

MR Serving cell fully offloading

- MR neighbor cell measurement

In RAN4#111 meeting, RAN4 agreed the MR serving cell fully offloading case as follows.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON.

According to the agreed case#1 fully offloading case in RAN4, it is not required to perform MR neighboring cell measurement either. In our understanding, MR neighbor cell measurements on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority are not required when MR serving cell measurement is offloading to LR. But whether MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped depends on RAN4’s further discussion.

Proposal 5: When the entry condition for serving cell measurement fully offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

- Relationship between MR Serving cell fully offloading and MR neighbor cell measurement off in legacy

In legacy, if the MR serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements including [1]:

-	If the serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In RAN2#127 meeting, RAN2 agreed that entry conditions for MR serving cell fully offloading (i.e., there is no serving cell measurement by MR) can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Entry conditions for serving cell measurement offloading should not be worse than conditions for legacy MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority, considering serving cell measurement offloading should be careful. Therefore, MR RSRP threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP, as shown in Figure 2. And before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement off no matter if MR serving cell measurement is relaxed or not, i.e. if the MR serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform MR intra-frequency measurements; If the MR serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform MR measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Proposal 6: MR RSRP threshold in the entry conditions for serving cell measurement fully offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 7: Before entry conditions for serving cell measurement fully offloading are fulfilled, the UE can follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Figure 2: One example of the serving cell measurement offloading coverage

Conclusion

According to the analysis in Section 2, we propose:

Relationships among conditions:

Proposal 1: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation.

MR RRM measurement relaxation:

Proposal 2: Same relaxation criteria are applied to both MR serving cell and neighbor cell measurement for UEs capable of LP-WUS.

Proposal 3: Apart from MR, LR needs to be considered for the entry/exit condition of MR RRM relaxation for UEs capable of LP-WUS.

Proposal 4: RAN2 discuss which option is adopted for MR RRM relaxation

- Option 1: Serving cell quality measured by MR is higher than relaxation threshold, while serving cell quality measured by LR is higher than relaxation threshold. No separate exit condition.

- Option 2: Entry condition: based on MR serving cell quality and optional LR serving cell quality.

Exit condition: based on LR serving cell quality.

MR Serving cell fully offloading:

Proposal 5: When the entry condition for serving cell measurement fully offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 6: MR RSRP threshold in the entry conditions for serving cell measurement fully offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 7: Before entry conditions for serving cell measurement fully offloading are fulfilled, the UE can follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.