# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500257
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.5.3

Source:	Apple

Title:	Remaining issues on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The study objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state becomes normative WI objective as copied:

In this contribution, we share our view on remaining issues from below aspects:

- How to handle UEs in RRC_CONNECTED state

- Area specific UL WUS configuration

- Access restriction for legacy UEs

- Paging interruption during OD-SIB1 acquisition

# 2 Discussion

## 2.1 How to handle UEs in RRC_CONNECTED state

In RAN2#128 [2], how to handle RRC_CONNECTED UEs was discussed but it was not concluded because it is not clear whether it is in WID scope.

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

Then, it was discussed in RANP#106 [3], and the following conclusion in RP-243297 was endorsed to task RAN2 to only address the RLF case (i.e. when T311 is running):

From RP-243297:

On RRC Connected mode

• The scope of WID will not be updated to include RRC Connected.

• However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission.  No need to change the scope of WI to indicate this.

As highlighted above, RANP has guided RAN2 to use the same procedure as IDLE/INACTIVE UE for this case. However, we think there are still some followed spec changes:

- After T311 starts, spec change is needed in Section 5.3.7.2 of TS 38.331 [4] to specify the UE doesn’t release the stored UL WUS configuration.

#### 5.3.7.2	Initiation

Omit unrelated part

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	stop timer T421, if running;

1>	if UE is not configured with attemptCondReconfig; and

1>	if UE is not configured with attemptLTM-Switch:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

Omit unrelated part

According to existing Section 5.3.7.2 of TS 38.331, when initiating RRC re-establishment procedure, the UE releases spCellConfig which is expected to include UL WUS configuraiton. Thus, the spec changes are necessary.

- RAN2 need to discuss the condition to trigger UL WUS transmission during RRC re-establishment procedure. Similar to agreed IDLE/INACTIVE UE behavior, we think the legacy cell selection criteria can be reused.

Note that the below agreement on IDLE/INACTIVE UEs in RAN2#128 [2] can’t cover RRC_CONNECTED UE in RLF because cell selection (rather than cell reselection) is performed during RRC re-establishment procedure, according to Section 5.3.7.3 of TS 38.331 [4].

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- With above spec impacts of 1) and 2), we think the same OD-SIB1 acquisition behavior of IDLE/INACTIVE UE can be reused.

Thus, we propose:

Proposal 1: Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 starts, the UE doesn’t release the stored UL WUS configuration and is allowed to trigger the OD-SIB1 acquisition procedure towards the selected NES cell during cell selection.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

Then, we think the validity of the stored UL WUS configuration needs further discussion. In RAN2#128 [2], the validity mechanism of IDLE/INACTIVE UEs was agreed:

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

We think the similar principle can be reused, i.e. discarding after directly succeeding cell selection (including selecting NES cell or normal cell). Meanwhile, we think there is one difference between cell reselection and cell selection (during RRC re-establishment):

- In cell reselection, the UE can always get the latest UL WUS config from camping Cell, e.g. right before triggering OD-SIB1 acqusition procedure in target NES cell.

- In cell selection (during RRC re-establishment), it may take a longer time than cell reselection and the UE doesn’t know when to acquire an updated UL WUS configuraiton. Thus, it is possible the stored UL WUS configuration is out of date when the UE identify NES cell (e.g. RLF is detected when the new SIB with UL WUS configuration is already stored for almost 3 hours).

Observation 1: Cell selection (during RRC re-establishment) may take a longer time than cell reselection and it is possible that RLF is detected when new SIB with UL WUS configuration is already stored for almost 3 hours.

According to Section 5.2.2.2.1 of TS 38.331 [4], the stored SIB is regarded as “non-valid” when either of the following conditions is met:

- After 3 hours from the moment it was successfully confirmed as valid.

- The version tag of serving cell’s SIB is not identical to the stored one.

- The cellIdentity of serving cell’s SIB is not identical to the stored one when areaScope is not present (i.e. cell specific SIB).

- The systemInformationAreaID of serving cell’s SIB is not identical to the stored one when areaScope is present (i.e. area specific SIB).

- The first PLMN-Identity in the PLMN-IdentityInfoList of serving cell’s SIB is not identical to the stored PLMN-Identity.

In above conditions, we think:

- 1) can be reused for CONNECTED UE in RLF.

- 2)/4)/5) can’t be reused because these information for checking validity (i.e., version tag, systemInformationAreaID and PLMN-Identity) are included in SIB1.

- 3) is aligned with the principle agreed for IDLE/INACTIVE UEs, i.e. directly succeeding cell (re)selection.

Thus, we propose to add 1) as one validity condition of CONNECTED UEs in RLF.

Proposal 2: When T311 is running, the UE discards the stored UL WUS configuration (i.e. the stored UL WUS configuration is regarded as not valid) when either of below conditions is met:

- Cell selection to either NES cell or normal cell (i.e. same as RRC_IDLE/RRC_INACTIVE UEs)

- After 3 hours from the moment the new SIB including UL WUS configuration was successfully confirmed as valid.

## 2.2 Area specific UL WUS configuration

In this section, we discuss the following FFS agreed in RAN2#128 [2]:

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

First, please note that RAN1#118 has agreed to have a unified configuration format for cell specific UL WUS configuration and area specific UL WUS configuration [5].

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Then, we do an analysis on payload size of UL WUS configuration based on latest RAN1 agreement [6] in Appendix of this contribution. It shows that at least 369 bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2952 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit, which may not be sufficient if only cell specific UL WUS configuration is allowed. It will cause coverage issue.

Observation 2: RAN1#118 has agreed to have a unified configuration format for cell specific UL WUS configuration and area specific UL WUS configuration.

Observation 3: According to analysis on payload size of RAN1 agreed UL WUS configuration, at least 369bit is required for one single UL WUS configuration. Thus, if only cell specific UL WUS configuration is allowed, at most 7 NES cell’s UL WUS configuration can be included, which may cause coverage issue.

Based on Observation 2 and 3, we propose to support area specific UL WUS configuration.

Proposal 3: Support area specific UL WUS configuration.

##### 5.2.2.2.1	SIB validity

Omit unrelated part

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

Omit unrelated part

Then, we discuss solution details. The existing area specific SIB should be a good reference. According to Section 5.2.2.2.1 of TS 38.331 [4], the UE applies the area specific SIB when areaScope is present. As highlighted below, the UE regards the stored SIB is valid only when the first PLMN-Identity, valueTag and systemInformationAreaID in the serving cell’ SIB are identical to the stored ones. This mechanism can’t be reused for area specific UL WUS configuration because these IEs are all indicated in SIB1.

Observation 4: In existing area specific SIB, the UE regards the stored SIB is valid only when the first PLMN-Identity, valueTag and systemInformationAreaID in the serving cell’ SIB are identical to the stored ones. This mechanism can’t be reused for area specific UL WUS configuration because these IEs are all indicated in SIB1.

We think the simplest solution is to allow more than one PCI(s) to be associated with one UL WUS configuration. For example:

ULWUSConfigInfoList ::=     SEQUENCE (SIZE (1..maxULWUS)) OF ULWUSConfigInfo

ULWUSConfigInfo ::=         SEQUENCE {

CarrierFreq               ARFCN-ValueNR,

NESCellList ::=          SEQUENCE (SIZE (1..maxNESCell)) OF PhysCellId,

ULWUSConfig               UlWUSConfig

}

As PCI information is included in PBCH, the UE can determine whether the stored UL WUS configuration is applicable via decoding PBCH during OD-SIB1 acquisition. In this way, there is no spec impacts on MIB. This solution is applicable to both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs (when T311 is running).

Proposal 4: Specify the following mechanism of area specific UL WUS configuration for both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs (when T311 is running):

- Configuration: more than one PCI(s) can be associated with one UL WUS configuration.

- UE behavior: during OD-SIB1 acquisition, the UE checks the PCI in NES cell’s PBCH to determine whether the stored UL WUS configuration is applicable (i.e. no need of spec changes in MIB).

## 2.3 Access restriction for legacy UEs

### 2.3.1 Issue 1: Whether “no SIB1 indication” can bar legacy UEs

The following agreement on legacy UE barring was agreed in RAN2#127b [7]:

Agreements on OD-SIB1

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES Ues is introduced.

In our understanding, the highlighted part is intended for RAN2 to check whether current RAN1 design of “no SIB1 indication” can bar legacy UE. However, it seems that some companies in RAN1 misunderstood that this agreement was intended to request RAN1 to design new solution for legacy UEs. And RAN1#119 had discussed it offline under the issue 8 “How to bar legacy UEs from the NES cell” in FL summary [8].

Observation 5: Legacy UE barring agreement in RAN2#127b is intended to check whether current RAN1 design of “no SIB1 indication” can bar legacy UE. However, some companies in RAN1 misunderstood that it was intended to request RAN1 to design new solution for legacy UEs and discussed it offline without conclusion.

Give the misunderstanding of RAN1, we think it is necessary to clarify RAN2 understanding based on latest RAN1 agreement on “no-SIB1 indication”. In RAN1#118b [5], the following related agreements were made:

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

According to above agreements, our understanding is:

- If NES cell’s SSB is transmitted in sync raster, its K_SSB will take value K_SSB > 23 for FR1 and K_SSB >11 for FR2.

- Whether SSB of NES cell can be transmitted off sync raster is FFS in RAN1.

We further provide our analysis on 1) in Table.1 based on existing TS 38.213 [9] and TS 38.304 [10].

|  | K_SSB Value on NES cell | Legacy UE behavior according to existing spec | Rel-19 NES UE behavior according to existing spec and RAN1/RAN2 status |
| --- | --- | --- | --- |
| Sub-case 1 | K_SSB <= 23 for FR1 and K_SSB <= 11 for FR2 | - Regard it is CD-SSB and detect SIB1 (38.213) - Camp in the NES cell (38.304) | - Regard it is CD-SSB and detect SIB1 (38.213) - Camp in the NES cell (38.304) |
| Sub-case 2 | 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 | - Regard it is NCD-SSB, and detect another SSB according to PDCCH-ConfigSIB1 (38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB, and detect another SSB according to PDCCH-ConfigSIB1 (38.213) - NW implementation may indicate SSB info of Cell A in PDCCH-ConfigSIB1. - May regard the cell is barred up to 300s (38.304) |
| Sub-case 3 | K_SSB = 30 for FR1 and K_SSB = 14 for FR2 | - Regard it is NCD-SSB, and no further behavior is specified (reserved K_SSB value in 38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB (38.213) - RAN1 may define new UE behavior and repurpose PDCCH-ConfigSIB1. - May regard the cell is barred up to 300s (38.304) |
| Sub-case 4 | K_SSB = 31 for FR1 and K_SSB = 15 for FR2 | - Regard it is NCD-SSB and no SIB1 (38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB and no SIB1 (38.213) - May regard the cell is barred up to 300s (38.304) |

Table.1 Analysis of legacy UE and Rel-19 UE behavior when NES cell takes different value of K_SSB

RAN1 agreed K_SSB value are sub-cases 2/3/4. According to Table.1, legacy UEs may bar the NES Cell up to 300s in all these sub-cases. Meanwhile, Rel-19 UE may be specified with new behavior (highlighted in red font) which is being discussed in RAN1. RAN2 can just wait RAN1 and then discuss whether any RAN2 spec change. However, irrespective of whether RAN1 agree Rel-19 UE new behavior, the legacy UE barring behavior is not impacted.

Observation 6: According to existing 38.213 and 38.304, legacy UEs may bar the NES Cell in all the K_SSB value range agreed by RAN1 (i.e. K_SSB > 23 for FR1 and K_SSB >11 for FR2 when SSB is transmitted in sync raster). Meanwhile, Rel-19 UE may be specified with new behavior which is being discussed in RAN1.

Although whether SSB of NES cell can be transmitted off sync raster is FFS in RAN1, we think it will not impact legacy UE barring NES cell because cell reselection is based on SSB in sync raster according to TS 38.300 [11]:

9.2.1.2	Cell Reselection

A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following:

- Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

Observation 7: Irrespective of whether RAN1 conclude SSB of NES cell can be transmitted off sync raster, it will not impact legacy UE barring NES cell because cell reselection is based on SSB in sync raster.

Thus, based on Observation 6 and 7, we think “no SIB1 indication” in MIB can bar legacy UEs based on latest RAN1 agreements, and no separate barring bit is needed.

Proposal 5a: RAN2 clarify that the legacy UE barring agreement in RAN2#127b is not intended to request RAN1 to design new barring solution for legacy UEs.

Proposal 5b: RAN2 confirm that “no SIB1 indication” in MIB of NES cell can bar legacy UEs according to latest RAN1 agreements on K_SSB value, and thereby no separate barring bit is needed.

### 2.3.2 Issue 2: How to determine whether NES cell is broadcasting OD-SIB1

The following agreements to check NES cell broadcasting status was agreed in RAN2#127b [7]:

Agreements on OD-SIB1

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Similar to legacy UE barring issue, some companies in RAN1 misunderstood that this agreement was intended to request RAN1 to design new solution to detect whether SIB1 is currently being broadcasted. And RAN1#119 had discussed it offline under the issue 7 “Indication of broadcast status of OD-SIB1” in FL summary [8].

Observation 8: Similar to legacy UE barring issue, some companies in RAN1 misunderstood that the agreement on checking OD-SIB1 broadcasting status was intended to request RAN1 to design new solution to detect whether SIB1 is currently being broadcasted. And RAN1#119 had discussed it offline without conclusion.

Give the misunderstanding of RAN1, we think it is necessary to clarify RAN2 understanding. In our understanding, the above agreement doesn’t intend to request RAN1 to design a blind detection solution. It is sufficient to rely on UE implementation to check if SIB1 is currently being broadcasted. Thus, we propose:

Proposal 6a: RAN2 clarify that RAN2#127b agreement on checking broadcasting status of OD-SIB1 is not intended to request RAN1 to design blind detection solution.

Proposal 6b: It is up to UE implementation how to check if SIB1 is currently being broadcasted or provided on demand before requesting SIB1 of that cell.

## 2.4 Paging interruption during OD-SIB1 acquisition

This issue was raised by some company contribution in study phase [12], but it was not treated. In details, after the UE sends OD-SIB1 request in NES cell, it can’t monitor paging in NES Cell until OD-SIB1 is acquired. This is an extra paging interruption compared with legacy cell reselection. We think it is a valid issue because TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”) [13]:

4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell.

These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Observation 9: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

We think there are two alternatives:

- Alt-1: leave it to UE implementation. But RAN2 need to send LS to RAN4 to check whether the paging interruption requirement needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Since RAN2 has not discussed this issue before, we suggest RAN2 to discuss these 2 alternatives.

Proposal 7: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

# 3 Conclusion

In this contribution, we discuss remaining issues of on-demand SIB1. Our observations are:

Observation 1: Cell selection (during RRC re-establishment) may take a longer time than cell reselection and it is possible that RLF is detected when new SIB with UL WUS configuration is already stored for almost 3 hours.

Observation 2: RAN1#118 has agreed to have a unified configuration format for cell specific UL WUS configuration and area specific UL WUS configuration.

Observation 3: According to analysis on payload size of RAN1 agreed UL WUS configuration, at least 369bit is required for one single UL WUS configuration. Thus, if only cell specific UL WUS configuration is allowed, at most 7 NES cell’s UL WUS configuration can be included, which may cause coverage issue.

Observation 4: In existing area specific SIB, the UE regards the stored SIB is valid only when the first PLMN-Identity, valueTag and systemInformationAreaID in the serving cell’ SIB are identical to the stored ones. This mechanism can’t be reused for area specific UL WUS configuration because these IEs are all indicated in SIB1.

Observation 5: Legacy UE barring agreement in RAN2#127b is intended to check whether current RAN1 design of “no SIB1 indication” can bar legacy UE. However, some companies in RAN1 misunderstood that it was intended to request RAN1 to design new solution for legacy UEs and discussed it offline without conclusion.

Observation 6: According to existing 38.213 and 38.304, legacy UEs may bar the NES Cell in all the K_SSB value range agreed by RAN1 (i.e. K_SSB > 23 for FR1 and K_SSB >11 for FR2 when SSB is transmitted in sync raster). Meanwhile, Rel-19 UE may be specified with new behavior which is being discussed in RAN1.

Observation 7: Irrespective of whether RAN1 conclude SSB of NES cell can be transmitted off sync raster, it will not impact legacy UE barring NES cell because cell reselection is based on SSB in sync raster.

Observation 8: Similar to legacy UE barring issue, some companies in RAN1 misunderstood that the agreement on checking OD-SIB1 broadcasting status was intended to request RAN1 to design new solution to detect whether SIB1 is currently being broadcasted. And RAN1#119 had discussed it offline without conclusion.

Observation 9: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

Based on observations, our proposals are:

How to handle UEs in RRC_CONNECTED state

Proposal 1: Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 starts, the UE doesn’t release the stored UL WUS configuration and is allowed to trigger the OD-SIB1 acquisition procedure towards the selected NES cell during cell selection.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

Proposal 2: When T311 is running, the UE discards the stored UL WUS configuration (i.e. the stored UL WUS configuration is regarded as not valid) when either of below conditions is met:

- Cell selection to either NES cell or normal cell (i.e. same as RRC_IDLE/RRC_INACTIVE UEs)

- After 3 hours from the moment the new SIB including UL WUS configuration was successfully confirmed as valid.

Area specific UL WUS configuration

Proposal 3: Support area specific UL WUS configuration.

Proposal 4: Specify the following mechanism of area specific UL WUS configuration for both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs (when T311 is running):

- Configuration: more than one PCI(s) can be associated with one UL WUS configuration.

- UE behavior: during OD-SIB1 acquisition, the UE checks the PCI in NES cell’s PBCH to determine whether the stored UL WUS configuration is applicable (i.e. no need of spec changes in MIB).

Access restriction for legacy UEs

Proposal 5a: RAN2 clarify that the legacy UE barring agreement in RAN2#127b is not intended to request RAN1 to design new barring solution for legacy UEs.

Proposal 5b: RAN2 confirm that “no SIB1 indication” in MIB of NES cell can bar legacy UEs according to latest RAN1 agreements on K_SSB value, and thereby no separate barring bit is needed.

Proposal 6a: RAN2 clarify that RAN2#127b agreement on checking broadcasting status of OD-SIB1 is not intended to request RAN1 to design blind detection solution.

Proposal 6b: It is up to UE implementation how to check if SIB1 is currently being broadcasted or provided on demand before requesting SIB1 of that cell.

Paging interruption during OD-SIB1 acquisition

Proposal 7: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#128, Chair Notes.

[3] RANP#106, Chair Notes.

[4] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[5] RAN1#118b, Chair Notes.

[6] RAN1#119, Chair Notes.

[7] RAN2#127b, Chair Notes.

[8] R1-2410680, FL summary 4 for on-demand SIB1 in idle/inactive mode, Moderator (MediaTek).

[9] TS 38.213-v18.0.0, Physical layer procedures for control, 2023-12.

[10] TS 38.304-v18.0.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state, 2023-12.

[11] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[12] R2-2406470, Remaining details to enable on-demand SIB1, Intel Corporation.

[13] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

# Appendix: Analysis of overhead of UL WUS configuration

The following Table.2 provides an estimation based on latest RAN1 agreement in RAN1#119 [6]:

| Purpose | Approximate payload size analysis | Note | Total payload size |
| --- | --- | --- | --- |
| Identify NES cell(s) | PhysCellId (10bit) ARFCN-ValueNR (22bit) | This is only value of one NES cell. Please note that RAN2 agreed multiple NES cells’ UL WUS config can be included in on Cell A. | 32bit |
| RACH resource parameter for SIB1 request | rach-ConfigGeneric (40bit) - prach-ConfigurationIndex (8bit) - msg1-FDM (2bit) - msg1-FrequencyStart (9bit) - zeroCorrelationZoneConfig (4bit) - preambleReceivedTargetPower (8bit) - preambleTransMax (4bit) - powerRampingStep (2bit) - ra-ResponseWindow (3bit) ssb-perRACH-Occasion (3bit) si-RequestPeriod (3bit) SI-RequestResources (14bit) - ra-PreambleStartIndex (6bit) - ra-AssociationPeriodIndex (4bit) - ra-ssb-OccasionMaskIndex (4bit) Other RACH config (22bit) - msg1-SubcarrierSpacing (3bit) - prach-RootSequenceIndex (10bit) - rsrp-ThresholdSSB (7bit) - n-TimingAdvanceOffset (2bit) - TDD-UL-DL-ConfigCommon (~30bit) - ss-PBCH-BlockPower (7bit) - restrictedSetConfig (1bit) | - Assume up to 275 PRBs. - n-TimingAdvanceOffset is missed. | 120bit |
| BWP info of initial UL BWP | BWP (~20bit), including locationAndBandwidth, SCS, cyclicPrefix. | It is necessary to identify PRACH frequency location because PRACH resource is configured per BWP. | 20bit |
| UL frequency info | FrequencyInfoUL-SIB (~133bit) - MultiFrequencyBandListNR (80bit) - absoluteFrequencyPointA (22bit) - scs-SpecificCarrierList (~24bit) - p-Max (6bit) - frequencyShift7p5khz (1bit) | - It is necessary to obtain carrier information of PRACH transmission and related setting (e.g. UL power). - Assume up to 8 bands - Assume only 1 SCS. | 133bit |
| SIB1 reception | pdcch-ConfigSIB1 (8bit) | It seems conflicted with latest RAN1 agreement | 8bit (TBD) |
| RAR reception | RA-SearchSpace (6bit) monitoringSlotPeriodicityAndOffset (16bit) duration (12bit) monitoringSymbolsWithinSlot (14bit) nrofCandidates (6bit) | - Assume CORESET#0 is used for RAR reception (i.e. no extra common CORESET configuration). - Assume only one search space is configured for RAR. | 52bit |
| Total payload size: 369bit×Num of NES Cell (e.g. If 8 NES Cells’ UL WUS config are included, 2952 bit is required) * * Optimization of area-specific configuration is not considered, i.e. one NES cell has one UL WUS config in this analysis. |  |  |  |

Table.2 Analysis on required payload size of UL WUS configurations