# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500258
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.5.4

Source:	Apple

Title:	Discussion on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

Up to now, only paging adaptation was discussed in RAN2. SSB adaptation and RACH adaptation were not discussed due to time limitation.

In this contribution, we further discuss RAN2 work on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Adaptation of SSB in time domain

In RAN1, the key agreements on SSB adaptation with potential RAN2 impacts are listed below:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement (RAN1#118) For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion (RAN1#118b) There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

In our understanding, the key points can be summarized as follows:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Observation 1: Key RAN1 progress on SSB adaptation with RAN2 impacts can be summarized as below:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Although RAN1 discussion on SSB adaptation is not finished, we think potential RAN2 impacts include:

- MAC-CE format design if RAN1 agree to use MAC-CE to indicate SSB periodicity adaptation.

- L3 measurement due to SSB adaptation, including RRM, RLM and BFD.

On 1), although RAN1 agreed to use MAC-CE to activate / deactivate OD-SSB which is similar to SSB adaptation in Rel-19, both of the two WID objectives are RAN1 led. Thus, RAN2 is not in position to conclude whether the unified signalling is applied. Thus, RAN2 should wait RAN1 progress.

Proposal 1: On SSB adaptation, RAN2 wait RAN1 conclusion on whether to use MAC-CE to indicate SSB adaptation (i.e. RAN2 doesn’t discuss whether the unified signaling is used in OD-SSB and SSB adaptation).

On 2), we first discuss L3 RRM impacts due to SSB periodicity adaptation. Please note that RAN4 has asked RAN2 whether SMTC configuration needs specification changes in their LS (R4-2416911) [2].

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

We think the issue is similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

Observation 2: L3 measurement impacts on SSB adaptation is similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

As we discussed in our companion contribution on OD-SSB [3], we prefer to keep legacy SMTC based L3 RRM framework. We basically have the same justifications to keep SMTC based framework for L3 measurement in SSB adaptation. The specific reasons why we prefer to keep legacy SMTC based L3 framework can be summarized in Observation 3 and Observation 4:

Observation 3: To support UE autonomous SSB measurement outside of SMTC, it will introduce significant RAN2/RAN4/RAN1 specification changes:

  - RAN2: it essentially challenges the per-frequency L3 RRM framework and introduces significant RAN2 specification impacts in Section 5.5 of TS 38.331 and Section 9.2.4 of TS 38.300.

  - RAN4: all L3 measurement requirements need specification changes because all L3 measurement requirements are specified with SMTC configuration rather than SSB configuration in legacy.

  - RAN1: it will result in multiple RAN1 specification changes in Section 5.1 of TS 38.215.

Observation 4: The UE autonomous SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when SSB is partially overlapped or has small timing gap with SMTC.

Thus, we have similar proposal as OD-SSB:

Proposal 2: On L3 measurement on SSB adaptation, keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC).

As we discussed in our companion contribution on OD-SSB [3], we prefer to introduce dynamic SMTC adaptation in L3 measurement on OD-SSB. We basically have the same justifications to introduce dynamic SMTC adaptation for L3 measurement in SSB adaptation. The specific reasons why we think dynamic SMTC adaptation is needed can be summarized in Observation 5 and Observation 6:

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover all SSBs with different periodicities without causing negative UE impacts (e.g. extra power consumption if NW configures a dense SMTC to cover all SSBs).

Observation 6: In legacy, SMTC can only be reconfigured by RRC signaling. However, because SSB adaptation is expected to dynamically change periodicity of SSB via MAC-CE / DCI, one single SMTC can’t quickly align with the dynamic change of SSB periodicity.

Due to the similarity between OD-SSB and SSB adaptation, we prefer to reuse the same dynamic SMTC adaptation solution for SSB adaptation. Thus, we propose:

Proposal 3: On L3 measurement on SSB adaptation, introduce dynamic adaptation between different SMTC configurations. FFS signaling details of how to configure multiple SMTC configurations.

As discussed before, RAN4 has asked RAN2 whether SMTC configuration needs specification changes in their LS (R4-2416911) [2]. RAN2 need to inform RAN4 if Proposal 2 and Proposal 3 are agreed. We provide an example of reply LS in Appendix.

Proposal 4: Send reply LS (as response to RAN4 LS R4-2416911) to inform RAN4 on Proposal 2 and Proposal 3 in Appendix.

Finally, different from OD-SSB which is supported only in SCell, SSB adaptation may be supported in PCell. Thus, we think RAN2 impacts on RLM and BFD need further discussion:

- RLM: In legacy RLM, several SSB(s) can be configured as RLM-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RLM (e.g. one actually transmitted beam configured as RLM-RS is turned off after SSB adaptation).

- BFD: In legacy BFD in SCell, several SSB(s) can be configured as BFD-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing BFD (e.g. one actually transmitted beam configured as BFD-RS is turned off after SSB adaptation).

Because RLM and BFD rely on configured SSB periodicity rather than SMTC, we think maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

Observation 7: Because RLM and BFD rely on configured SSB periodicity rather than SMTC, maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

However, because SSB adaptation in PCell is still not finalized in RAN1, we think RAN2 can further discuss their impacts. Thus, we propose:

Proposal 5: RAN2 study the spec impacts on RLM and BFD due to SSB adaptation.

## 2.2 Adaptation of paging occasions

In RAN2#127b, option-b (i.e. extend value range of N and Ns) is agreed as baseline [4]:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

And in RAN2#128 [9], baseline values of Ns and N were agreed with some candidate values as FFS. Meanwhile, it was agreed to revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion.

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In this section, we discuss these issues:

- Detailed values of extended N and Ns

- How to support PEI when Ns>4

### 2.2.1 Detailed values of extended N and Ns

According to Table. 1, only if we support extended N to 64, 128 and 256, it can achieve the target of option b (i.e., concentrate all POs of one paging cycle in on PF).

| Value of 1/N | Number of PFs in one paging cycle |  |  |  | Required Ns if fully compensate |
| --- | --- | --- | --- | --- | --- |
| 1/N | rf32 | rf64 | rf128 | rf256 |  |
| 1/16 | 2 | 4 | 8 | 16 | 4 (no need to extend Ns) |
| 1/8 | 4 | 8 | 16 | 32 | 8 |
| 1/4 | 8 | 16 | 32 | 64 | 16 |
| 1/2 | 16 | 32 | 64 | 128 | 32 |
| 1 | 32 | 64 | 128 | 256 | 64 |

Table.1 All possible cases of PF number in one paging cycle

Meanwhile, unlike Ns, increasing values of N doesn’t bring RRC overhead issue. Thus, we propose:

Proposal 6: N value is extended with 32, 64, 128, and 256.

Then, we discuss Ns. According to offline discussion of RAN2#128 [9], we observe that there were some concerns on RRC signaling overhead and NW complexity to allow Ns =16. Although we think supporting Ns=16 can bring paging latency benefit with manageable cost, we can compromise for progress. Thus, we propose:

Proposal 7: Ns value is extended only with 8.

### 2.2.2 How to support PEI when Ns>4

According to offline discussion of RAN2#128 [9], 3 solutions for PEI can be considered:

- Alt-1: Based on one single PEI-O configuration, and rely on network implementation to have separate PEIs for legacy UEs and Rel-19 UEs

- Alt-2: Introduce one separate PEI-O configuration different from legacy one:

  - Alt 2-1: It is up to NW configuration which one is monitored by legacy UEs.

  - Alt-2-2: Rel-19 UEs only monitor the PEI-O according to the new PEI-O configuration.

One example of Alt-1 and Alt-2-1 can be found in Figure.1

Figure.1 Illustration of two solutions of supporting PEI in paging adaptation

Between them, we prefer Alt-2 due to the following concern on Alt-1:

- Alt-1 may put a restriction on the NW configuration of PEI-O, e.g. the legacy PEI needs to cover all 8 POs with last 4 POs (PO 5-8) only visible to Rel-19 UEs.

- Alt-1 may force legacy UEs to receive PEI information on invisible POs (e.g. PO5-8 in Figure 1), which is bad for legacy UE power saving.

- Alt-1 put extra requirement on the subgroup configuration. In Figure. 1, NW needs to ensure that none of subgroups with legacy UEs are mapped to PO 5-8. We think it is difficulty for both CN based and RAN based subgrouping because gNB is not aware of the differentiation of legacy UE IDs and Rel-19 UE IDs.

So, Alt-2 is the cleanest solution. Compared with Alt 2-1, the benefit of Alt-2-2 is not clear but puts extra complexity in both UE and NW. Thus, we propose to support Alt 2-1:

Proposal 8: Introduce a separate PEI configuration. Rel-19 UEs only monitor the PEI-O according to the new PEI-O configuration while legacy UEs only monitor the PEI-O according to the legacy PEI-O configuration.

One followed issue is how to support Ns=8 in the new PEI-O configuration. As highlighted below, the existing symbol level offset IE (i.e. firstPDCCH-MonitoringOccasionOfPEI-O-r17) is configured per maxPEI-perPF-r17. maxPEI-perPF-r17 is fixed to 4 which means the maximum PO number in one PF associated with one PEI.

PEI-O configuration

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

As Ns is increased to 8, the above legacy symbol level offset IE may not well support the new PEI-O for Rel-19 UEs. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI. The root cause is due to hard coding of ASN.1 with maxPEI-perPF-r17=4.

Observation 8: The legacy PEI framework can’t work when Ns is increased to 8. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

We think the simplest way to resolve this issue is to increase the value maxPEI-perPF to 8 in Rel-19 PEI-O configuration. As we propose to introduce a new PEI-O configuration, there is no legacy issue. Meanwhile, we also don’t see strong concern on increasing RRC payload size because it is only increased from 21bit to 42bit. Thus, we propose:

Proposal 9: The new PEI-O configuration uses maxPEI-perPF = 8 in symbol level offset IE firstPDCCH-MonitoringOccasionOfPEI-O (i.e. allow one PEI to associate with 8 POs in one PF when Ns=8).

## 2.3 Adaptation of PRACH in time domain

RAN1#116, RAN1#116b and RAN1#117 had discussed RACH adaptation [6][7][8], and a lot of agreements were made. Based on RAN1 agreements, we understand that RAN1 progress can be highlighted as below:

- Introduce adaptation mechanism between legacy PRACH resources and one new additional PRACH resources for NES-capable UE in all RRC states.

  - RAN1 agreed the adaptation signaling can be L1/DCI signaling and RRC/SIB.

- The additional PRACH resources are provided via a prach-ConfigurationIndex and a separate SSB-RO mapping is performed for the additional PRACH resources.

  - It is still under discussion whether to allow any overlapping between legacy PRACH resources and the additional PRACH.

  - prach-ConfigurationIndex for the additional PRACH resources can be same as or different from the one for the legacy resource.

Based on latest RAN1 progress, we share our view on potential RAN2 work in the followed sub-sections.

### 2.3.1 Legacy UE compatibility

First, we discuss the legacy compatibility issue. Because RAN1 agreements have considered legacy UE impacts, our understanding is that RAN2 don’t need to introduce new legacy UE barring mechanism for RACH adaptation. Meanwhile, same as paging adaptation, we don’t think it makes sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of RACH adaptation because it may lead to coverage holes and goes against the intention of NES.

Proposal 10: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

### 2.3.2 Discussion on RACH framework

Firstly, we discuss whether RACH adaptation is applicable to CBRA or CFRA. Currently, RAN1 study focus on CBRA.  Thus, we propose to first study CBRA.

Proposal 11: RAN2 first study CBRA in RACH adaptation.

For CFRA, if the UE dedicated RO/preamble is configured in RRC message (e.g. HO), it seems that the benefit of RACH adaptation is not clear because NW can reconfigure the UE dedicated RO/preamble via RRC. However, shared CFRA resource pool is configured in RRC in some cases of CFRA (e.g. PDCCH order RACH). In such cases, similar mechanism of RACH adaptation may be reused for CFRA to achieve NES gain. Thus, we think it can be studied if time allows.

Proposal 12: If time allows, RAN2 extends the CBRA adaptation mechanism to some type of CFRA with shared CFRA resource pool in RRC (e.g. PDCCH order RACH).

Then, another issue is whether to introduce a new RACH feature combination in the RACH portioning framework. In Rel-17, a general RACH partitioning framework was introduced. As illustrated in Figure.2, a list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Figure.2 Illustration of RRC configuration of multiple RACH partitions

In our previous RAN2 contribution [10], we proposed to clarify two different understandings on RACH adaptation:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

However, we later realized that it may not be necessary to introduce RACH adaptation as one RACH feature combination. In SBFD RACH, the similar framework was introduced to adapt between legacy RACH resource and RACH resource in SBFD sub-band, as illustrated in Figure. 3.

Figure.3 Illustration of similarities between RACH in NES and RACH in SBFD

Please note that SBFD RACH agreed not to introduce a new RACH feature combination in RAN2#127 [11]:

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The main technique justifications for the agreements are:

- No requirement is identified to support Msg 1 early identification of SBFD feature.

- General RACH portioning framework is already too complex to maintain.

Instead, one simpler option is to model it as one new RACH type similar to 2-step RACH. With such modeling, all the existing RACH features can enjoy the benefit of SBFD RACH.

Observation 9: SBFD RACH agreed not to introduce a new RACH feature combination due to:

- No requirement is identified to support Msg 1 early identification of SBFD feature.

- General RACH portioning framework is already too complex to maintain.

We think these two justifications are also applicable to RACH adaptation. Thus, we propose:

Proposal 13: Same as SBFD RACH, RACH adaptation is not introduced as a new RACH feature combination in RACH partitioning framework.

### 2.3.3 Potential RAN2 work

Based on latest RAN1 progress, we think that at least the following RRC and MAC impacts are foreseen:

RRC impacts:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

Proposal 14: In RRC, introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

MAC impacts:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

  - For example, Rel-19 UE may always start from additional PRACH resource, considering legacy UE can only use legacy PRACH resource.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

  - RAN2 need to discuss whether only the same PRACH resource set (i.e. legacy vs additional) as initial transmission can be selected.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

For above MAC impacts, we think there are two alternatives of adaptation framework between additional RACH resource and legacy RACH resource:

- Alt-1 (specified solution): the adaptation solution agreed in SBFD RACH is reused as baseline, including RACH resource set selection in Msg1 (re)transmission and fallback mechanism.

  - Note that SBFD RACH has agreed to specify UE behavior in RAN2#127b and RAN2#128, which can be baseline of RACH adaptation.

- RAN2#127b agreement:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- Alt-2 (unspecified solution): it is up to UE implementation how to select RACH resource set for Msg1 initial transmission.

  - However, one issue is whether only the same set selected in Msg1 initial transmission can be used in Msg1 retransmission due to consideration of power ramping continuity.

We think the specification work of these two alternatives are similar and are open for discussion.

Proposal 15: RAN2 discuss the following two alternatives of adaptation framework between additional RACH resource and legacy RACH resource:

- Alt-1 (specified solution): the adaptation solution agreed in SBFD RACH is reused as baseline, including RACH resource set selection in Msg1 (re)transmission and fallback mechanism.

- Alt-2 (unspecified solution): it is up to UE implementation how to select RACH resource set for Msg1 initial transmission. FFS whether restrict to the same set in Msg1 retransmission due to power ramping.

If Alt-1 of Proposal 15 is agreed, we think it is straight forward to model the additional RACH resource as a new RACH type parallel with 2-step RACH in MAC. One example is illustrated in Figure.4.

Figure.4 Example of modelling additional RACH resource (i.e. rach-SetB-ConfigCommon) as new RACH type

Proposal 16: If RAN2 agree to support specified RACH resource set selection, the additional RACH resource is modelled as a new RACH type parallel with 2-step RACH in MAC.

# 3 Conclusion

In this contribution, we further discuss RAN2 work on common signal transmission adaptation. Our observations are:

Observation 1: Key RAN1 progress on SSB adaptation with RAN2 impacts can be summarized as below:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Observation 2: L3 measurement impacts on SSB adaptation is similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

Observation 3: To support UE autonomous SSB measurement outside of SMTC, it will introduce significant RAN2/RAN4/RAN1 specification changes:

  - RAN2: it essentially challenges the per-frequency L3 RRM framework and introduces significant RAN2 specification impacts in Section 5.5 of TS 38.331 and Section 9.2.4 of TS 38.300.

  - RAN4: all L3 measurement requirements need specification changes because all L3 measurement requirements are specified with SMTC configuration rather than SSB configuration in legacy.

  - RAN1: it will result in multiple RAN1 specification changes in Section 5.1 of TS 38.215.

Observation 4: The UE autonomous SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when SSB is partially overlapped or has small timing gap with SMTC.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover all SSBs with different periodicities without causing negative UE impacts (e.g. extra power consumption if NW configures a dense SMTC to cover all SSBs).

Observation 6: In legacy, SMTC can only be reconfigured by RRC signaling. However, because SSB adaptation is expected to dynamically change periodicity of SSB via MAC-CE / DCI, one single SMTC can’t quickly align with the dynamic change of SSB periodicity.

Observation 7: Because RLM and BFD rely on configured SSB periodicity rather than SMTC, maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

Observation 8: The legacy PEI framework can’t work when Ns is increased to 8. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

Observation 9: SBFD RACH agreed not to introduce a new RACH feature combination due to:

- No requirement is identified to support Msg 1 early identification of SBFD feature.

- General RACH portioning framework is already too complex to maintain.

Based on observations, our proposals are:

Adaptation of SSB in time domain

Proposal 1: On SSB adaptation, RAN2 wait RAN1 conclusion on whether to use MAC-CE to indicate SSB adaptation (i.e. RAN2 doesn’t discuss whether the unified signaling is used in OD-SSB and SSB adaptation).

Proposal 2: On L3 measurement on SSB adaptation, keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC).

Proposal 3: On L3 measurement on SSB adaptation, introduce dynamic adaptation between different SMTC configurations. FFS signaling details of how to configure multiple SMTC configurations.

Proposal 4: Send reply LS (as response to RAN4 LS R4-2416911) to inform RAN4 on Proposal 2 and Proposal 3 in Appendix.

Proposal 5: RAN2 further study the spec impacts on RLM and BFD due on SSB adaptation.

Adaptation of paging occasions

Proposal 6: N value is extended with 32, 64, 128, and 256.

Proposal 7: Ns value is extended only with 8.

Proposal 8: Introduce a separate PEI configuration. Rel-19 UEs only monitor the PEI-O according to the new PEI-O configuration while legacy UEs only monitor the PEI-O according to the legacy PEI-O configuration.

Proposal 9: The new PEI-O configuration uses maxPEI-perPF = 8 in symbol level offset IE firstPDCCH-MonitoringOccasionOfPEI-O (i.e. allow one PEI to associate with 8 POs in one PF when Ns=8).

RACH adaptation

Proposal 10: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

Proposal 11: RAN2 first study CBRA in RACH adaptation.

Proposal 12: If time allows, RAN2 extends the CBRA adaptation mechanism to some type of CFRA with shared CFRA resource pool in RRC (e.g. PDCCH order RACH).

Proposal 13: Same as SBFD RACH, RACH adaptation is not introduced as a new RACH feature combination in RACH partitioning framework.

Proposal 14: In RRC, introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

Proposal 15: RAN2 discuss the following two alternatives of adaptation framework between additional RACH resource and legacy RACH resource:

- Alt-1 (specified solution): the adaptation solution agreed in SBFD RACH is reused as baseline, including RACH resource set selection in Msg1 (re)transmission and fallback mechanism.

- Alt-2 (unspecified solution): it is up to UE implementation how to select RACH resource set for Msg1 initial transmission. FFS whether restrict to the same set in Msg1 retransmission due to power ramping.

Proposal 16: If RAN2 agree to support specified RACH resource set selection, the additional RACH resource is modelled as a new RACH type parallel with 2-step RACH in MAC.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] R4-2416911, LS on SSB adaptation.

[3] R2-2500256, Discussion on on-demand SSB for SCell, Apple.

[4] RAN2#127b, Chair Notes.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] RAN1#116, Chair Notes.

[7] RAN1#116b, Chair Notes.

[8] RAN1#117, Chair Notes.

[9] RAN2#128, Chair Notes.

[10] R2-2409942, Discussion on common signal transmission adaptation, Apple.

[11] RAN2#127, Chair Notes.

# Appendix

# 3GPP TSG RAN WG2 Meeting #129      	                           R2-250xxxx
Athens, Greece, 17th – 21th Feb., 2025

Title:	Reply LS on SSB adaptation

Response to:	R4-2416911

Release:	Release - 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Peng Cheng

E-mail Address:	< pcheng24@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 thanks RAN4 for the LS on SSB adaptation. Regarding the following question from RAN4:

“However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.”

RAN2 conclude to keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC). And dynamic adaptation between different SMTC configurations can be introduced to adapt to dynamic change of SSB periodicity.

2. Actions:

To RAN2.

ACTION: 	RAN2 respectfully asks RAN4 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2#129	     17 February - 21 February 2025		 Athens, GR

3GPP TSG RAN WG2#129b	     7 April - 11 April 2025		            Wuhan, China