# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500260
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.1.2.3

Source:	Apple

Title:	Remaining issues on LCM procedure of AI/ML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

The baseline of LCM procedure for AI positioning case 1 was agreed in RAN2#127b [2]. Then, some further agreements were made in RAN#128 [3]. We identify the following unaddressed FFSs:

- FFS 4: Order of inference config and applicability reporting

- FFS 5: Need of functionality activation

Meanwhile, there is one on-going post-meeting email discussion on key remaining issues [4], including whether to send LS to RAN1 on some issues. Although most issues seem to converge to majority view, we think there is no conclusion on below issues:

- FFS 1: Whether support on-demand request for NW-side additional condition

- FFS 2: Whether support reporting UE-side additional condition

- FFS 3: Whether support fallback configuration

In this contribution, we discuss these remaining issues on the LCM procedure of AI/ML based positioning.

# 2 Discussion

Based on RAN2#127b and RAN2#128 agreement [2][3], the baseline LCM procedure can be illustrated in Figure. 1. We highlight the FFSs which are discussed in this contribution.

Figure.1: Baseline LCM procedure of UE-sided model for AI/ML positioning Case 1

## 2.1 FFS 1: On-demand request for NW-side additional condition

This is one issue discussed in post-meeting email discussion [4], but no conclusion is made.

First, on-demand request for DL-PRS assistance data is already supported from Rel-17. Please note that RAN1#119 agreed all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE [5]:

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7) are reused.

Then, we think RAN2 can reuse the legacy on-demand request for DL-PRS assistance data. We don’t see need to support on-demand request for NW-side additional conditions

Proposal 1: On NW-side additional condition, support the legacy on-demand request for DL-PRS assistance data. No need to support on-demand request for other NW-side additional condition.

## 2.2 FFS 2: Whether support reporting UE-side additional condition

This is one issue discussed in post-meeting email discussion [4], but no conclusion is made.

In RAN2#127 [6], it was agreed that the UE determines applicable functionalities based on NW-side additional condition, UE-side additional condition and model availability in AI/ML based beam management:

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

We understand that this agreement means the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW. The logic behind this agreement is because the UE-side additional condition includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

We think the same principle can be reused to AI/ML based positioning. Thus, we propose:

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

## 2.3 FFS 3: Whether support fallback configuration

This FFS was captured in agreement of RAN2#128 [3]:

Agreements 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

This issue was also discussed in post-meeting email discussion [4], but no conclusion is made.

We think it is sufficient to rely on error cause:

- With UE reporting error code, LMF can already take corresponding reaction (e.g. configure non-AI/ML positioning) as legacy. Thus, fallback configuration is an optimization.

- The main benefit of fallback configuration is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction. So, we are not convinced its necessity.

Observation 4: The main benefit of fallback configuration is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction.

Thus, we propose not to support fallback configuration.

Proposal 3: Not support fallback configuration, i.e. it is sufficient to rely on legacy LMF reconfiguration upon reception of error cause.

## 2.4 FFS 4: order of inference config and applicability reporting

This FFS is captured in RAN2#127 post-meeting email discussion summary [7]:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

Please note that existing TS 38.305 has captured below principle for LPP procedures [8]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

We see no reason to challenge above principle of LPP in AI/ML based positioning. Thus, we propose:

Proposal 4: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

## 2.5 FFS 5: need of functionality activation

The existing LPP Location Information Transfer procedure can be found in Section 5.3.1 of 37.355 [9]:

### 5.3.1 Location Information Transfer procedure The Location Information Transfer procedure is shown in Figure 5.3.1-1. Figure 5.3.1-1: LPP Location Information transfer procedure 1. The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS. 2. The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE. 3. If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

As we can see, the UE immediately sends response via ProvideLocationInformation to LMF upon reception of RequestLocationInformation. Thus, it can be regarded that the functionality is activated when the LPP message RequestLocationInformation (i.e. configuration) is received, i.e. no extra activation procedure after reception of configuration.

Observation 5: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

We see no reason to introduce a new activation procedure on top of the existing procedure for AI/ML based positioning. Thus, we propose:

Proposal 5: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7) are reused.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Observation 4: The main benefit of fallback configuration is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction.

Observation 5: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

Based on observations, our proposals can be found below:

Proposal 1: On NW-side additional condition, support the legacy on-demand request for DL-PRS assistance data. No need to support on-demand request for other NW-side additional condition.

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

Proposal 3: Not support fallback configuration, i.e. it is sufficient to rely on legacy LMF reconfiguration upon reception of error cause.

Proposal 4: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

Proposal 5: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#127b, Chair Notes.

[3] RAN2#128, Chair Notes.

[4] R2-25xxxx, Summary of LCM Procedure for Positioning Case1 (Ericsson), Ericsson.

[5] RAN1#119, Chair Notes.

[6] RAN2#127, Chair Notes.

[7] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[8] 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[9] 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".