# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500263
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.1.5

Source:	Apple

Title:	Discussion on parameters/model transfer in two-sided model

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- CSI feedback enhancement [RAN1]:

  - For CSI temporal prediction: further update TR 38.843 with additional evaluations.

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

The new Rel-19 SID of AI/ML for air interface Phase 2 (SID RP-243245) was agreed in RAN#106 [1]. The SI objective on model transfer/delivery and two-sided model can be found below:

Although RAN2 is not listed in SI objective of CSI compression, RAN1 sent LS (R1-2410922) [2] to request RAN2 to evaluate feasibility of identified solutions on two-sided model. The detailed action to RAN2 is copied below:

To: RAN2 CC: SA2/SA3/SA5 Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

In this contribution. we provide our view from the following 3 aspects:

- Overall procedure

- Feasibility of OTA and non-OTA signaling

- Feasibility of 3 options on parameter and/or dataset transfer

We also provide a TP to capture in TR 38.843 in Appendix.

## 2.1 Overall procedure

Before RAN2 impact evaluation, it is necessary to align company understanding on how the CSI compression works. According to RAN1 conclusion, the overall procedure of Direction A includes the following 4 steps:

- NW-side model training: NW training entity first trains the NW part of the CSI compression model with dataset collected by gNB (e.g., named as dataset A).

  - Dataset A includes target CSI collected by the gNB.

  - The NW-part model performs as a decoder, where its input is CSI feedback and its output is target CSI.

  - The NW training entity may be gNB, or OAM or CN NF (e.g. NWDAF) or a server out of 3GPP.

- NW training entity shares different alternatives of training information with UE / UE-side server.

  - RAN LS listed below 3 options on shared training information [2]:

    - Option 1: Dataset sharing consisting of {(Target CSI, CSI feedback)}

    - Option 2: Encoder parameter sharing

    - Option 3: Encoder parameter sharing + dataset sharing consisting of {target CSI}

  - Depending on whether to first share training information with UE or UE-side server, we have OTA and non-OTA signalling:

    - OTA signaling: the forwarding path is NW-training entity->gNB->UE->UE-side server.

    - Non-OTA signalling: the forwarding path is NW-training entity->UE-side server.

- UE-side model training: UE-sider server trains the UE part of the CSI compression model with both shared information from NW in 2) and dataset collected by the UE (e.g., named as dataset B).

  - Dataset B includes target CSI collected by the UE, which is different from dataset A collected by gNB.

  - The UE-part model performs as an encoder, where its input is target CSI and its output is CSI feedback.

  - We skip the detailed procedure of UE first training a nominal decoder and then training the actual encoder against the nominal decoder because it doesn’t impact RAN2 evaluation.

- Deploy models for inference: UE-side server delivers the trained UE-part model to the UE and NW training entity delivers the NW-part model to the NW for inference.

Based on above analysis, we illustrate OTA signalling and non-OTA signalling in Figure. 1 and Figure. 2, respectively. We have the following two clarifications on step 3 of OTA signalling:

- Procedure of “UE selection”: it is necessary because the training of UE part model is per device type per UE vendor (i.e. only a subset of UEs from the same device type of the same UE vendor need to involve in the training procedure of two-sided model).

  - The UE selection procedure has RAN2 impacts (e.g. UE capability), which is analysed in Section 2.2.

- Procedure of “dataset splitting”: it was proposed by multiple companies in RAN1 to alleviate the overhead issue of OTA signalling (i.e. gNB splits the overall dataset into many subsets each with a limited number of data samples within RRC message payload limitation, which can be separately sent to different UEs).

  - As it is not mentioned in RAN1 LS [2], we put an FFS mark and discuss its RAN2 impact in Section 2.2.

Observation 1: In OTA signalling, it is necessary to specify UE selection procedure (e.g. at least UE capability) because the model training of UE part model is per device type per UE vendor (i.e. only a subset of UEs from the same device type of the same UE vendor need to involve in the training procedure of two-sided model).

Observation 2: In OTA signalling, although it is not mentioned in RAN1 LS, “dataset splitting” procedure was proposed by multiple companies in RAN1 to alleviate the overhead issue (i.e. gNB splits the overall dataset into many subsets each within RRC message payload limitation to be separately sent to different UEs).

For non-OTA signalling, please note that “UE selection” procedure is not needed because it is assumed that UE-side server is controlled by the UE and thereby can identify the UE’s device type and vendor information.

Observation 3: In non-OTA signalling, “UE selection” procedure is not needed because it is assumed that UE-side server is controlled by the UE and thereby can identify the UE’s device type and vendor information.

As this is Rel-19 study item, we propose to capture Figure.1 and Figure.2 as baseline of OTA and non-OTA signaling respectively in TR38.843 [3].

Proposal 1: Capture Figure.1 and Figure.2 as baseline of OTA signaling and non-OTA signaling respectively in TR 38.843.

Figure.1. Illustration of baseline of OTA signaling

Figure.2. Illustration of baseline of non-OTA signaling

## 2.2 Feasibility of OTA and non-OTA signalling

### 2.2.1 Feasibility of OTA signaling

We first analyse OTA signalling in Figure. 1. The key problem is signalling overhead issue (including CP and UP solution). According to RAN1 LS [2], the overhead of option 1/2/3 can be summarized as below:

- Option 1 (dataset sharing): its payload size is roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be larger if the dataset consists of supporting multiple configurations.

- Option 2 (parameter sharing): its payload size is 5.8M*(16/8) = 11.6 MB, based on the medium model size and the use of float16.

- Option 3 (dataset and parameter sharing): its payload size is N2 * (2000 bits) / (8bits/Byte) + 11.6 MB, where N2 is the number of samples of the dataset with only target CSI. Note that N2 <= N1 (sample number of option 1) because N1 includes both target CSI and feedback CSI.

  - If we assume N2=N1, then the payload size is roughly in the order of 600K * (2000 bits) / (8bits/Byte) + 11.6MB = 161.6MB.

If there is no “dataset splitting” procedure, the gNB needs to transfer the above required payload size in OTA message to the UE. We think it is not feasible for either CP or UP signalling:

- CP signalling:

  - It is not feasible because the existing RRC message only allows up to 45KB with 5 segments in DL, which is too far from the minimum payload size indicated in RAN1 LS (i.e. 11.6 MB in option 2).

  - If we consider increasing DL RRC segment number, it is also not feasible because the maximum DL segment number needs to increase to at least 1320 (for 11.6 MB in option2), which is impossible and a high burden for both gNB and UE (e.g. memory requirement).

- UP signaling:

  - Irrespective of whether performing dataset split, it is not feasible because existing UP protocol stack is between UE and UPF rather than between UE and gNB. Considering it is late stage of 5G, we don’t think that 3GPP can introduce a new protocol stack.

Observation 4: If dataset split is not performed, it is infeasible to extend DL RRC segment number as it needs to increase to 1320 which is impossible and a high burden for both gNB and UE (e.g. memory requirement).

Thus, we propose:

Proposal 2: RAN2 conclude that CP solution of OTA signaling without dataset splitting is not feasible because existing RRC only allows up to 45KB with 5 segments in DL, which has a significant gap compared with the minimum payload size indicated in RAN1 LS (i.e. 11.6 MB in option 2).

Proposal 3: Irrespective of whether performing dataset split, RAN2 conclude that UP solution of OTA signaling is not feasible because existing UP protocol stack is between UE and UPF rather than between UE and RAN.

Then, if “dataset splitting” can be performed in step 3 of Figure.1, we think it is still questioned whether OTA signalling is feasible due to below concerns:

- It is infeasible for Option 2 and Option 3 because model parameter can’t be split. The UE / UE-side server needs the whole model parameters from NW to perform UE part model training.

- It is not clear how to select a subset of UEs from the same device type of the same UE vendor.

  - Please note that existing UE capability do not include the UE’s vendor information and the device type information because they are UE’s internal information and can’t be exposed to NW.

    - Here, the device type information means different UE baseband and/or RF implementation of the same vendor rather than 3GPP specified UE type (e.g. redcap).

    - If gNB randomly selects UEs, it can’t ensure one particular UE vendor can get sufficient dataset to train a qualified UE part model.

- It is not clear how to aggregate datasets of multiple UEs from the same device type of the same vendor.

  - To support dataset aggregation, “subset ID” may be needed, but it is not clear whether such ID is in RRC or application layer. Furthermore, “subset ID” may not be sufficient if we have requirement of in-order delivery in dataset aggregation.

Based on 1), we propose:

Proposal 4: RAN2 conclude that CP solution of OTA signaling Option 2 and Option 3 with dataset split is not feasible because model parameter in Option 2 and 3 can’t be split.

Based on 2) and 3), we have below observation:

Observation 5: Existing UE capability do not include the UE’s vendor information and the device type information because they are UE’s internal information and should not be exposed to NW.

Observation 6: If gNB randomly selects UEs, it can’t ensure one particular UE vendor can get sufficient / complete datasets to train a qualified UE part model.

Thus, we propose:

Proposal 5: RAN2 conclude that CP solution of OTA signaling Option 1 with dataset split may not be feasible due to below concerns:

- It is not clear how to select a subset of UEs from the same device type of the same UE vendor, given that UE vendor and device type information are UE’s internal info which can’t be exposed to NW.

- It is not clear how to aggregate datasets of multiple UEs from the same device type of the same vendor.

### 2.2.2 Feasibility of non-OTA signaling

Compared with OTA signalling, there is no signalling overhead issue for non-OTA signalling. The key issue is how to support 3GPP standardized tunnel between NW training entity and UE-side server, to alleviate inter-vendor training collaboration issue.

Observation 7: Compared with OTA signalling, there is no signalling overhead issue for non-OTA signalling. The key issue is how to support 3GPP standardized tunnel between NW training entity and UE-side server, to alleviate inter-vendor training collaboration issue.

We think there are two kinds of options which are illustrated in Figure. 3 and 4:

- Option 1: non-OTA signalling with UE involvement

  - In Step 1, the UE needs to provide its UE-side server information (e.g. IP address of the server) and permission of inter-vendor training collaboration to the NW. Depending on whether the signalling terminated point is in RAN or CN, it can be RRC or NAS signalling or UP traffic. Note that if the signalling is terminated in RAN, we will have RAN2 impact.

    - One clarification is that the UE-side server information is per UE vendor rather than per UE.

  - In Step 2, based on the UE-side server information obtained in Step 1, NW can coordinate with NW training entity and UE-side server to establish a tunnel between them for dataset/parameter transfer.

    - The tunnel establishment procedure can be 3GPP standardized or out of 3GPP. In the former case, some SA2 spec impacts are expected. And in the latter case, it can be one IP tunnel established based on NW implementation.

  - In Step 4, the NW training entity directly shares its dataset/parameters with the UE-side server via the established tunnel in Step 2.

- Option 2: non-OTA signalling without UE involvement

  - In Step 1, the UE-side server is registered in CN as one AF, similar to existing eNA framework [4].

  - In Step 2, NW can coordinate with NW training entity and AF (i.e. UE-side server in this option) to establish a tunnel between them for dataset/parameter transfer.

    - Similar to option 1, the tunnel establishment procedure can be 3GPP standardized or out of 3GPP.

  - In Step 4, same as Option 1, the NW training entity directly shares its dataset/parameters with the UE-side server via the established tunnel in Step 2.

  - Option 2 doesn’t have RAN2 impact.

In our understanding, both options are technique feasible from RAN2 perspective. Although they need SA2 involvements on detailed procedure design on tunnel establishment, we don’t see showstopper to make conclusion of Rel-19 study from RAN2 perspective.

Proposal 6: RAN2 conclude that it is technique feasible to support 3GPP standardized non-OTA signaling from RAN2 perspective. And their details can be left to normative phase.

As this is Rel-19 study item, we propose to capture Figure. 3 and Figure. 4 as baseline of non-OTA signaling with and without UE involvement in TR 38.843 [3].

Figure.3. Example of non-OTA signaling with UE involvement

Figure.4. Example of non-OTA signaling without UE involvement

Proposal 7: Capture Figure. 3 and Figure. 4 as baseline of non-OTA signaling with and without UE involvement in TR 38.843:

- Option 1: non-OTA signaling with UE involvement:

  - The UE provides its UE-side server information (e.g. server address) and permission to the NW via 3GPP signalling (e.g. RRC/NAS/UP), which is used by the NW to coordinate with NW training entity and UE-side server to establish a tunnel between them for dataset/parameter transfer.

- Option 2: non-OTA signaling without UE involvement:

  - The UE-side server is registered in CN as one AF, similar to existing eNA. NW coordinates with NW training entity and AF (i.e. UE-side server in this option) to establish a tunnel between them for dataset/parameter transfer.

### 2.2.3 Comparison between OTA and non-OTA signaling

Comparing between the OTA signalling in Figure. 1 and non-OTA signalling in Figure. 2, we think OTA signalling has below downsides against non-OTA signalling:

- Higher UE power consumption caused by OTA reception of parameters/dataset from NW.

- Higher signaling latency caused by duplicated transfer from RAN to UE and transfer from UE to UE-side server with RAN forwarding.

- Potential radio resource congestion caused by OTA reception of parameters/dataset with large payload size.

As this is Rel-19 study item, we propose to capture the above comparison in TR 38.843 [3].

Proposal 8: Capture the following compassion between OTA signalling and non-OTA signalling in TR 38.843:

- Higher UE power consumption caused by OTA reception of parameters/dataset from NW.

- Higher signaling latency caused by duplicated transfer from RAN to UE and transfer from UE to UE-side server with RAN forwarding.

- Potential radio resource congestion caused by OTA reception of parameters/dataset with large payload size.

## 2.3 Feasibility of 3 options on parameter and/or dataset transfer

According to our Proposal 2/3/4/5, we conclude that OTA signalling is feasible for none of the 3 dataset/parameter transfer options. Thus, we propose:

Proposal 9: OTA signaling is not feasible for all the 3 dataset/parameter transfer solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1).

If non-OTA signalling is adopted, we understand that there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Observation 8: For non-OTA signaling, there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Thus, we propose:

Proposal 10: For non-OTA signaling, RAN2 conclude that all the 3 solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1) are feasible. It is up to RAN1 to determine which option to adopt.

## 2.4 TP for TR 38.843

Based on our proposals in this contribution, we draft a TP to capture in TR 38.843 in Appendix. Thus, we propose:

Proposal 11: Adopt the TP in Appendix as RAN2 conclusion of Rel-19 study on parameter/dataset transfer in two-sided model.

# 3 Conclusion

In this contribution, we share our view on dataset/parameter transfer in two-sided model. Our observations are:

Observation 1: In OTA signalling, it is necessary to specify UE selection procedure (e.g. at least UE capability) because the model training of UE part model is per device type per UE vendor (i.e. only a subset of UEs from the same device type of the same UE vendor need to involve in the training procedure of two-sided model).

Observation 2: In OTA signalling, although it is not mentioned in RAN1 LS, “dataset splitting” procedure was proposed by multiple companies in RAN1 to alleviate the overhead issue (i.e. gNB splits the overall dataset into many subsets each within RRC message payload limitation to be separately sent to different UEs).

Observation 3: In non-OTA signalling, “UE selection” procedure is not needed because it is assumed that UE-side server is controlled by the UE and thereby can identify the UE’s device type and vendor information.

Observation 4: If dataset split is not performed, it is infeasible to extend DL RRC segment number as it needs to increase to 1320 which is impossible and a high burden for both gNB and UE (e.g. memory requirement).

Observation 5: Existing UE capability do not include the UE’s vendor information and the device type information because they are UE’s internal information and should not be exposed to NW.

Observation 6: If gNB randomly selects UEs, it can’t ensure one particular UE vendor can get sufficient / complete datasets to train a qualified UE part model.

Observation 7: Compared with OTA signalling, there is no signalling overhead issue for non-OTA signalling. The key issue is how to support 3GPP standardized tunnel between NW training entity and UE-side server, to alleviate inter-vendor training collaboration issue.

Observation 8: For non-OTA signaling, there is no much difference among the 3 dataset/parameter transfer options from RAN2 perspective because the tunnel between NW training entity and UE-side server (e.g. IP tunnel) can easily handle the maximum payload size identified by RAN1 (i.e. 225 MB in option 1).

Based on observations, our proposals can be found below:

Overall procedure

Proposal 1: Capture Figure.1 and Figure.2 as baseline of OTA signaling and non-OTA signaling respectively in TR 38.843.

Feasibility study on OTA signaling

Proposal 2: RAN2 conclude that CP solution of OTA signaling without dataset splitting is not feasible because existing RRC only allows up to 45KB with 5 segments in DL, which has a significant gap compared with the minimum payload size indicated in RAN1 LS (i.e. 11.6 MB in option 2).

Proposal 3: Irrespective of whether performing dataset split, RAN2 conclude that UP solution of OTA signaling is not feasible because existing UP protocol stack is between UE and UPF rather than between UE and RAN.

Proposal 4: RAN2 conclude that CP solution of OTA signaling Option 2 and Option 3 with dataset split is not feasible because model parameter in Option 2 and 3 can’t be split.

Proposal 5: RAN2 conclude that CP solution of OTA signaling Option 1 with dataset split may not be feasible due to below concerns:

- It is not clear how to select a subset of UEs from the same device type of the same UE vendor, given that UE vendor and device type information are UE’s internal info which can’t be exposed to NW.

- It is not clear how to aggregate datasets of multiple UEs from the same device type of the same vendor.

Feasibility study on non-OTA signaling

Proposal 6: RAN2 conclude that it is technique feasible to support 3GPP standardized non-OTA signaling from RAN2 perspective. And their details can be left to normative phase.

Proposal 7: Capture Figure. 3 and Figure. 4 as baseline of non-OTA signaling with and without UE involvement in TR 38.843:

- Option 1: non-OTA signaling with UE involvement:

  - The UE provides its UE-side server information (e.g. server address) and permission to the NW via 3GPP signalling (e.g. RRC/NAS/UP), which is used by the NW to coordinate with NW training entity and UE-side server to establish a tunnel between them for dataset/parameter transfer.

- Option 2: non-OTA signaling without UE involvement:

  - The UE-side server is registered in CN as one AF, similar to existing eNA. NW coordinates with NW training entity and AF (i.e. UE-side server in this option) to establish a tunnel between them for dataset/parameter transfer.

Comparison between OTA and non-OTA signaling

Proposal 8: Capture the following compassion between OTA signalling and non-OTA signalling in TR 38.843:

- Higher UE power consumption caused by OTA reception of parameters/dataset from NW.

- Higher signaling latency caused by duplicated transfer from RAN to UE and transfer from UE to UE-side server with RAN forwarding.

- Potential radio resource congestion caused by OTA reception of parameters/dataset with large payload size.

Feasibility of 3 options on parameter and/or dataset transfer

Proposal 9: OTA signaling is not feasible for all the 3 dataset/parameter transfer solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1).

Proposal 10: For non-OTA signaling, RAN2 conclude that all the 3 solutions identified by RAN1 (i.e. option 4-1 and two variants of option 3a-1) are feasible. It is up to RAN1 to determine which option to adopt.

# 4 References

[1] RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

[2] R1-2410922, LS on signalling feasibility of dataset and parameter sharing.

[3] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[4] TS 23.288- v19.1.0, Architecture enhancements for 5G System (5GS) to support network data analytics services (Release 19), 2024-12.

# Appendix

### 7.3.5 Dataset/Parameter transfer in two-sided model

To support inter-vendor training collaboration in CSI compression, two solutions (OTA signaling and non-OTA signaling) are identified, which are illustrated in Figure.7.3.5-1 and Figure.7.3.5-2. In step 2 of OTA signaling and step 3 of non-OTA signaling, the identified options of dataset/parameter transfer include:

- Option 1: Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Option 2: Encoder parameter sharing

- Option 3: Encoder parameter sharing + dataset sharing consisting of {target CSI}

Figure.7.3.5-1. Illustration of baseline of OTA signaling

Figure.7.3.5-2. Illustration of baseline of non-OTA signaling

RAN2 study the feasibility of CP solution of OTA signaling, and conclude:

- If without dataset splitting, all 3 options are not feasible because existing RRC only allows up to 45KB with 5 segments in DL, which has a significant gap compared with the minimum payload size (i.e. 11.6 MB in option 2).

- Option 2 and Option 3 with dataset split is not feasible because model parameter in Option 2 and 3 can’t be split.

- Option 1 with dataset split may not be feasible due to below concerns:

  - It is not clear how to select a subset of UEs from the same device type of the same UE vendor, given that UE vendor and device type information are UE’s internal info which can’t be exposed to NW.

  - It is not clear how to aggregate datasets of multiple UEs from the same device type of the same vendor.

RAN2 also study the feasibility of UP solution of OTA signaling, and conclude:

- Irrespective of whether performing dataset split, it is not feasible because existing UP protocol stack is between UE and UPF rather than between UE and RAN.

RAN2 also study the feasibility of non-OTA signaling. Two solutions in Figure.7.3.5-3 and Figure.7.3.5-4 are identified with the following explanation:

- Option 1: non-OTA signaling with UE involvement:

  - The UE provides its UE-side server information (e.g. server address) and permission to the NW via 3GPP signalling (e.g. RRC/NAS/UP), which is used by the NW to coordinate with NW training entity and UE-side server to establish a tunnel between them for dataset/parameter transfer.

- Option 2: non-OTA signaling without UE involvement:

  - The UE-side server is registered in CN as one AF, similar to existing eNA. NW coordinates with NW training entity and AF (i.e. UE-side server in this option) to establish a tunnel between them for dataset/parameter transfer.

RAN2 conclude that it is technique feasible to support 3GPP standardized non-OTA signaling from RAN2 perspective. And their details can be left to normative phase.

As comparison, the OTA signaling has the following downsides against non-OTA signaling:

- Higher UE power consumption caused by OTA reception of parameters/dataset from NW.

- Higher signaling latency caused by duplicated transfer from RAN to UE and transfer from UE to UE-side server with RAN forwarding.

- Potential radio resource congestion caused by OTA reception of parameters/dataset with large payload size.

Figure.7.3.5-3. Example of non-OTA signlaing with UE involvement

Figure.7.3.5-4. Example of non-OTA signaling without UE involvement