3GPP TSG RAN WG2 Meeting #129            	                          R2-2500264

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Functionality Applicability Reporting and Management

Document for:	Discussion and Decision

# Introduction

In RAN2 #127 meeting, RAN2 discussed the unified framework supporting functionality applicability reporting, and RAN1 replied our questions in R1-2410898 [2].

In this contribution, we analyse the overall procedure of functionality applicability reporting based on RAN1 input.

Furthermore, we also discuss remaining applicable functionality management for UE-sided beam management, e.g. handling in mobility and RRC_ICLE/IDLE state, and training data collection configuration.

# Discussion

## Functionality Applicability Reporting

According to RAN1 reply LS, in this section, we further analyse the detailed information carried in each step of functionality applicability reporting procedure.

### Functionality Capability Reporting

As asked in R2-2407848 [1], RAN2 wonders what is the granularity of functionality reported in UECapabilityInformation (i.e., Step 2). According to RAN1 reply LS in R1-2410898 [2], RAN1 expects the reported UE-capability information is the same as other UE capability information, i.e., Features/FGs with components and corresponding value ranges. Similar as other features, RAN1 expects such feature/FGs will be discussed later phase of Rel-19.

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

On one hand, the granularity of reported functionalities in Step 4 is associated with granularity of functionalities configured in Step 3. We further observe that, based on current RAN2 progress, it seems the definition of functionality granularity is not a hard show-stopper of RAN2 discussion on applicable functionality reporting. On the other hand, capability of Feature/FGs may implicitly indicate supporting a use case/sub-use case (i.e. instead of defining a specific use case/sub-use case as capability), which is up to RAN1 discussion. Hence, it seems there’s no need to further discuss the granularity of functionality in RAN2.

Proposal 1: It is up to RAN1 how to define granularity of functionality and Feature/FGs for AI/ML air.

### Functionality Applicability Reporting

The main leftover issue of functionality applicability reporting is about what information is carried in Step 3 and how/when UE reports applicable functionalities in Step 4.

Step 3

As replied in RAN1 reply LS [2], RAN1 defines two types of applicability reporting configuration in Step 3. However, it is not clear whether Cat A or Cat B should both be provided or only one configuration is needed. In below section, based on RAN1 reply, we further discuss the need of providing Cat A and Cat B in Step 3.

- The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

According to TR 38.843 [3], the model could be designed for specific Set A/B of beams, as they are key information deciding AI/ML model input and output. Such information could include:

- Model input: Set B of beam information (e.g. beam index)

- Model output: granularity of output (e.g. predicted L1-RSRP, and/or beam index), Set A of beam information (beam index)

Observation 1: According to TR 38.843, models for UE-side beam management can be differentiated by Set A/B of beam information and granularity of output.

Then, let’s further discuss what information is needed in Step 3. In our understanding, the information provided in Step 3 at least needs to support differentiation of model input/output among different AI/ML models.

On one hand, in Cat A), recalling that CSI-ReportConfig contains a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet¸ where all report configuration types (i.e. periodic, semi-persistent, aperiodic), report quantity, codebook configurations and other configurations are provided. Each CSI resource set includes a set of CSI-RS resources and corresponding IDs, where CSI-RS resource element mapping in time/frequency domain is provided. Assuming network can support multiple functionalities (i.e., different groups of Set A and Set B of beams), network needs to provide all CSI-RS report configurations for different group of Set A and Set B of beams in Step 3, where a significant signaling overhead may be introduced based on the functionalities that gNB can support.

Moreover, the functionalities provided in Step 3 may not be applicable at UE-side. Providing full inference configuration for those potential non-applicable functionalities further brings a substantial waste of air interface resources.

Observation 2: Some information (e.g. codebook config, report config type, etc) in CSI-ReportConfig is not needed for applicable functionality decision. Providing Cat A) full inference configuration (i.e. one or more of CSI-ReportConfig) in Step 3 may introduce significant signaling overhead and waste of air interface resource.

To reduce signaling overhead over the air interface and use air interface resource efficiently, network only needs to transfer the necessary information for functionality applicability reporting in Step 3. Based on the above discussion, at least beam index of Set A/B and output information (e.g. report content related information, time instance, etc), i.e. Cat B) information is necessary for UE to decide applicable functionalities, while more comprehensive inference configurations (i.e. Cat A)) can be provided to UE after functionality applicability reporting.

Cat B) configuration for functionality applicability reporting can be provided in OtherConfig, together with UAI configuration, where associated ID is used to identify different combination of Set A/B of beams.

Proposal 2: In Step 3, except configuring UE is allowed to do UAI reporting via OtherConfig, NW also provides Cat B (i.e. Associated ID and selected inference related parameters for applicability report only, e.g. Set A/B related information, report content, time instance if applicable) in OtherConfig. Cat A (i.e. inference configuration) is not provided in Step 3.

Even if Cat A) full inference configuration is provided to UE for applicable functionality decision, the configured inference configuration in Step 3 should not be applied by UE immediately after receiving RRCReconfiguration, which is different from normal CSI framework. Therefore, network needs to differentiate CSI-ReportConfig for legacy CSI framework and CSI-ReportConfig for inference configuration in Step 3, at least a different IE is used for inference configuration, so that UE can perform differently upon receiving them.

Proposal 3: If Cat A) is agreed to be provided in Step 3, a separate IE (decoupled from current CSI framework) is used for CSI-ReportConfig for inference configuration to allow UE to differentiate the configuration and will not apply it immediately.

Step 4

Recalling that RAN2 agreed to support both UAI and RRCReconfigurationComplete message for applicable functionality reporting with unified design.

RAN2 #127bis meeting: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content.

As discussed in Step 3 and proposed above, all mandatory information (i.e. Cat B)) for UE to decide applicable functionality reporting can be provided in OtherConfig, where UAI can be used to report associated ID of applicable functionalities to network upon being configured and upon change of applicable functionality. Only if Cat A) information is agreed to be provided in RRCReconfiguration, as a response, UE can include applicable functionalities in RRCReconfigurationComplete for the corresponding configured functionalities.

Proposal 4: In “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured and upon change of applicable functionality via UAI;

2) Only if Cat A (i.e. inference configuration of supported functionalities in CSI-ReportConfig) is agreed to be configured in Step 3, UE reports applicable functionalities in RRCReconfigurationComplete.

It was further agreed in RAN2 that UE reports functionality applicability reporting under below two conditions:

- Non-applicable functionality becomes applicable

- Applicable functionality becomes non-applicable

It is FFS applicability reporting content and whether UE explicitly or implicitly report “non-applicable” functionality.

RAN2 #127bis - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - FFS if inference configuration can be signalled in step3. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) RAN2 #128 - When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - FFS detailed design - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

Upon being configured for functionality applicability reporting, for the first time UE sends functionality applicability reporting, UE reports all applicable functionalities. Upon change of NW-side additional condition, or UE-side additional condition or UE side model availability, the applicable functionalities at UE side can also be changed accordingly.

If there’s new applicable functionalities available at UE side, UE can directly add such functionality in UAI message. On the other hand, if some functionalities become non-applicable, UE can directly remove the corresponding functionality from the reporting list. Based on the delta functionalities reported in two UAI messages, network can know which functionality becomes non-applicable and release its inference configuration accordingly for deactivation.

Proposal 5: Upon being configured, for the first time UE sends functionality applicability reporting, UE reports all applicable functionalities. UE reports applicable functionality based on delta applicable functionalities reported in UAI (i.e., UE reports implicitly when a functionality becomes non-applicable by removing the non-applicable functionality from UAI).

The message flow of functionality applicability reporting can be summarized as below:

Initial State of applicable functionality after Step 4

In current CSI framework, UE has three types of CSI reports:

- Periodic CSI reporting, configured by RRC

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

If full inference configuration (i.e. Cat A) is not provided in Step 3, UE behaviour of receiving inference configuration is the same as legacy CSI framework. However, if full inference configuration (i.e. Cat A) is provided in Step 3, as discussed above, UE cannot activate periodic CSI reporting upon receiving configuration in Step 3. We here summarize the initial state of applicable functionality and how to activate the corresponding functionality.

|  | full inference configuration (i.e. Cat A) is not provided in Step 3 |  | full inference configuration (i.e. Cat A) is provided in Step 3 |  |
| --- | --- | --- | --- | --- |
|  | Before applicable functionality reporting | After applicable functionality reporting | Before applicable functionality reporting | After applicable functionality reporting |
| Periodic | Inactive | Inactive (NW considers it is activated upon receiving RRCReconfigurationComplete in Step 5) | Inactive | Active (NW considers it as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.) |
| Semi-persistent | Inactive | Inactive (NW activates the corresponding resource following legacy CSI framework) | Inactive | Inactive (NW activates the corresponding resource following legacy CSI framework) |
| Aperiodic | Inactive | Inactive (NW activates the corresponding resource following legacy CSI framework) | Inactive | Inactive (NW activates the corresponding resource following legacy CSI framework) |

Proposal 6: Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

      - Semi-persistent reporting, activated by MAC CE/DCI

      - Aperiodic CSI reporting, activated by DCI

## Applicability reporting during handover

RAN2 #128 - Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover. - Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI.

As proposed in Section 2.1, full inference configuration (i.e. Cat A) in Step 3 can be optionally provided. During handover, if full inference configuration is provided as part of HO command, UE can report its applicable functionalities to target cell after successful handover. On the other hand, if inference configuration is not provided in HO command, UE can report its applicable functionalities via UAI after successful handover.

Proposal 7: As baseline, UE reports its applicable functionalities via UAI after successful handover. Full inference configuration (i.e. Cat A) in RRCReconfigurationWithSync is optional. If configured, UE reports applicable functionality in RRCReconfigurationComplete as a response to HO command.

## Applicability reporting in RRC state change

Different from service continuity supported during handover in RRC_CONNECTED mode, connected UE does not need to continuously perform AI/ML of applicable functionality when it changes to RRC_IDLE or RRC_INACTIVE state.

UE is expected to deactivate all applicable functionalities when UE goes to RRC_IDLE or RRC_INACTIVE state, considering AI-based beam management is designed for UEs in RRC_CONNECTED state. When UE goes back to RRC_CONNECTED state, it is very likely that UE’s applicable functionalities have been changed based on dynamic change of connected cell’s NW-side additional condition, current UE additional condition and/or current model availability. This further indicates that configurations of previous applicable functionalities may not be valid anymore. Therefore, UE does not need to restore configurations of applicable functionalities when UE goes to RRC_IDLE/INACTIVE.

Proposal 8: UE releases configurations of applicable functionalities when UE goes to RRC_INATIVE and RRC_IDLE state.

When UE goes back to RRC_CONNECTED state, similar to initial connection, UE can follow the same procedure and report applicable functionality to network via UAI and/or RRCReconfigurationComplete. One may say that UE could report its applicable functionalities in RRCResumeComplete/RRCSetupComplete message, so that network may know the applicable functionalities supported by the UE earlier and configure them accordingly. However, considering AI/ML is an add-on feature to further improve network performance, before UE connects to network successfully, there’s no urgency to enable AI/ML functionalities immediately after UE goes back to RRC_CONNECTED mode.

Proposal 9: Functionality applicability reporting procedure in RRCResumeRequest/Complete and RRCSetupRequest/Complete is not supported.

# Conclusion

In this contribution, we discussed the detailed information carried in functionality applicability reporting procedure and discussed UE behavior during handover and change of RRC state. We observed and proposed the followings:

Capability

Proposal 1: It is up to RAN1 how to define granularity of functionality and Feature/FGs for AI/ML air.

Functionality applicability reporting Step 3

Observation 1: According to TR 38.843, models for UE-side beam management can be differentiated by Set A/B of beam information and granularity of output.

Observation 2: Some information (e.g. codebook config, report config type, etc) in CSI-ReportConfig is not needed for applicable functionality decision. Providing Cat A) full inference configuration (i.e. one or more of CSI-ReportConfig) in Step 3 may introduce significant signaling overhead and waste of air interface resource.

Proposal 2: In Step 3, except configuring UE is allowed to do UAI reporting via OtherConfig, NW also provides Cat B (i.e. Associated ID and selected inference related parameters for applicability report only, e.g. Set A/B related information, report content, time instance if applicable) in OtherConfig. Cat A (i.e. inference configuration) is not provided in Step 3.

Proposal 3: If Cat A) is agreed to be provided in Step 3, a separate IE (decoupled from current CSI framework) is used for CSI-ReportConfig for inference configuration to allow UE to differentiate the configuration and will not apply it immediately.

Functionality applicability reporting Step 4

Proposal 4: In “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured and upon change of applicable functionality via UAI;

2) Only if Cat A (i.e. inference configuration of supported functionalities in CSI-ReportConfig) is agreed to be configured in Step 3, UE reports applicable functionalities in RRCReconfigurationComplete.

Proposal 5: Upon being configured, for the first time UE sends functionality applicability reporting, UE reports all applicable functionalities. UE reports applicable functionality based on delta applicable functionalities reported in UAI (i.e., UE reports implicitly when a functionality becomes non-applicable by removing the non-applicable functionality from UAI).

Activation/Deactivation

Proposal 6: Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

      - Semi-persistent reporting, activated by MAC CE/DCI

      - Aperiodic CSI reporting, activated by DCI

Handover and RRC_IDLE/INACTIVE

Proposal 7: As baseline, UE reports its applicable functionalities via UAI after successful handover. Full inference configuration (i.e. Cat A) in RRCReconfigurationWithSync is optional. If configured, UE reports applicable functionality in RRCReconfigurationComplete as a response to HO command.

Proposal 8: UE releases configurations of applicable functionalities when UE goes to RRC_INATIVE and RRC_IDLE state.

Proposal 9: Functionality applicability reporting procedure in RRCResumeRequest/Complete and RRCSetupRequest/Complete is not supported.

# Reference

[1] R2-2407848 LS on applicable functionality reporting for beam management UE-sided model

[2] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

[3] TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface