3GPP TSG RAN WG2 Meeting #129                                             R2-2500268

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.12.2

Source:	Xiaomi

Title:	Discussion on asymmetric DL sTRP and UL mTRP

Document for:	Discussion and Decision

# Introduction

During RAN2 #128 meeting, following agreements were made for asymmetric DL sTRP and UL mTRP:

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In this contribution, we further discuss the RAN2 impact of asymmetric DL sTRP and UL mTRP.

# Discussion

## New MAC CE Design

As mentioned above, RAN2 has agreed to use a new MAC CE to update PL offset for asymmetric DL sTRP/UL mTRP. In this section, we mainly discuss the detailed design of this new MAC CE.

One/More TCI state within a MAC CE

As agreed by RAN2 #128 meeting, PL offset value can be updated for any configured TCI states (i.e. not just activated TCI state), which means PL offset for multiple TCI states can be updated. There are mainly two options:

- Option 1 (fixed size): One MAC CE updates pathloss offset value for one joint/UL TCI state. If pathloss update is needed for multiple TCI states, multiple MAC CEs are needed.

- Option 2 (variable size): One MAC CE updates pathloss offset value(s) for one or more joint/UL TCI state(s).

Technically both options can work. However, in Option 1, gNB has to provide multiple MAC CEs to update the pathloss offset values for more than one joint/UL TCI states which may cause additional signaling overheads over the air interface. Option 2 MAC CE with variable size can provide more flexibiltiy compared to Option 1.

Proposal 1: One MAC CE can include pathloss offset value(s) update for one or more joint/UL TCI-state(s).

ServingCell ID and BWP ID

Considering TCI state ID is cell/BWP specific, similar as other TCI state indication MAC CE, it is nature to include serving cell ID that the MAC CE applied for and UL/DL BWP ID.

Following legacy TCI state related MAC CE, pathloss offset update MAC CE is also designed per cell, where different serving cells will use different MAC CEs to indicate its pathloss offset update separately.

Regarding to BWP ID, for joint TCI state, DL BWP ID is indicated, while UL BWP ID is indicated for UL TCI state. To differenciate whether the BWP ID is for DL or UL, an additional bit is introduced to indicate whetherBWP ID is for DL or UL.

TCI state ID or codepoint

The next question is whether to use codepoint or TCI state ID directly in MAC CE for the TCI state with udpated pathloss offset. In our understanding, to save more bits of MAC CE, codepoints is sufficient as 1:1 mapping is defined between active TCI state and codepoint, where 16-bit is needed for sDCI method for asymmetric DL sTRP/UL mTRP.

With above statement, the following design of MAC CE can be considered to update pathloss offset.

Proposal 2: The new pathloss offset update MAC CE consists of the following fields:

- Serving cell ID: This field indicates the identity of the serving cell for which the MAC CE applies.

- D/U: This field indicates whether BWP ID is for UL or DL.

- DL/UL BWP ID: DL BWP ID should be indicated for the joint TCI state and the UL BWP ID should be indicated for the UL TCI state.

- Codepoint: Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th joint TCI state is present or not,

- Pathloss offset value

## PHR

According to the WID [1] as quoted below, only one pathloss reference is configured for both UL-only TRP and UL+DL TRP:

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

It is further agreed in RAN2 #128 meeting that a new MAC CE is introduced to indicate the absolute value of PL offset for asymmetric DL sTRP and UL mTRP.

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

According to PHR procedure defined in 38.321 [2], when the path loss is changed above a certain threshold (i.e. phr-Tx-PowerFactorChange), the UE should trigger a power headroom report.

### 5.4.6 Power Headroom Reporting … A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In the asymmetric mTRP scenario, the pathloss offset(s) associated with joint/UL TCI states can be updated via MAC-CE. The update may be necessary when the UL TRP is changed or the UL Tx beam is changed due to UE movements. This all leads to the change on pathloss offset value(s) or the PL RS(s) related to TCI states. To avoid significant network scheduling impact, it is better for UE to trigger PHR based on the latest pathloss after considering pathloss offset.

Pathloss for the two TRPs of asymmetric DL sTRP/UL mTRP after pathloss offset can be calculated as below:

- PathlossUL_DL_TRP = PowerTx_UL_DL_TRP - RSRPPathloss_Reference

- PathlossUL _TRP = PathlossUL_DL_TRP + Pathloss_OffsetRRC + Pathloss_OffsetMAC_CE

For UL_DL_TRP, the pathloss change is due to the change of DL measurement of pathloss reference (i.e. RSRPPathloss_Reference). However, for UL_TRP, the pathloss change is not only impacted by DL measurement of pathloss reference (i.e. RSRPPathloss_Reference), RRC configuration of pathloss offset (i.e. Pathloss_OffsetRRC), but also impacted by MAC CE update of pathloss offset (i.e. Pathloss_OffsetMAC_CE).

Based on our understanding, all parameters impacting the pathloss calculation/change need be counted for the triggering of the PHR, so that the network can get the latest UE UL Tx power status in time.

Proposal 3: Pathloss offset (including RRC configured offset and MAC CE updated offset) needs to be considered in PHR trigger condition.

## RACH

According to the RAN1 agreements, the pathloss offset is indicated via the PDCCH-order RACH procedure.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

According to the RACH procedure as quoted below, RSRP of downlink pathloss reference is used for the selection of different RACH procedures, NUL and SUL, and different PRACH resources.

TS 38.321 - rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type; - rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier; - msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP; - rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b); - rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

Calculation of pathloss for two TRPs of asymmetric DL sTRP/UL mTRP considering pathloss offset can be updated as below:

- PathlossUL_DL_TRP = PowerTx_UL_DL_TRP - RSRPPathloss_Reference

- PathlossUL _TRP

  - = PathlossUL_DL_TRP + Pathloss_Offset

  - = PowerTx_UL_DL_TRP – (RSRPPathloss_Reference - Pathloss_Offset)

  - = PowerTx_UL_DL_TRP – RSRPUL_TRP

Therefore, assuming that the UL_TRP is changed to UL_DL_TRP with the TRP transmission power of PowerTx_UL_DL_TRP, the virtual RSRP (i.e. RSRPUL_TRP reflecting the uplink coverage of the UL-only TRP) of the UL_TRP would be as follows:

- RSRPUL_TRP = RSRPPathloss_Reference - Pathloss_Offset

As PDCCH-order indicating pathloss offset is for the PRACH transmission to the UL-only TRP, in order to ensure the uplink coverage for the PRACH transmission to the UL-only TRP, the RSRP for the selection of the different RACH procedures, NUL and SUL, and different PRACH resources towards the UL-only TRP should combine the RSRP measured for the pathloss reference and the pathloss offset.

Proposal 4: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: One MAC CE can include pathloss offset value(s) update for one or more joint/UL TCI-state(s).

Proposal 2: The new pathloss offset update MAC CE consists of the following fields:

- Serving cell ID: This field indicates the identity of the serving cell for which the MAC CE applies.

- D/U: This field indicates whether BWP ID is for UL or DL.

- DL/UL BWP ID: DL BWP ID should be indicated for the joint TCI state and the UL BWP ID should be indicated for the UL TCI state.

- Codepoint: Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th joint TCI state is present or not,

- Pathloss offset value

Proposal 3: Pathloss offset (including RRC configured offset and MAC CE updated offset) needs to be considered in PHR trigger condition.

Proposal 4: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.

# Reference

[1] RP-242394, Samsung, “WID revision: NR MIMO Phase 5”

[2] TS 38.321 Medium Access Control (MAC) protocol specification